README.DOC                                   updated July 21, 1995

Backup Exec for NetWare

---------------------------------------------------------------------

CAREFULLY FOLLOW THE INSTRUCTIONS IN SECTIONS 1-2 BELOW TO COMPLETE

THE INSTALLATION PROCESS.

YOU MAY WISH TO PRINT A COPY OF THIS README.DOC FILE, WHICH IS

LOCATED IN YOUR SERVER'S SYS:BKUPEXEC SUBDIRECTORY.  FOR EXAMPLE:

NPRINT [ insert server name ]/SYS:BKUPEXEC\README.DOC

=====================================================================

TABLE OF CONTENTS

=================

Section#  Topic

--------  ------------------------------------------------------

1.   Installation Notes

2.   Release Notes

3.   What's new in Version 7.01?

4.   Compatibility Information for Drives and Controllers

5.   Limitations

6.   Frequently Asked Questions

7.   Prompt Timeouts

=====================================================================

1.  INSTALLATION NOTES

=====================================================================

Version 7.01 clients (BEWIN.EXE, BEOS2.EXE, BEDOS.COM and

BE.NLM) may not be used with previous versions of Backup Exec

for NetWare.  New functionality added into version 7.01 has

caused various system database formats to change, along with an

entirely new synchronization mechanism based on NetWare Core

Protocol Extensions (NCPE).  Because of this, the new clients

cannot access old databases, nor can old clients access the new

databases.  If you need to run both versions of Backup Exec

clients, you will have to install the version 7.01 client(s) to

different locations on your local hard disk.

2.  RELEASE NOTES

=====================================================================

Conner (Maynard) SCSI Controller

--------------------------------

The Conner (Maynard) 16-bit SCSI controller does not support

concurrent operations.

Existing Jobs

Backup Exec v5.0x Only

----------------------

Existing jobs will be targeted to the first available partition

by default.  To choose a specific partition for a job, edit the

job using the job schedule feature in one of the Backup Exec

clients.

Existing Jobs

Backup Exec v5.0x with Advanced Autoloader Module

-------------------------------------------------

Existing jobs will be targeted to an invalid partition by

default.  You will be prompted to either delete the jobs or

retarget them to a valid partition.  These jobs will not run

until a valid partition is chosen.

Windows 95 Agent

----------------

Using the Backup Exec Windows 95 agent in a token-ring/ethernet

network environment:

Backup Exec for NetWare Version 7.01 resolves the problem

of losing connections when performing backups of a Windows

95 workstation across a token-ring/ethernet bridge.

If both the Windows 95 agent and the backup server are on

the same type of network (both on token-ring or both on

ethernet) then no additional steps are required for the

Windows 95 agent to function properly.

However, if the backup server is on an ethernet network

which uses a network bridge to backup workstations on the

token-ring network, then you must modify the value of the

"GRFS_max_buffer_size" option within the backup server's

options file (BESRVR.CFG).  You should set the value

of this option to "240".

Example: GRFS_max_buffer_size = 240

No steps are required if the backup server is on the

token-ring network and the Windows 95 agent is on the

ethernet network.

3.  WHAT'S NEW IN VERSION 7.01?

=====================================================================

Concurrent device support

-------------------------

Up to twenty-eight (28) simultaneous operations are supported on

each NetWare backup server.  This allows you to simultaneously

perform multiple backup and restore operations on multiple tape

drives.  Support for multi-drive autoloaders has been added to

the Advanced Autoloader Module.

Console command execution before and after a job

------------------------------------------------

Backup Exec jobs can be configured to execute a system console

command on the backup server to stop or start other programs on

the server.  For example, you can automatically stop a database

application before a backup job begins, and automatically

restart the database application after the backup job ends.

Software compression with Agent Accelerator(TM)

-----------------------------------------------

Now, with Agent Accelerator, you can configure Backup Exec to

compress the data on the remote server before sending it to the

backup server.  This improves performance dramatically on slower

network wire technologies, such as 4MB/sec token ring or when

backing up over a WAN.  Agent Accelerator uses proven STAC

Electronics LZS compression for software data compression.

Automatic job history log printing capability

---------------------------------------------

Backup Exec can automatically route job results to any network

printer, even on a different server.  (Note: Only bindery based

print queues are supported.)

SNMP Agent

----------

Centralized status monitoring is available from a NetWare SNMP

compatible console, such as NetWare NMS.

Job Manager monitor

-------------------

The status of the tape system and all tape sessions can be

monitored from any workstation running a Backup Exec client as

well as from the server console.  A "gas gauge" indicates the

progress of the current operation by showing the percentage

completed.

Ability to read SIDF and ARCserve (version 4 and 5) tapes

---------------------------------------------------------

With the ability to read SIDF and ARCserve (version 4 and 5)

tapes, Backup Exec makes it easy to transition from Cheyenne

ARCserve, Novell's Sbackup utility or other SIDF compliant

backup solutions.  (Note: Only those SIDF tapes which conform

to ECMA-208 are supported. Also, ARCserve backups created with

Novell SMS enabled cannot be restored.  ARCserve's default is

not to use SMS for backup.)

Flat file restore

-----------------

This feature allows the flexibility either to retain the

directory tree during a restore or simply to restore all

selected files to a single subdirectory.

Hardware compression enabling/disabling

---------------------------------------

On a partition basis, the administrator can enable or disable

hardware compression on tape drives that support this feature.

Performance tuning

------------------

On a partition basis, the administrator can configure the block

size used to write to the tape media, the number of buffers

allocated from memory for data transfer, and the number of

buffers that are filled in memory prior to beginning data

transfer to tape.  Backup Exec automatically tailors these for

performance, and these values should only be changed in

consultation with Arcada Technical Support.

Job rerouting on fatal tape device failure

------------------------------------------

On a partition basis, the administrator can configure Backup

Exec to reroute jobs to an alternate partition on the same

backup server or a remote backup server in the event of a fatal

tape device error.

Enhanced Windows 95 Agent

-------------------------

An enhanced version of the Backup Exec Windows 95 agent is

provided in this release.  An option to display an agent status

and statistics window is included in the enhanced agent.

4.  COMPATIBILITY INFORMATION FOR DRIVES AND CONTROLLERS

=====================================================================

Loaders and tape drives can be used on the same backup server

by following these configuration rules:

1) The SCSI addresses of tape drives on a controller must

precede the loader SCSI address settings.

2) The SCSI ID of the loader arm must precede the SCSI address

of the drives in the loader.

3) In multiple drive loaders, the SCSI IDs must be in the same

order as the drive elements (i.e., Tape Drive 1 must have a

lower SCSI ID than Tape Drive 2).

Compatibility information for drives and controllers can be

found in a separate file named SYS:BKUPEXEC\HARDWARE.DOC after

installation of Backup Exec.  This file can also be found on the

first installation diskette.  You may wish to print a copy of

this file for later reference.

This file is also viewable from the Job Manager menus under

Options/Backup Exec Information/View Hardware Information, as

well as View/Backup Exec Information/View Hardware Information.

Both of these menus are selectable via key combinations from

the Job Manager's main screen.

5.  LIMITATIONS

=====================================================================

a.   When accessing servers with older versions of the NetWare

STREAMS and TLI NLM modules, Backup Exec clients or the

Job Manager may hang while scanning the network for servers

during initialization.  If this occurs, you should update

these modules to newer versions.

b.   If you are using the Backup Exec Windows client and your

screen "blacks out" followed by a workstation hang, you

must obtain updated IPX drivers from Novell's NetWire

CompuServe forum.  Updated files are found in the

VLMUPx.EXE, NET33x.EXE, WINDRx.EXE and NWDLLx.EXE self-

extracting archives.  These files are available from

the NOVFILES area in CompuServe's NetWire Forum.

c.   When using Backup Exec agent software, you may want to

first type CASTOFF ALL on the workstation.  If a message

is sent to a workstation that is running the Backup Exec

agent software, the Job Manager or Client software may lose

its connection to the agent, or performance may be severely

degraded.

To turn off broadcasts on a workstation logged into a 4.x

server, use SEND /A=N instead of CASTOFF ALL.

d.   If the backup server is running the NFS.NAM name space

during a restore, directories that are restored are

always restored in lower case.  This occurs with the

1.20c version of NFS.NAM on 3.x servers and the

1.27b version of NFS.NAM on 4.01 servers.  If you are

using name spaces on your server that have case

sensitive directory names other than NFS, you may want

to unload NFS.NAM prior to performing a restore.  This

problem may be fixed in a future release of NetWare.

e.   The Backup Exec DOS Agent (also used for Windows) does not

support the Microsoft-supplied MSIPX.COM driver which comes

standard with Windows for Workgroups version 3.1.  In order

to use the Backup Exec DOS agent under Windows for

Workgroups 3.1, you must install Novell's ODINSUP.COM

driver instead of the MSIPX.COM driver.

An outline of the procedure to modify a Windows for

Workgroups 3.1 environment to use the ODINSUP driver is

found in the README.TXT file that is installed on the

workstation as part of the Backup Exec Windows workstation

SETUP program.  If there are Windows for Workgroups 3.1

workstations that will run the Backup Exec DOS Agent, the

network administrator should review the procedure outlined

and create a customized procedure for the network users.

If you are using Windows for Workgroups version 3.11,

you should see the WFW3_11.WRI file in the WIN31 directory

for information about upgrading your network interface to

use the ODI drivers.  By default, this file is installed

into SYS:BKUPEXEC/WIN_WS/WIN31.

f.   If you are running the Backup Exec Job Manager with an

Adaptec controller and the tape drive is powered off

during an operation, you may need to DOWN your file

server to restart the Job Manager.  Adaptec's ASPI manager

does not correctly handle this problem and results in

the Job Manager appearing to be "hung" while trying to

complete an operation.  The file server will continue to

function properly, but you will need to DOWN the file

server and reboot it in order to use Backup Exec again.

g.   There is a problem with the 3Com 3C503 card installed in

a NetWare server that causes it to hang during extended

periods of heavy network traffic.  When this problem

occurs, you will receive "LAN receive buffer limit

reached.  All buffers are in use" errors while

performing remote server backups (this applies only to

Backup Exec - Enterprise Edition).  If you encounter this

error, you should upgrade your network card.

h.   If the Job Manager (BEMGR.NLM) or the NetWare Client

(BE.NLM) displays an "out of memory" error and you see a

message at the NetWare console "Alloc short term memory

allocator requests exceeded the configuration limit", you

must give NetWare more memory.  Failure to do so may cause

memory corruption or a server abend.

For 3.11 Servers: Type

"set maximum alloc short term memory=xxxxxxxx"

where xxxxxxxx is 50kb to 16mb.

It is set to 2mb by default.

For 3.12 Servers Type

"set maximum alloc short term memory=xxxxxxxx"

where xxxxxxxx is 50kb to 32mb.

It is set to 8mb by default.

i.   When you run out of disk space on your catalog volume, you

can move your catalogs to a different volume.  If you

do this and encounter the "Unable to initialize job" on

the Job Manager screen, you will need to go into the

<volume>:\BKUPEXEC\HISTORY directory and manually delete

any 0 byte files that exist.

j.   Some values in the E-mail options are not validated.

These include: server names, paths, login names and

passwords, and E-mail addresses.  If E-mail does not seem

to be working, you may have made an error entering E-mail

setup information in the options window.

k.   If the backup server runs out of short term memory while

backing up an SMS device, the files being processed

are backed up as corrupted files.

l.   The screen saver LOCK.EXE that comes with Novell DOS 7.0/

Personal NetWare is incompatible with the DOS Agent.  If

you are using the DOS Agent to back up this workstation,

this screen saver cannot be running.

m.   If you back up directories with the Delete-Inhibit or

Rename-Inhibit attributes set, and then attempt to restore

those directories, SMS errors will be written to the

history log.  The following message will appear in the

log file: "Invalid data (an unexpected directory

trailer) was found".  All data is correctly restored,

however, so the message can be safely ignored.  Novell is

aware of this condition.

n.   In the NFS name space on NetWare volumes and Unix

workstations, if there are multiple directories which

differ only by case (e.g., test_dir vs. TEST_DIR)

within a given subdirectory, only one of those

directories will be backed up.

o.   During disaster recovery, if you re-install the Bindery

or the NDS Tree containing the Backup Exec account and

queue information, the job scheduler will go offline

with error 254 or 156.  To recover from this condition,

either go into the options menu, or simply exit and

restart the Job Manager.  You may also encounter error

254 by running DSREPAIR.NLM while the Job Manager is

loaded.

p.   In the Job Manager SNMP configuration window, options appear

that allow you to set the priority at which traps are sent

to SNMP management workstations.  To retain true SNMP

functionality, the filter settings should be set directly at

the management workstation.  The Job Manager sends all traps

if the SNMP setting is set to	any value other than zero.

q.   When using HP OpenView, the ARCADA.TDF file should be

used instead of ARCADA.MIB.  This file is found in the

SYS:\BKUPEXEC directory and should be placed into the

HP OpenView Trap Manager directory (e.g., \OV\TRAPMGR).

This is the Trap Definition File required by HP OpenView to

receive SNMP traps from Backup Exec.  To replace this file

open the Monitor menu in HP OpenView and select

"Customize Traps...".  In the dialog that appears,

press the "Load Traps..." button.  Select ARCADA.TDF and

press OK.  From the Customize Trap Alarms dialog

press OK.  Backup Exec SNMP traps will now appear in the

Alarm Log.

6.  FREQUENTLY ASKED QUESTIONS

=====================================================================

Q.  I'm getting a "Tape Drive Not Found" message after running

BESTART.NCF.  Why am I getting this message?

A.  This message appears under the following conditions:

1. When you are trying to use an unsupported tape drive - Check

the HARDWARE.DOC for a list of supported tape drives and make

sure that your drive is in this list.

2. When the ASPI manager does not see the drive - Backup Exec

uses the tape drive's SCSI inquiry string to determine which

drivers to load for the drive.  If you have more than one

ASPI controller in the server, make sure that the drivers for

the controller to which the tape drive is attached are being

loaded in the STARTUP.NCF file.  You can run the BEDIAG

utility to view which controller the tape drives are attached

to, by using the following syntax:

LOAD BEDIAG /S /C

-----------

Q.  I'm getting an error message "Error on Tape drive x" during a

backup (or utility) operation.  What should I do?

A.  If the error is either "Error: Media Failure (3)" or "Error:

Bad Write (5)", first try cleaning the tape drive.  This is a

common problem with magnetic media drives and they should be

cleaned after approximately 20 hours of use.  You may also have a

bad tape.  If the tape has been in active use longer than six to

eight months, it is probably time to cycle the tape out of your

rotation scheme and get a new one.

If the error is "Hardware Failure (10)", the tape drive failed to

process the assigned command.  Power off the drive and turn it

back on.  If this error occurs on a regular basis, your tape

drive may need mechanical repair.

-----------

Q.  I get a 'Public Symbol Error BuildTCB, DeBuildTCB, etc.' when

attempting to run BESTART.NCF.  Why am I getting this?

A.  The ASPI manager for your controller is not loaded.  This is

common in controllers that do not load ASPI support by default.

Check the HARDWARE.DOC for currently supported controllers and

find your specific controller.  It should show you the ASPI

manager that needs to be loaded for your controller.

-----------

Q.  I have a Compaq on-board controller and get a 'Public Symbol

Error BuildTCB, DeBuildTCB, etc.' when attempting to run

BESTART.NCF.  Why am I getting this?

A.  The ASPI manager for your controller is not loaded.  Load the

NLM called CPQSASPI.NLM in your AUTOEXEC.NCF.  (See the previous

question.)

-----------

Q.  I get the error "There are no restore targets readily

available, please select restore target in the backup sources

window" when trying to submit a restore job from my Windows

client.  What should I do?

A.  You must login to the workstation through the Backup Sources

window before the Windows client will allow a restore.  This

error occurs when the password database has been disabled.  In

the Backup Sources window, double click on the server.  You will

be prompted for a login name and password.  Once the name and

password are provided, you should be able to submit the restore

job.

-----------

Q.  What are Backup Exec's Memory requirements for NetWare?

A.  Backup Exec requires a minimum of 2.5 MB of memory over NetWare

system requirements.  The amount of memory required increases

as additional tape devices are added.  NetWare system

requirements are affected by many variables.  To verify that

you have sufficient memory, type "load MONITOR.NLM" at the

server console.  Select the menu option "Resource Utilization".

The window at the top of the screen will change to "Server

Memory Statistics".  At the end of the "Cache Buffers" line, a

percentage number appears.  This number should be between 60%

and 65%.  A number below 60% can result in various problems.  To

increase this number, unload some other NLM's that may be using

memory, or add more memory (RAM) to the system.

-----------

Q.  I am running the Windows client under Windows for Workgroups

3.11.  When I load the Windows client it hangs on "Scanning for

Agents".  What's wrong?

A.  Refer to item 'e', under the Limitations section of this file

for information on resolving this issue.

-----------

Q.  I can see NetWare agents with the DOS client, but not the

Windows client.  Why?

A.  Your NetWare workstation requesters are out of date.  You will

need to get the following NetWare updates for your workstation:

VLMUPx.EXE

WINDRx.EXE

NWDLLx.EXE

NET33x.EXE (If running older NETX).

Each of these files is available on the Arcada BBS, the NetWire

forum on CompuServe, WWW.NOVELL.COM or FTP.NOVELL.COM.  These

self-extracting files have a text file contained with them that

explain how to properly set them up on your workstation.

7.  PROMPT TIMEOUTS

=====================================================================

The following section lists all of the prompts that have

timeouts, along with the default response that will be

supplied, should the timeout occur.  They are listed by

operation type.

Backup Operations

-----------------

The tape in drive #1 is not compatible.  Replace the tape in

drive #1.  Do you want to continue? (Y/N) N

The tape in drive #1 is write protected.  Replace the tape in

drive #1.  Do you want to continue? (Y/N) N

Would you like to place this job on hold? (Y/N) Y

Tape Rotation requires a prepared tape.  Insert a tape in

drive #1.  Do you want to continue? (Y/N) N

Only tape rotation jobs can be placed on tape rotation tapes.

Insert a tape in drive #1.  Do you want to continue? (Y/N) N

The wrong tape is inserted for this tape rotation job.  Insert

a tape in drive #1.  Do you want to continue? (Y/N) N

Client jobs are not allowed to overwrite prepared tape rotation

tapes.  Insert a tape in drive #1.  Do you want to continue?

(Y/N) N

Client jobs can only be appended to active (non-available)

tape rotation tapes.  Insert a tape in drive #1.  Do you want

to continue? (Y/N) N

This tape is not available for the current operation.  Insert

a tape in drive #1.  Do you want to continue? (Y/N) N

WARNING:  This tape was created with the TRANSFER option of

Backup Exec The information now on this tape may have been

permanently erased from your hard drive.  Do you want to

continue? (Y/N) N

The current tape has been rewound.  Please insert tape #2 for

this backup in partition "Partition Name".  Do you want to

continue? (Y/N) N

The current tape has been rewound.  Please insert tapes 3-4 for

this backup in partition "Partition Name".  Do you want to

continue? (Y/N) N

A tape was not found in drive #1.  Please insert a tape or make

sure that the tape is inserted properly.  Do you want to

continue? (Y/N) N

Backup and Restore

------------------

This is a once only job and it will not be scheduled again.

Would you like to put this job on hold? (Y/N) Y

Restore

-------

Do you wish to continue restoring files even though

information on the tape is corrupt? (Y/N) N

All Operations

--------------

The tape in drive #1 is not inserted properly. Do you want to

continue? (Y/N) Y

Replace the tape in drive #1.  Do you want to continue?

(Y/N) N

Catalog a Tape

--------------

Do you want to use On Tape Catalogs if they exist? (Y/N) Y

Do you want to catalog any backup sets not yet cataloged on

tape "Tape Name"? (Y/N) Y

WARNING: Tape #2 is out of sequence.  Do you want to continue?

(Y/N) Y

Quick Erase and Security Erase

------------------------------

Do you want to erase the information on the tape in drive #1?

(Y/N) N

WARNING: This tape has been found to be ACTIVE in the current

tape rotation scheme.  Do you want to continue? (Y/N) N

Do you want to erase another tape? (Y/N) N

The tape in drive #1 is blank or it may contain a backup which

this drive cannot recognize.  Do you want to erase the

information on the tape in drive #1? (Y/N) N

The tape in drive #1 is unrecognizable.  Do you want to erase

the information on the tape in drive #1? (Y/N) N

Verify a Tape

-------------

Do you want to verify this tape? (Y/N) Y

Prepare a Tape

--------------

Insert the next tape that you wish to prepare in partition

"Partition Name".  Do you want to continue? (Y/N) N

This tape is currently active in a tape rotation scheme.  It

must be erased before it can be prepared.  Replace the tape in

drive #1.  Do you want to continue? (Y/N) N

This tape is already a prepared tape rotation tape.  Do you

want to prepare this tape? (Y/N) N

The tape in drive #1 is unrecognizable.  WARNING: If you

prepare this tape, none of the information on this tape can be

restored.  Do you want to prepare this tape? (Y/N) Y

The tape in drive #1 is blank or it may contain a backup which

this drive cannot recognize.  Do you want to prepare this

tape? (Y/N) Y

Do you want to prepare another tape? (Y/N) N

Insert the next tape you wish to prepare in the tape drive.

Do you want to continue? (Y/N) N