Electric Circuits Lab

Purpose:

To confirm Kirchoff’s voltage and current laws experimentally, and to find out if the resistors used are ohmic.

Apparatus:

- Power Supply

- Wires

- Multimeter

- 1-150W resistor

- 2-100W resistors

- 1-270W resistor

Method:

1.)	The following circuit was hooked up in order to perform the circuit's lab.

source

150W

100W

100W

270W

2.)	The power supply was not turned on until everything was hooked up and checked 	out.  The source potential difference was then turned up to 4 V.

3.)	The potential difference was measured over each resistor.  V1, V2, V3, and V4 	were recorded for analysis.

4.)	The power supply was turned off again.  The ammeter was hooked up to measure		 the current through each of the resistor’s, and from the source.  The power supply 	was turned off each time the meter was moved around.  The values of I1, I2, I3, 	and I4 were recorded.

5.)	The power supply was turned of again, and the meter was unhooked.  Steps 3 and 	4 were repeated for a source voltage of 8.0 V.

6.)	Steps 3-5 were repeated for a source potential difference of 12.0 V.

Observations:

Source ® 4 V	I0 ® 0.011A

|  | Resistor (W) | Potential Difference (V) | Current (A) |
| --- | --- | --- | --- |
| V1 | 100 | 1/2 | 0.004 |
| V2 | 100 | 1/2 | 0.004 |
| V3 | 150 | 1 | 0.006 |
| V4 | 270 | 3 | 0.011 |

Source ® 8 V	I0 ® 0.021A

|  | Resistor (W) | Potential Difference (V) | Current (A) |
| --- | --- | --- | --- |
| V1 | 100 | 1 | 0.008 |
| V2 | 100 | 1 | 0.008 |
| V3 | 150 | 2 | 0.012 |
| V4 | 270 | 6 | 0.021 |

Source ® 12 V  I0 ® 0.034A

|  | Resistor (W) | Potential Difference (V) | Current (A) |
| --- | --- | --- | --- |
| V1 | 100 | 2 | 0.014 |
| V2 | 100 | 2 | 0.014 |
| V3 | 150 | 3 | 0.018 |
| V4 | 270 | 9 | 0.035 |

Analysis:

Sources of Error

1.)	The wrong full-scale could have been used on the multimeter, and the improper 	voltage and current readings were taken.

2.)	The resistors were not connected properly.  Thus, an incorrect circuit was used in the experiment.

3.)	The sporadic movements of the multimeters’ dial made eye judgment completely inconsistent.

4.)	Movement of the table, or any disruptions (e.g. moving hand away from the circuit) altered both of the current and voltage readings.

5.)	All of the current and voltage values were rounded-off to the nearest number.  Thus, several results are larger than they should be.

Experimental Improvements

1.)	The circuit could have been built on a bread-board so that the results would be more consistent.

2.)	The use of more professional, updated equipment (e.g. shorter wires) for the experiment in order to reduce any loss of current voltage would help improve the experiment.

3.)	Using a digital multimeter in order to take more accurate readings would help to improve the experiment.

Questions

1.)	Considering our results from the 4.0 V, 8.0 V, and 12.0 V batteries, our results did not support Kirchoff’s voltage law.  Examples of these results are proven in the following.

(i)V0 = V1+V2+V3+V4

...when V0=4.0 V	V1+V2+V3+V4=5 V

\5 V ¹ 4 V

(ii)

...when V0=8.0 V	V1+V2+V3+V4=10V

\10.0 V ¹ 8.0 V

(iii)

...when V0=12.0 V	V1+V2+V3+V4 = 16 V

\16.0 V ¹ 12.0 V

However, our experiment did confirm Kirchoff’s Current Law because I0=I4 (they 	were connected in series), and I4=I2+I3.  Although our results were rounded 	upwards slightly, when I2 and I3 were added together, they almost equalized I4.

2.)	The resistors proved to be ohmic.  Our results showed constant proof that the increase in current was directly proportional to the increase in voltage.  As the voltage increased by 4 volts, the current increased by a factor of 3.

Conclusions:

In conclusion, in this experiment, Kirchoff’s Voltage Law was not followed.  His Current Law, however, was followed.  As determined above, the resistors were ohmic, and the voltage was directly proportional to the increase of our results for the current.



®Andrew Likakis

®Michael Hancock

®Christina Mitchel

®Anuja ________