Guide to Novell TCP/IP Kernel

Information in this document is subject to change without notice and does not represent a commitment on the part of JSB Computer Systems Ltd.  The software described in this document is furnished under a license agreement or non-disclosure agreement.  The software may be used or copied only in accordance with the terms of the Software License Agreement.

Reprinted, with changes and additions, from the Novell TCP/IP Transport for DOS Guide with the permission of Novell, Inc.  Copyright 1993 Novell, Inc.

This document is copyright material and may not be copied in any form whatsoever.

Copyright ã JSB Computer Systems Ltd 1994.

All rights reserved.

Issue 4, August 1994.

JSB Computer Systems Ltd
Riverside
Mountbatten Way
Congleton
Cheshire
CW12 1DY
England

MultiView DeskTop, MultiView/X, Virtual Socket Library, MultiView Mascot and Host Support Server are trade names of JSB Computer Systems Ltd.

Novell, NetWare, LAN WorkPlace, DR DOS, Excelan, EXOS, and Script Director are registered trademarks and EXOS205L, EXOS205T, EXOS215T, Host Presenter, LAN WorkGroup, NE1000, NE2000, NE2100, NE3200, NE/2, NE/2-32, NetWare Loadable Module, NLM, Rapid Filer, RX-Net, RX-Net/2, Serving FTP, and Turbo RX-Net are trademarks of Novell, Inc.

All other trademarks are the property of their respective holders.

Document Reference:-  input.doc

Table of Contents

About This Guide		1

Documentation Conventions		1

Introduction		2

TCP/IP Transport Software		3

Network Drivers		4

Utilities		5

Installation		6

Prior to Installation		6

Installation Procedure		6

Configuration		9

Configuring Application Services		9

Setting Up the HOSTS File		10

Setting Up Domain Name System (DNS) Support		11

Optional Transport Configuration Files		14

Setting Up the PROTOCOL File		14

Setting Up the SERVICES File		15

Setting Up the NETWORKS File		17

Setting Up the ETHERS File		18

Modifying NET.CFG		19

Editing the NET.CFG File		19

Link Driver Section		20

Link Support Section		28

Standard Configuration		28

Optional Parameters		28

Protocol TCPIP Section		33

Allocating Sockets		32

Using a BOOTP Server		32

Using a Reverse ARP Server		32

Setting Up NetBIOS		33

Setting Up Multiple Network Interfaces		36

Setting Up SLIP and PPP		38

Applications Running over the SLIP_PPP Driver		39

Loading the SLIP_PPP Driver		40

NET.CFG Settings for the SLIP_PPP Driver		40

Link Driver SLIP_PPP Settings		41

ODINSUP		48

Setting Up the SNMP Agent		49

SNMP NET.CFG Summary		50

Community Names		50

SNMP Operations		50

Authentication		51

Example NET.CFG Configurations		51

Token Ring Configuration		52

ARCnet Configuration		52

SLIP and PPP Configurations		53

SNMP Agent Configuration		56

Multiple Network Configuration		56

Concurrent TCP/IP and IPX Configurations		59

Utilities		60

Running DIALUP		61

Running LWPCON		62

LWPCON Displays, Menus, and Control Keys		64

Checking Workstation and Remote Host Configuration		65

Listing and Closing TCP Connections		67

Determining Remote Host Status		68

Checking the Network Route to a Host		69

Checking Remote Host Services		70

Displaying DNS Information		71

Running PING		72

Troubleshooting		75

ODI to NDIS Protocol Conversion		68

Unlisted Card Drivers		68

Windows for Workgroups Startup Kit Network Card		68

Windows for Workgroups 3.10		69

Windows for Workgroups 311		70

Novell Lan WorkPlace 4.12 TCPIP and Noveel Netware IPX		72

Third Party Network Cards		73

Thick Ethernet - Transceivers		73

IBM LAN Support Program		73

IBM LAN Server		73

Microsoft Lan Manager		73

List of Figures

Figure  1	HOSTS File  		7

Figure  2	Remote Host Identification Flow  		 9

Figure  3	Sample RESOLV.CFG File  		 9

Figure  4	PROTOCOL File  		 11

Figure  5	SERVICES File  		 12

Figure  6	 NETWORKS File  		 13

Figure  7	Sample NET.CFG File for Token Ring  		 46

Figure  8	Sample NET.CFG File for ARCnet  		 47

Figure  11	NET.CFG File Using SLIP Frame Type  		47

Figure  12	NET.CFG File Using PPP Frame Type  		48

Figure  13	Sample NET.CFG File Using SLIP and
	Ethernet Connection Concurrently  		49

Figure  15	NET.CFG with SNMP Settings  		50

Figure  16	Example NET.CFG File for Multiple Network Connections  		51

Figure  17	Example Batch File for Multiple Network Connections  		52

Figure  18	Example Sign-On Display for Multiple Network Drivers  		53

Figure  19	Sample NET.CFG File for Concurrent TCP/IP and
	IPX Over ECONFIG  		53

Figure  20	Sample NET.CFG File for Concurrent TCP/IP and
	IPX Over Ethernet 802.3  		54

Figure  21	Sample NET.CFG File for Concurrent TCP/IP and
	IPX Over Token Ring  		54

About This Guide

This guide describes how to install this package, how to use each transport utility, how to configure the transport software for the supported network types and a troubleshooting section covering common problems and solutions when trying to configure the transport software.

Documentation Conventions

This guide uses the following xe "Notation"§xe "Conventions"§conventions:

| Convention | Meaning |
| --- | --- |
| command | Boldface characters are used to show items that you type, such as xe "Command\:syntax"§commands and options. You can use any combination of uppercase and lowercase letters. For example: PING |
| [ option ] | In syntax examples, boldface type enclosed in square brackets marks command options that you can type as needed. For example: FTP [ –D ] [ –F ] |
| ¦ | In syntax examples, a bar separating two command options indicates that you can choose one of the options. For example: –S ¦ –R ... N.B. N.B. Do not type the bar. |
| { options } | In syntax examples, curly braces indicate that you are required to choose one of the enclosed options. For example, the following notation means that you must include a 0 or a 1 in the command: { 0 ¦ 1 } |
| ... | Ellipses in syntax examples indicate that a parameter can be repeated. |
| parameter | Italic type marks descriptive item names, such as command parameters, that you replace with appropriate values. For example, in the following command: FTP –F remote_host you type the name of a computer on your network in place of remote_host. |
| Section headings, Parameter settings | xe "NET.CFG file\:syntax and notation"§xe "Configuring\:NET.CFG syntax and notation"§In the NET.CFG file, section headings and parameter settings are identified by italics. Because interpretation of this file is not case-sensitive, you can type its contents in uppercase or lowercase letters. |
| emphasis | Italics are also used in text for emphasis. |

| Convention | Meaning |
| --- | --- |
| DOS COMMANDS | xe "Command\:typographic conventions"§xe "DOS\:command notation"§xe "DOS\:command notation"§xe "Command\:DOS notation"§DOS commands and command option letters are shown in uppercase letters. For example, FTPD. Because DOS is not case-sensitive, you can type DOS commands in uppercase or lowercase letters. |
| DOS FILENAMES | xe "DOS\:filename notation"§xe "File\:notation"§DOS filenames are shown in uppercase letters. For example, AUTOEXEC.BAT. Because DOS is not case-sensitive, you can type DOS filenames in uppercase or lowercase letters. |
| UNIX commands | xe "UNIX\:command notation"§xe "Command\:UNIX notation"§UNIX commands are shown in boldface letters. For example, vi. Because UNIX is case-sensitive, commands are usually lowercase. Type UNIX commands exactly as shown. |
| UNIX filenames | xe "UNIX\:filename notation"§xe "File\:UNIX notation"§UNIX filenames and path names are shown in italics. For example, /etc/hosts. Because UNIX is case-sensitive, filenames usually are in lowercase letters. Type UNIX filenames exactly as shown. |
| <Key> | xe "Keystroke notation"§Angle brackets < > surround the name of a key or key sequence. For example, <Enter> corresponds to the Enter key on your keyboard. <Ctrl-C> means hold down the <Ctrl> key and simultaneously type the letter C (in uppercase or lowercase). |
| system response | Monospace type shows system-generated responses that appear on your workstation screen. For example: TNVT220 > |
|  | Identifies a procedure. Follow the steps to accomplish the specified result. |
|  | Noteworthy information is emphasized in the typeface used in this sentence, and marked with the Note icon to the left of this text. |
|  | Important information that deserves particular attention is emphasized with the Important icon to the left of this text. |
|  | Suggestions are guidelines that may be applicable to your site, but might not be appropriate at all sites. |
|  | Warnings to the user are marked with the Warning icon to the left of this text. |

Introduction

The Novell TCP/IP Transport for DOS consists of the following software components:

·	TCP/IP Transport software

·	Network drivers (including DOS ODI drivers)

·	Utilities: DIALUP, PING, and LWPCON

The following sections describe these components.

TCP/IP Transport Software

The following programs provide the TCP/IP transport:

·	TCP/IP Transport driver: TCPIP.EXE

·	RFC-1001/1002 compliant NetBIOS driver: RFCNBIOS.EXE

·	Reverse ARP server: RARPD.EXE

·	SNMP MIB-2 agent program (TSR): SNMP.EXE

·	Windows Sockets API support: WINSOCK.DLL and NOVASYNC.EXE

xe "TCP/IP\:Transport software"§The TCP/IP Transport software provides data-communications services to Berkeley 4.3 xe "Socket"§socket and Windows Sockets applications and NetBIOS users. In the context of the Open Systems Interconnection (OSI) xe "OSI reference model"§xe "Network\:OSI reference model"§networking reference model from the International Organization for Standardization (ISO), the TCP/IP protocols support Level 3 (the Network Layer) through Level 4 (the Transport Layer).

xe "Protocols\:used by the TCP/IP Transport"§The Novell TCP/IP Transport for DOS provides the following standard protocols:

·	xe "TCP protocol"§Transmission Control Protocol (TCP)

·	xe "UDP protocol"§User Datagram Protocol (UDP)

·	xe "IP protocol"§xe "Internet protocol (IP)"§Internet Protocol (IP)

·	xe "ICMP protocol"§Internet Control Message Protocol (ICMP)

·	xe "ARP protocol"§Address Resolution Protocol (ARP)

·	xe "RARP protocol"§Reverse Address Resolution Protocol (RARP)

·	BOOTP (standard protocol that provides TCP/IP configuration information)

·	Simple Network Management Protocol (SNMP)

·	Management Information Base (MIB)

·	xe "NetBIOS"§NetBIOS Service Interface using the RFC-1001/1002 compliant xe "B-node NetBIOS"§B-node (broadcast) type of the NetBIOS Protocol (NetBIOS)

|  | A workstation running NetBIOS software must also be running the TCP/IP Transport software. |
| --- | --- |

The TCP/IP Transport software also includes support for Windows Enhanced Mode operation.

xe "Network\:types supported"§The TCP/IP Transport software operates on any xe "ODI driver\:types available"§ODI driver that supports any of the following data link encoding standards:

·	Ethernet DIX

·	Ethernet 802.2 and SNAP

·	Token Ring 802.2 and SNAP

·	ARCnet

·	IBM PCN2

·	FDDI

·	SLIP or PPP asynchronous serial line

For more information about IEEE 802 Networks, refer to RFC 1042, A Standard for the Transmission of IP Datagrams over IEEE 802 Networks.

Network Drivers

The TCP/IP Transport software supplies the ODI-compatible software required to access a TCP/IP network from DOS and Windows. This software includes the following:

·	ODI drivers for supported network boards (see the file DRIVER.LST on the distribution diskette for a complete list)

·	Link Support Layer program used with ODI drivers: xe "LSL.COM driver"§LSL.COM

·	Program that supports the coexistence of the Network Driver Interface Specification (NDIS) and the Open Datalink Interface (ODI) Specification: ODINSUP.COM

Utilities

The following utilities are provided for use with the TCP/IP Transport software:

·	Utility that establishes dialin connections over SLIP/PPP: DIALUP.EXE

·	Utility that checks remote host connection and status: PING.EXE

·	Utility that displays local and remote configuration information, network statistics and TCP/IP service status:LWPCON.EXE.

Configuration

This section explains both required and optional configuration for the TCP/IP Transport software.

Configuring Application Services

xe "Configuring\:application services"§When you configure the Novell TCP/IP Transport software you must set up the required configuration and database files. Usually the files require customization to accommodate the specific features of your network.

Table  1 lists the service configuration files that you must set up before you can run your applications.

| Table 1 Required Service Configuration Files |  |  |  |
| --- | --- | --- | --- |
| File | Location | Description |
| HOSTS | \MVW\TCP | xe "HOSTS file"§xe "File\:HOSTS"§Hostname database file that lists the names, IP addresses, and aliases for the remote hosts on your network. The location of this file can be changed using the PATH TCP_CFG setting in the NET.CFG file. You must configure either HOSTS or RESOLV.CFG to be able to resolve symbolic hostnames. |
| RESOLV.CFG | \MVW\TCP | xe "RESOLV.CFG file"§xe "File\:RESOLV.CFG"§Defines the network’s xe "Domain Name System (DNS)"§DNS domain name and name servers. This file is required only if the network uses a name server. The location of this file can be changed using the PATH TCP_CFG setting in the NET.CFG file. You must configure either RESOLV.CFG or HOSTS to be able to resolve symbolic hostnames. |

The hostname database contains the names, aliases (optional), and network addresses of the computers on your network.

When using the TCP/IP Transport software, you can connect to a remote host by specifying its Internet address. If a hostname database exists, you can also connect to a remote host by specifying its name or one of its aliases.

You can set up a hostname database in one of two ways:

·	By placing a host table on each workstation

·	By having one or more Domain Name System (DNS) name servers on the networkxe "Domain Name System (DNS)\:name server"§

Your network is probably a mixed environment where some workstations or hosts running TCP/IP will use a DNS name server and other systems will have only a host table. You might require that each workstation or host on your network use both methods, use a DNS name server and have a host table on each workstation as a backup for the DNS.

Setting Up the HOSTS File

A host table is a listing of IP addresses, corresponding host names, and aliases. The TCP/IP Transport host table is the HOSTS file in the \MVW\TCP directory on each workstation. The HOSTS file on each workstation or server must contain an entry for every host that will be accessed. For example, a UNIX user might have the names of all UNIX hosts. The HOSTS file can also contain entries for target hosts on remote networks.

Figure  1 is an example xe "File\:HOSTS"§HOSTS file.

Figure 1 HOSTS File

## IP address Hostname Aliases# 127.0.0.1 loopback lb 129.47.5.1 engr engineering 129.47.7.3 mktg marketing 129.47.10.25 manfg manufacturing

The general format for each line in the HOSTS file is as follows (with each parameter separated by one or more blanks or tabs):xe "HOSTS file"§

Internet_address     hostname [alias [...]]

xe "Address\:Internet format"§Internet_address is a 4-byte address in standard dotted notation. Each byte is a decimal or hexadecimal value and is separated by a period. Hexadecimal numbers must start with the character pair 0x or 0X. Each number in the Internet address must be a value in the range 0 to 255 (decimal) or 0x00 to 0xFF (hexadecimal). Each entry should have a unique IP address.

|  | Serious problems can occur if two workstations are assigned the same IP address.xe "Internet address format"§ |
| --- | --- |

The hostname xe "Hostname\:defined"§is the name of the system associated with this Internet address. It can contain any printable character other than space, tab, backspace, CR-LF, end-of-line character, or comment character (the number sign). The hostname is an arbitrary string, preferably long enough for each name to be unique across the network.

The alias xe "Alias"§is another name for the same system. Typically, this is a shorter name and should be unique within each user’s domain (subset of computers to be used). A single host can have more than one alias. For example, the host “sales” can have the following address and aliases:

129.47.9.5 sales sa sell

The first item on the line is the host’s Internet address.xe "HOSTS file\:Internet address entry"§ The second item is the host’s name (sometimes referred to as the primary name).xe "HOSTS file\:hostname entry"§ Any items following the name are aliases for the host.xe "HOSTS file\:alias entry"§

A number sign (#) in the HOSTS file, at any position on a line, indicates the beginning of a comment. Characters from the number sign up to the CR-LF or end-of-line character are ignored by functions that search the file.

xe "HOSTS file\:comments"§You can create or modify a HOSTS file using any text editor.xe "HOSTS file\:creating"§ It should be in the format shown in Figure  1.xe "HOSTS file\:sample"§

Setting Up Domain Name System (DNS) Support

The preferred way for the TCP/IP Transport software to identify remote hosts is to use a DNS name serverxe "Domain Name System (DNS)\:name server"§. To use a xe "Name server (DNS)"§DNS name server, your workstation has to call the name server on a remote host to get address information. Like a host table, a name server has network addresses for each computer on the network. The only type of name server that the TCP/IP Transport software supports is a Domain Name System (DNS) name server.xe "Domain Name System (DNS)\:name server"§ The software accesses a DNS name server through the information provided in the RESOLV.CFG file.

Even if you have a DNS name server on your network, you may want to create a xe "HOSTS file"§HOSTS file on each individual workstation or server in case the DNS name server fails.

|  | If your site does not have a DNS name server, you must have a HOSTS file on each workstation. |
| --- | --- |

The TCP/IP Transport software is designed to identify a remote host by first looking for a DNS name server. If a valid entry is in the DNS, a connection is made. If the TCP/IP protocol stack does not find a name server or if a name server exists but the software cannot find the requested name, the protocol stack looks in the workstation’s HOSTS file, in the \MVW\TCP directory.

Figure  2 diagrams the logical flow for remote host identification.

Figure  2 Remote Host Identification xe "Hostname\:resolution flowchart"§xe "Remote host\:name search"§Flow

µ §To access a Domain Name System (DNS) name server on the network, you must have a resolver configuration file on your workstation. This is the RESOLV.CFG file in the \MVW\TCP directory. It contains entries that specify the Internet address of the system on which the name server is running.

xe "RESOLV.CFG file\:nameserver entry"§Two types of entries are available in the resolver file: domain and nameserver. All entries must be lowercase. Figure  3 shows both types of entries.

Figure 3 Sample RESOLV.CFG xe "RESOLV.CFG file\:example"§File

;COMPANY RESOLVER FILE domain company.com;;maxcache (Fast caching only nameserver for COMPANY.COM - internal only) nameserver 129.47.8.1;;secroot (secondary root nameserver for COMPANY.COM - visible to Internet) nameserver 129.47.4.2;;primroot (Primary root nameserver for COMPANY.COM - visible to Internet) nameserver 129.47.4.1

The xe "RESOLV.CFG file\:domain entry"§DOMAIN entry is a single entry with the following format:

domain domain_name

The xe "Domain"§xe "Domain Name System (DNS)"§domain keyword is required. The domain_name has the extension  .COM or .CO.xx for commercial organizations,  .MIL for a military organization,  .EDU for a school, or  .GOV for a government organization. If your organization has an Internet domain assigned (for example, NOVELL.COM), the domain_name must be entered here if you need to communicate with the Internet.

xe "RESOLV.CFG file\:nameserver entry"§The nameserver entry is a multiple entry (you can have more than one) with the following format:

nameserver Internet_address

xe "Domain Name System (DNS)\:name server configuration"§The nameserver keyword is required. Internet_address is the Internet address of the name server in dotted notation.

The domain entry lets you specify a default domain to apply to all names without a domain. The nameserver entry lets you specify the IP address of a name server for the resolver functions (for example, nameserver 129.47.8.1). Comments are preceded by the semicolon (;) character. For example:

nameserver 129.47.8.1      ; COMPANY RESOLVER FILE

You can include a maximum of three nameserver entries in the resolver configuration file. If more than one Internet address is listed, the resolver tries them in the order listed. For example, in Figure  3, the resolver tries to locate an IP address on the maxcache name server first. If unsuccessful, the resolver checks the secroot name server, followed by the primroot name server.

To disable your domain name server, you can rename the RESOLV.CFG file in the \MVW\TCP directory (for example, renaming the file to RESOLV.OFF turns your DNS off).

Optional Transport Configuration Files

Other configuration files are optional to customize the operation of the utilities and applications. These files are as follows:

·	\MVW\TCP\PROTOCOL — associates protocol name and aliases with standard Internet protocol numbers

·	\MVW\TCP\SERVICES — associates service name and aliases with standard Internet protocol and port numbers

·	\MVW\TCP\NETWORKS — associates network names and aliases with Internet IP addresses

·	\MVW\TCP\ETHERS — used only by the RARP server, RARPD.EXE, to associate IP addresses with workstation MAC addresses.

Setting Up the PROTOCOL File

xe "PROTOCOL file\:content and format"§The PROTOCOL file contains information about the known protocols used on the Internet network. Each protocol is represented by a single line with the following parameters separated by one or more blanks:

protocol_name protocol_number [  alias  [...]  ]

The protocol_name is the name of the Internet protocol associated with this protocol number. It can contain any printable character other than a field delimiter (space or tab), newline character, or number sign (#).

The protocol_number is the number of the Internet protocol.

The alias is an alternate name for the protocol.

For example, the TCP protocol has the following entry in the PROTOCOL file:

tcp        6    TCP    #  transmission control protocol

Figure  4 is a sample xe "PROTOCOL file"§xe "File\:PROTOCOL"§PROTOCOL file.

Figure 4 PROTOCOL File

#Name Number Aliases# ip 0 IP # internet protocol, pseudo protocol number icmp 1 ICMP # internet control message protocol igmp 2 IGMP # internet group multicast protocol ggp 3 GGP # gateway-gateway protocol tcp 6 TCP # transmission control protocol pup 12 PUP # PARC universal packet protocol udp 17 UDP # user datagram protocol

Setting Up the SERVICES File

The xe "SERVICES file"§xe "File\:SERVICES"§SERVICES file contains information about the known services used on the Internet network. Each service is represented by a single line with the following parameters, each separated by one or more blanks:

service_name port_number/protocol_name [  alias  [...]  ]

The service_name is the name of the service associated with this port number and protocol name. It can contain any printable character other than a field delimiter (space or tab), newline character, or number sign (#). These services are generally application, presentation, or session-level modules, such as TFTP, FTP, SMTP, and TELNET.

The port_number is the number of the Internet port used by the service. The protocol_name is the protocol with which the service is associated. This protocol is generally a transport-level or network-level module, such as TCP or UDP. Separate the port number and the protocol name with a slash.

The alias is an alternate name for the service.

A number sign (#) in the SERVICES file indicates the beginning of a comment. Characters between the number sign and the newline character are ignored by the functions that search the file.

For example, the following line is the entry for the SMTP mail protocol:

SMTP 25/TCP    MAIL

Figure  5 is a sample SERVICES file.

Figure 5 SERVICES File

domain 53/tcp hostnames 101/tcp hostname # usually to sri-nic sunrpc 111/udp sunrpc 111/tcp ##Host-specific functions# tftp 69/udp rje 77/tcp finger 79/tcp link 87/tcp tty link supdup 95/tcp iso-tsap 102/tcp x400 103/tcp # ISO Mail x400-snd 104/tcp csnet-ns 105-tcp pop-2 109/tcp # Post Office uucp-path 117/tcp nntp 119/tcp usenet # Network News Transfer ntp 123/tcp # Network Time Protocol NeWS 144/tcp news # Window System ## UNIX-specific services## these are NOT officially assigned# exec 512/tcp login 513/tcp shell 514/tcp cmd # no passwords used printer 515/tcp spooler # experimental courier 530/tcp rpc # experimental biff 512/udp comsat

who 513/udp whod syslog 514/udp talk 517/udp route 520/udp router routed new-rwho 550/udp new-wh # experimental rmonitor 560/udp rmonitord # experimental monitor 561/udp # experimental ingreslock 1524/tcp

Setting Up the NETWORKS File

xe "NETWORKS file"§xe "File\:NETWORKS"§The NETWORKS file contains information about the known networks that make up the Internet network. Each network is represented by a single line with the following parameters separated by one or more blank spaces:

network_name Internet_number [  alias  [...]  ]

The network_name is the name of the network associated with this Internet network number. It can contain any printable character other than a field delimiter (space or tab), newline character, or number sign (#). The network name is an arbitrary string, preferably long enough for each name to be unique.

The Internet_number is the number of the Internet network. Hexadecimal numbers start with the character pair 0x or 0X.

The alias is another name for the same network.

For example, the network “cornet” might have the following entry in the NETWORKS file:

cornet    144.61            horn

The NETWORKS file can be created from the official host database maintained at the Network Information System Center (NISC), although local changes may be required to bring it up to date regarding unofficial aliases and unknown networks. Such a file should be centrally administered for consistency.

Figure  6 is a sample NETWORKS file.

Figure 6 NETWORKS File

## Network-Name Internet-Number Aliases# loopback 127 company-net 129.47 subnet-A 129.47.4 subnet-B 129.47.8

# Internet networks arpanet 10 arpa ucb-ether 46 ucbether

Setting Up the ETHERS File

xe "File\:ETHERS"§The configuration table read by RARPD when it is loaded is the text file xe "ETHERS file (RARPD)"§xe "RARP server\:ETHERS file format"§ETHERS in the \MVW\TCP directory.

Each line of this file contains one MAC address/IP address pair, separated by tabs or spaces. The MAC address is 6 hexadecimal bytes separated by colons. The IP address can be in dotted notation, or a symbolic hostname can be supplied.

Hostnames are resolved when RARPD is loaded through the Domain Name System (DNS) or by lookup in the HOSTS file. xe "HOSTS file\:RARPD server and"§

Lines in the ETHERS file that start with the pound sign ( # ) are interpreted as comments.

The following is an example ETHERS file:

# Ethernet Address		IP Address

08:00:14:11:34:2c			lab
102:60:8c:0d:fa:df		129.47.5.98

Modifying NET.CFG

xe "File\:NET.CFG"§xe "Configuring\:NET.CFG file"§xe "NET.CFG file\:customizing"§The NET.CFG file in the root directory of a workstation’s boot drive has three sections that are used by the TCP/IP Transport software:

·	Link Driver — xe "Link Driver section (NET.CFG file)"§xe "NET.CFG file\:Link Driver section"§configures xe "ODI driver\:NET.CFG file section"§ODI drivers. You can set up multiple drivers if you have more than one network board or want to run SLIP/PPP over a COM port serial link.

·	Link Support — xe "Link Support section (NET.CFG file)"§xe "NET.CFG file\:Link Support section"§determines the sizes of the xe "LSL.COM driver\:NET.CFG section"§LSL memory pool and buffers. You can change these values to suit your network configuration and application usage.

·	Protocol TCPIP — xe "Protocol TCPIP section (NET.CFG file)"§xe "NET.CFG file\:Protocol TCPIP section"§contains settings used by the xe "TCP/IP\:NET.CFG configuration"§TCPIP.EXE module. You can modify this section to support optional features.

The NET.CFG file is used by xe "NetWare\:NET.CFG file use"§NetWare as well as the TCP/IP Transport software. It can contain additional Protocol sections to configure other protocols, such as IPX. For example, the Protocol RFCNBIOS section is used to specify fixed NetBIOS name to IP address mappings.

The sections that follow explain the NET.CFG format and discuss NET.CFG settings that you can customize in the Link Driver, Link Support, and Protocol TCP/IP sections. These discussions are followed by a selection of example configurations.

Editing the NET.CFG File

xe "NET.CFG file\:editing procedure"§You can use any text editor, such as DOS 5.0 EDIT, to update this file.The NET.CFG file uses the following format:

·	xe "NET.CFG file\:syntax"§Section headings start at the beginning of a line (first column) and precede the parameter settings that apply to the section:

Section Heading

·	Parameter settings listed under each section heading are indented with a tab or one or more spaces:

Section Heading

Parameter setting  parameter value(s)

·	Type numbers for parameter values in decimal notation unless directed otherwise.

·	Section headings and parameter settings can be typed in any combination of uppercase and lowercase letters.

·	xe "NET.CFG file\:comments"§Comments can be added to the file by preceding them with a number sign ( # ) or semicolon ( ; ).

·	Each line ends with a carriage return.

After changing NET.CFG, you might also need to change AUTOEXEC.BAT file or other workstation batch file to load any new drivers you are configuring. The drivers read the information in NET.CFG when they are loaded into memory. For your NET.CFG changes to take effect, you must unload and reload the drivers and the xe "NET.CFG file\:rebooting to implement changes"§TCPIP.EXE so that they read the new NET.CFG information. If you configure these programs to load from AUTOEXEC.BAT, rebooting the workstation has the same effect.

Link Driver Section

xe "Link Driver section (NET.CFG file)"§xe "NET.CFG file\:Link Driver section"§The Link Driver section is used for naming the ODI drivers and specifying various hardware and software settings for each driver. The installation program sets up the Link Driver section for your network board using the parameter settings necessary to run the Transport software. Other settings are optionally available to configure the board and its driver for use by other software.

Table  2 summarizes the most frequently used Link Driver settings.

| Table 2 Summary of NET.CFG Link Driver Settings |  |
| --- | --- |
| Parameter Syntax | Explanation |
| Link Driver drivername | Section heading for xe "ODI driver\:naming in NET.CFG"§ODI driver drivername. This heading is followed by parameter settings that apply to this driver. The drivername parameter is the name of the ODI driver file. For example, if the driver file is NE2000.COM, drivername is NE2000. |
| Int [ #1 ¦ #2 ] IRQ_number | Specifies the xe "Int NET.CFG Link Driver setting"§hardware xe "Interrupt"§interrupt number, IRQ_number, used by the network board. |
|  | The Int parameters are as follows: |
|  | [ #1 ¦ #2 ] Optionally specifies which network board channel you are configuring. Use this parameter only if you are configuring both communications channels on a board that supports two channels, or two network boards of the same type. |
|  | IRQ_number Decimal interrupt request number. |
| Port [ #1 ¦ #2 ] address [ ports ] | Specifies the startingxe "Port NET.CFG Link Driver setting"§ xe "Port\:NET.CFG Link Driver setting"§xe "Port\:workstation"§port address and number of ports used by the board. |
|  | The Port parameters are as follows: |
|  | [ #1 ¦ #2 ] Optionally specifies which network board channel you are configuring. Use this parameter only if you are configuring both communications channels on a board that supports two channels, or two network boards of the same type. |
|  | address Address, in hexadecimal, of the hardware port to use or the first port in a range of ports. |
|  | [ ports ] Number of ports in the range, in hexadecimal. |
| DMA [ #1 ¦ #2 ] channel | Specifies the xe "DMA NET.CFG Link Driver setting"§DMA xe "DMA\:channel"§xe "Channel\:DMA"§channel to be used by the network board. |
|  | The DMA parameters are as follows: |
|  | [ #1 ¦ #2 ] Optionally specifies which network board channel you are configuring. Use this parameter only if you are configuring both communications channels on a board that supports two channels, or two network boards of the same type. |
|  | channel Decimal DMA channel number. |

| Parameter Syntax | Explanation |
| --- | --- |
| Mem [ #1 ¦ #2 ] address [ length ] | Specifies the range of xe "Mem NET.CFG Link Driver setting"§xe "Memory\:offset for network board"§DOS conventional memory to be reserved for use by the board. |
|  | The Mem parameters are as follows: |
|  | [ #1 ¦ #2 ] Optionally specifies which network board channel you are configuring. Use this parameter only if you are configuring both communications channels on a board that supports two channels, or two network boards of the same type. |
|  | length Length of the memory segment, in hexadecimal. |
| Slot number | For xe "Slot NET.CFG Link Driver setting"§xe "Micro Channel Architecture network boards"§xe "Network\:MCA bus boards"§MCA boards, specifies the number, in decimal, of the slot into which you inserted the network board. This setting is required only if your workstation has more than one board of the same type. |
| Connector DIX | xe "Connector DIX NET.CFG Link Driver setting"§For the 3C503.COM driver, configures the board to use the DIX (thick xe "Network\:thick/thin Ethernet connection (3C503 board)"§xe "Ethernet\:thick/thin connection (3C503 network board)"§Ethernet) connector. If this setting is not included, the BNC/TP (thin) connector is used. |
| Frame frame_type | xe "Frame NET.CFG Link Driver setting"§xe "Frame format\:NET.CFG file setting"§Specifies the frame type used by the network board. See the installation checklist in Chapter 2 for a complete list of the frame types for network boards. This setting can be included more than once for boards and drivers that support multiple frame types. |
| Protocol name protocol_ID frame_type | xe "Protocol NET.CFG Link Driver setting"§Specifies the network protocol to be supported by the ODI driver. The Protocol parameters are as follows: |
|  | name Name of the network protocol to be supported. |
|  | protocol_ID Hexadecimal ID number assigned to this protocol (see Table 3 for a list of the ID numbers). |
|  | frame_type Frame type used by this protocol. See the preceding Frame setting. |
|  | address Starting memory address, in hexadecimal. |

Additional settings are used by individual ODI drivers and may be required or optional. Refer to the ODI section of the hardware documentation for your network board for information about what Link Driver settings the driver uses and which settings are required. Another source of this information is the  .INS file for each driver. These files, which have the same name as the driver  .COM file and are stored in the same directory, are ASCII text files that often contain information and comments about driver configuration.

The sections that follow describe the most frequently used Link Driver parameters.

| Link Driver name | xe "ODI driver\:naming in NET.CFG"§Each ODI driver that you configure has a separate Link Driver section in the NET.CFG file. Replace name with the executable filename of the driver you are using, omitting the extension (for example, NE2000 for NE2000.COM). |
| --- | --- |

xe "Network\:boards supported by each ODI driver"§xe "ODI driver\:network boards supported by each"§The names of the drivers provided with the TCP/IP Transport software can be found in the file MANIFEST.TXT on the distribution diskette.

| Int[ #1 / #2] IRQ_umber | xe "Int NET.CFG Link Driver setting"§The Int setting (called IRQ, for interrupt request, by some vendors) specifies which xe "Workstation\:interrupt"§xe "Interrupt"§interrupt your network board uses. For example, to specify interrupt 5 on an NE2000 board, place the following lines in your NET.CFG file: |
| --- | --- |

Link Driver NE2000
Int #1 5

To specify the IRQ_number for both of a board’s communications channels, use the following syntax in your NET.CFG file:

Link Driver 3C505
Int #1 2 Int #2 3

You do not need to include the characters #1 if you are specifying a value for only one network board communications xe "Channel\:network board"§channel. For example, if you are configuring only the first communications channel on a 3C505 board, and this channel uses Interrupt 5, you can place the following lines in your NET.CFG file:

Link Driver 3C505
Int 5

If, however, you are using more than one channel or network board, each of which has an Int value, you must include the characters #1 for the first channel’s value and #2 for the second channel’s value.

| Port [ #1 / #2 ] address[ports] | xe "Port NET.CFG Link Driver setting"§Use the Port setting to specify the starting xe "Address\:port (workstation)"§xe "Port\:workstation"§port (address, a hexadecimal number) and the number of ports in the range ( ports, also hexadecimal). |
| --- | --- |

For example, suppose you want to specify the starting port for the first communications channel on your board. The port address for the board is 300. Place the following lines in the NET.CFG file:

Link Driver NE2000
Port #1 300

The number of ports (ports) is optional. The following example specifies port ranges for both of a board’s communications channels with 32 ports (20 hexadecimal) in each range:

Link Driver 3C505
Port #1 300 20
Port #2 700 20

You do not need to include the characters #1 if you are specifying a value for only one network board communications xe "Channel\:network board"§channel. For example, if you are configuring only the first communications channel on a 3C505 board, and this channel uses 32 ports starting at address 300, you can place the following lines in your NET.CFG file:

Link Driver 3C505
Port 300 20

If, however, you are using more than one channel or network board, each of which has a Port address and range, you must include the characters #1 for the first channel’s values and #2 for the second channel’s values.

| DMA[ #1 / #2 ] channel | xe "DMA NET.CFG Link Driver setting"§The xe "DMA"§DMA (direct memory access) option specifies the hardware xe "Workstation\:DMA channel"§xe "Channel\:DMA"§xe "DMA\:channel"§DMA channel setting of the network board. This option lets you configure DMA channels by entering the channel number (channel, a decimal number) to be used. The default value is the first DMA channel (channel = 1). |
| --- | --- |

|  | Do not confuse your workstation’s xe "Channel\:DMA"§DMA channels with the communications channels on your network board. Some xe "Channel\:network board"§network boards have more than one network communications channel, each of which can be connected to a separate network communications line. DMA channels, on the other hand, are a feature of your workstation. A set number of DMA channels are allocated among all controller boards in your workstation that make use of this facility. |
| --- | --- |

For example, if the first communications channel on your board uses DMA channel 3 and the second communications channel uses DMA channel 4, place the following lines in your NET.CFG file:

Link Driver 3C505
DMA #1 3
DMA #2 4

You do not need to include the characters #1 if you are specifying a value for only one network board communications channel. For example, if you are configuring only the first communications channel on a 3C505 board, and this channel uses DMA channel 3, you can place the following lines in your NET.CFG file:

Link Driver 3C505
DMA 3

If, however, you are using more than one channel or network board, each of which has a DMA value, you must include the characters #1 for the first channel’s value and #2 for the second channel’s value.

| Mem [ #1 / #2 ] address[length] | xe "Mem NET.CFG Link Driver setting"§The Mem option specifies a range of conventional DOS xe "Workstation\:memory for network board"§xe "Memory\:offset for network board"§memory to be reserved for use by the network board. |
| --- | --- |

Use the hexadecimal physical (absolute) xe "Address\:memory offset for network board"§address of the memory used by the board for the address parameter. This address must match the starting address configured on the board. Enter the optional length parameter in hexadecimal paragraphs (a paragraph is 16 bytes) of the memory segment used by the board.

For example, if your board addresses the memory segment from D0000 to D4000, the starting address is D0000 and the range is 400 hexadecimal paragraphs. You would place the following lines in your NET.CFG file for an EXOS board:

Link Driver EXOS
Mem D0000 400

You do not need to include the characters #1 if you are specifying a value for only one network board communications xe "Channel\:network board"§channel. For example, if you are configuring only the first communications channel on a 3C505 board, and this channel uses memory address C0000, you can place the following lines in your NET.CFG file:

Link Driver EXOS
Mem C0000

If, however, you are using more than one channel or network board, each of which has a Mem value, you must include the characters #1 for the first channel’s value and #2 for the second channel’s value:

Link Driver EXOS
Mem #1 C0000
Mem #2 D0000

Some network boards, such as the 3C503 board, can be configured to use either shared DOS memory (requiring a Mem setting) or their own on-board memory. Refer to the manufacturer’s documentation for your network board to determine how best to configure your board.

| Slot number | xe "Slot NET.CFG Link Driver setting"§In slot-based computers, the driver usually locates the board by scanning through the xe "Network\:slot location of board"§xe "Workstation\:slot location of network board"§slots in order from lowest to highest. The Slot option directs the driver to eliminate the scan and locate the board in the specified slot. |
| --- | --- |

Use the number of the slot into which you inserted the board for the number parameter (a decimal value). The slot number is usually found on the back of the workstation.

For example, if you are using two NE/2 boards in the same workstation and you insert one board into slot 1 and the other into slot 2, place the following lines in your NET.CFG file:

Link Driver NE2
Slot 1

Link Driver NE2
Slot 2

You would then need to load the NE2.COM driver twice. The Slot option directs the first NE2 driver loaded to use the NE/2 board in slot 1 and directs the second NE2 driver to use the NE/2 board in slot 2.

| Connector DIX | xe "Connector DIX NET.CFG Link Driver setting"§The Connector DIX option configures the 3C503 LAN driver to use the DIX (thick) connector. BNC/TP (thin) is the default value. To specify the DIX connector, place the following lines in your NET.CFG file: |
| --- | --- |

Link Driver 3C503
Connector DIX

DIX can also be selected in the command line that loads the 3C503.COM driver, as in the following example:

3C503 /D

| Frame frame_type | xe "Frame NET.CFG Link Driver setting"§The Frame option enables the xe "Frame format\:network types that support each available"§frame types used by the network board. You can use this option more than once with network boards that support multiple frame types.. |
| --- | --- |

For example, to enable the ETHERNET_II frame type on an NE1000 board, place the following lines in the NET.CFG file:

Link Driver NE1000
Frame ETHERNET_II

Use the following frame formats for TCP/IP networks:

| Ethernet | ETHERNET_II or ETHERNET_SNAP |
| --- | --- |
| Token Ring | TOKEN-RING_SNAP |
| ARCnet | NOVELL_RX-NET |
| IBM PCN2 | IBM_PCN2_SNAP |
| FDDI | FDDI_SNAP |

For OSI networks, use frame type ETHERNET_802.2. For IPX networks, Ethernet LAN drivers default to the ETHERNET_802.3 frame type; xe "Token Ring\:frame types supported"§token ring LAN drivers default to the TOKEN-RING frame type.

Table  3 lists the network types that support each NET.CFG frame type parameter. Additional frame types are supported by the SLIP_PPP driver; see page  41.

| Table 3 IPX Frame Types and Corresponding Network Types |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Frame Type | Hex ID Number | Ethernet | Token Ring | ARCnet | IBM PC-NET | FDDI |
| ETHERNET_II | 8137 | Supported | — | — | — | — |
| ETHERNET_SNAP | 8137 | Supported | — | — | — | — |
| ETHERNET_802.2 | E0 | Supported | — | — | — | — |
| ETHERNET_802.3 | 0 | Supported | — | — | — | — |
| TOKEN-RING_SNAP | 8137 | — | Supported | — | — | — |
| TOKEN-RING | E0 | — | Supported | — | — | — |
| NOVELL_RX-NET | FA | — | — | Supported | — | — |
| IBM_PCN2_802.2 | E0 | — | — | — | Supported | — |
| IBM_PCN2_SNAP | 8137 | — | — | — | Supported | — |
| FDDI802.2 | E0 | — | — | — | — | Supported |
| FDDI_SNAP | 8137 | — | — | — | — | Supported |
| Legend: — Indicates that this frame type is not supported on this network type. |  |  |  |  |  |  |  |  |  |  |  |

| Protocol name protocol_ID frame_type | xe "Protocol NET.CFG Link Driver setting"§The Protocol option lets existing ODI drivers handle new network protocols. Replace name with the name of the new protocol. |
| --- | --- |

Replace protocol_ID with the hexadecimal protocol ID number that the protocol is assigned (see Table  3).

Replace frame_type with the frame type that the new protocol ID applies to. Make sure the Link Driver section for this driver contains a Frame setting enabling support of this frame type.

For example, if you want to use a protocol IPX with a 3c503 network board, include the following lines in the NET.CFG file:

Link Driver 3c503
Frame ETHERNET_II

Protocol IPX 8137 Ethernet_II

Link Support Section

xe "Link Support section (NET.CFG file)"§xe "NET.CFG file\:Link Support section"§The TCP/IP Transport software uses the buffer management functions provided by the xe "LSL.COM driver"§LSL driver. The Link Support section of the NET.CFG file includes parameters that determine the number and sizes of buffers available for the TCP/IP protocol. IPX does not use these buffers.

Standard Configuration

The installation program sets up the Link Support section as follows:

Link Support
Buffers 8 1500
MemPool 4096

| Buffers number[size] | xe "Buffers NET.CFG Link Support setting"§The Buffers setting controls the number and size of receive packet buffers. The default number of receive buffers is 8, and the default size is 1500 bytes. You can reduce the number of receive buffers if xe "Receive buffers (TCP)"§xe "TCP protocol\:receive buffers"§ TCP connections are used sequentially. If you make heavy use of multiple TCP connections, you can increase the number of buffers for better performance. |
| --- | --- |
| MemPool Size | xe "MemPool NET.CFG Link Support setting"§The MemPool setting controls the size of an internal send-buffer pool used by xe "TCP/IP\:send buffer pool"§TCP/IP. xe "TCP protocol\:send data buffering"§TCP send data is copied into this pool before being sent to the remote system. The default size is 4096 bytes. You might need to increase the MemPool size when communicating with remote TCP hosts that offer large receive windows and delay acknowledgments. |

Optional Parameters

If you are configuring more than one network board or plan to use multiple protocol stacks, you can set two additional parameters to configure the required LSL support.

| Max Boards number | This option configures the maximum number of logical boards the LSL program can support.. |
| --- | --- |

Each driver and frame type supported by the driver uses one LSL logical board resource. For example, if you are configuring NE1000.COM to load all possible Ethernet frame types (ETHERNET_II, ETHERNET_802.3, ETHERNET_80202, and ETHERNET_SNAP), 4 board resources are used.

Include the Max Boards setting only if you require more than 4 logical boards (the default). The range of values for number is from 1 to 16

| Max Stacks number | This option configures the maximum number of logical protocol stacks LSL can support. Each protocol stack uses one or more stack ID resources. |
| --- | --- |

If a protocol stack fails to load because of an out-of-resource condition, you can solve the problem by increasing this value. Because support for additional stacks increases the amount of memory used by LSL.COM, you can conserve some memory by reducing this value if your system is not using all stack support allocated.

Include the Max Stacks setting only if you require more than 4 logical stacks (the default). The range of values for number is from 1 to 16.

Protocol TCPIP Section

xe "TCPIP.EXE program\:specifying NET.CFG file location"§The Protocol TCPIP section contains settings used by TCPIP.EXE. TCPIP.EXE reads NET.CFG for this information when it is loaded. By default the NET.CFG file in the root directory of the workstation’s boot drive is used. You can specify a different location for NET.CFG by loading TCPIP with the –C drive:path command-line option. The ODI driver and LSL, however, always use the NET.CFG file in the root directory of the boot drive.

xe "Protocol TCPIP section (NET.CFG file)"§xe "NET.CFG file\:Protocol TCPIP section"§The parameters that can be set in the Protocol TCPIP section of the NET.CFG file to configure TCPIP.EXE are summarized in Table  4.

| Table 4 Summary of NET.CFG Protocol TCPIP Settings |  |
| --- | --- |
| Parameter Syntax | Explanation |
| Board Settings |  |
| Bind driver [ #board frame_type network_name ] | xe "Bind NET.CFG Protocol TCPIP setting"§Specifies the network interface board for the TCP/IP protocol stack. This parameter is needed only if you are running more than one ODI driver to support TCP/IP (for example, a workstation that uses both an Ethernet connection and a SLIP/PPP serial link, or a network gateway machine with two network boards). |
|  | If this parameter is not specified, TCP/IP uses the first driver it finds in memory (the first one loaded). |

| Parameter Syntax | Explanation |
| --- | --- |
|  | The Bind parameters are as follows. Parameters #board, frame_type, and network_name are optional. Include them only when configuring TCP/IP to run over multiple network interfaces. |
|  | driver Name of the ODI driver for this board. For example, if the driver is NE1000.COM, the value for the driver parameter is NE1000. |
|  | #board Number of the board to bind to this driver. This is the physical load sequence number of the board (not the same as the logical board number displayed by the driver when it is loaded). Using board number 0 instructs TCP/IP to bind to the first board it finds that supports TCP/IP and also supports the specified frame_type. |
|  | frame_type The frame format used for this network connection. This is the same as the frame value in the Link Driver section for this driver. |
|  | network_name A descriptive name for this network connection. The network_name is used with the ip_address, ip_router, and ip_netmask settings to distinguish between the values for each network connection. |
| Address Settings |  |
| ip_address address [ network_name ] | xe "IP address\:specifying local workstation’s"§xe "ip_address NET.CFG Protocol TCPIP setting"§The IP xe "Address\:IP (workstation)"§address of the workstation. The value for this parameter is supplied during installation. The ip_address parameter has no default value. |
|  | The ip_address parameters are as follows: |
|  | address IP address in dotted notation. If this parameter is missing or is 0.0.0.0, the protocol stack uses BOOTP or Reverse ARP to determine the IP address. |
|  | network_name The descriptive network name used in the Bind setting for this network connection. The network_name is required only if you are configuring multiple ODI drivers for TCP/IP. |
| ip_router address [ network_name ] | xe "IP router, specifying default address"§xe "ip_router NET.CFG Protocol TCPIP setting"§The defaultxe "Address\:IP router"§xe "Router"§ router address to send all packets destined for remote networks. All other gateways are dynamically discovered by using the ICMP redirect mechanism. The value for this parameter is supplied during installation. The ip_router parameter has no default value. |
|  | The ip_router parameters are as follows: |
|  | address IP address in dotted notation. |

| Parameter Syntax | Explanation |
| --- | --- |
|  | network_name The descriptive network name used in the Bind setting for this network connection. The network_name is required only if you are configuring multiple ODI drivers for TCP/IP. |
| ip_netmask mask [ network_name ] | xe "Subnetwork mask"§xe "Subnetwork\:mask"§xe "ip_netmask NET.CFG Protocol TCPIP setting"§Specifies the subnetwork mask if subnetworks are used. The value for this parameter is supplied during installation. The ip_netmask parameter has no default value. |
|  | The ip_netmask parameters are as follows: |
|  | mask Subnetwork mask in dotted notation. |
|  | network_name The descriptive network name used in the Bind setting for this network connection. The network_name is required only if you are configuring multiple ODI drivers for TCP/IP. |
| Miscellaneous Settings |  |
| PATH TCP_CFG [ [ drive: ] path [ ; ... ] ] | xe "PATH TCP_CFG NET.CFG Protocol TCPIP setting"§Specifies the directory or directories that contain the database configuration files HOSTS, NETWORKS, PROTOCOL, SERVICES, and RESOLV.CFG. The syntax is the same as the DOS PATH command. The default value is \MVW\TCP. |
| no_bootp | xe "BOOTP system\:bypassing"§xe "no_bootp NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:no_bootp setting"§Specifies whether to bypass a BOOTP system, if one is present on the network, and use xe "RARP server"§Reverse ARP to identify a workstation’s IP address. The transport uses the address specified by the ip_address parameter first, if this parameter is present; if not, it sends out a BOOTP request packet; and if no response is received from a BOOTP server, Reverse ARP is assumed. Including the no_bootp parameter eliminates the BOOTP request packet step. |
| Socket Settings |  |
| tcp_sockets number | xe "TCP protocol\:concurrent connections"§xe "tcp_sockets NET.CFG Protocol TCPIP setting"§xe "Socket\:TCP"§Specifies the maximum number of concurrent TCP connections. If you configure multiple ODI drivers for TCP/IP, the number of tcp_sockets you specify is the total shared by all drivers. The default value is 8. The range of values is 0 through 64. |
| udp_sockets number | xe "UDP protocol\:concurrent connections"§xe "udp_sockets NET.CFG Protocol TCPIP setting"§xe "Socket\:UDP"§Specifies the maximum number of concurrent UDP connections. You should have at least 1 UDP socket for each concurrently run application if you are planning to use the xe "Domain Name System (DNS)\:UDP socket requirements"§Domain Name System (DNS). xe "NetBIOS\:UDP socket requirements"§NetBIOS requires at least 2 UDP sockets. If you configure multiple ODI drivers for TCP/IP, the number of udp_sockets you specify is the total shared by all drivers. The default value is 8. The range of values is 0 through 32. |

| Parameter Syntax | Explanation |
| --- | --- |
| raw_sockets number | xe "raw_sockets NET.CFG Protocol TCPIP setting"§xe "Socket\:raw"§Specifies the maximum number of raw xe "IP protocol\:raw connections"§IP connections. You should have 1 raw IP socket if you are planning to use the PING commandxe "PING utility"§ or the applications that use this capability (Serving FTP, IP Resolver, LWPCON). If you configure multiple ODI drivers for TCP/IP, the number of raw_sockets you specify is the total shared by all drivers. The default value is 1. Valid values are 0 and 1. |

Allocating Sockets

xe "Socket\:allocating"§The default NET.CFG file configured by the installation program provides for 8 TCP sockets, 8 UDP sockets, and 1 raw socket. These default allocations are intended to be sufficient for most configurations. If you expect to make heavy concurrent use of applications that use sockets, you might want to modify the number of xe "Socket\:TCP"§TCP sockets (tcp_sockets).

xe "Socket\:UDP"§UDP sockets are used by LWPCON and by all applications that query DNS name servers. Because UDP sockets are used briefly to send and receive datagrams, then released, you need more than the default 8 UDP sockets only if you make very heavy use of DNS.

xe "Socket\:raw"§One raw socket is required for use by PING, LWPCON, and other applications that have ping functions.

Using a BOOTP Server

xe "BOOTP system"§If your network has a BOOTP server that is configured to supply your workstation with its IP address, router address, and subnetwork mask, omit the ip_address, ip_router, and ip_netmask parameters from your NET.CFG file. If these parameters are missing, the TCP/IP Transport software expects their values to be supplied by a BOOTP server.

Using a Reverse ARP Server

xe "Configuring\:RARPD"§RARPD is a Reverse Address Resolution Protocol server program. It allows you to assign workstation IP addresses from a centrally administered IP address database on a single workstation.

RARPD maintains a table in memory associating IP addresses with physical hardware (MAC) addresses of workstations on the network. A workstation configured as a RARPD client queries the RARPD server on startup, supplying its MAC address. RARPD responds with the corresponding IP address.

xe "RARP server\:NET.CFG configuration"§If your network has a reverse ARP server, such as RARPD, that provides your workstation IP address when you restart the workstation, you need to configure the following in your NET.CFG file:

·	Do not include the ip_address parameter.

·	If your network also has a BOOTP server, include the no_bootp parameter. If this parameter is present and the ip_address parameter is absent, the TCP/IP Transport software expects to receive the IP address from a Reverse ARP server.

Setting Up NetBIOS

xe "Configuring\:NetBIOS"§xe "NetBIOS"§RFCNBIOS.EXE implements the NetBIOS B-node protocols as defined in RFCs 1001 and 1002. (The B-node protocols use broadcast datagrams for network management and some NetBIOS datagram operations.) With this software, you can run any workstation network application designed for the standard NetBIOS interface.

|  | All directly-connected NetBIOS nodes that you plan to communicate with must use an RFC1001/1002-compliant NetBIOS program. Also, all NetBIOS implementations must be B-node configurations. |
| --- | --- |

The NetBIOS software is customized in the Protocol TCP/IP and Protocol RFCNBIOS sections of NET.CFG. Table  5 summarizes the NetBIOS settings.

| Table 5 NetBIOS NET.CFG Settings |  |
| --- | --- |
| Parameter Syntax | Explanation |
| Protocol TCPIP Section |  |
| nb_sessions number | xe "NetBIOS\:concurrent sessions"§xe "nb_sessions NET.CFG Protocol TCPIP setting"§Specifies the maximum number of concurrent NetBIOS sessions. When you set this parameter, you must increase the number of TCP sockets you allocate (tcp_sockets parameter) to at least nb_sessions + 1. The default value is 4. The range of values is 0 through 64. |
| nb_commands number | xe "NetBIOS\:asynchronous commands"§xe "nb_commands NET.CFG Protocol TCPIP setting"§Specifies the maximum number of asynchronous NetBIOS commands. The default value is 20. The range of values is 0 through 80. |
| nb_domain domain_name | xe "nb_domain NET.CFG Protocol TCPIP setting"§Specifies the name of the lxe "NetBIOS\:logical domain"§ogical domain for this NetBIOS station. The default domain_name (null) is used if this setting is not specified. The value of this parameter is an ASCII string. |
| nb_adapter { 0 ¦ 1 } | xe "nb_adapter NET.CFG Protocol TCPIP setting"§Specifies which xe "NetBIOS\: specifying board to use"§NetBIOS board to use. |
| nb_brdcast { 0 ¦ 1 } | xe "Broadcast address (NetBIOS)"§xe "Address\:broadcast (NetBIOS)"§xe "NetBIOS\:broadcast address"§xe "nb_brdcast NET.CFG Protocol TCPIP setting"§Specifies the format of IP broadcasts used when RFCNBIOS.EXE sends broadcast messages. |

| Parameter Syntax | Explanation |
| --- | --- |
|  | The value 0 sets the broadcast address to use zeroes for the host portions of the IP address. For example, a Class B network with network address 122.44 would have broadcast address 122.44.0.0. |
|  | The value 1 sets the broadcast address to use 255 for the host portions of the IP address. For example, a Class B network with network address 122.44 would have broadcast address 122.44.255.255. |
| Protocol RFCNBIOS Section | xe "NetBIOS\:remote broadcast networks"§xe "RemoteName NET.CFG Protocol RFCNBIOS setting"§Specifies an IP address on a different broadcast network from the workstation. This address is preloaded in the NetBIOS name cache. The value of n can be from 1 to 16, allowing up to 16 addresses to be preloaded. |
| RemoteNamen |  |

The following paragraphs describe the configuration of the NetBIOS settings in the NET.CFG file.

| nb_adapter | xe "NetBIOS\:specifying board to use"§xe "nb_adapter NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:nb_adapter setting"§Specifies which NetBIOS adapter number to bind to TCP/IP. This value must be either 0 or 1 (the first network board is 0, the second is 1). The format is nb_adapter { 0 ¦ 1 }. For example: |
| --- | --- |

Protocol TCP/IP
ip_address 129.47.6.84
nb_adapter 0

The default value is 0.

| nb_brdcast | xe "NetBIOS\:broadcast address"§xe "nb_brdcast NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:nb_brdcast setting"§Specifies the format of IP broadcasts used when RFCNBIOS.EXE sends broadcast messages. The value 0 sets the broadcast address to use zeroes for the host portions of the IP address. Therefore, a Class B network with a network address of 129.47 would have a broadcast address of 129.47.0.0. The value 1 sets the broadcast address to use 255 for the host portions of the IP address. Therefore, a Class B network with a network address of 129.47 would have a broadcast address of 129.47.255.255. |
| --- | --- |

The format is nb_ brdcast { 0 | 1 }. For example:

Protocol TCP/IP
ip_address 129.47.6.84
nb_brdcast 0

| nb_commands | xe "NetBIOS\:asynchronous commands"§xe "nb_commands NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:nb_commands setting"§Specifies the maximum number of asynchronous NetBIOS commands. Each pending NetBIOS command requires 32 bytes of memory. The range of values is 0 through 80. |
| --- | --- |

The format is nb_commands number. For example:

Protocol TCP/IP
ip_address 129.47.6.84
nb_commands 16

The default value is 8.

| nb_domain | xe "NetBIOS\:logical domain"§xe "nb_domain NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:nb_domain setting"§Specifies the name of the logical domain for this NetBIOS station. All stations in the same logical domain must use the same value for nb_domain. The value is a string of up to 80 characters. |
| --- | --- |

The format is nb_domain domain_name. For example:

Protocol TCP/IP
ip_address 129.47.6.84
nb_domain biosphere_2

The default value is a null string (no value).

| nb_sessions | xe "NetBIOS\:concurrent sessions"§xe "nb_sessions NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:nb_sessions setting"§Specifies the maximum number of concurrent NetBIOS sessions. Your tcp_sockets setting must be at least nb_sessions + 1. The range of values is 0 through 64. |
| --- | --- |

The format is nb_sessions number. For example:

Protocol TCP/IP
Bind NE1000
ip_address 129.47.6.84
nb_sessions 8

The default value is 4 sessions.

| RemoteNamen | xe "Broadcast address (NetBIOS)"§xe "Address\:broadcast (NetBIOS)"§xe "NetBIOS\:broadcast address"§xe "nb_brdcast NET.CFG Protocol TCPIP setting"§This parameter is used in the Protocol RFCNBIOS section of NET.CFG. It specifies an IP address that is not on the same broadcast network as your workstation. This address is preloaded in the NetBIOS name cache. The range of values for n is 1 through 16, allowing you to preload up to 16 addresses. |
| --- | --- |

For example:

Protocol RFCNBIOS
RemoteName1 129.57.52.6
RemoteName2 129.57.32.8

Setting Up Multiple Network Interfaces

The TCP/IP Transport software can support up to four simultaneous xe "Network\:connections, multiple"§network connections. You can install up to four network boards in your workstation and communicate simultaneously with networks of the same or different types.

|  | Configuring multiple network connections is not sufficient to designate a workstation as a xe "Router"§router. A workstation connected to more than one network can communicate on each of the networks simultaneously, but it cannot route packets between these connections for other workstations. |
| --- | --- |

You enable support for multiple networks by loading multiple ODI drivers and configuring them in the NET.CFG file. Depending on the network boards you install, you might xe "ODI driver\:configuring multiple network connection support"§load different ODI drivers, or load the same ODI driver more than once. Each time you load an ODI driver, a separate Link Driver section in the NET.CFG file provides the configuration parameters.

xe "Configuring\:multiple network support"§For example, if your workstation has both a token-ring network board and an NE2000 Ethernet network board, you can connect to two different networks. You need to load both TOKEN.COM and NE2000.COM. Your NET.CFG file would have two Link Driver sections similar to the following:

Link Driver TOKEN
Frame TOKEN-RING_SNAP

Link Driver NE2000
Int #1 5
Port #1 320
Frame ETHERNET_II

You need only one Link Support section to configure xe "LSL.COM driver"§LSL.COM, which automatically supports both ODI drivers.

Each network connection has its own xe "IP address\:multiple network connections"§IP address and its own optional router address and subnetwork mask. In the Protocol TCPIP section of NET.CFG, you need to designate which settings apply to each connection. To do this you assign a name to each network for use within NET.CFG. You can then use the name with each TCP/IP setting and use xe "Bind NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:Bind setting"§Bind statements to associate the settings with their network connections.

For example, you can assign the name FINANCE-NET to the token-ring connection, and assign MARKETING-NET to the Ethernet connection. You can then set up TCP/IP address settings for the two names:

Protocol TCPIP
ip_address 144.52.6.5
FINANCE-NET
ip_router 144.52.6.6
FINANCE-NET
ip_netmask 255.255.0.0
FINANCE-NET
ip_address 122.44.9.7
MARKETING-NET
ip_router 122.44.3.97
MARKETING-NET
ip_netmask 255.255.0.0
MARKETING-NET

You then use Bind to associate the appropriate driver (board instance) with its setting (network name):

Bind token #1 token-ring_snap
FINANCE-NET
Bind ne2000 #1 ethernet_II MARKETING-NET

The xe "Bind NET.CFG Protocol TCPIP setting"§xe "NET.CFG file\:Bind setting"§Bind statement specifies the driver, board instance, frame format, and network name for each connection. In this example, two different ODI drivers are used, and each is the first instance of its board type.

The board instance numbers are used by Bind to distinguish different instances of the same ODI driver. They are not the same as the logical board numbers displayed by the ODI drivers when they are loaded. For example, if you have two ARCnet boards in your workstation, you load the TRXNET.COM driver twice, once to support each board. Board instance #1 is the first TRXNET.COM loaded into memory, and instance #2 identifies the second copy of TRXNET.COM in memory.

|  | When multiple network interfaces are configured and an application tries to reach a destination host that is outside of the immediate networks of the interfaces connected, the protocol stack sends the packet through the default interface which is the first network specified in the NET.CFG file. This network is specified in the ip_router setting of the Protocol TCPIP section. |
| --- | --- |

Setting Up SLIP and PPP

xe "SLIP_PPP ODI driver\:configuring"§The TCP/IP Transport software supports asynchronous serial line xe "Configuring\:SLIP_PPP"§media by implementing the Serial Line Internet Protocol (SLIP) and Point-to-Point Protocol (PPP) in an ODI driver. SLIP and PPP operate on a workstation’s COM devices. This driver supports the two most popular link layer protocols: SLIP and PPP. This support includes optional Van Jacobson TCP/IP header compression.

xe "PPP protocol\:reference documents"§xe "SLIP protocol\:reference documents"§The following documents provide background information for the SLIP and PPP protocols.

·	RFC 1055, A Nonstandard for Transmission of IP Datagrams over Serial Lines

·	RFC 1144, Compressing TCP/IP Headers for Low-Speed Serial Links

·	RFC 1331, The Point-to-Point Protocol for the Transmission of Multi-Protocol Datagrams over Point-to-Point Links

·	RFC 1332, The PPP Internet Protocol Control Protocol

The TCP/IP protocol stack receives the same functional support from the SLIP_PPP driver as from any other LAN driver except xe "BOOTP system\:use with SLIP_PPP"§BOOTP and xe "RARP server\:use with SLIP_PPP"§RARP support is disabled when TCP/IP is operating on the SLIP_PPP driver. The TCPIP driver will not load if an ip_address is not specified in the NET.CFG file when the SLIP frame type is used. However, when PPP frame type is used, the TCPIP driver will load successfully in anticipation of IP address assignment from a remote host through PPP options negotiation.

The normal load sequence of the ODI driver should be followed when activating the protocol stack; that is, the SLIP_PPP driver must be loaded after the LSL driver but before the TCPIP driver. The SLIP_PPP driver can be configured using parameter settings in the Link Driver SLIP_PPP section of your NET.CFG file.

|  | For simple communications tasks, such as terminal access or file transfer, using TCP/IP over SLIP or PPP may be inefficient. It may be more effective to use a direct serial connection with terminal emulators and Kermit or Zmodem type protocols for xe "File transfer protocols for SLIP_PPP connections"§file transfer. However, if you plan to use more sophisticated network services or connecting to an internetwork, TCP/IP over SLIP or PPP is very effective. |
| --- | --- |

A workstation-to-LAN or workstation-to-internetwork connection is the most probable configuration for the SLIP_PPP driver. A user with a personal computer at home or in a dormitory or remote office, or with a laptop while traveling, can dial in to the company or university LAN and instantly become the equivalent of a local workstation on the LAN. A remote user can access the same services that are available to the local workstation and gain Internet access if Internet service is available. A dial-up router using either the SLIP or PPP protocol is required on the LAN side.

Applications Running over the SLIP_PPP Driver

xe "COM port\:Windows access over SLIP_PPP"§xe "Windows\:exclusive COM port access for SLIP_PPP"§Windows applications that use the TCP/IP Transport software operate normally if you configure Windows to give you exclusive access to the COM port that the SLIP_PPP driver is running on. This is automatically done by the SLIP_PPP driver when it is loaded by zeroing out the corresponding COMn: port base I/O address in the ROM BIOS data area. The same result can be accomplished by adding the following parameters to the [386Enh] portion of your Windows SYSTEM.INI file:

ComNAutoAssign=0
ComNIrq=-1
ComNBase=0

N represents the appropriate COMn: port number. For SLIP_PPP to run reliably in Windows 386 enhanced mode, a COM device based on 16550A UART (Universal Asynchronous Receiver/Transmitter) is recommended. The 16550 includes a FIFO (first in, first out) buffer that improves throughput and reduces interrupt overhead. If you experience errors when using COM devices based on the slower 8250 or 16450 UART, try to lower the communication line speed by setting a lower baud rate value. Also, lowering mru (maximum receive and transmit unit) size in your NET.CFG file might correct the problem.

Loading the SLIP_PPP Driver

xe "SLIP_PPP ODI driver\:loading"§The SLIP_PPP driver must be loaded after the LSL driver but before the TCPIP driver. You must configure SLIP or PPP by setting parameters in your NET.CFG file. A sample NET.CFG file for the SLIP_PPP driver is shown in Figure  16.

NET.CFG Settings for the SLIP_PPP Driver

xe "SLIP_PPP ODI driver\:NET.CFG file settings"§xe "NET.CFG file\:SLIP_PPP settings"§The SLIP_PPP driver has several Link Driver settings in addition to those illustrated in Figure  16. All SLIP_PPP settings are summarized in Table  6 and described in the following section.

| Table 6 NET.CFG Settings for SLIP and PPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Keyword | Value | Default | Description |
| xe "Frame NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:Frame setting (SLIP_PPP)"§frame | SLIP or PPP | SLIP | Defines frame type on serial line media. Select SLIP or PPP. |
| xe "accm NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:accm setting (SLIP_PPP)"§accm [ remote ] | 4-byte hexadecimal number | ffffffff | Asynchronous control character map. |

| Keyword | Value | Default | Description |
| --- | --- | --- | --- |
| xe "accomp NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:accomp setting (SLIP_PPP)"§accomp [ remote ] | Yes or No | No | xe "PPP protocol\:frame compression"§PPP frame address and control fields compression enabling. |
| xe "NET.CFG file\:pcomp setting (SLIP_PPP)"§pcomp [ remote ] | Yes or No | No | PPP frame protocol field compression enabling. |
| xe "authen NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:authen setting (SLIP_PPP)"§authen [ remote ] | PAP username password | None | User information for Password Authentication Protocol (PAP) authentication; allow 32-byte maximum usernames and passwords. |
| xe "baud NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:baud setting (SLIP_PPP)"§baud | 300, 1200, 2400, 4800, 9600, 14400, 19200, 38400 | 2400 | Serial line speed, in decimal. |
| xe "counter NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:counter setting (SLIP_PPP)"§counter [ protocol ] | timeout max_config max_term max_nak | 3 10 2 10 | The values are defined as follows: timeout - restart timer timeout in seconds; max_config -retry counter for configuration-request; max_term - retry counter for terminate-request; max_nak – retry counter for configuration-nak protocol can be LCP, IPCP, or PAP. All four parameter numbers must be specified; max_term is not used by PAP. |
| xe "dial NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:dial setting (SLIP_PPP)"§dial | Phone number | None | Valid only when direct is No; signifies the active end of a modem connection. |
| xe "direct NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:direct setting (SLIP_PPP)"§direct | Yes or No | Yes | Select whether the link is direct. |
| xe "int NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:int setting (SLIP_PPP)"§int | IRQ level | 4 | Interrupt level of the COMn: device. |
| xe "ipaddr NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:ipaddr setting (SLIP_PPP)"§ipaddr [ remote ] | IP address | None | Specify the IP addresses for negotiation. |
| xe "listen NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:listen setting (SLIP_PPP)"§listen | None | None | Valid only when direct is No; signifies passive end of a modem connection. |

| xe "magic NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:magic setting (SLIP_PPP)"§magic [ remote ] | 4-bytes hexadecimal numbers | Not enabled | Initial seed number for process recognition. |
| --- | --- | --- | --- |
| xe "mru NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:mru setting (SLIP_PPP)"§mru | n [m] | 1500 | Maximum receive and transmit unit for SLIP_PPP where n is bytes in decimal; the valid range is 76 through 1500 bytes. The optional parameter m instructs SLIP_PPP to assume the largest receive size character mapping scenario. |

| Keyword | Value | Default | Description |
| --- | --- | --- | --- |
| xe "open NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:open setting (SLIP_PPP)"§open | Active or passive | Active | LCP and IPCP open mode. |
| xe "port NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:port setting (SLIP_PPP)"§port | Base I/O address | 3F8 | Default address is the hexadecimal equivalent of the COM1: port. |
| xe "tcpipcomp NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:tcpipcomp setting (SLIP_PPP)"§tcpipcomp [ remote ] | VJ or No slots comp_slot | No 1 60 | xe "SLIP_PPP ODI driver\:Van Jacobson header compression"§Enable the Van Jacobson (VJ) header compression as follows: VJ or No – compress or not to compress; slot – 1 through 16 conversation slots; comp_slot – 0 not to compress slot ID; or 1 to compress slot ID) |

Link Driver SLIP_PPP Settings

The SLIP_PPP driver is customized in the Link Driver section of the NET.CFG file.

xe "PPP protocol\:NET.CFG file settings"§xe "SLIP protocol\:NET.CFG file settings"§The following NET.CFG settings are common to both SLIP and PPP:

| Frame | int | port |
| --- | --- | --- |
| baud | listen | protocol |
| dial | mru | tcpipcomp |
| direct |  |  |

The following NET.CFG settings are specific to PPP:

| accm | counter | open |
| --- | --- | --- |
| accomp | ipaddr | pcomp |
| authen | magic |  |

xe "SLIP_PPP ODI driver\:obtaining configuration settings from the remote site"§The accm, accomp, ipaddr, magic, pcomp, and tcpipcomp settings use the remote option. If remote is not specified, the setting defaults to local. The remote specifier instructs SLIP_PPP to use the value entered in response to the remote site configuration requests.

The following paragraphs describe these NET.CFG settings.

| Frame | xe "NET.CFG file\:Frame setting (SLIP_PPP)"§xe "SLIP_PPP ODI driver\:frame type selection"§xe "Frame NET.CFG file setting (SLIP_PPP)"§The Frame setting lets you select either the SLIP or the PPP protocol. SLIP defines a sequence of characters that frame IP packets on a serial line. It provides no addressing, packet type identification, error detection or correction, or compression. PPP conveys multiple protocol packets on the same point-to-point link and provides error detection. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
baud 9600
tcpipcomp VJ
Frame SLIP

The default value is SLIP.

| accm[ remote ] | xe "SLIP_PPP ODI driver\:asynchronous control character map"§xe "accm NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:accm setting (SLIP_PPP)"§The accm [ remote ] setting lets you specify whether the PPP protocol should use the asynchronous control character map (ACCM). ACCM is a configuration option that provides a way to negotiate the use of control characters (0x0 through 0x1F) for mapping on an asynchronous link. |
| --- | --- |

When sending data, each character with a value less than 0x20 that is flagged in the remote ACCM is replaced by a 2-byte sequence consisting of the Control-Escape character (0x7D) and the original character with bit 6 complemented. Conversely, when receiving, the mapped 2-character sequence is converted back to the original character.

The eight hexadecimal bytes in ACCM use bit numbering from 31 to 0 from left to right. Each numbered bit corresponds to a control character with the same number value. When the bit is set to 0, the character does not need to be mapped; when the bit is set to 1, the character needs to be mapped.

The following example asks the remote host to escape XON (0x11) and XOFF (0x13) characters. For example:

Link Driver SLIP_PPP
Int 4
Port 3F8
baud 9600
accm 000A0000
Frame PPP

XON and XOFF have decimal values of 17 and 19, respectively, and the 17th and 19th bits of ACCM are set to one to request the remote host to send the appropriate 2-character sequences (0x7d, 0x31 and 0x7d, 0x33) when an XON or XOFF is encountered in packet data. The default value is ffffffff.

| accomp [ remote ] | xe "SLIP_PPP ODI driver\:address/control header compression"§xe "accomp NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:accomp setting (SLIP_PPP)"§The accomp [ remote ] setting lets you instruct the PPP protocol to compress the address and control field of the PPP header. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
accomp Yes
Frame PPP

The default value is No.

| authen [ remote ] | xe "SLIP_PPP ODI driver\:authentication"§xe "authen NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:authen setting (SLIP_PPP)"§The authen [ remote ] setting lets you specify whether the PPP protocol should use authentication. If selected, PPP uses the name and password to identify the local system to the peer. This process is based on the Password Authentication Protocol (PAP). If the remote option is specified, a local name and password database are formed to authenticate a remote host. Up to five remote host entries can be entered in the NET.CFG file. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
authen PAP brian bang
Frame PPP

The default value is None.

| baud | xe "SLIP_PPP ODI driver\:baud rate"§xe "baud NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:baud setting (SLIP_PPP)"§The baud setting lets you select a baud rate for the SLIP_PPP driver to use. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
Frame SLIP

The default value is 2400 baud.

| counter [protocol] |  |  | xe "SLIP_PPP ODI driver\:timeouts and retries"§xe "counter NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:counter setting (SLIP_PPP)"§The counter setting lets you specify a timeout value and three retry counters for the PPP protocol. All values are in decimal. The available values and descriptions are as follows: |  |
| --- | --- | --- | --- | --- |
|  | timeout | The timeout value for restarting the transmission of request packets. The default value is 3 seconds. |  |  |
|  | max_config | The number of retries for sending a configuration request. The default value is 10 times. |  |  |
|  | max_term | The number of retries for sending a terminate request. The max_term value is not used by PAP. The default value is 2 times. |  |  |
|  | max_nak | The number of retries for sending a configuration NAK. The default value is 10 times. |  |  |

The protocol option can be any of the following:

·	LCP — Link control protocol

·	IPCP — Internet protocol control protocol

·	PAP — Password authentication protocol

If the protocol option is not specified, changes are applied to all protocols.

For example:

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
counter LCP 5 10 5 10
Frame PPP

| dial | xe "SLIP_PPP ODI driver\:phone number to dial"§xe "dial NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:dial setting (SLIP_PPP)"§The dial setting lets you specify a phone number for the SLIP_PPP driver to call when it gets loaded. This setting can be used only when direct is set to No. |
| --- | --- |

The phone number entered can include dial modifiers supported by your modem up to the maximum of 30 characters. For the Hayes Smartmodem these include

0-9,*,#,A,B,C,D and -,(,),,,P,T.

For example:

dial 9,(801)123-4567

dial p123-4567

where p indicates pulse dialing; the default is tone dialing.

For example:

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
direct No
dial 999-1234
Frame SLIP

A phone number is required with this setting.

| direct | xe "SLIP_PPP ODI driver\:direct link"§xe "direct NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:direct setting (SLIP_PPP)"§The direct setting lets you select whether your SLIP or PPP link is direct. This setting must be No if you plan to use the dial or listen setting. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
direct No
listen
Frame SLIP

The default value is Yes.

| int | xe "COM port\:specifying for SLIP_PPP"§xe "SLIP_PPP ODI driver\:COM port to use"§xe "int NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:int setting (SLIP_PPP)"§The int setting lets you specify the interrupt value of your COMn: device for the SLIP_PPP driver. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 3
Port 2F8
Baud 9600
Frame SLIP

The default value is 4 (for COM1:).

|  | The SLIP_PPP driver’s internal dial or answer capability (when set up to use an INDIRECT connection) requires that the modem be set up to track the status of a carrier detect signal (with the &C1 AT command in effect). SLIP_PPP will assume the line is up and not dial or answer if the modem’s DCD is asserted. |
| --- | --- |

| ipaddr [ remote ] | xe "IP address\:SLIP_PPP"§xe "SLIP_PPP ODI driver\:IP address"§xe "ipaddr NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:ipaddr setting (SLIP_PPP)"§The ipaddr [ remote ] setting lets you specify the IP addresses for PPP negotiation (in dotted decimal notation). If the remote option is specified, PPP tries to assign the address to the remote host if the remote host did not specify an address to use in its configuration request. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
ipaddr 129.47.30.6
Frame PPP

There is no default value for this setting.

| listen | xe "Modem connection (SLIP_PPP)"§xe "SLIP_PPP ODI driver\:passive (modem) connection"§xe "listen NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:listen setting (SLIP_PPP)"§The listen setting signifies the passive end of a modem connection for the SLIP or PPP protocol; listen is valid only when direct is No. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
direct nolisten
Frame PPP

There is no default value for this setting.

| magic [ remote ] | xe "SLIP_PPP ODI driver\:looped-back links"§xe "magic NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:magic setting (SLIP_PPP)"§The magic [ remote ] setting provides a way to detect looped-back links and other datalink anomalies. The value specified is used by the SLIP_PPP driver as a seed number to generate a unique number to identify itself. If remote is specified, PPP tries to get the peer to use the number specified; that is, if the peer does not indicate that it will use the magic number in its initial configuration request. The magic setting is a 4-byte hexadecimal number. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
magic 1234FFFF
Frame PPP

The default value is to not use the magic setting.

| mru n [ m ] | xe "SLIP_PPP ODI driver\:transmit and receive units"§xe "mru NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:mru setting (SLIP_PPP)"§The mru setting lets you specify the maximum receive and transmit units for the SLIP or PPP protocol. |
| --- | --- |

The mru value (format of n[ m ] ) is a decimal number within the range of 76 through 1500 bytes. The optional parameter m (for mapped) instructs the SLIP_PPP driver to assume the worst case character mapping scenario where every data byte in a packet could be mapped into two bytes as required by the ACCM configuration (for example, a 500-byte packet could double its size to 1000 bytes). SLIP_PPP uses half of the mru as its largest receive size, and the protocol stack uses that as the maximum datagram size it can send. The optional parameter m applies to PPP only. For example:

Link Driver SLIP_PPP
Int 4
Port 3F8
baud 9600
mru 1500
Frame SLIP

The default value is 1500 for PPP and 1006 bytes for SLIP.

| open | xe "SLIP_PPP ODI driver\:active connection"§xe "open NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:open setting (SLIP_PPP)"§The open setting lets you select either the active or the passive open mode for the PPP protocol. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
open passive
Frame PPP

The default value is active.

| port | xe "SLIP_PPP ODI driver\:port address"§xe "port NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:port setting (SLIP_PPP)"§The port setting lets you specify the base I/O address of your COMn: device for the SLIP_PPP driver to use. For example: |
| --- | --- |

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
Frame SLIP

The default value for this setting is 3F8 (for COM1:).

The normal interrupt levels and port I/O addresses for COMn: ports are as follows:

|  |  |  |  | Int (Interrupt) | Port |
| --- | --- | --- | --- | --- | --- |
|  |  | COM1: |  | 4 | 3F8 |
|  |  | COM2: |  | 3 | 2F8 |
|  |  | COM3: |  | 4 | 3E8 |
|  |  | COM4: |  | 3 | 2E8 |
|  | tcpipcomp |  | xe "Van Jacobson header compression (SLIP_PPP)"§xe "SLIP_PPP ODI driver\:Van Jacobson header compression"§xe "tcpipcomp NET.CFG file setting (SLIP_PPP)"§xe "NET.CFG file\:tcpipcomp setting (SLIP_PPP)"§The tcpipcomp setting enables Van Jacobson (VJ) header compression for the SLIP or PPP protocol as follows: |  |  |

·	VJ or No — compress or not to compress

·	slot — 1 through 16 conversation slots

·	comp_slot — 0 not to compress slot ID; 1 to compress slot ID

The format is as follows:

tcpipcomp {VJ | No} slots comp_slot

If No is specified (for no compression), the slots and comp_slot values can be omitted. If they are specified with the No option, they are ignored. For example:

Link Driver SLIP_PPP
Int 4
Port 3F8
Baud 9600
tcpipcomp VJ 16 1
Frame SLIP

The default values for tcpipcomp are No, 16, and 0.

ODINSUP

xe "Configuring\:ODINSUP"§xe "ODINSUP"§The ODINSUP program is provided on the second installation diskette. This program supports the coexistence of the Network Driver Interface Specification (NDIS) and the Open Datalink Interface (ODI) Specification. It allows NDIS protocol stacks to run unmodified over the ODI LSL.

ODINSUP configuration is described in the file ODINSUP.DOC in your chosen installation directory (usually C:\MVW).

Setting Up the SNMP Agent

xe "Configuring\:SNMP"§SNMP is a Simple Network Management Protocol (SNMP) agent TSR program. It lets a remote Network Management Station (NMS) monitor the TCP/IP protocol stack running on the workstation. SNMP allows remote and local SNMP clients complete access to the TCP/IP portion of the Management Information Base (MIB). SNMP supports the MIB-II variables.

The LWPCON utility can be used for SNMP access as described on page  62. LWPCON does not require the SNMP agent to be running on the workstation to return local statistics. The SNMP TSR is required if you want remote SNMP client programs to be able to gather statistics from your workstation.

The SNMP protocol is fully described in RFC 1157, Simple Network Management Protocol (SNMP).The MIB-II standard is described in RFC 1213, Management Information Base for Network Management of TCP/IP-Based Internets: MIB-II.

To use the SNMP agent on a workstation, you must first load and bind TCP/IP to the interface it will be using. The typical loading sequence is as follows:

LSL NE2000 (or appropriate driver) TCPIPSNMP

The agent is a terminate and stay resident (TSR) program that can be loaded by typing the following command:

SNMP <Enter>

To remove the agent from memory execute the following command:

SNMP U <Enter>

You configure the SNMP agent by adjusting settings in the NET.CFG file. If NET.CFG is not configured for SNMP, it runs with the MonitorCommunity variable set to public (all clients can read the MIB values) and ControlCommunity set to disabled (client write access to the MIB values is not allowed).

SNMP NET.CFG Summary

xe "NET.CFG file\:SNMP agent settings"§xe "SNMP agent program\:NET.CFG file settings"§The NET.CFG configuration file provides five SNMP settings under the xe "NET.CFG file\:Protocol SNMP section"§Protocol SNMP section. Table  7 summarizes the valid and default values of these settings.

Table  7 NET.CFG Settings for SNMP

| Parameter | Explanation |
| --- | --- |
| xe "SNMP agent program\:contact name"§xe "sysContact NET.CFG file setting (SNMP)"§xe "NET.CFG file\:sysContact setting (SNMP)"§sysContact | Specifies the name of the person who is responsible for this machine. Normally, this will be the name of the person using this machine. |
| xe "SNMP agent program\:domain name"§xe "sysName NET.CFG file setting (SNMP)"§xe "NET.CFG file\:sysName setting (SNMP)"§sysName | Specifies the full domain name. |
| xe "SNMP agent program\:system physical location"§xe "sysLocation NET.CFG file setting (SNMP)"§xe "NET.CFG file\:sysLocation setting (SNMP)"§sysLocation | Specifies the location of the machine. This parameter is used by NMS to determine the physical location of the machine. |
| xe "SNMP agent program\:read-only community name"§xe "monitorCommunity NET.CFG file setting (SNMP)"§xe "NET.CFG file\:monitorCommunity setting (SNMP)"§monitorCommunity | Sets the community name for read-only access. By default, this community is public. |
| xe "SNMP agent program\:read/write community name"§xe "controlCommunity NET.CFG file setting (SNMP)"§xe "NET.CFG file\:controlCommunity setting (SNMP)"§controlCommunity | Sets the community name for read and write access. By default, this community is disabled. |

Community Names

xe "Community names (SNMP)"§xe "SNMP agent program\:community names"§Community names are used to authenticate SNMP request messages received at the agent. The community name in a message requesting a given access type must match the name defined for that access type by one of the SNMP community options.

Community names are arbitrary 32-character text strings. You can use the string noAccess to disable the appropriate community. This name is reserved and is not case-sensitive.

SNMP Operations

The SNMP protocol allows four operations:

·	Retrieve specific management information.

·	Retrieve general management information.

·	Manipulate management information.

·	Report extraordinary events.

The Trap operation has not been implemented in this release.

Authentication

xe "Authentication (SNMP)"§xe "SNMP agent program\:authentication"§SNMP defines a community as a relationship between an SNMP agent and one or more SNMP managers. At the present, only trivial authentication mechanisms are available with SNMP. This means that community name is placed, in clear text, in an SNMP message. If the community name corresponds to a community known to the receiving SNMP entity, the sending SNMP entity is considered to be authenticated as a member of that community.

The SNMP agent lets you specify any community names you require. Two different communities are used by the agent. The monitor community is the used for read-only access; the control community is used for read-write access. If community name is not known to the agent, the request from the SNMP manager station is ignored.

By default, the monitor community is set to public and the control community is disabled. To override these default settings, edit the NET.CFG file.

Example NET.CFG Configurations

This section contains examples of NET.CFG files set up for different purposes. You can copy the appropriate parts of these examples into your workstation’s NET.CFG file if you have a similar configuration.

This section provides the following examples:

·	Setting up a token ring configuration

·	Setting up an ARCnet configuration

·	Configuring a SLIP or PPP asynchronous serial line

·	Configuring the SNMP agent

·	Setting up multiple network connections

·	Running IPX and IP concurrently

Token Ring Configuration

xe "Token Ring\:example configuration"§Figure  7 shows how to configure NET.CFG for a token ring TCP/IP network using the xe "TOKEN.COM driver, example configuration"§TOKEN.COM driver. IPX support is not configured in this example. The frame type used on token ring TCP/IP networks is always TOKEN-RING_SNAP. The increased buffer size (4202) is recommended only if problems are encountered using the default size (1500).

Figure 7 Sample NET.CFG File for Token Ring

Link Driver TOKEN Frame TOKEN-RING_SNAP Link Support Buffers 8 4202 MemPool 4096 Protocol TCPIP PATH TCP_CFG C:\NET\TCP ip_address 144.52.6.5 ip_router 144.52.6.6 ip_netmask 255.255.0.0

ARCnet Configuration

xe "ARCnet example configuration"§Figure  8 shows how to configure NET.CFG for an ARCnet network. IPX support is not configured in this example. This example uses a TRXNET board with NOVELL_RX-NET frame format, which is the frame format always used on ARCnet TCP/IP networks. The increased buffer size (4202) is recommended only if problems are encountered using the default size (1500).

Figure 8 Sample NET.CFG File for ARCnet

Link Driver TRXNET Frame NOVELL_RX-NET Link Support Buffers 8 1500 MemPool 4096 Protocol TCPIP PATH TCP_CFG C:\NET\TCP ip_address 144.52.6.5 ip_router 144.52.6.6 ip_netmask 255.255.0.0

SLIP and PPP Configurations

xe "NET.CFG file\:SLIP_PPP configuration example"§xe "SLIP_PPP ODI driver\: example NET.CFG file"§This section includes examples of the NET.CFG file for frame types SLIP and PPP.

xe "SLIP protocol\:example NET.CFG file"§Figure  11 provides an example NET.CFG using SLIP frame type.

Figure 11 NET.CFG File Using SLIP Frame Type

Link Support Buffers 8 1500 MemPool 4096 Link Driver SLIP_PPP INT 4 PORT 3F8 baud 2400 mru 800 tcpipcomp vj direct no dial 456-7890 Frame SLIP

Protocol TCPIP ip_address 129.47.236.1 ip_netmask 255.255.255.0 ip_router 129.47.236.11 tcp_sockets 8 udp_sockets 8 raw_sockets 1

The configuration in Figure  11 installs SLIP_PPP to run on COM1: at 2400 baud, use maximum packet size 800 bytes, enable Van Jacobson Header Compression, and dial up phone number 456-7890 when it was loaded.

SLIP is not capable of exchanging configurations so the configurations have to be statically set and pre-agreed upon among peers. Configuration values such as mru and whether VJ compression is used need to be set the same as your peer. You should know your peer’s IP address beforehand and use it to set the ip_router address in the Protocol TCPIP section of your NET.CFG file.

xe "PPP protocol\:example NET.CFG file"§Figure  12 provides an example NET.CFG using the PPP frame type.

Figure 12 NET.CFG File Using PPP Frame Type

Link Support Buffers 8 1500 MemPool 4096 Link Driver SLIP_PPP INT 3 PORT 2F8 baud 9600 mru 1000 accm 00000000 accomp yes pcomp yes tcpipcomp VJ 16 1 direct yes Frame PPP Protocol TCPIP tcp_sockets 8 udp_sockets 8 raw_sockets 1

The configuration in Figure  12 installs SLIP_PPP to run on COM2: at 9600 baud, use the maximum packet size of 1000 bytes, and attempt to tell the remote host the following about the local workstation:

·	None of the ASCII control characters need to be mapped

·	Handles compressed PPP frames

·	Interprets compressed TCP/IP packets using the Van Jacobson algorithm.

The driver will run on directly connected null-modem lines.

PPP negotiates link layer and network layer configurations among peers. In the NET.CFG file in Figure  12 we request that our peer assign an IP address for us to use (no local ipaddr is specified). The protocol stack TCP/IP will use the negotiated local and remote IP addresses to set its operating ip_address, ip_netmask, ip_router. They are not statically configured in the Protocol TCPIP section of the NET.CFG file.

Because you can configure multiple ODI drivers for use with TCP/IP, you can set up your workstation for communications over both a network board and a COMn: port link simultaneously. Figure  13 llustrates a NET.CFG file that configures an Ethernet board (NE2000) and the COM1: port for xe "NET.CFG file\:SLIP_PPP configuration example"§xe "SLIP protocol\:sample NET.CFG file"§SLIP TCP/IP use.

Figure 13 Sample NET.CFG File Using SLIP and an Ethernet Connection Concurrently

Link Driver NE2000 Int #1 2 Port #1 340 10 Frame Ethernet_II Frame Ethernet_802.3 Protocol IPX 0 Ethernet_802.3 Link Driver SLIP_PPP Int 4 Port 3F8 Baud 9600 tcpipcomp vj Frame SLIP Link Support Buffers 8 1500 MemPool 4096 Protocol TCPIP PATH TCP_CFG C:\NET\TCP ip_address 129.48.6.51 ENGINEERING-NET ip_router 129.48.4.254 ENGINEERING-NET ip_netmask 255.255.0.0 ENGINEERING-NET bind ne2000 #0 ETHERNET_II ENGINEERING-NET ip_address 129.47.6.51

LOCAL-NET ip_router 129.47.4.254 LOCAL-NET ip_netmask 255.255.0.0 LOCAL-NET bind slip_ppp #0 SLIP LOCAL-NET

This NET.CFG file performs the following operations:

·	Configures the SLIP_PPP driver to operate on the COM1: port using the LOCAL-NET address parameters (ip_address, ip_router, ip_netmask).

·	Configures the NE2000 driver to use the ENGINEERING-NET address parameters. This driver is configured to use both Ethernet_II and Ethernet_802.3 frame types, and run concurrently with IPX.

|  | Configuring multiple ODI drivers is not sufficient to make a workstation into a xe "Router"§router. The workstation with the NET.CFG file in Figure 13 can operate solely as a workstation on one LAN connection and one serial connection simultaneously. It cannot route packets between these two connections for other workstations. |
| --- | --- |

SNMP Agent Configuration

xe "SNMP agent program\:example NET.CFG file"§xe "NET.CFG file\:SNMP agent example"§Figure  15 provides an example NET.CFG for configuring the SNMP agent.

Figure 15 NET.CFG with SNMP Settings

Link Support Buffers 8 1500 MemPool 4096 Protocol TCPIP Bind NE2000 Protocol SNMP monitorCommunity public controlCommunity novelladmin sysContact Horace Felton sysName horacef@success.com sysLocation Building #1 - Office 209 Link Driver NE2000 Frame Ethernet_II

Multiple Network Configuration

Figure  16 is an example of a xe "NET.CFG file\:multiple network connection example"§NET.CFG file that configures two different network connections, one through an NE2000 Ethernet board and one through asynchronous serial ports.

Figure 16 Example NET.CFG File for Multiple Network Connections

Link Support Buffers 8 1500 MemPool 4096 Link Driver NE2000 Int #1 2 Port #1 340 10 Frame Ethernet_II Frame Ethernet_802.3 Protocol IPX 0 Ethernet_802.3 Link Driver SLIP_PPP Int 4 Port 3F8 Band 2400 MRU 800 TCPIPcomp VJ Frame SLIP Direct No Dial 456-0684 Protocol TCPIP tcp_sockets 8 udp_sockets 8 raw_sockets 1 ip_address 122.44.9.7 LOCAL-NET ip_router 122.44.3.97 LOCAL-NET ip_netmask 255.255.0.0 LOCAL-NET ip_address 93.57.6.168 ENGINEERING-NET ip_router 93.57.4.254 ENGINEERING-NET ip_netmask 255.255.0.0 ENGINEERING-NET bind slip_ppp #1 SLIP LOCAL-NET bind ne2000 #1 ETHERNET_II ENGINEERING-NET

This NET.CFG file does the following:

·	Configures LSL to support all connections (LSL will direct network packets to and from each of the three interfaces).

·	Configures the NE2000 driver to support two protocol stacks simultaneously: TCP/IP (Ethernet_II frame type) and IPX (Ethernet_802.3 frame type). This driver becomes NE2000 board instance #1 for the Bind statement later in the NET.CFG file.

·	Configures the SLIP_PPP driver to operate on the COM1: port (interrupt 4).

·	Allocates eight TCP sockets, eight UDP sockets, and one raw socket for use by both network connections.

·	Assigns the name ENGINEERING-NET to the NE2000 connection, and assigns the name LOCAL-NET to the first SLIP_PPP connection.

The network names are arbitrary and are used only within the NET.CFG file. Their only purpose is to identify which parameter settings to apply to each network connection.

·	Assigns an IP address, IP router address, and subnetwork mask for each network name.

·	Specifies that the first SLIP_PPP driver is to use the LOCAL-NET parameters and frame type SLIP (Bind statement).

·	Specifies that the NE2000 connection is to use the ENGINEERING-NET parameters and frame type ETHERNET_II.

The batch file that loads the drivers configured by this NET.CFG file loads both the NE2000.COM and the SLIP_PPP driver, as in Figure  17.

Figure 17 Example Batch File for Multiple Network Connections

C:\yesno “Do you want to load the networking software?” If errorlevel 1 goto noload PATH C:\MVW\BIN; %path% C:\lsl.com C:\NE2000.com C:\slip_ppp.com TCPIP.EXE:noload

xe "TCPIP.EXE program\:sign-on display, multiple network connections"§When the batch file is executed, sign-on messages similar to Figure  18 are displayed.

Figure 18 Example Sign-On Display for Multiple Network Drivers

Novell TCP/IP Transport v4.1 (920910)(C) Copyright 1992 Novell, Inc. All Rights Reserved Network Name: ENGINEERING-NET Bind: NE2000IP Address:93.57.6.168 Board Number: 1Subnet Mask: 255.255.0.0 Board Instance: 1Default Router: 93.57.4.254 Frame: ETHERNET_II Network Name: LOCAL-NET Bind: SLIP_PPPIP Address: 122.44.9.7 Board Number: 2Subnet Mask: 255.255.0.0 Board Instance: 1Default Router: 122.44.3.97 Frame: SLIP

Concurrent TCP/IP and IPX Configurations

xe "NetWare\:concurrent use of IPX and TCP/IP"§xe "Protocols\:concurrent TCP/IP and IPX"§xe "IPX protocol\:TCP/IP used concurrently with"§xe "TCP/IP\:IPX used concurrently with"§ODI supports concurrent TCP/IP and IPX protocol stacks. The Link Driver section of the NET.CFG file must be modified if you plan to run both TCP/IP and IPX.

xe "Ethernet\:concurrent TCP/IP and IPX configuration"§Figure  19 is a sample NET.CFG file for using TCP/IP and IPX in an Ethernet configuration (ECONFIG) environment. In this example, both IPX and TCP/IP use the ETHERNET_II frame format.

Figure 19 Sample NET.CFG File for Concurrent TCP/IP and IPX Over ECONFIG

Link Driver NE1000 Int #1 5 Port #1 320 Frame ETHERNET_II # Allow Ethernet II packets

Link Support Buffers 8 1500 MemPool 4096

Protocol TCPIP PATH TCP_CFG C:\NET\TCP ip_address 129.47.6.129 ip_router 129.47.4.254 ip_netmask 255.255.0.0

Figure  20 is a sample NET.CFG file for using TCP/IP and IPX in the default Ethernet 802.3 environment. The following line configures IPX to run over Ethernet 802.3:

Protocol IPX 0 Ethernet_802.3

A Protocol IPX setting must be present to run IPX with TCP/IP, when IPX is operating in an Ethernet 802.3 environment. In this example, TCP/IP uses the ETHERNET_II frame format, and IPX uses the ETHERNET_802.3 frame format.

Figure 20 Sample NET.CFG File for Concurrent TCP/IP and IPX Over Ethernet 802.3

Link Driver NE1000 Int #1 5 Port #1 320 Frame ETHERNET_II # Allow Ethernet II packets Frame ETHERNET_802.3 # Allow 802.3 packets Protocol IPX 0 Ethernet_802.3 # Make IPX use 802.3 Link Support Buffers 8 1500 MemPool 4096 Protocol TCPIP PATH TCP_CFG C:\NET\TCP ip_address 129.47.6.129 ip_router 129.47.4.254 ip_netmask 255.255.0.0

xe "Token Ring\:concurrent TCP/IP and IPX configuration"§Figure  21 shows how to configure NET.CFG for a token ring network running both TCP/IP and IPX. In this example, TCP/IP uses the TOKEN-RING_SNAP frame format and IPX uses the TOKEN-RING frame format.

Figure 21 Sample NET.CFG File for Concurrent TCP/IP and IPX Over Token Ring

Link Driver TOKEN Frame TOKEN-RING_SNAP Frame TOKEN-RING Protocol IPX E0 TOKEN-RING

Link Support Buffers 8 4202 MemPool 4096

Protocol TCPIP PATH TCP_CFG C:\NET\TCP ip_address 144.52.6.5 ip_router 144.52.6.6 ip_netmask 255.255.0.0

Utilities

This section explains how to run the DIALUP, LWPCON, and PING utilities.

Running DIALUP

xe "DIALUP utility"§DIALUP is a Hayes Smartmodem (or compatible) control program. It provides service to dial, redial, answer, hang up, or set line format to establish a modem connection in either batch mode or conversational mode for the asynchronous xe "SLIP_PPP ODI driver\:DIALUP utility"§SLIP_PPP driver.

DIALUP operates in batch mode (dial only) when a hostname is specified.  The hostname is a name entry in the file DIALUP.TXT which should be in the \MVW\HSTACC directory. The DIALUP.TXT file has a phone number paired with each hostname. DIALUP looks up the hostname in DIALUP.TXT and dials its associated phone number. The syntax for entries in the xe "DIALUP utility\:DIALUP.TXT file format"§xe "SLIP_PPP ODI driver\:DIALUP.TXT file format"§xe "DIALUP.TXT file"§DIALUP.TXT file is as follows:

hostname1	phone_number1

hostname2	phone_number2

hostnamen	phone_numbern

The result of the batch dialing can be interrogated using the DOS batch file command ERRORLEVEL.

For example:

lsl slip_ppp DIALUP engineering-router
if errorlevel 1 goto noconnection
tcpip telapi:noconnection

DIALUP operates in conversational mode when a hostname is not specified, either through a menu or as a dumb terminal emulator.

The DIALUP syntax is as follows:

DIALUP [ –? ][ –n ] [ –Snnnnn ] [ –A | –H | –T | ][ –T ] [ hostname ]

The DIALUP options and parameters are:

| –? or –H | Display DIALUP usage help. |
| --- | --- |
| –n | Specify the resident SLIP-PPP to use; n is the board number returned when SLIP_PPP is loaded. |
| –Snnnnn | Set the baud rate to nnnnn to start modem initialization. If not selected, modem initialization starts with the current baud rate setting in the Link Driver SLIP_PPP section of the NET.CFG file. |
| –A | Put the modem in blind answer mode and return. |
| –E | Hang up the line and end the session. |
| –T | Request dumb terminal emulator. |

Dumb terminal emulator is enabled with the –T command line option. xe "DIALUP utility\:entering modem commands (Ctrl-L)"§AT commands can be directly entered to control the modem, and pressing <Ctrl-L> anytime lets you change the baud rate. Pressing <Ctrl-X> lets you exit.

Menu driven control is enabled when the –T command line option is omitted. The menu options are as follows:

| L | Line settings |
| --- | --- |
| D | Dial a phone number |
| R | Redial the last phone number |
| A | Answer incoming calls |
| H | Hang up |
| Q | Quit |

|  | DIALUP requires the asynchronous SLIP_PPP MLdriverID to be loaded to operate, and its sole purpose is to establish or tear down modem connections for SLIP_PPP. Although dumb terminal mode is supported, it should only be used to set modem parameters and establish modem connections. |
| --- | --- |

Running LWPCON

xe "Command\:LWPCON"§LWPCON (LAN WorkPlace Console) is a DOS menu-based application that lets you display TCP/IP xe "Configuration information (LWPCON)"§xe "Information\:LWPCON utility"§configuration, statistics, and service information for your workstation and xe "TCP/IP\:information, using LWPCON"§xe "Network\:information, using LWPCON"§xe "Remote host\:information (LWPCON)"§remote hosts on your network. You can do the following tasks with xe "LWPCON"§xe "LWPCON\:tasks, typical"§LWPCON:

·	Check the configuration parameters for your workstation and remote hosts on your network.

·	Force closure of TCP connections.

·	Check whether remote hosts are available.

·	Check the network route used to connect you to a remote host.

·	List the services available on a remote host.

·	Determine which remote host services are currently available.

·	Determine what remote host information is available from your network’s DNS servers.

This section explains how to perform these tasks.

In addition to these capabilities, LWPCON provides several statistical displays that network administrators can use to monitor network traffic and isolate problems. For a complete description of the statistical displays and information on LWPCON system administration features and procedures, refer to the LAN WorkPlace for DOS Administrator’s Guide.

You can start xe "LWPCON\:starting"§LWPCON by double-clicking the LWPCON Windows icon or entering the LWPCON command at the DOS prompt. From Windows, LWPCON runs in a DOS window. In either case, LWPCON displays its initial screen and menu:

The first line of the LWPCON screen identifies the program, LAN WorkPlace Console, followed by the current date and time. The last line is a reverse-video display of the special keys and function keys available for that particular screen. Above the last line is a two-line message area for help information about a particular screen or highlighted item.

To exit from the initial xe "LWPCON\:quitting"§LWPCON screen and return to DOS, press the <Esc> key.

xe "LWPCON\:displays and menus"§LWPCON Displays, Menus, and Control Keys

LWPCON menus have a title followed by a list of options. You can select a menu option using the up or down arrow keys, then pressing the <Enter> key. You cannot use a mouse to make selections in LWPCON. In some cases, information about the highlighted option is displayed in the message area. Pressing the <F1> key provides additional help.

LWPCON can also display lists or tables containing a varying number of items. You can highlight and select list items as you do for menu selections. In some cases, you can make insertions (using <Insert>), deletions (using <Delete>), and modifications (using <Enter>).

To let you enter new information, LWPCON presents the user with various forms. An LWPCON form contains data fields. You can edit and change some fields, although others are used only for display. After you type new information in a field, press the <Esc> key to exit the field and continue the operation.

xe "LWPCON\:control keys"§The following paragraphs describe the control keys used by LWPCON.

·	The arrow keys move the highlight through menu and list items and scroll through lists one line at a time.

·	The <PgUp> and <PgDn> keys scroll through long lists one screen at a time. You can press <Ctrl-PgDn> to scroll to the bottom of a list or <Ctrl-PgUp> to scroll to the top of a list.

·	The <Delete> key removes an existing entry from a list, if the list can be changed. You can also use the <Delete> key to select and clear fields that can be changed.

·	The <Enter> key selects the currently highlighted menu or list item. Selecting an entry from a list of statistics causes any additional information associated with the entry to be displayed.

·	The <Esc> key returns you to the previous menu level or ends a field selection.

·	The <F1> key displays online help for the current display. In some cases, help screens are available for list selections.

·	The <Insert> key adds a new entry to a list that can be changed. Fill in the form displayed by the <Insert> key, then press <Esc>.

·	The <Tab> key toggles host displays between symbolic name and dotted decimal IP address representation.

Table  9 summarizes these keys.

| Table 9 LWPCON Control Keys |  |
| --- | --- |
| Key | Description |
| <>, <¯> | Moves the highlight through menu or list items. |
| <Delete> | Removes an existing entry from a list or select and clear a field. |
| <Enter> | Selects the currently highlighted menu item or selects a hostname from a list. |
| <Esc> | Ends field entry or returns to the previous menu level. |
| <F1> | Accesses online help. |
| <Insert> | Adds a new entry to a list. |
| <PgUp>, <PgDn> | Scrolls through list entries |
| <Tab> | Toggles between the hostname and the IP address display. |

Checking Workstation and Remote Host Configuration

xe "Configuration information (LWPCON)"§xe "Information\:configuration"§You can check the xe "TCP/IP\:configuration information"§TCP/IP configuration settings for your workstation or a remote host on your network. Follow these steps:

|  | 1. Select either Local Workstation or Remote Host from the Available Options menu on the initial LWPCON screen: |
| --- | --- |

If you select Remote Host, you are prompted to enter the remote hostname:

2.	If prompted, type the name or IP address (in dotted notation) of a host on your network.

The remote host must be running a server program, such as TCPIP.NLM for a NetWare server or SNMP.EXE for a PC workstation.

After you enter a valid remote hostname or IP address, the Remote Host Options menu appears.

If you selected Local Workstation from the Available Options menu, the Local Workstation Options menu appears.

The Change Host option is not available to you if you selected Local Workstation.

3.	To display configuration information, select Overview.

A general information screen similar to the following one appears:

The xe "Overview screen (LWPCON)"§overview screen supplies the following information:

·	Hostname—the fully qualified name of the remote host or local workstation.

·	IP address—the address configured for this host or workstation.

·	Subnet mask—the subnetwork mask configured for this host or workstation, if your network has subnetworks.

·	Router—the address of the IP router, if your network has one.

·	Interface name—usually displays the name of the device driver providing network connection support.

·	Interface address—the hardware or MAC address of the host or workstation.

4.	To exit the overview display, press the <Esc> key.

The other choices on the options menus (Interfaces, Protocols, Tables, and SNMP) provide statistical displays that are of interest to the network administrator. Refer to the LAN WorkPlace for DOS Administrator’s Guide for more information.

Listing and Closing TCP Connections

Occasionally a TCP xe "Connections\:closing TCP"§xe "Connections\:listing current"§connection might not be closed properly; for example, when a remote connection is severed unexpectedly. This can result in a shortage of TCP resources (sockets) on your workstation.

To display a list of the current TCP connections for your workstation and close any that are hung up, follow these steps:

|  | 1. Select Local Workstation from the Available Options menu on the initial LWPCON screen. |
| --- | --- |

2.	Select Tables from the Local Workstation Options menu:

3.	Select TCP Connection Table from the TCP/IP Tables menu:

A table of current TCP connections appears, similar to the following example:

For each connection, the workstation (Local Host) address and port, remote host address and port, and status are displayed.

4.	Use the arrow keys to move the highlight to the connection you want to close, and press the <Delete> key.

Use caution in deleting established connections. When you delete a connection that is in use, data can be lost that has not been saved by the user who is communicating on the connection.

5.	Confirm that you want to close the connection by pressing <Enter>:

The connection is closed and deleted from the table.

6.	Press <Esc> to exit from the TCP connection table.

Determining Remote Host Status

xe "Remote host\:status (LWPCON)"§xe "LWPCON\:status of remote host"§You can check whether a remote host is currently running and available on your network. Follow these steps:

|  | 1. Select Services from the Available Options menu on the initial LWPCON screen. |
| --- | --- |

You are prompted for a hostname:

2.	Type the name or IP address (in dotted notation) of a host on your network.

Information similar to the following appears at the top of the screen:

If the remote host is available, the Host Up? field contains Yes.

3.	Press <Esc> to exit the status display.

Checking the Network Route to a Host

xe "LWPCON\:route to remote host"§xe "Route to remote host (LWPCON)"§xe "Network\:route to host (LWPCON)"§xe "Remote host\:route to"§xe "Information\:route to host (LWPCON)"§You can check the interconnections used to route your communication with a remote host. This information can be useful if you are having difficulty connecting to a host or the response from the host is slow.

To check the interconnections to a host, follow these steps:

|  | 1. Select Services from the Available Options menu on the initial LWPCON screen. |
| --- | --- |

You are prompted for a hostname:

2.	Type the name or IP address (in dotted notation) of a host on your network.

Below the remote host information, the Services Options menu appears.

3.	Use the arrow keys to move the highlight to Trace Route, and press <Enter>.

LWPCON displays information about the route followed to reach this host, as in the following example:

`

The display lists the name and IP address of each intermediate host through which your connection is routed, starting with your router (hop 1) and ending with the remote host you specified. The amount of time in milliseconds required to make each intermediate connection is also listed. A time of 0 indicates that less than 1 millisecond was required to connect, as is often the case for a LAN connection.

If a host in the route does not respond to LWPCON within its time limit, the message “Not Available” is displayed. You can use these messages to determine where a problem is occurring in reaching a remote host. In the preceding example, if both hop 6 and hop 7 are not available, you might suspect that network 144.202 is down.

4.	Press <Esc> to exit from the trace route display.

Checking Remote Host Services

xe "Information\:remote host services"§xe "LWPCON\:services on remote host"§xe "Remote host\:services available (LWPCON)"§You can determine what network services a remote host makes available and check the current availability of each service. Follow these steps:

|  | 1. Select Services from the Available Options menu on the initial LWPCON screen. |
| --- | --- |

You are prompted for a hostname:

2.	Type the name or IP address (in dotted notation) of a host on your network.

Below the remote host information, the Services Options menu appears:

3.	Select Check Active TCP Services from the Services Options menu.

LWPCON displays a list of TCP services similar to the following:

The service numbers correspond to the TCP well-known port numbers. For example, the File Transfer Protocol service, usually service (port) 21, provides services used by Rapid Filer and FTP. The Terminal Connection service, usually service 23 (the Telnet port), is used by Host Presenter and TNVT220.

4.	Use the arrow keys to highlight the service you want to check and press the <Enter> key.

If the service is available, LWPCON replies with a message similar to the following:

If the service is inactive, a problem exists on the remote host. The server program that supports this service might not be running on the remote host. The remote host administrator can check and activate the service.

5.	To check a service for which you know the port number, highlight Other at the end of the List of TCP Services and press <Enter>.

LWPCON prompts you to enter a TCP port number:

6.	Type a port number and press <Enter>.

LWPCON replies with a message indicating whether the service is active, as in Step 4.

7.	Press <Esc> to exit the display.

Displaying DNS Information

xe "Information\:DNS"§xe "LWPCON\:DNS information"§xe "DNS information (LWPCON)"§You can determine whether a remote host is known to your network DNS name servers, and if so, display the information stored about the remote host.

Follow these steps:

|  | 1. Select Services from the Available Options menu on the initial LWPCON screen. |
| --- | --- |

You are prompted for a hostname:

2.	Type the name or IP address (in dotted notation) of a host on your network.

Below the remote host information, the Services Options menu appears.

3.	Use the arrow keys to highlight Query Name Service on the Services Options menu, and press <Enter>.

LWPCON responds with a list of the DNS information for the host, similar to the following:

The information available depends on what information is stored in your network DNS database.

4.	Press <Esc> to exit the display.

For information about the LWPCON system administration features that are not covered in this manual, refer to the LAN WorkPlace for DOS Administrator’s Guide.

Running PING

The PING utility sends “round-trip” (ICMP ECHO_REQUEST) packets to the network host specified. PING uses the ICMP protocol’s mandatory ECHO_REQUEST datagram to get an echo response from the specified host or network router. ECHO_REQUEST datagrams, or “pings,” have an IP and ICMP header, followed by a structure timeval. The remaining unfilled bytes are padded with random values.

Executing PING with the optional node_name command-line parameter prints HOST IS ALIVE if the host responds. Otherwise, it repeatedly tries to send ICMP PING packets once every second up to 20 seconds. If no response is received from the host, it displays NO ANSWER FROM host.

The optional node_name parameter can be a name or an Internet address. If this value is given a valid Internet name, the PING packet is sent across the Internet.

For example:

C:\> PING la.company.com

Output:

la.company.com is alive

or

C:\> PING mynode.company.com

Output:

no answer from mynode.company.com

If the given host cannot be resolved by DNS or by the host file, then PING displays UNKNOWN HOST. If an error occurs in any of the socket functions used by PING, the error is displayed and PING exits.

PING has a default delay of 1 second.

The following examples illustrate the PING utility.

To PING node ISOSUN using the node name, enter the following command:

C:\> PING ISOSUN

Output:

no answer from isosun

The above response indicates that ISOSUN is not reachable.

To PING node 129.47.6.1 using the IP address, enter the following command:

C:\> PING 129.47.6.1

Output:

129.47.6.1 is alive

The above response indicates that 129.47.6.1 is available.

Troubleshooting

This section describes some of the common problems, and solutions, when trying to configure the TCP/IP Kernel.

ODI to NDIS protocol conversion

For a detailed description of the ODI to NDIS protocol conversion required when running with NDIS protocols (such as Windows for Workgroups) please see the file ODINSUP.DOC in your chosen installation directory (usually C:\MVW).

Unlisted card drivers

If your network card falls into the Unlisted Driver category, during installation you will need a disk that contains an ODI driver and the relevant NET.CFG configuration file.  If your network card does not come with an ODI driver you may be able to obtain one from the vendors Bulletin Board System (BBS), failing this you should contact your dealer.  If you have an ODI driver without a NET.CFG, you will need to create a file (see below for some examples of this).  If your NET.CFG file is not shown below, refer to the Configuration section of this guide for information on how to create a NET.CFG.

NOTE: If you wish to proceed with the installation without a NET.CFG file, you will need to create a null file on the same disk that contains your driver.

The Windows for Workgroups Startup Kit network card

You can download the ODI driver (EXP16ODI.COM) for this network card from the Intel Bulletin Board System (BBS) at (USA) (503) 645-6275.  Currently there is no NET.CFG that can be downloaded at the same time so it will be necessary for you to create a NET.CFG file containing the following information:

Link Support

Buffers 8 1500

MemPool 4096

Link Driver EXP16ODI

INT   #1 5

PORT  #1 300

Frame Ethernet_II

Frame Ethernet_802.2

Frame Ethernet_802.3

Frame Ethernet_snap

Protocol TCPIP

ip_address     nnn.n.n.n	(Add your IP address)

ip_router      0.0.0.0		(Add your IP router address)

ip_netmask     255.255.255.0

tcp_sockets    8

udp_sockets    8

raw_sockets    1

Copy the NET.CFG file to the disk containing your down loaded driver and follow the installation process to install both the driver and the NET.CFG file

Windows for Workgroups 3.10

If you are having difficulty installing the TCP/IP kernel with Windows for Workgroups, the following information may be helpful:

1.	When you install Windows for Workgroups, choose LANMAN network as your transport.

2.	Install the TCP/IP Kernel.

3.	Remove any reference to NDIS network drivers from the system CONFIG.SYS.

For example:

DEVICE=C:\WINDOWS\NE2000.DOS

4.	Edit the AUTOEXEC.BAT file and add the following line AFTER the ODI drivers (e.g. LSL, 3C503) and BEFORE the NET START and TCPIP commands.

ODINSUP

For example, your AUTOEXEC.BAT may look like:

LSL

3c503		(Example ODI driver name)

ODINSUP

NET START

TCPIP

5.	Reboot your PC.

If you still encounter difficulties, follow these instructions:

6.	Make a note of the following line displayed during PC boot:

mac=[x3c503]

Where x3c503 will reflect the name of your relevant driver.

7.	Edit the Windows PROTOCOL.INI file and add the driver name as shown in the example below:

[MS$NETBEUI]
	BINDINGS=x3C503

Where x3c503 should be replaced by the driver name shown in step 6.  If your driver name has a trailing space, it may be necessary to enclose the name in quotes, e.g. BINDINGS="PCMSM "

8.	Reboot your PC.

NOTE: If you configure your network through Windows for Workgroups, you should audit the text files that get changed. For example, the PROTOCOL.INI file is changed, but you are not informed of the change. It is possible that the BINDINGS entry in this file will be changed and this will result in the TCP/IP Kernel not working, see point 7 above.

Windows for Workgroups 3.11

If you are installing Windows for Workgroups 3.11, the following steps will help to configure your network card.

1.	When you install Windows for Workgroups .3.11, choose NETBEUI network as your transport.

2.	Install the TCP/IP Kernel.

3.	Within Windows, choose the Network group and select Network Setup.

4.	Select Drivers and then Setup.

5.	Change the Driver Type to ODI Driver.  Windows will make the necessary changes to your AUTOEXEC.BAT and PROTOCOL.INI.

Detailed below is a sample of the configuration files that can be used with a 3C503 network card.

AUTOEXEC.BAT

c:\WINDOWS\SMARTDRV

SET MVWDIR=C:\MVW		- DeskTop installation directory

SET PATH=%MVWDIR%;%PATH%

SET EXCELAN=C:\MVW	- Network installation directory

C:\WINDOWS\NET START

LSL				-Link Support Level driver

3c503				-Network card driver

C:\WINDOWS\ODIHLP		-Windows for Workgroups driver

TCPIP				-TCP/IP TSR

NET.CFG

Link Support		-LSL configuration options
Max Boards 4
Buffer 8 1500

Max Boards 4
Buffers 8 1500
Mempool 4096

Link Driver 3C503		-Card driver options
INT 	5			-Interrupt
MEM 	D4000		-Memory address use
PORT 	300			-Port address
Frame Ethernet_802.2	-Frame types
Frame Ethernet_II
Frame Ethernet_snap
Frame Ethernet_802.3
Frame IPX 0 Ethernet_802.3

Protocol TCP/IP
ip_address	192.179.109.29	-PC's IP address
ip_router	0.0.0.0	-If you have a router
ip_netmask	255.255.255.0
tcp_sockets	8
udp_sockets	8
raw_sockets	1

SYSTEM.INI

[386Enh]
device=vtcpip.386

Novell Lan WorkPlace 4.12 TCPIP and Novell Netware IPX

To configure Novell Lan WorkPlace 4.1.2 TCPIP and Novell Netware IPX as a dual stack, proceed as follows:

1.	Novell Lan WorkPlace uses the ODI standard for network communications.  As a result, the IPX.COM file created when running WSGEN with Novell Netware is no longer used.  Replace this file with IPXODI.COM found on the WSGEN disk in the DOSODI directory.

2.	Change your AUTOEXEC.BAT to reflect these changes as follows:

SET MVWDIR=c:\MVW

PATH=%MVWDIR%;%PATH%

LSL

3c503

IPXODI

NETX

f:

LOGIN

TCPIP

SET EXCELAN=c:\MVW

3.	Your NET.CFG file should contain the following and be placed in your root directory.  In the Link Driver section, add this following line:

PROTOCOL IPX protocol_id frame_type

The Protocol IPX line has the following format:

The settings are shown below and on page 27.

| Frame_Type | Protocol_id | Frame_Type | Protocol_id |
| --- | --- | --- | --- |
| Ethernet_802.3 | 0 | TOKEN-RING_SNAP | 8137 |
| Ethernet_802.2 | E0 | IBM_PCN2_802.2 | E0 |
| Ethernet_II | 8137 | IBM_PCN2_SNAP | 8137 |
| Ethernet_SNAP | 8137 | NOVELL_RX-NET | FA |
| TOKEN-RING | E0 | FDD1802.2 | E0 |
| FDDI_SNAP | 8137 |  |  |

Third Party network cards

If you are having difficulty installing the TCP/IP kernel with a Third Party network card there are later versions of the LSL.COM and the ODINSUP.COM programs available from the Novell Bulletin Board System (BBS) that may solve your problem.  You can access the Novell BBS using ftp to the system ftp novell.com using the account name ftp.  The update can be found in the directory sjf-lwp/odi and the file will be called dosupN.zip, where N is the release number.

Thick Ethernet - transceivers

If you are using Thick Ethernet connections the 3c503 ODI card driver needs a parameter added when it is loaded.  Edit the NET.CFG to add the DIX parameter.  For example:

Link Driver 3c503

Connector DIX

NOTE: This is only required for network cards that are configured by software rather than jumpers.

IBM LAN support program

If you are using the IBM LAN support program for Token Ring, the following device drivers need to be loaded in the system CONFIG.SYS file:

DEVICE=DXMA0MOD.SYS

DEVICE=DXMC0MOD.SYS

IBM Lan Server

At this release it is not possible to use the TCP/IP kernel in conjunction with the IBM Lan Server.  If you selected Reconfigure during the installation process please ensure that you restore your original AUTOEXEC.BAT before rebooting your PC.

Microsoft Lan Manager

At this release it is not possible to use the TCP/IP kernel in conjunction with Microsoft Lan Manager 2.0 or later. If you selected Reconfigure during the installation process please ensure that you restore your original AUTOEXEC.BAT before rebooting your PC.

Expand Utility

This DOS command allows you to uncompress files that are found on the DeskTop installation disks which have not been installed, for example slip.ex_ and ppp.ex_.  Files with the extension .xx_ are compressed files that may be expanded, for example:

expand a:\slip.ex_ c:\mvw\tcp\slip.exe

Index

A

accm NET.CFG file setting (SLIP_PPP)  34, 36

accomp NET.CFG file setting (SLIP_PPP)  35, 37

Address

broadcast (NetBIOS)  28, 30

Internet format  7

IP (workstation)  25

IP router  25

memory offset for network board  19

port (workstation)  18

Alias  7

ARCnet example configuration  47

ARP protocol  3

authen NET.CFG file setting (SLIP_PPP)  35, 38

Authentication (SNMP)  45

B

baud NET.CFG file setting (SLIP_PPP)  35, 38

Bind NET.CFG Protocol TCPIP setting  24, 32

B-node NetBIOS  3

BOOTP system  27

bypassing  26

use with SLIP_PPP  33

Broadcast address (NetBIOS)  28, 29

Buffers NET.CFG Link Support setting  23

C

Channel

DMA  16, 19

network board  18, 19, 20

COM port

specifying for SLIP_PPP  40

Windows access over SLIP_PPP  34

Command

DOS notation  2

LWPCON  56

syntax  1

typographic conventions  2

UNIX notation  2

Community names (SNMP)  45

Configuration information (LWPCON)  56, 59

Configuring

application services  6

multiple network support  31

NET.CFG file  14

NET.CFG syntax and notation  1

NetBIOS  28

ODINSUP 43

RARPD  27

SLIP_PPP  33

SNMP  43

Connections

closing TCP  61

listing current  61

Connector DIX NET.CFG Link Driver setting  17, 21

controlCommunity NET.CFG file setting (SNMP)  45

Conventions  1

counter NET.CFG file setting (SLIP_PPP)  35, 38

D

dial NET.CFG file setting (SLIP_PPP)  35, 39

DIALUP utility  55

DIALUP.TXT file format  55

entering modem commands (Ctrl-L)  56

DIALUP.TXT file  55

direct NET.CFG file setting (SLIP_PPP)  35, 40

DMA  19

channel  16, 19

DMA NET.CFG Link Driver setting  16, 19

DNS information (LWPCON)  65

Domain  8

Domain Name System (DNS) 8, 9

name server  6, 8

name server configuration  9

UDP socket requirements  26

DOS

command notation  2

filename notation  2

E

Ethernet

concurrent TCP/IP and IPX configuration  53

thick/thin connection (3C503 network board)  17

ETHERS file (RARPD)  14

F

File

ETHERS  14

HOSTS  6. 7

NET.CFG  14

NETWORKS  13

notation  2

PROTOCOL  11

RESOLV.CFG  9

SERVICES  11

UNIX notation  2

File transfer protocols for SLIP_PPP connections  33

Frame format

NET.CFG file setting  17

network types that support each available  21

Frame NET.CFG file setting (SLIP_PPP)  34 36

Frame NET.CFG Link Driver setting  16, 21

H

Hostname

defined  7

resolution flowchart  9

HOSTS file  6, 9, 10

alias entry  7

comments  8

creating  8

hostname entry  8

Internet address entry  8

RARPD server and  14

sample  8

I

ICMP protocol  3

Information

configuration  59

DNS  65

LWPCON utility  56

remote host services  63

route to host (LWPCON)  62

int NET.CFG file setting (SLIP_PPP)  35, 40

Int NET.CFG Link Driver setting  16, 18

Internet address format  7

Internet protocol (IP)  3

Interrupt  16, 18

IP address

multiple network connections  31

SLIP_PPP  40

specifying local workstation’s  25

IP protocol  3

raw connections  27

IP router, specifying default address  25

ip_address NET.CFG Protocol TCPIP setting  25

ip_netmask NET.CFG Protocol TCPIP setting  26

ip_router NET.CFG Protocol TCPIP setting  25

ipaddr NET.CFG file setting (SLIP_PPP)  35, 40

IPX protocol

TCP/IP used concurrently with  53

K

Keystroke notation  2

L

Link Driver section (NET.CFG file)  14, 15

Link Support section (NET.CFG file)  14, 23

listen NET.CFG file setting (SLIP_PPP)  35, 41

LSL.COM driver  4, 23, 31

NET.CFG section  14

LWPCON  56

control keys  58

displays and menus  58

DNS information  65

quitting  60

route to remote host  62

services on remote host  63

starting  56

status of remote host  65

tasks, typical  56

M

magic NET.CFG file setting (SLIP_PPP)  35, 41

Mem NET.CFG Link Driver setting  17, 20

Memory

offset for network board  17, 19

MemPool NET.CFG Link Support setting  23

Micro Channel Architecture network boards  17

Modem connection (SLIP_PPP)  40

monitorCommunity NET.CFG file setting (SNMP)  45

mru NET.CFG file setting (SLIP_PPP)  35, 41

N

Name server (DNS)  8

nb_adapter NET.CFG Protocol TCPIP setting  28, 29

nb_brdcast NET.CFG Protocol TCPIP setting  28, 29

nb_commands NET.CFG Protocol TCPIP setting  28, 30

nb_domain NET.CFG Protocol TCPIP setting  28, 30

nb_sessions NET.CFG Protocol TCPIP setting  28, 30

NET.CFG file

accm setting (SLIP_PPP)  34, 37

accomp setting (SLIP_PPP)  35, 37

authen setting (SLIP_PPP)  35, 38

baud setting (SLIP_PPP)  35, 38

Bind setting  32

comments  15

controlCommunity setting (SNMP)  45

counter setting (SLIP_PPP)  35, 38

customizing  15

dial setting (SLIP_PPP)  35, 39

direct setting (SLIP_PPP)  35, 40

editing procedure  15

Frame setting (SLIP_PPP)  34, 36

int setting (SLIP_PPP)  35, 40

ipaddr setting (SLIP_PPP)  35, 40

Link Driver section  15, 16

Link Support section  19, 28

listen setting (SLIP_PPP)  35, 41

magic setting (SLIP_PPP)  35, 41

monitorCommunity setting (SNMP)  45

mru setting (SLIP_PPP)  35, 41

multiple network connection example  52

nb_adapter setting  29

nb_brdcast setting  29

nb_commands setting  30

nb_domain setting  30

nb_sessions setting  30

no_bootp setting  26

open setting (SLIP_PPP)  36, 42

pcomp setting (SLIP_PPP)  35

port setting (SLIP_PPP)  36, 42

Protocol SNMP section  44

Protocol TCPIP section  14, 24

rebooting to implement changes  15

SLIP_PPP configuration example  47, 48

SLIP_PPP settings  34

SNMP agent example  50

SNMP agent settings  44

syntax  15

syntax and notation  1

sysContact setting (SNMP)  44

sysLocation setting (SNMP)  44

sysName setting (SNMP)  44

tcpipcomp setting (SLIP_PPP)  36, 42

NetBIOS  4, 28

asynchronous commands  28, 30

broadcast address  28, 29

concurrent sessions  28, 30

logical domain  28, 30

remote broadcast networks  29

specifying board to use  29

UDP socket requirements  26

NetWare

NET.CFG file use  14

Network

boards supported by each ODI driver  17

connections, multiple  31

information, using LWPCON  56

MCA bus boards  17

OSI reference model  3

route to host (LWPCON)  62

slot location of board  20

thick/thin Ethernet connection (3C503 board)  17

types supported  4

NETWORKS file  13

no_bootp NET.CFG Protocol TCPIP setting  26

Notation  1

O

ODI driver

configuring multiple network connection support  31

naming in NET.CFG  14, 16

NET.CFG file section  14

network boards supported by each  17

types available  4

ODINSUP  43

open NET.CFG file setting (SLIP_PPP)  36, 42

OSI reference model  3

Overview screen (LWPCON)  60

P

PATH TCP_CFG NET.CFG Protocol TCPIP setting  26

PING utility  27

Port

NET.CFG Link Driver setting 16, 18

workstation  15, 17

port NET.CFG file setting (SLIP_PPP)  36, 42

PPP protocol

example NET.CFG file  49

frame compression  35

NET.CFG file settings  34

reference documents  33

PROTOCOL file  10

content and format  11

Protocol NET.CFG Link Driver setting  16, 22

Protocol TCPIP section (NET.CFG file)  14, 24

Protocols

concurrent TCP/IP and IPX  53

used by the TCP/IP Transport  3

R

RARP protocol  3

RARP server  27

ETHERS file format  14

NET.CFG configuration  28

use with SLIP_PPP  33

raw_sockets NET.CFG Protocol TCPIP setting  27

Receive buffers (TCP)  23

Remote host

information (LWPCON)  56

name search  9

route to  62

services available (LWPCON)  65

status (LWPCON)  62

RemoteName NET.CFG Protocol RFCNBIOS setting  29

RESOLV.CFG file  6

domain entry  9

example  9

nameserver entry  9, 10

Route to remote host (LWPCON)  62

Router  25, 31

S

SERVICES file  10

SLIP protocol

example NET.CFG file  47

NET.CFG file settings  35

reference documents  33

sample NET.CFG file  49

SLIP_PPP ODI driver

active connection  42

address/control header compression  37, 38

asynchronous control character map  37

authentication  38

baud rate  38

COM port to use  40

configuring  33

DIALUP utility  55

DIALUP.TXT file format  55

direct link  40

example NET.CFG file  48

frame type selection  36

IP address  40

loading  34

looped-back links  41

NET.CFG file settings  34

obtaining configuration settings from the remote site  36

passive (modem) connection  41

phone number to dial  39

port address  42

timeouts and retries  38

transmit and receive units  41

Van Jacobson header compression  36 42

Slot NET.CFG Link Driver setting  15 20

SNMP agent program

authentication  45

community names  45

contact name  45

domain name  50

example NET.CFG file  51

NET.CFG file settings  50

read/write community name  45

read-only community name  45

system physical location  45

Socket  3

allocating  27

raw  27

TCP  26, 27

UDP  26, 27

Subnetwork

mask  27

Subnetwork mask  27

sysContact NET.CFG file setting (SNMP)  44

sysLocation NET.CFG file setting (SNMP)  44

sysName NET.CFG file setting (SNMP)  44

T

TCP protocol  3

concurrent connections  26

receive buffers  23

send data buffering  23

TCP/IP

configuration information  59

information, using LWPCON  56

IPX used concurrently with  53

NET.CFG configuration  14

send buffer pool  23

Transport software  3

tcp_sockets NET.CFG Protocol TCPIP setting  26

TCPIP.EXE program

sign-on display, multiple network connections  52

specifying NET.CFG file location  24

tcpipcomp NET.CFG file setting (SLIP_PPP)  36, 42

Token Ring

concurrent TCP/IP and IPX configuration  53

example configuration  46

frame types supported  21

TOKEN.COM driver, example configuration  46

U

UDP protocol  3

concurrent connections  26

udp_sockets NET.CFG Protocol TCPIP setting  26

UNIX

command notation  2

filename notation  2

V

Van Jacobson header compression (SLIP_PPP)  42

W

Windows

exclusive COM port access for SLIP_PPP  36

Workstation

DMA channel  19

interrupt  18

memory for network board  20

slot location of network board  20