DayStar Digital LT200 MC (MicroChannel) LocalTalk Card Installation Notes

=========================================================================

1. IMPORTANT NOTES

---------------

To use this MLID, DSTARLT.COM and DTALK.EXE are the software drivers you

need to run PhoneNET PC in your system.

DSTARLT.COM is the driver to be used to initialize the card.

DTALK.EXE is the software to be used to configure the interrupt and

port settings.  If you want to change the default settings, you MUST

run this software before loading the driver DSTARLT.COM, otherwise, the

card will not work correctly.

2. GENERAL INFORMATION

-------------------

The allowable interrupt levels for this card are :

3, 5, 6, 7

NOTE:

You MUST use DTALK.EXE to configure the card if the interrupt is set

to 5.  DO NOT use DTALK.EXE to configure other interrupts.

Here is how DTALK.EXE is used to configure the card with interrupt 5

DTALK /CardInt=5

If you change the default address of the card, you MUST use DTALK.EXE to

configure to card as follows:

DTALK /Address=240

The allowable Address are 240, 340, and 300.

Note that in all cases, the actual parameter settings you specify in

DTALK.EXE and the parameters specified in the INSTALL program must match

or the MLID will not work.

For more details about configuring and installing this card, please

refer to the "LT200 Connection Interface Board Installation/User's

Manual" that comes with your adapter.