3Com EtherLink III (3C529) Adapter Installation Notes

=====================================================

1. IMPORTANT NOTES

---------------

If you use this MLID, 3C509.COM is the only driver you need.  You

should NOT install or load any other programs supplied by 3Com for your

EtherLink III adapter.  (You may have to delete the line in your

CONFIG.SYS that loads 3Com's ETH509.SYS ("DEVICE=ETH509.SYS") or its

equivalent).  This MLID will not work if another driver for the same

adapter is loaded.

2. GENERAL INFORMATION

-------------------

The EtherLink III adapter will work on either Thin Ethernet cabling

(connected via an BNC connector) or thick Ethernet cabling (connected

via a DIX connector).  Note you must set which connector you wish to

use by booting the reference disk and configuring the card to use the

appropriate connector.

For more information about installing and configuring your adapter,

please refer to the "3Com EtherLink III Adapter Installation Guide"

that comes with your adapter.