IBM Token-Ring Network 16/4 Adapter II Installation Notes

=========================================================

1. IMPORTANT NOTES

---------------

It is assumed that you will be running in Windows Enhanced mode.  By

default, the PhoneNET PC Installer will add the line "ENHANCED YES" in

the NET.CFG file.  If you are running Windows on a 80286 processor or

in Windows Standard mode on a 80386 or higher processor, change the

NET.CFG line to read "ENHANCED NO".

If you are using NETBIOS on a token-ring network, you must load

TOKENBM.COM before running any application that uses the NETBIOS

interface.  (The NETBIOS driver may be loaded by CONFIG.SYS, but the

adapter will not be initialized until an application sends a command to

the NETBIOS interface.  The TOKENBM MLID must initialize the adapter.)

Most adapter open parameters are set to the default values.  If your

NETBIOS application runs before TOKENBM.COM, the open parameters it

sets will prevent TOKENBM.COM from running.

2. GENERAL INFORMATION

-------------------

There is one configuration menu for the I/O port configuration you set

in the dip switches on the card (#1-#3).  Be sure that the port

address you have selected on the card, is the one you supply the

installation program for PhoneNET PC.

For instructions on installing the IBM Token-Ring adapter II, consult

your "Guide to Operations" or other documentation that comes with your

Token-Ring adapter or Local Area Network Support Program.

3. IBM LICENSE AGREEMENT FOR PRODUCTIVITY AIDS

-------------------------------------------

IF YOU DOWNLOAD OR USE THIS PROGRAM YOU AGREE TO THESE TERMS.

International Business Machines Corporation grants you a license to use

the Program only in the country where you acquired it.  The Program is

copyrighted and licensed (not sold).  We do not transfer title to the

Program to you.  You obtain no rights other than those granted you

under this license.

Under this license, you may:

1. use the Program on one or more machines at a time;

2. make copies of the Program for use or backup purposes within your

Enterprise;

3. modify the Program and merge it into another program; and

4. make copies of the original file you downloaded and distribute it,

provided that you transfer a copy of this license to the other

party.  The other party agrees to these terms by its first use of

the Program.

You must reproduce the copyright notice and any other legend of

ownership on each copy or partial copy, of the Program.

You may NOT:

1. distribute the Program over electronic networks (except internally),

or by means of electronic "bulletin boards";

2. sublicense, rent, lease, or assign the Program; and

3. reverse assemble, reverse compile, or otherwise translate the

Program.

We do not warrant that the Program is free from claims by a third party

of copyright, patent, trademark, trade secret, or any other

intellectual property infringement.

Under no circumstances are we liable for any of the following:

1. third-party claims against you for losses or damages;

2. loss of, or damage to, your records or data; or

3. economic consequential damages (including lost profits or savings)

or incidental damages, even if we are informed of their possibility.

Some jurisdictions do not allow these limitations or exclusions, so

they may not apply to you.

We do not warrant uninterrupted or error free operation of the Program.

We have no obligation to provide service, defect correction, or any

maintenance for the Program.  We have no obligation to supply any

Program updates or enhancements to you even if such are or later become

available.

IF YOU DOWNLOAD OR USE THIS PROGRAM YOU AGREE TO THESE TERMS.

THERE ARE NO WARRANTIES, EXPRESS OR IMPLIED, INCLUDING THE IMPLIED

WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE.

Some jurisdictions do not allow the exclusion of implied warranties, so

the above exclusion may not apply to you.

You may terminate this license at any time.  We may terminate this

license if you fail to comply with any of its terms.  In either event,

you must destroy all your copies of the Program.

You are responsible for the payment of any taxes resulting from this

license.

You may not sell, transfer, assign, or subcontract any of your rights

or obligations under this license.  Any attempt to do so is void.

Neither of us may bring a legal action more than two years after the

cause of action arose.

If you acquired the Program in the United States, this license is

governed by the laws of the state of New York.  If you acquired the

Program in Canada, this license is governed by the laws of the Province

of Ontario. Otherwise, this license is governed by the laws of the

country in which you acquired the Program.