Hayes Smartmodem 300 and 1200

The Hayes Microcomputer Products Smartmodem 300 and 1200 can be

used with Softerm by selecting Hayes Smartmodem as the modem

dialer type.

The originate baud rate when using the Smartmodem 1200 is

determined from the current Softerm communications Speed parameter

which can select either 300 or 1200 baud.  The configuration

switches to establish the operating parameters for the standalone

Smartmodem 300 and 1200 should be set as follows when operating

with Softerm:

Switch	  Setting	Function

1	  Up		Normal DTR operation

2	  User Option	Word/Digit result codes

3	  User Option	Quiet/Result codes sent

4	  User Option	Echo/No Echo in command state

5	  User Option	Auto-Answer Enabled/Disabled

6	  Up		Normal DCD operation

7	  User Option	RJ11/(RJ12 or RJ13) jack

8	  User Option	Disable/Enable command recognition

Hayes Smartmodem 1200B

The Hayes Microcomputer Products Smartmodem 1200B internal board

modem can be used with Softerm by selecting Hayes Smartmodem as

the modem dialer type.  Refer to the description of Hayes

Smartmodem 300 and 1200 external modems for additional

information.

U.S.  Robotics Password 300/1200

The U.S.  Robotics Password modem can be used with Softerm by

selecting Hayes Smartmodem as the modem dialer type.  For proper

operation, set all configuration switches to the Off position.