Auto-Dial Modems

Anchor Automation SIGNALMAN MARK XII

The Anchor Automation SIGNALMAN MARK XII auto-dial modem can be

used with Softerm by selecting SIGNALMAN MARK XII as the modem

dialer type.

The MARK XII does not allow local control of the DTR signal.  This

means that a call cannot be disconnected by dropping DTR by

selecting local mode on the Softerm status line.  To hangup a

connection, use the Phone Book Utilities Hangup option or enter

the escape to command mode sequence +++ followed by a hangup

command AT H in the terminal mode.

The use of the + character in a Softerm phone number will cause a

blind 2 second wait.  The @ character in a phone number is not

supported.  The use of the @ character in a phone number will

cause a blind 4 second wait.

AT&T Model 4000

The AT&T Model 4000 autodial modem can be used with Softerm by

selecting AT&T Model 4000 as the modem dialer type.  When starting

Softerm, the modem must be in a newly powered on state or the

serial I/O parameters must match those of the current state of the

modem.

Cermetek INFO-MATE 212A

The Cermetek INFO-MATE 212A auto-dial modem can be used with

Softerm by selecting INFO-MATE 212A as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch	  Setting	Function

1	  On		Asynchronous Operation

2	  Off		7 or 8 Bit Data with 2 Stop Bits

3	  Off		Normal CTS/DSR/DCD Operation

4	  On		Host Controlled DTR

5	  User Option	Battery Backup

6	  User Option	Exclusion Key Telephone Control

When using this modem on systems with serial interfaces which will

not transmit without the CTS signal, such as the Tandy 2000, the

CTS signal from the modem must be disconnected and CTS provided to

the serial interface through a local loopback of the RTS signal.

The use of the + character in a Softerm phone number will cause a

blind 2 second wait.  The @ character in a phone number is not

supported.

Concord Data Systems CDS 224

The Concord Data Systems CDS 224 autodial 1200/2400 baud modem can

be used with Softerm by selecting CDS 224 Autodial as the modem

dialer type.  This modem is normally shipped from the factory with

all internal option switches set to OFF.  Set BANK SW8 to ON to

provide a true DCD (carrier) signal.

Modems                       - 1 -             Modems

Datec 212 Auto-Dial Modem

The Datec 212 Auto-Dial modem can be used with Softerm by

selecting Datec 212 as the modem dialer type.

Other Datec modems with auto-call units (ACU) may be compatible

with the Softerm driver.  For proper operation, ACU firmware

Revision 3.5 or later is required and the configuration switches

of the ACU must be set as follows:

Switch		Setting

1		On

2-8		Off

Hayes Smartmodem 300 and 1200

The Hayes Microcomputer Products Smartmodem 300 and 1200 can be

used with Softerm by selecting Hayes Smartmodem as the modem

dialer type.

The originate baud rate when using the Smartmodem 1200 is

determined from the current Softerm communications Speed parameter

which can select either 300 or 1200 baud.  The configuration

switches to establish the operating parameters for the standalone

Smartmodem 300 and 1200 should be set as follows when operating

with Softerm:

Switch	  Setting	Function

1	  Up		Normal DTR operation

2	  User Option	Word/Digit result codes

3	  User Option	Quiet/Result codes sent

4	  User Option	Echo/No Echo in command state

5	  User Option	Auto-Answer Enabled/Disabled

6	  Up		Normal DCD operation

7	  User Option	RJ11/(RJ12 or RJ13) jack

8	  User Option	Disable/Enable command recognition

Hayes Smartmodem 2400

The Hayes Microcomputer Products Smartmodem 2400 can be used with

Softerm by selecting Hayes Smartmodem2400 as the modem dialer

type.  Softerm automatically initializes all internal soft

switches to the appropriate values for operation with this modem.

Intel SatisFAXtion

The Intel SatisFAXtion modem can be used with Softerm by selecting

Intel SatisFAXtion as the modem dialer type.  Softerm automatically

initializes all internal soft switches to the appropriate values

for operation with this modem.  In order to use at 14,400 you need

to set the baud rate to 19,200.

Multi-Tech MT212AH Auto-Dial Modem

The Multi-Tech MT212AH auto-dial modem can be used with Softerm by

selecting Multi-Tech MT212AH as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch-3  Setting	Function

1	   User Option	Enable/Disable auto-answer

2	   Open		Normal DTR Operation

3	   Open		Normal DCD Operation

Modems                       - 2 -             Modems

Switch-4  Setting	Function

1	   User Option	Word/Digit result codes

2	   User Option	Enable/Suppress command responses

3	   User Option	Enable/Disable command echo

4	   User Option	Enable/Disable command mode

Multi-Tech MT212C Auto-Dial Modem

The Multi-Tech MT212C auto-dial modem can be used with Softerm by

selecting Multi-Tech MT212C as the modem dialer type.

The MT212C provides 1200 baud operation and pulse dialing only.

The ACU in this modem allows a maximum phone number length of 16

characters.

NEC N212BR Auto-Dial Auto-Logon Modem

The NEC N212BR Auto-Dial Auto-Logon modem can be used with Softerm

by selecting NEC N212BR as the modem dialer type.

Novation SMART-CAT Modem

The Novation 103 and 103/212 SMART-CAT modems can be used with

Softerm by selecting Novation SMART-CAT as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch	  Setting	Function

1	  On		Set command character to %

2	  User Option	Verbose/terse response codes

3	  User Option	High/low data rate

4	  User Option	Enable/Disable auto answer

5	  On		Normal CTS/DSR/DCD operation

Systems which will not transmit without the CTS signal from the

modem may require a special cable with a local loopback of the RTS

to CTS signal.  The use of the + character in a Softerm phone

number will cause a blind 5 second wait.

Prentice POPCOM

The Prentice POPCOM 300/1200 autodial modem can be used with

Softerm by selecting Prentice POPCOM as the modem dialer type.

PROMETHEUS ProModem 1200

The PROMETHEUS ProModem 1200 modem can be used with Softerm by

selecting ProModem as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Modems                       - 3 -             Modems

Switch	  Setting	  Function

1	  On		  Enable Command Recognition

2	  User Option	  RJ11/(RJ12 or RJ13) jack

3	  User Option	  Initialized by Softerm

4	  User Option	  Enable/Disable auto answer

5	  User Option	  Echo/No Echo in command state

6	  User Option	  Initialized by Softerm

7	  User Option	  Verbal/Numeric result codes

8	  Off		  Normal DTR operation

9	  Off		  Prevent modem automatically redialing

10	  Off		  Normal DCD operation

The ProModem includes true wait for dial tone capability.

Racal-Vadic VA212 Auto-Dial Modem

The Racal-Vadic VA212 auto-dial modem can be used with Softerm by

selecting Racal-Vadic VA212 as the modem dialer type.

The Softerm driver expects the standard factory option settings.

The modem will automatically select pulse or tone dialing.  If a T

or P is the first character of a Softerm phone number, the

appropriate command option will be issued to override the

automatic selection and use the type of dialing indicated.  The

VA212 has no separate delay command and the only delay available

is wait for dial tone.  This can be changed to a 5-second delay by

changing option 23 in the modem to enable blind dialing.

Radio Shack TRS-80 Modem II

The Radio Shack TRS-80 Modem II can be used with Softerm by

selecting TRS-80 MODEM II as the modem dialer type.

The FORCE DTR switch on the rear of the modem should be set to

OFF.  The front panel AUTO/MAN should be set to AUTO for automatic

dialing.  The ORIG/ANS switch can be set to either position.

The use of the @ character in a Softerm phone number will cause a

blind 4 second wait.

Radio Shack TRS-80 DC-1200 Modem

The Radio Shack TRS-80 DC-1200 Modem with auto-dial module can be

used with Softerm by selecting TRS-80 DC-1200 as the modem dialer

type.

The internal option switches should be set as follows to operate

with Softerm:

Switch	  Setting	  Function

1	  Closed	  Enable serial data out

2	  User Option	  Not used

3	  User Option	  Enable/Disable character echo

4	  User Option	  Status responses/Modem II compatible

5	  User Option	  Not used

6	  User Option	  7 bits, even parity

8 bits, no parity

7	  User Option	  Disable/Enable A-A1 control

8	  Open		  Disable forced local carrier

Modems                       - 4 -             Modems

When using this modem on systems with serial interfaces which will

not transmit without the CTS signal, such as the Tandy 2000, the

CTS signal from the modem must be disconnected and CTS provided to

the serial interface through a local loopback of the RTS signal.

The use of the @ character in a Softerm phone number will cause a

blind 4 second wait.

Radio Shack TRS-80 DC-2212 Modem

The Radio Shack TRS-80 DC-2212 1200 Baud Modem can be used with

Softerm by selecting TRS-80 DC-2212 as the modem dialer type.  For

proper operation, the Auto-Answer switch on the rear of the modem

must be set to IN.

Rixon R212A Intelligent Modem

The Rixon R212A Intelligent Modem can be used with Softerm by

selecting Rixon R212A as the modem dialer type.  The modem also

includes a Hayes Smartmodem compatible mode which can be used with

the Hayes Smartmodem dialer type.

The Softerm driver is designed to work with the default Rixon

options in the Rixon mode.  Since carrier is constantly present in

the Rixon mode, PAUSE or XMIT:WAIT commands must be used when

using a DIAL command in a macro.  However, the Rixon modem option

"CTS ON" can be set to "N", and a modified cable can be used to

allow Softerm to wait for carrier and detect disconnects properly.

To modify the cable, disconnect CTS (Pin 5) from the modem and

jumper RTS (Pin 4) to CTS so that characters can still be

transmitted to the modem.

Tandy DCM-5 Modem

The Tandy DCM-5 external modem can be used with Softerm by

selecting TANDY DCM-5 as the modem dialer type.  The Auto-Answer

switch (on rear of modem) must be set to IN to provide a true DCD

signal.

When using this modem on systems with serial interfaces which will

not transmit without the CTS signal, such as the Tandy 2000, the

CTS signal from the modem must be disconnected and CTS provided to

the serial interface through a local loopback of the RTS signal.

The use of an @ or + character in a Softerm phone number will

cause a blind 3 second wait.

Universal Data Systems 212A/D Auto-Dial Modem

The Universal Data Systems 212A/D modem can be used with Softerm

by selecting UDS 212A/D as the modem dialer type.

U.S.  Robotics Auto-Dial 212A

The U.S.  Robotics Auto-Dial 212A modem can be used with Softerm

by selecting USR AD 212A as the modem dialer type.  For proper

operation, the configuration switches must be set as follows:

Modems                       - 5 -             Modems

Switch		Setting

RNG		Use as specified in manual

8HI		On

BAL		Use as specified in manual

DTR		Off

Pin 4 may have to be looped back to Pin 5 on the serial I/O

interface, depending on the revision of the modem.

U.S.  Robotics Password 300/1200

The U.S.  Robotics Password modem can be used with Softerm by

selecting Hayes Smartmodem as the modem dialer type.  For proper

operation, set all configuration switches to the Off position.

Ven-Tel MD212 Plus Auto-Dial Modem

The Ven-Tel MD212 Plus auto-dial modem can be used with Softerm by

selecting Ven-Tel MD212 Plus as the modem dialer type.

The MD212 Plus does not allow software control of the type of

dialing used.  Switch H1 on the standalone MD212 Plus if UP will

cause pulse dialing and if DOWN will cause tone dialing.  On the

rackmount version, Switch S7-1 will cause pulse dialing if ON and

tone dialing if OFF.

Zenith EazyPC

Select this modem driver if you are using the Modem/Memory/

Serial Accessory with the Zenith eazy-pc computer.  This driver

specifically includes the command ATC0 in the initialization

string to disable the modem forcing Carrier Detection.

ZyXEL U-1496

The ZyXEL U-1496 modem can be used with Softerm by selecting

ZyXEL U-1496 as the modem dialer type.  Softerm automatically

initializes all internal soft switches to the appropriate values

for operation with this modem.  In order to use at 14,400 you need

to set the baud rate to 19,200.

PC-Compatible Board Modems

Softerm includes support for a selection of PC-compatible internal

board modems which provide automatic dialing capability.  This

type of modem requires a PC expansion slot and functions as a

combination modem and COM adaptor.  The automatic dialing

capability can be accessed from the keyboard using special

firmware built-in to the modem board, or by using the dialer

drivers included with Softerm.

Bizcomp PC:IntelliModem

The Bizcomp PC:IntelliModem modem card can be used with Softerm by

selecting PC:IntelliModem as the modem dialer type.

Although the PC:IntelliModem allows connections to be made at 110,

300, and 1200 baud, the board always communicates at 1200 baud

with the software.  The Softerm Speed communications parameter in

terminal setup should be set to 1200 baud for proper operation

when using the modem manually.  Phone numbers dialed using the

Softerm phone book will force the serial interface speed to 1200

but will issue the proper command for dialing at 110, 300, or 1200

baud.

Modems                       - 6 -             Modems

The use of the + character in a Softerm phone number will cause a

blind 2 second wait.

Hayes Smartmodem 1200B

The Hayes Microcomputer Products Smartmodem 1200B internal board

modem can be used with Softerm by selecting Hayes Smartmodem as

the modem dialer type.  Refer to the description of Hayes

Smartmodem 300 and 1200 external modems for additional

information.

Qubie PC 212A/1200

The Qubie PC 212A/1200 modem card can be used with Softerm by

selecting Qubie PC 212A as the modem dialer type.

The internal option switches should be set as follows to operate

with Softerm:

Switch	  Setting	  Function

1	  Open		  Normal DTR operation

2	  User Option	  Word/Digit result codes

3	  User Option	  Quiet/Result codes sent

4	  User Option	  Echo/No Echo in command state

5	  User Option	  Auto-Answer Enabled/Disabled

6	  Open		  Normal DCD operation

7	  User Option	  RJ11/(RJ12 or RJ13) jack

8	  User Option	  Disable/Enable 1200 baud

9	  Closed	  External serial port off

Tandy 300 Internal Modem

The Tandy 300 Internal Modem card can be used with Softerm by

selecting TANDY 300 INTERNAL as the modem dialer type.  When using

this modem, if a connection breaks while in the Communications

Agent Terminal Mode, the modem will not auto-answer any incoming

calls until a Phone Book Utilities HANGUP command or a DIAL

command is issued.

Using an @ or + character in a phone number will cause a blind 3

second wait.

Tandy 1200 Internal Modem

The Tandy 1200 Internal Modem card can be used with Softerm by

selecting TANDY 1200 INTERNAL as the modem dialer type.  For

proper operation, the modem switches should be set as follows:

Switch	  Setting	Function

1	  OFF		Carrier detect active only when carrier

present

2	  OFF		Modem will auto-answer telephone

3	  ON/OFF	Selects COM1 or COM2

4	  Not Used

Texas Instruments Internal Modem

The Texas Instruments Internal Modem card can be used with Softerm

by selecting TI INTERNAL as the modem dialer type.

Modems                       - 7 -             Modems