Qubie PC 212A/1200

The Qubie PC 212A/1200 modem card can be used with Softerm by

selecting Qubie PC 212A as the modem dialer type.

The internal option switches should be set as follows to operate

with Softerm:

Switch	  Setting	  Function

1	  Open		  Normal DTR operation

2	  User Option	  Word/Digit result codes

3	  User Option	  Quiet/Result codes sent

4	  User Option	  Echo/No Echo in command state

5	  User Option	  Auto-Answer Enabled/Disabled

6	  Open		  Normal DCD operation

7	  User Option	  RJ11/(RJ12 or RJ13) jack

8	  User Option	  Disable/Enable 1200 baud

9	  Closed	  External serial port off