Novation SMART-CAT Modem

The Novation 103 and 103/212 SMART-CAT modems can be used with

Softerm by selecting Novation SMART-CAT as the modem dialer type.

The internal option switches should be set as follows for

operation with Softerm:

Switch	  Setting	Function

1	  On		Set command character to %

2	  User Option	Verbose/terse response codes

3	  User Option	High/low data rate

4	  User Option	Enable/Disable auto answer

5	  On		Normal CTS/DSR/DCD operation

Systems which will not transmit without the CTS signal from the

modem may require a special cable with a local loopback of the RTS

to CTS signal.  The use of the + character in a Softerm phone

number will cause a blind 5 second wait.