Radio Shack TRS-80 DC-1200 Modem

The Radio Shack TRS-80 DC-1200 Modem with auto-dial module can be

used with Softerm by selecting TRS-80 DC-1200 as the modem dialer

type.

The internal option switches should be set as follows to operate

with Softerm:

Switch	  Setting	  Function

1	  Closed	  Enable serial data out

2	  User Option	  Not used

3	  User Option	  Enable/Disable character echo

4	  User Option	  Status responses/Modem II compatible

5	  User Option	  Not used

6	  User Option	  7 bits, even parity

8 bits, no parity

7	  User Option	  Disable/Enable A-A1 control

8	  Open		  Disable forced local carrier

When using this modem on systems with serial interfaces which will

not transmit without the CTS signal, the CTS signal from the modem

must be disconnected and CTS provided to the serial interface

through a local loopback of the RTS signal.

The use of the @ character in a Softerm phone number will cause a

blind 4 second wait.