Tandy DCM-5 Modem

The Tandy DCM-5 external modem can be used with Softerm by

selecting TANDY DCM-5 as the modem dialer type.  The Auto-Answer

switch (on rear of modem) must be set to IN to provide a true DCD

signal.

When using this modem on systems with serial interfaces which will

not transmit without the CTS signal, such as the Tandy 2000, the

CTS signal from the modem must be disconnected and CTS provided to

the serial interface through a local loopback of the RTS signal.

The use of an @ or + character in a Softerm phone number will

cause a blind 3 second wait.