NetBIOS Extended Communications File

XCM Name:                NETBIOS.XCM

Softerm Revision:        3.00.00 or above

Base System File:        IBMPC.SYS

The NetBIOS extended communications file provides support for

multiple asynchronous ports controlled by a NetBIOS compatible

communications server.

The INSTALL program provided with Softerm is normally used to build

a system file containing this XCM module.  Use the "Customize"

option to build the system file with the INSTALL program.  However,

one may be built manually using the following DOS command:

COPY IBMPC.SYS/B+NETBIOS.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the

command any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+NETBIOS.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+NETBIOS.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG

program. This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program

interogates the system file to determine which XCM module is

present, and provides the appropriate configuration options.

Define the LAN type, number of communications ports, and local

system name. The local system name must be unique on the network.

SOFTERM PC will append _COMx to the system name defined to provide

each communications port with a unique NetBIOS network name.

The lockup question would normally be answered as 'NO' but should

be set to 'YES' if SOFTERM lockups are experienced.  It is not

possible at this time to provide a list of environments where

SOFTERM may exhibit problems other than to note that they occur in

some non 100% IBM configurations.  This option, when selected,

forces SOFTERM to use different internal network access mechanisms.

NetBIOS Extended Communications File                       Page 2

The "Auto-Connect on Assign" option provides a mechanism by which

to have Softerm establish a connection as soon as the port is

assigned for use. If you answer this option as "Yes", Softerm will

attempt to establish a connection to the "Auto-Connect Call ID".

This call id is in one of the following formats:

NetBIOS Server:

- Servername

Auto-Connection is tried only once; if it fails, you must use the

Softerm Phone Book Utilities or the File Transfer "Dial" command to

establish a connection.

Any physical COM ports present on the PC will still be available

under their usual designations, provided that the number of network

ports plus the number of physical ports does not exceed 4.  If it

does, the physical ports will be deleted, starting at the highest

numbered port, until all of the network ports can be accessed.

After running NETCONFG, if you have not used the INSTALL program,

YOU MUST EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications

environment.

The NetBios extended communications file supports the use of

Softerm's Phone Book and auto-dialer.  You may enter the connection

information in the number field for a phone book entry, and Softerm

will establish the connection for you.  You may also terminate a

connection using the emulation status line command "Break

Connection", toggling to local mode, or using the Phone Book

"Hangup" command.

To establish a connection through the network, invoke the Softerm

dialing directory function from Softerm's Online mode of execution

by pressing Alt-2 followed by function key 2.  Select the "Dial

Number" option, and press return.  Press return again to position

the cursor at the number field.  This is where to enter the

information for establishing the connection.

The File Transfer MONITOR command is an example of a NetBIOS

compatible server that may be 'dialed' and accessed by any other

Softerm client on the Local Area Network that is configured with

the NETBIOS.XCM capability.

The call id for this environment is as follows:

- Servername_COMx

where:

Servername is the NetBIOS name defined using

NETCONFG for the SOFTERM executing the MONITOR mode

and x is the number of the Softerm communications

port that the MONITOR command is active on.