Sytek SyTermX Extended Communications File

XCM Name:              SYTEK.XCM

Softerm Revision:      3.00.00 or above

Base System File:      IBMPC.SYS

The SYTEK extended communications file provides access to one port via the

SyTermX interface.  This XCM uses the Interrupt 014H interface.

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  Use the "Customize" option to build the

system file with the INSTALL program.  However, one may be built manually using

the following DOS command:

COPY IBMPC.SYS/B+SYTEK.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+SYTEK.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+SYTEK.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.  In the case of the SYTEK.XCM module, the

only option is the COM port number to which communications for SyTermX

will be directed.  This is defined as the "Mapped to Port #" on the

configuration menu, and, for the last version of SyTermX tested, should be

set to 1.  However, if your SyTermX software allows it, the number may be

changed to 2, 3 or 4.  This defines the COM port number that will be used

to communicate with the Interrupt 014H software provided by Sytek.  As far

as Softerm PC is concerned, it will always know the SyTermX port as COM4.

This is the port name that you should pick when you wish to use the SyTermX

connection.  Any physical COM ports present on the PC will still be available

under their usual designations, provided that the number of network ports

plus the number of physical ports does not exceed 4.  If it does, the

physical ports will be deleted, starting at the highest numbered port, until

all of the network ports can be accessed.

After running NETCONFG, if you have not used the INSTALL program, YOU MUST

EXECUTE SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

The Sytek extended communications file does not support the use of

Softerm's Phone Book and auto-dialer.