Ungermann-Bass Net One Extended Communications File

XCM Name:                UB.XCM

Softerm Revision:        3.00.00 or above

Base System File:        IBMPC.SYS

The Ungermann-Bass Net One extended communications file provides access to up

to 3 ports via the UB NETCI interface.  This XCM uses the Interrupt 06BH

interface.

The INSTALL program provided with Softerm is normally used to build a system

file containing this XCM module.  However, one may be built manually using

the following DOS command:

COPY IBMPC.SYS/B+UB.XCM/B SOFTERM.SYS

If an extended text module is also desired, it may be added in the command

any place after the IBMPC.SYS entry and before the space:

COPY IBMPC.SYS/B+HERC.XTX/B+UB.XCM/B SOFTERM.SYS

or

COPY IBMPC.SYS/B+UB.XCM/B+HERC.XTX/B SOFTERM.SYS

After building the system file, it is configured using the NETCONFG program.

This program is invoked by the following command line:

NETCONFG

No parameters are required or expected.  The NETCONFG program interogates the

system file to determine which XCM module is present, and provides the

appropriate configuration options.  In the case of the UB.XCM module, the

only option is the number of network communications ports desired.  This may

be a number from one to three, inclusive, and defines the number of different

sessions Softerm may use via NETCI.  Each session will be available on a

different Softerm COM port, mapped in the following manner:

for 1 network COM port:

Softerm's COM4 is NETCI session 0

for 2 network COM ports:

Softerm's COM4 is NETCI session 1

Softerm's COM3 is NETCI session 0

for 3 network COM ports:

Softerm's COM4 is NETCI session 2

Softerm's COM3 is NETCI session 1

Softerm's COM2 is NETCI session 0

Any physical COM ports present on the PC will still be available under their

usual designations, provided that the number of network ports plus the number

of physical ports does not exceed 4.  If it does, the physical ports will be

deleted, starting at the highest numbered port, until all of the network ports

can be accessed.

Ungermann-Bass Net One Extended Communications File                    Page 2

After running NETCONFG, without using the INSTALL program, YOU MUST EXECUTE

SOFTERM WITH THE /N SWITCH:

SOFTERM/N

This forces Softerm to recognize the reconfigured communications environment.

The Ungermann-Bass extended communications file does not support the use of

Softerm's Phone Book and auto-dialer.