ADDS Regent 20

Introduction

The ADDS Regent 20 display terminal can be emulated using Softerm

by selecting ADDS Regent 20 for the terminal emulation when

generating a configuration in terminal setup.  Softerm supports

all the general capabilities of the Regent 20 including the full

128 character set and 80 X 24 display, conversational half or full

duplex transmission, all addressable cursor modes, auxiliary port

operation with a printer, and special terminal command sequences.

Features

The display format of the ADDS Regent 20 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Monitor Mode.  Softerm

supports all available keyboard functions and provides additional

keyboard functions for Cursor Up, Cursor Down, Cursor Forward,

Cursor Home, Erase to End of Line, Erase to End of Page, and

Monitor Mode On/Off.

Softerm supports all remote terminal commands and switch settings

with the following exceptions:

Switch selectable settings not supported by Softerm include

Audible Key Feedback and Character Display.  The switch setting

for Keyboard Lock/Unlock is not supported but Softerm will respond

to both Regent 20 and Consul 580 keyboard lock/unlock functions.

Softerm includes support for the Auxiliary Port if the terminal

configuration includes a printer.  If either a AUX port ON or

Transparent Print ON remote command is received, all subsequent

data is sent to the printer until a AUX port OFF command is

received.  If a Transparent Print OFF command is received, the

Auxiliary Port will be returned to the state it was in before the

Transparent Print ON command was received.  In the transparent

mode data is sent only to the printer and not displayed, otherwise

the received data is both displayed and printed.

ADDS Regent 20               - 1 -             Terminal Emulation

ADDS Regent 20 Keyboard Functions

The following table defines the standard keyboard functions for

the ADDS Regent 20 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Right Arrow		Cursor Forward

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt M			Monitor Mode On/Off

Backspace		BS or DEL

Shift Backspace 	DEL or BS

Home			Home Cursor

PgUp			Erase All

PgDn			Erase to End of Page

End			Erase to End of Line

ADDS Regent 20               - 2 -             Terminal Emulation