ADDS Regent 60

Introduction

The ADDS Regent 60 display terminal can be emulated using Softerm

by selecting ADDS Regent 60 for the terminal emulation when

generating a configuration in terminal setup.  Softerm supports

all the general capabilities of the Regent 60 including the full

128 character set and 80 X 24 display, conversational half or full

duplex transmission, buffered transmission in Page, Message,

Forms, and Modify operating modes, all addressable cursor modes,

auxiliary port operation with a printer, visual attributes, line

drawing, function keys, editing, and special terminal command

sequences.

Features

The display format of the ADDS Regent 60 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Monitor Mode.  The ADDS

Regent 60 also incorporates a 25th line used as a Status Line

which is supported by Softerm.

Softerm supports all remote terminal commands, keyboard functions

and switch settings with the following exceptions:

Remote terminal commands which are recognized but not processed

are Set Aux Baud Rate, On-Line Mode, Off-Line Mode, and Video ON

and OFF.

Switch selectable settings not supported by Softerm include

Character Display, Upper/Lower Case, Line Turnaround and Reverse

Channel.

The operating mode keys CONV, FORM, MSG and PAGE are supported

through the Status Line Display which permits toggling of the

operating mode.

Softerm includes support for the Auxiliary Port if the terminal

configuration includes a printer.  If either a Printer ON or

Transparent Mode Enable remote command is received, all subsequent

data is sent to the printer until a Printer OFF command is

received.  If a Transparent Mode Disable command is received, the

Auxiliary Port will be returned to the state it was in before the

Transparent Mode Enable command was received.  In the transparent

mode data is sent only to the printer and not displayed, otherwise

the received data is both displayed and printed.  The Auxiliary

port may also be enabled by the Ctrl PrtSc Softerm function which

emulates the AUX Key capability of the Regent 60.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

���� Adds Regent 60 Emulation Options ��Ŀ

Line Terminator Code: CR

ADDS Regent 60               - 1 -             Terminal Emulation

The emulation option Line Terminator Code allows the character

code(s) which terminate locally initiated function key sequences

as well as the Read Status and Transmit remote functions to be

defined.  Selecting CR will cause a carriage return ($0D) to

become the terminator.  Selecting a CR/ETX will cause a carriage

return and end of text sequence to be the terminator and Softerm

will send  $0D $03 after each function key sequence, status

message, or transmit function.  Selecting a CR/EOT will cause a

carriage return and end of transmission sequence to be the

terminator and Softerm will send $0D $04 after each function key

sequence, status message, or transmit function.

Operator Status Line Display

The ADDS Regent 60 terminal emulation includes a status display

similar to the Status Line displayed on line 25 of the Regent 60

terminal.  The status information displayed includes the Operating

Mode, Field Type, Attribute, and Row-Column Position.  The status

line can be displayed by pressing Alt T during online terminal

operation.

While the status is displayed, Softerm allows the Operating Mode

of the regent 60 to be toggled by pressing the M key.  Online

terminal operation is suspended and input is enabled on the status

line whenever a blinking  appears in the first column of the

status line.  For example, to toggle from the conversational mode

to one of the buffered modes (message, page, forms), first display

the status by entering Alt T and then press the M key until the

desired operating mode (Conv, Msg, Page, Forms) appears in the

operating mode field.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required. If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

ADDS Regent 60 Keyboard Functions

The following table defines the standard keyboard functions for

the ADDS Regent 60 terminal emulation.  The function key

terminator code is set by the Regent 60 emulation option Line

Terminator Code described previously.

Keyboard		Function

Enter			CR, LF, or CR/LF

Alt Enter		Transmit

Ctrl NumLock		Stop Display

Right Arrow		Cursor Forward

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Shift PrtSc		Print ALL

Alt PrtSc		Print Variable

Ctrl PrtSc		AUX

ADDS Regent 60               - 2 -             Terminal Emulation

Backspace		BS or DEL

Shift Backspace	DEL or BS

Tab			Tab

Shift Tab		Back Tab

Alt M			Monitor Mode On/Off

Alt R			Reset Modified Field

Home			Home Cursor

Alt T			Display Regent 60 Status

PgUp			Erase All

Ctrl PgUp		Erase Variable Data

PgDn			Erase to End of Page

End			Erase to End of Line/Field

Del			Delete Character (Line/Field)

Ctrl Del		Delete Character (Page)

Alt Del		Delete Line

Ins			Insert Character (Line/Field)

Ctrl Ins		Insert Character (Page)

Alt Ins		Insert Line

ADDS Regent 60               - 3 -             Terminal Emulation