Digital Equipment VT100

Introduction

The Digital Equipment Corporation VT100 terminal can be emulated

using Softerm PC by selecting DEC VT100 for the terminal emulation

when defining a configuration from the Setup Options menu.

Softerm supports all the general capabilities of the VT102 model

including the full 128 character set and 80 X 24 or 132 X 24

display, Smooth (Slow) Scrolling, Special Graphics Character Set,

user definable answerback message, ANSI or VT52 mode, Auto

XON/XOFF, Line/Local, New Line, Tabs, Wraparound, Auxiliary Keypad

and Cursor Mode Functions, Program function Keys, the additional

character attributes of the Advanced Video Option, and the print

and host editing features provided by the Printer Port Option.

Features

The display format of the VT102 allows for 24 lines by 80 or 132

characters.  Softerm supports the 132 column format using Extended

Text Mode or horizontal scrolling.  The VT102 has an operator

accessible Setup mode which allows terminal features to be

selected and then saved in non-volatile memory.  Softerm provides

the equivalent of this mode in the Terminal Parameters option of

the Setup Options menu and allows all VT102 features to be

specified except for:  AUTO REPEAT, INTERLACE, KEYCLICK, and

SCREEN BRIGHTNESS.  In addition, Softerm allows only 1 speed to be

selected and used for both receive and transmit.

Softerm supports all available keyboard functions and provides

additional keyboard functions for Cursor Home, Erase all of the

Display, Erase to End of Line, Erase to End of Screen, Print

Screen, and Enable/Disable Auto Print.

Softerm recognizes and processes all escape and control sequences

of the VT102 in both ANSI and VT52 compatible mode.  However,

hardware limitations prevent Softerm from providing a full

implementation of some features.  Some control sequences are

completely ignored while others are processed in a limited manner.

Control Sequences which Softerm recognizes but ignores in the ANSI

mode include Request Terminal Parameters, Auto Repeat Mode, and

Invoke Confidence Test.  Softerm recognizes and processes as a

special case control sequences for Double Height Line, Double

Width Line, and Single Width Line.

For double height lines, only the top half line is displayed.  For

double width lines, each displayable character is followed by a

space.

With the PC's Numlock key in the Off position, the VT100's Cursor

Key Mode is simulated with the cursor control keys on the PC's

numeric keypad.

The VT100's Keypad Application Mode is simulated in Softerm using

the Alt key in combination with a 0-9, minus, period, comma, and

Enter for the auxiliary keypad functions.  However, the Keypad

Application mode must be enabled by the host or the Keypad Mode

DEC VT100                    - 1 -             Terminal Emulation

Emulation Option (described below) before the numeric keypad

Alt-key functions are enabled.  Softerm also enables and disables

the auxiliary keypad functions and generates the appropriate codes

for the VT52 mode commands Enter alternate keypad mode and Exit

alternate keypad mode.

In the VT52 compatibility mode, Softerm supports all control

sequences.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������������� DEC VT100 Emulation Options �����������Ŀ

Operating Mode: VT102

Print Extent: Full Screen

Print Terminator: None

Number of Columns: 80

Screen Display: Normal

Scroll Rate: Jump

Character Set: U.S. ASCII

Margin Bell: Yes

BREAK Enabled: Yes

Keypad Mode: Numeric

Local Horizontal

Scroll Increment: 1

Horizontal Scroll

Delay (1/10 Secs.): 2

The emulation option Operating Mode allows the initial operating

mode of the terminal to be selected, and to determine the response

to the Identity or Device Attributes control sequence.  If the

VT52 operating mode is selected, the VT100 emulation will be

initialized in the VT52 mode.  If the Enter ANSI Mode command is

received, the emulation will switch to the ANSI VT102 operating

mode.  However, any ANSI mode control sequence which would cause

the terminal to return to its initial state, will cause the

terminal to return to the VT52 mode.  If VT102 or VT100 is

selected, the emulation will be initialized in the ANSI mode and

respond accordingly.

The Print Extent option determines what portion of the screen is

printed when the Shift PrtSc keyboard function is executed or a

print screen command is received.  If this option is specified as

Full Screen, the full screen will be printed.  If this option is

specified as Scrolling Region, only the area defined in the

current scrolling region will be printed.

The Print Terminator option specifies if a form feed character is

is generated after the screen is printed using the Shift PrtSc

keyboard function or when a print screen command is received.  If

this option is specified as Form Feed, a form feed character ($0C)

is appended to the data sent to the printer by the print screen

function. If this option is specified as a None, no form feed is

included.

DEC VT100                    - 2 -             Terminal Emulation

The Number of Columns option allows the initial column mode of the

VT100 emulation to be selected.  If 80 is selected, the emulation

will be initialized in 80-column mode.  If 132 is selected, the

emulation will be initialized in 132-column mode.

The Screen Display option allows the initial screen display mode

to be selected as Normal or Reverse.  If Normal is selected, the

foreground and background screen colors are displayed normally.

If Reverse is selected, the foreground and background colors are

reversed.

The Scroll Rate option allows the scroll rate to be selected.  If

Jump is selected, data will be scrolled at a fast jumpy rate.  If

6 Lines/Second is selected, smooth scrolling is simulated by

scrolling at a slower rate.

The Character Set option allows either the U.S. ASCII or United

Kingdom character set to be selected.  If United Kingdom is

selected, the U.S. # character is displayed as the British

currency symbol, �.

The Margin Bell option selects whether the audible alarm is

sounded as characters are keyed when approaching the right margin.

If this option is set to Yes, the audible alarm is sounded when a

character is entered at column 74.  If this option is set to No,

no alarm is issued.

The BREAK Enabled option determines if the BREAK key Alt B is

enabled.  If this option is set to No, the BREAK function is

disabled.

The option Keypad Mode allows the default operating mode of the

numeric keypad to be selected.  If set to Numeric and NumLock is

On, the keypad will generate numbers.  If NumLock is Off, the

keypad will generate Local Edit Functions.  If set to Application

and NumLock is On, the special keypad applications mode is enabled

and generates special code sequences.  If NumLock is Off, the

keypad will generate Local Edit Functions unless used with the Alt

key, in which case an Application Mode sequence will be generated.

Note:  The host may override the setting of this option.

The emulation option Local Horizontal Scroll Increment selects the

number of columns the screen will be scrolled left or right when

the Alt Ctrl Left Arrow or Alt Ctrl Right Arrow local horizontal

scroll keys are used.  It may be any value in the range 1-20.

Note:  These keys are disabled if a Hercules-type graphics adapter

card supporting 132 columns is installed in the PC and you have

chosen the 132-column Extended Text Mode option.

The emulation option Horizontal Scroll Delay determines the amount

of time in one tenth second increments that Softerm will delay

before horizontal scrolling is initiated when no characters have

been received for the amount of time specified by the delay.  This

option prevents unnecessary shifting of the screen when a

continuous stream of characters is being received.

DEC VT100                    - 3 -             Terminal Emulation

After the preceding options have been specified, press the Alt

Enter key and the following additional option screen is displayed:

������������ DEC VT100 Emulation Options �����������Ŀ

�������������������� Tab Stops ���������������������

Columns           T       T       T       T

1-40      1234567890123456789012345678901234567890

Columns   T       T       T       T       T

41-80     1234567890123456789012345678901234567890

Columns   T       T       T       T       T

81-120    1234567890123456789012345678901234567890

Columns   T       T

121-132   123456789012

Softerm allows default tab stops to be defined which are set at

initialization, when Ctrl Break is pressed, or any control

sequence is received which would return the terminal to its

initial state.  The tab format consists of 4 string entry fields,

for columns 1-40, 41-80, 81-120, and 121-132.  Entering any

non-blank character will result in a tab stop being set at the

current location.

Operator Status Line Display

The DEC VT100 terminal emulation includes a status line which may

be displayed on the 25th line by pressing the Alt T key.  The

status information displayed includes Row and Column, Operating

Mode, 80 or 132 Column Mode, Replace or Insert mode, Printer

Status, Hold indicator, Keyboard Lock indicator, and LED

indicators L1, L2, L3, and L4.

The Operating Mode setting can be toggled while the status is

displayed by pressing the M key.  Online terminal operation is

suspended and input is enabled on the status line whenever a

blinking  appears in the first column position.  For example, to

toggle from the VT102 mode to the  VT52 mode, first display the

status by entering Alt T and then press the M key until VT52 is

displayed on the status line.  Pressing the C key will cause the

column mode to be toggled between 80 and 132.  Pressing the K key

will cause the LOCK (Keyboard Lock) setting to be toggled.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

DEC VT100                    - 4 -             Terminal Emulation

DEC VT100 Keyboard Functions

The following table defines the standard keyboard functions for

the Digital Equipment VT100 terminal emulation.  Refer to the VT52

Keyboard Function Table for keyboard functions in VT52

compatibility mode.

Keyboard		Function

Enter			CR, LF, or CR/LF

Tab			Tab

Right Arrow		Cursor Forward

Alt Ctrl Right Arrow	Scroll Right

Left Arrow		Cursor Backward

Alt Ctrl Left Arrow	Scroll Left

Up Arrow		Cursor Up

Alt Ctrl Up Arrow	Home Left

Down Arrow		Cursor Down

Alt Ctrl Down Arrow	Home Right

Home			Cursor Home

Backspace		Delete

Shift Backspace	Backspace

PgUp			Clear

PgDn			Erase to End of Page

End			Erase to End of Line

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Shift PrtSc		Print Screen

Ctrl PrtSc		Auto/Print On/Off

Alt T			Display VT100 Status

Note:  In general, the PC's F1 key functions as the "Gold Key" on

the VT100 keyboard.

F1	PF1		F3	PF3

F2	PF2		F4	PF4

The following functions are entered on the numeric keypad except

for the comma (,) and minus (-) which are entered on the main

keyboard:

Alt 0     Keypad Function 0   	Alt ,		Keypad Function ,

Alt 1     Keypad Function 1   	Alt PrtSc	Keypad Function ,

Alt 2     Keypad Function 2

Alt 3     Keypad Function 3   	Alt -		Keypad Function -

Alt 4     Keypad Function 4   	Alt .		Keypad Function .

Alt 5     Keypad Function 5

Alt 6     Keypad Function 6   	Alt Enter	Keypad Function ENTER

Alt 7     Keypad Function 7  	Alt +		Keypad Function ENTER

Alt 8     Keypad Function 8

Alt 9     Keypad Function 9

DEC VT100                    - 5 -             Terminal Emulation