Data General D200

Introduction

The Data General DASHER D200 display terminal can be emulated

using Softerm by selecting Data General D200 for the terminal

emulation when generating a configuration in terminal setup.  The

D200 emulation is compatible and may be used with any application

which uses a Data General 6052, 6053, DASHER D100, or DASHER D200.

All the models listed are upward compatible with the functions and

features provided by the D200 display terminal.

Features

D200 keyboards can transmit the 96 displayable characters of the

ASCII character set but only 30 of the 32 control characters.  The

two control characters the keyboard cannot independently transmit

are the unit separator ($1F) and the record separator ($1E).

These character codes are used as header codes for function

sequences.  Softerm allows all 128 ASCII character codes to be

generated from the keyboard.

Display commands for the D200 are divided into five groups.  These

are screen control, cursor position, character attribute, terminal

identification, and diagnostic and print commands.

Softerm supports all screen control commands with the exception of

enable blink and disable blink, all cursor position commands, and

all character attribute commands.  The diagnostic commands enter

remote test and exit remote test are not supported.

The terminal identification command model report request is

supported and Softerm will respond with the following 6 character

sequence if there is no printer in the configuration:

$1E,$6F,$23,$21,$52,$A0

If the configuration includes a printer, the following sequence

will be transmitted:

$1E,$6F,$23,$21,$4A,$A0

Softerm supports all printer commands if a printer is configured

on the system.  This includes the print form command which prints

only the information displayed at full intensity.

As an extended feature of the Softerm D200 emulation, the host-

controlled Enable Printer Pass Through and Disable Printer Pass

Through commands compatible with Data General D400 and D450

terminals is provided.  If the terminal configuration includes a

printer, the following three character sequence when received will

enable the printer pass through mode:

$1E $46 $60

Data General D200            - 1 -             Terminal Emulation

If printer pass through is enabled, the following three character

sequence will disable printer pass through and return Softerm to

normal display operation:

$1E $46 $61

Softerm will respond to the host computer with a $06 character

when the disable printer pass through command is received and

printing is complete.  The printer pass through enabled by the

host computer operates in a transparent mode so that all data

received is sent to the printer and not the display until the

disable printer pass through function is received.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

��� Data General D200 Emulation Options ��Ŀ

Unshifted PrtSc Key Definition: $2A

The emulation option Unshifted PrtSc Key Definition allows the

code generated by the PrtSc key to be selected.  The default value

is $2A which is an asterisk (*) character.  It may be convenient

to set this value to a $0D (CR) or $0A (LF) depending on how the

Enter Key Sends terminal emulation option has been set.

Data General D200 Keyboard Functions

The following table defines the standard keyboard functions for

the Data General D200 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Shift PrtSc		Print

Alt Shift PrtSc	Print Form

Right Arrow		Cursor Right

Ctrl Right Arrow	Shift Cursor Right

Left Arrow		Cursor Left

Ctrl Left Arrow	Shift Cursor Left

Up Arrow		Cursor Up

Ctrl Up Arrow		Shift Cursor Up

Down Arrow		Cursor Down

Ctrl Down Arrow	Shift Cursor Down

Backspace		DEL

Tab			Tab

Home			Home

Ctrl Home		Shift Home

Data General D200            - 2 -             Terminal Emulation

PgUp			Erase Page

PgDn			Erase Page

End			Erase to End of Line

Alt B                  Break

F1	FK1		Ctrl F1		Ctrl FK1

F2	FK2		Ctrl F2		Ctrl FK2

F3	FK3		Ctrl F3		Ctrl FK3

F4	FK4		Ctrl F4		Ctrl FK4

F5	FK5		Ctrl F5		Ctrl FK5

F6	FK6		Ctrl F6		Ctrl FK6

F7	FK7		Ctrl F7		Ctrl FK7

F8	FK8		Ctrl F8		Ctrl FK8

F9	FK9		Ctrl F9		Ctrl FK9

F10	FK10		Ctrl F10	Ctrl FK10

Alt F1	FK11		Alt Ctrl F1	Ctrl FK11

Alt F2	FK12		Alt Ctrl F2	Ctrl FK12

Alt F3	FK13		Alt Ctrl F3	Ctrl FK13

Alt F4	FK14		Alt Ctrl F4	Ctrl FK14

Alt F5	FK15		Alt Ctrl F5	Ctrl FK15

Shift F1	Shift FK1	Shift Ctrl F1	Shift Ctrl FK1

Shift F2	Shift FK2	Shift Ctrl F2	Shift Ctrl FK2

Shift F3	Shift FK3	Shift Ctrl F3	Shift Ctrl FK3

Shift F4	Shift FK4	Shift Ctrl F4	Shift Ctrl FK4

Shift F5	Shift FK5	Shift Ctrl F5	Shift Ctrl FK5

Shift F6	Shift FK6	Shift Ctrl F6	Shift Ctrl FK6

Shift F7	Shift FK7	Shift Ctrl F7	Shift Ctrl FK7

Shift F8	Shift FK8	Shift Ctrl F8	Shift Ctrl FK8

Shift F9	Shift FK9	Shift Ctrl F9	Shift Ctrl FK9

Shift F10	Shift FK1	Shift Ctrl F10	Shift Ctrl FK10

Alt Shift F1	Shift FK11	Alt Shift Ctrl F1  Shift Ctrl FK11

Alt Shift F2	Shift FK12	Alt Shift Ctrl F2  Shift Ctrl FK12

Alt Shift F3	Shift FK13	Alt Shift Ctrl F3  Shift Ctrl FK13

Alt Shift F4	Shift FK14	Alt Shift Ctrl F4  Shift Ctrl FK14

Alt Shift F5	Shift FK15	Alt Shift Ctrl F5  Shift Ctrl FK15

The following functions are generated on the numeric keypad:

Alt 1		C1	Alt Shift 1	Shift C1

Alt 2		C2	Alt Shift 2	Shift C2

Alt 3		C3	Alt Shift 3	Shift C3

Alt 4		C4	Alt Shift 4	Shift C4

Data General D200            - 3 -             Terminal Emulation