Datamedia DT80/2 Video Terminal

Introduction

The Datamedia DT80/2 Video Terminal can be emulated using Softerm

by selecting Datamedia DT80/2 for the terminal emulation when

defining a configuration from the Setup Options menu.  The DT80/2

general features supported by Softerm include the full 128-

character US ASCII set, the United Kingdom character set, the 32-

character special graphics set, 80 X 24 and 132 X 14 display

modes, visual attributes, cursor control and keypad numeric and

application modes, and function keys.

Features

The display format of the DT80/2 is either 24 lines by 80 columns

or 14 lines by 132 columns with the ability to display all of the

displayable standard ASCII characters and the 32 special graphics

characters.

The DT80/2 emulation can operate in either standard ANSI command

mode or an Alternate Terminal Support (ATS) command mode.  While

in one mode, the other mode's commands are ignored.

The DT80/2 emulation recognizes and supports all ANSI and ATS

operational commands and functions with the following exceptions:

ANSI command sequences not supported are: Set LEDs, Auto Repeat

Set/Repeat, and Power Up Self Tests.

Setup Options not supported are: CRT Saver, Keyclick, Margin Bell,

Screen Brightness, and Tape Mode.

After the last standard screen in the Terminal Parameters mode in

Terminal Setup, the following screen will be displayed:

����� DT80/2 Emulation Options ����Ŀ

Word Wrap : Yes

Screen : Normal

Terminal Mode : ANSI

80/132 Columns : 80

CR Action : CR and LF

Scroll : Smooth

The emulation option Word Wrap may be toggled to Yes or No.  If

set to Yes, characters will wrap from the last column position of

one row to the first column position of the next row.  If the row

is the last displayable row, the screen will scroll up.  If set to

No, characters will overwrite the last displayable character on a

row.

The emulation option Screen may be toggled to Normal or Reverse.

If set to Normal, light characters will be displayed on a dark

background.  If set to Reverse, the display will be dark

characters on a light background.

Datamedia DT80-2             - 1 -             Terminal Emulation

The emulation option Terminal Mode may be toggled to ANSI or ATS.

If set to ANSI, all control sequences used will be a subset of

those specified in ANSI X-3.41-1974 and ANSI X-3.64-1977.  When in

ANSI Mode, none of the ATS control sequences will function

properly.  If set to ATS, the Alternate Terminal Support Mode

control sequences will be in effect and the ANSI control sequences

will not function properly.  The ANSI and ATS control sequences

are detailed in the Datamedia Corporation DT80/2 Video Terminal

Operator's Handbook, copyright November 1980 by Datamedia

Corporation.

The emulation option 80/132 Columns may be toggled to 80 or 132.

If set to 80, the screen will display 24 rows and 80 columns of

characters.  If set to 132, the screen will display 14 rows and

132 columns of characters.  Horizontal scrolling will be used to

display characters in columns 81 through 132.  The display will

scroll up when the end of row 14 is reached.

The emulation option CR Action may be toggled to CR and LF or LF

only.  If set to CR and LF, the Enter key will send a carriage

return/line feed sequence, positioning the cursor to column 1 of

the next line.  If the cursor is already on the last displayable

row, the display will scroll up one line.  If set to LF only, the

Enter key will send only a line feed sequence, positioning the

cursor to the next row.  If the cursor is already on the last

displayable row, the display will scroll up one line.

The emulation option Scroll may be toggled to Smooth or Jump.  If

set to Smooth, scrolling takes place at approximately 6 lines per

second, allowing you to scan data as it is received.  If set to

Jump, scrolling takes place as fast as data is received.

After the preceding options have been specified, press the Alt

Enter key combination and the following additional options menu is

displayed:

��������������� DT80/2 Emulation Options ������������Ŀ

Local Horizontal Scroll Increment: 5

Horizontal Scroll Delay (1/10 Secs.): 10

��������������������� Tab Stops ���������������������

Columns          T       T       T       T       T

1-40      1234567890123456789012345678901234567890

Columns          T       T       T       T       T

41-80     1234567890123456789012345678901234567890

Columns          T       T       T       T       T

81-120    1234567890123456789012345678901234567890

Columns          T

121-132   123456789012

Datamedia DT80-2             - 2 -             Terminal Emulation

The emulation option Local Horizontal Scroll Increment defaults to

5 and may be set to any integer value from 1 through 20.  When the

display is set to 132 columns, this option defines the number of

columns that will be scrolled left or right when the local

horizontal scroll keys are used.

The emulation option Horizontal Scroll Delay (1/10 Secs.)

determines the amount of time in one-tenth second increments that

Softerm will delay before horizontal scrolling is initiated when

no characters have been received for the amount of time specified

by the delay.  This option prevents unnecessary shifting of the

screen when a continuous stream of characters is being received.

This option has no effect when using the keyboard for input.

The emulation option Tab Stops defines the number of columns the

cursor will move when the Tab key is pressed.  This option

defaults to every 8 columns.  To change the settings, move the

highlighted block to the Columns row and use the Space bar or

arrow keys to move the cursor to the row(s) at which you want to

place a Tab Stop.  Type an upper-case "T" to define a Tab Stop.

To delete a Tab Stop, merely space over the "T."  Repeat this

process for each applicable set of defined columns.

DT80/2 Keyboard Functions

The following table defines the standard keyboard functions for

the Datamedia DT80/2 terminal emulation:

Keyboard		Function

Enter			CR, CR/LF

Backspace		BS

Tab			Tab

Right Arrow		Cursor Right

Alt Ctrl Right Arrow	Scroll Right

Left Arrow		Cursor Left

Alt Ctrl Left Arrow	Scroll Left

Up Arrow		Cursor Up

Alt Ctrl Up Arrow	Home Left

Down Arrow		Cursor Down

Alt Ctrl Down Arrow	Home Right

F1   F1		F3   F3

F2   F2		F4   F4

Use the numeric keypad for the following special functions:

0 to 9			Keypad Function 0 to 9

+			Keypad Function Enter

-			Keypad Function ,

*			Keypad Function -

Datamedia DT80-2             - 3 -             Terminal Emulation