Esprit 6310

Introduction

The Esprit 6310 terminal can be emulated using Softerm PC by

selecting ESP6310 for the terminal emulation when defining a

configuration from the Setup Options menu.  Softerm supports all

the general capabilities of the 6310 model including the full 128

character set and 80 X 24 display, Special Line Drawing Character

Set, user definable answerback message, Auto XON/XOFF, Line/Local,

New Line, Tabs, Auxiliary Keypad and Cursor Mode Functions,

Program Function Keys, Batch Mode, End of Message Character, 1 or

3 pages of memory, Lead-In Character Selection, Host Writable

Status Line, and the print and host editing features provided by

the Printer Port Option.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

�����������  Esprit 6310 Emulation Options ����������Ŀ

End of Message Character: None

Batch Mode: No

Lead-In Character: ESC

Pages of Display Memory: 1

�������������������Function Keys���������������������

F1: 1�ESC�@�cM�       Shifted F1: 1�ESC��cM�

F2: 1�ESC�A�cM�       Shifted F2: 1�ESC�a�cM�

F3: 1�ESC�B�cM�       Shifted F3: 1�ESC�b�cM�

F4: 1�ESC�C�cM�       Shifted F4: 1�ESC�c�cM�

F5: 1�ESC�D�cM�       Shifted F5: 1�ESC�d�cM�

F6: 1�ESC�E�cM�       Shifted F6: 1�ESC�e�cM�

F7: 1�ESC�F�cM�       Shifted F7: 1�ESC�f�cM�

F8: 1�ESC�G�cM�       Shifted F8: 1�ESC�g�cM�

F9: 1�ESC�H�cM�       Shifted F9: 1�ESC�h�cM�

F10:1�ESC�I�cM�       Shifted F10:1�ESC�i�cM�

F11:1�ESC�J�cM�       Shifted F11:1�ESC�j�cM�

The emulation option End of Message Character may be toggled to

None, CR ($0D), EOT ($04), or ETX ($03) and defines which

character is to be used to signal the end of messages sent in

Batch mode.

The emulation option Batch Mode may be toggled to No ro Yes to

select the mode in which the emulation will operate.  If set to

Yes, emulation to run in Batch (Block) mode.

The emulation option Lead-In Character may be toggled to ESC ($1B)

or TILDE ~ ($07E) and sets which lead in character is used for

command seqences.

The emulation option Pages of Memory may be toggled to allow the

emulation to run with either 1 or 3 pages of memory.

Esprit 6310                  - 1 -             Terminal Emulation

The emulation option Function Keys lets you define all normal and

shifted function keys.  Note: The first character always is a 1,

2, or 3.  This first character is a transmission flag for the

function key.  The flags are LOCAL, REMOTE, and LOCAL/REMOTE

respectively.

After the preceding options have been specified, press Alt Enter

and the following additional option screen is displayed:

������������� ESP 6310 Emulation Options ������������Ŀ

�������������������� Tab Stops ����������������������

Tab Function: Field

Columns   T       T       T       T       T

1-40      1234567890123456789012345678901234567890

Columns   T       T       T       T       T

41-80     1234567890123456789012345678901234567890

����������������� Attribute Setup �������������������

Screen Display: NORMAL

Foreground Attribute: NORMAL

Background Attribute: LOW

������������������� Keypad Setup ��������������������

Keypad Mode: NORMAL

The emulation option Tab Function may be toggled to FIELD or

SETUP.  If FIELD is selected, tab stops are considered to be at

the beginning of each attributes field.  If SETUP is chosen, the

tab stops will be determined by the following settings.  Softerm

allows default tab stops to be defined which are set at

initialization, when Ctrl Break is pressed, or any control

sequence is received which would return the terminal to its

initial state.  The tab format consists of two string entry

fields, for columns 1-40 and 41-80.  Entering any non-blank

character will result in a tab stop being set at the cursor's

position.

The emulation option Screen Display allows the screen to be

displayed in normal or reverse video.

Both the Foreground Attribute and Background Attributes options

let you define which attribute, or combination of attributes, you

want to use.  The available attributes are UNDERLINE, BLINK, LOW,

and INVERSE.

The emulation option Keypad Mode may be toggled to NORMAL, MODE 1,

MODE 2, or MODE 3 and sets the mode in which the keypad will

operate.

Esprit 6310                  - 2 -             Terminal Emulation

Operator Status Line Display

The Esprit 6310 terminal emulation includes a status line which

may be displayed on the 25th line by pressing the Alt-T key

combination.  The status information displayed includes: cursor

position; Local or Online operating mode, Keyboard Lock (KLock) On

or Off, and Aux Off or On.

In addition, while the Status Line menu is displayed, you can

press the letter U to Display User Status.

If you exit the menu by pressing Alt-T again, the Esprit 6310

status line will remain.  If you exit the menu by pressing Alt-

Esc, the Esprit 6310 status line will be cleared.

Esprit 6310 Keyboard Functions

Key			Function

Enter			CR

Alt Enter		SEND

Shift Enter		SEND LINE

Ctrl Enter		SEND PAGE

BackSpc		Backspace

Shift BackSpc		Delete

Tab			Tab

Shift Tab		Backtab

Alt Ins		Insert Line

Alt Del		Delete Line

Alt F			FUNCTION

Home			Home Cursor

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

PgUp			Clear Screen

Ctrl PgUp		Clear Foreground

PgDn			Clear to End Of Screen

End			Clear to End Of Line

Shift PrtSc		PRINT

Ctrl PrtSc		Online Print On/Off

Alt PgUp		Previous Page

Alt PgDn		Next Page

Esprit 6310                  - 3 -             Terminal Emulation

F1		F1

F2		F2

F3		F3

F4		F4

F6		F6

F7		F7

F8		F8

F9		F9

F10		F10

F11		F11

Shift F1	Shift  F1

Shift F2	Shift  F2

Shift F3	Shift  F3

Shift F4	Shift  F4

Shift F5	Shift  F5

Shift F6	Shift  F6

Shift F7	Shift  F7

Shift F8	Shift  F8

Shift F9	Shift  F9

Shift F10	Shift  F10

Shift F11	Shift  F11

Esprit 6310                  - 4 -             Terminal Emulation