Hazeltine 1520

Introduction

The Hazeltine 1520 Video Display Terminal can be emulated using

Softerm by selecting Hazeltine 1520 for the terminal emulation

when generating a configuration in terminal setup.  The Hazeltine

1520 general capabilities supported by Softerm include the full

128 character set with 80 X 24 display, full duplex, half duplex,

and format modes, selectable function lead-in character,

selectable EOM character, automatic line feed, wraparound,

scrolling, absolute cursor positioning, printer operations

including print screen and online print with or without display,

and special control character sequences including all remote

commands.

Features

The display format of the Hazeltine 1520 is 24 lines by 80

characters with the capability to display 94 of the 128 ASCII

character codes.  The basic mode of operation for the Hazeltine

1520 terminal emulation is character-by-character when set for

half or full duplex.  In this case, data which is entered at the

keyboard is sent directly to the computer.  In the format mode of

operation, the terminal becomes a buffered editing terminal,

allowing the operator to enter data with characters transmitted to

the computer only after the SEND key is pressed.  Thus, before any

data is actually transmitted, the operator may review it for

errors, omissions and misplaced data, and take corrective action.

After making the corrections, the operator may initiate a serial

buffered transmission of the data by pressing the SEND key.

The Auto LF-CR setup switch option corresponds to the Softerm LF

After CR option, and the Wraparound option corresponds to the

Softerm Auto Line Wrap option.

Setup switch options available on the Hazeltine 1520 and not

supported by Softerm include U/L CASE-UP and STD VIDEO-REV.

Softerm supports only upper and lower case operation and standard

video display of light characters on a dark background.

Softerm provides support for the keyboard functions ESC, DEL,

RETURN, LINE FEED, BACK SPACE, TAB, BACK TAB, FUNCTION, BREAK,

RESET, HOME, CLEAR, Clear Foreground, Clear End of Line, Clear End

of Screen, Up Cursor, Down Cursor, Right Cursor, Left Cursor,

Insert Line, Delete Line, and SEND.  Send, Send Line, and Send

Page keyboard functions are supported and the transmit symbol is

displayed as a rectangular block.  The SHIFT SEND or PRINT

function is also provided and the print symbol is displayed as a

diamond.

Pressing the FUNCTION key on the Hazeltine 1520 conditions the

terminal for a three-character transmission which has no effect on

the display.  This capability is provided in Softerm using the Alt

F key.  On pressing the Alt F key, an audible alarm will sound

alerting the operator to enter any ASCII code.  Once an additional

character is entered, the sequence ESC, ASCII code, and EOM

character is transmitted.

Hazeltine 1520               - 1 -             Terminal Emulation

The remote command features of the Hazeltine 1520 provide the user

with the capability to control the terminal via the host computer

software.  For the terminal to execute a remote command, the

command code must be preceded in some cases by a lead-in code

which is either a tilde ($7E) character or an ESC ($1B) character.

Softerm includes support for all remote functions.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����� Hazeltine 1520 Emulation Options ���Ŀ

End of Message Character: None

Format Mode: No

Lead-In Character: ESC

The emulation option End of Message Character selects the

character which will be inserted at the end of read cursor

address, function, batch, line, and PAGE transmissions to signal

the end of message (EOM).  The character may be selected as None

indicating no character, CR ($0D), EOT ($04), or ETX ($03).

The emulation option Format Mode overrides the half or full duplex

character mode and selects the format mode for keyboard entry and

transmission.  In format mode data entered on the display can be

edited before the contents of the display is transmitted.  Yes

selects the format mode of operation while No selects

conversational full or half duplex operation.

The emulation option Lead-In Character allows the lead-in

character for remote commands to be specified as either an ESC

($1B) or a Tilde ($7E) character code.

Hazeltine 1520 Keyboard Functions

The following table defines the standard keyboard functions for

the Hazeltine 1520 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Shift Enter		SEND LINE

Alt Enter		SEND

Ctrl Enter		SEND PAGE

Shift PrtSc		PRINT

Ctrl PrtSc		Online Print Enable/Disable

Backspace		BS or DEL

Shift Backspace	DEL or BS

Tab			Tab

Shift Tab		Back Tab

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Hazeltine 1520               - 2 -             Terminal Emulation

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Alt F			FUNCTION

Alt B			Break

Home			Home Cursor

PgUp			Clear Screen

Ctrl PgUp		Clear Foreground

PgDn			Clear to End of Screen

End			Clear to End of Line

Alt Del		Delete Line

Alt Ins		Insert Line

Hazeltine 1520               - 3 -             Terminal Emulation