Honeywell VIP7205

Introduction

The Honeywell VIP7205 Video Display Terminal can be emulated using

Softerm by selecting Honeywell VIP7205 for the terminal emulation

when generating a configuration in terminal setup.  The Honeywell

VIP7205 general capabilities supported by Softerm include the full

128 character set with 80 X 24 display, full duplex, full duplex

with local display, and buffered transmission modes, function

keys, full cursor control, automatic carriage return/line feed,

page or roll mode, buffered line or page operation with local

edit, and extension port.

Features

The display format of the Honeywell VIP7205 is 24 lines by 80

characters with the capability to display 95 of the 128 ASCII

character codes.  The VIP7205 provides selectable operating modes

including full duplex with no local display, full duplex with

local display, half duplex, half duplex buffered mode, and local

display.  Softerm supports all modes of operation using a full

duplex communications link but does not support half duplex with

two-way alternate capability and line turnaround.  The mode of

operation is character-by-character when set for full duplex or

full duplex with local echo.  In this case, data which is entered

at the keyboard is sent directly to the computer.  In the buffered

mode of operation, the terminal becomes a buffered editing

terminal, allowing the operator to enter data with characters

transmitted to the computer only after the XMIT key is pressed.

Thus, before any data is actually transmitted, the operator may

review it for errors, omissions and misplaced data, and take

corrective action.  After making the corrections, the operator may

initiate a serial buffered transmission of the data by pressing

the XMIT key.

The Softerm Page Mode option corresponds to the VIP7205 ROLL/PAGE

setup switch and the Softerm Duplex option corresponds to the

VIP7205 LOCAL COPY switch.

Setup switch options available on the Honeywell VIP7205 and not

supported by Softerm include LOWERCASE and HALF DUPLEX.  Softerm

supports only upper and lower case operation and full duplex

communications.

Softerm provides support for all keyboard functions.  Since

Softerm does not support half duplex communications facilities,

the END OF MESSAGE key is not supported.

Softerm supports all host processor commands and terminal response

codes of the VIP7205.  When the cursor is moved to a phantom

position such as column 81, the function will be processed

appropriately.  However, the cursor will be displayed at Column 80

on the screen even when it is actually at Column 81.

Honeywell VIP7205            - 1 -             Terminal Emulation

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

���� Honeywell VIP7205 Emulation Options ���Ŀ

End-of-Message Character: CR ($0D)

Operating Mode: Character

Transmit Mode: Page

Enable Blink: No

The emulation option End of Message Character selects the

character which will be inserted at the end of a line or page

buffered transmission to signal the end of message (EOM).  This

character should be set to a NUL ($00), CR ($0D), LF ($0A), EOT

($04), or ETX ($03).

The emulation option Operating Mode allows the selection of either

Character or Buffer mode for keyboard entry and transmission.  In

buffer mode, data entered on the display can be edited before the

contents of the display is transmitted.  This option is equivalent

to the VIP7205 CHARACTER/BUFFER switch.

The emulation option Transmit Mode is equivalent to the VIP7205

LINE/PAGE switch.  If this option is selected as Line, when a

buffered transmit is executed the line containing the cursor up to

but not including the cursor position is transmitted.  If this

option is selected as Page, page mode is assumed and transmission

starts at the home position and proceeds up to the cursor

position.

The emulation option Enable Blink if specified as Yes will cause

characters or fields proceeded by set low intensity command to

blink.  This option is equivalent to the VIP7205 INHIBIT/ENABLE

BLINK internal switch option.

Operator Status Line Display

The Softerm VIP7205 terminal emulation includes a status line

which may be displayed on the 25th line by pressing Alt T during

online terminal operation.  The status line display indicates the

current Row and Column, Character Mode or Buffer Mode, Page Xmit

or Line Xmit when operating in buffer mode, and the current End of

Message character.

While the status line is displayed, Softerm allows the Operating

Mode to be toggled by pressing the B key.  Online terminal

operation is suspended and input is enabled on the status line

whenever a blinking Right Arrow appears in the first column of the

status line.  For example, to to toggle from the character mode to

the buffer mode, first display the status by entering Alt T and

then press the B key until Buffer Mode appears on the status line.

You can also toggle the buffer transmit mode from Line to Page by

pressing the L key.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

Honeywell VIP7205            - 2 -             Terminal Emulation

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

Honeywell VIP7205 Keyboard Functions

The following table defines the standard keyboard functions for

the Honeywell VIP7205 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Alt Enter		XMIT

Alt B			Break

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Backspace		BS or DEL

Shift Backspace	DEL or BS

Home			Home Cursor

PgUp			Clear Screen

PgDn 			Clear to End of Screen

End			Clear to End of Line

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Alt T			Display VIP7205 Status Line

F1	F1		Shift F1	Shift F1

F2	F2		Shift F2	Shift F2

F3	F3		Shift F3	Shift F3

F4	F4		Shift F4	Shift F4

F5	F5		Shift F5	Shift F5

F6	F6		Shift F6	Shift F6

F7	F7		Shift F7	Shift F7

Honeywell VIP7205            - 3 -             Terminal Emulation