Honeywell VIP7801

Introduction

The Honeywell VIP7801 Video Display Terminal can be emulated using

Softerm by selecting Honeywell VIP7801 for the terminal emulation

when generating a configuration in terminal setup.  The Honeywell

VIP7801 general capabilities supported by Softerm include the full

128 character ASCII set with 80 X 24 display and simulated 80-

character status line; asynchronous character, text, and form

transmission modes; visual and form attributes; full keyboard

emulation including cursor control, editing, device control, and

function keys; auxiliary port printer support; support for the

VAF7821 Buffered Printer Adapter Option and support for the

VDF7811 72-Line Scroll Option.  This option allows three pages of

data to be stored.  Any consecutive 24 lines of the data space may

be displayed at one time.

Features

The display format of the Honeywell VIP7801 is 24 lines by 80

characters with the capability to display 139 characters including

the 95 ASCII character set, 11 business graphic symbols, and 33

symbols used only in communications display mode.  Softerm

supports display of the standard ASCII character set and line

graphic characters but not the special communications symbols.

The VIP7801 also displays a 25th line called the Status Line,

which is used to indicate the current operational status of the

terminal, and as a message line separate from the data space,

providing a one-line window for dialog between the operator and

the host.  Softerm provides full support for the status line

capabilities.

Softerm supports all operational modes of the VIP7801 including

Character, Text, and Form mode of operation.  The Character mode

of operation provides the capability to transmit a code or codes

immediately as they are generated by a keystroke.  In the

Character Non-echo mode, the terminal reacts to codes from the

keyboard as they are transmitted to the host.  In the Character

Echo mode, the terminal does not react to code sequences generated

by the keyboard unless these codes are echoed by the host computer

and received as normal data.

The Text mode of operation allows the operator to enter data into

the terminal where it is stored and displayed, but not sent to the

host until a transmit is initiated by the operator using the

TRANSMIT or RETURN key if the Transmit on Return option is set.

In Text mode, messages may be composed, visually checked for

correctness, edited if necessary, and then transmitted.  A message

can be variable in length encompassing a single character, a part

of a line, several lines, or an entire screen.

The Form mode of operation derives its name from the pre-printed

paper form on which headings, labels, and instructions on filling

in the blank spaces are not changeable by users of the form.  In

the VIP7801, these fields are considered protected.  Unlike the

paper form, the terminal provides for validation of the data as it

is entered into the blank spaces or unprotected fields.  This

Honeywell VIP7801            - 1 -             Terminal Emulation

validation is controlled by the Form attributes which can be

assigned in combinations as required for different fields.

Softerm supports all form attributes with the exception of Omit

Print.  Softerm supports all visual attributes including inverse,

low intensity, underline, hide, and blink if the video interface

and display driver used support these capabilities.  Only the

unprotected data is normally transmitted to the host in Form mode

when the TRANSMIT key is entered.

Softerm supports all setup switch options available on the

Honeywell VIP7801 with the exception of Test Mode, Display All

Mode, and Hold/Drop DTR in Local.  However, Softerm provides a

special local mode of operation which does not drop the DTR signal

causing a disconnect.  Softerm can be toggled between the special

local and online mode using the VIP7801 Status Display initiated

with the Alt T keyboard function.

Softerm recognizes and processes all host-initiated VIP7801

operational commands except for: Firmware Version Display (will

cause Clear) and Test Results Display.

Host initiated VAF7821 buffered printer adapter commands which

Softerm recognizes but ignores include Print Multiple Copies and

Print Transmit All.  Any form following a Print Transmit All

command will be processed normally.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������ Honeywell VIP7801 Emulation Options ����Ŀ

End-of-Message Character: ETX ($03)

CR/LF Delimiters: No

Operating Mode: Character

Space Suppress: Yes

Full Field Delimiter

W/Space Suppress Set: US ($1F)

Block Transmit Mode: No

Transmit On Return: No

Transmit Delay (1/10 Secs.): 0

Echo Mode: Yes

Number Of Rows: 24

Automatic Tabbing: Delayed

Roll Mode: Yes

Print All: Yes

Print Control Characters:

Start:     None

End:     None

Send Key Definition: Alt Enter

User Defined Function Keys: No

Print Extent: Screen

The emulation option End-of-Message Character selects the

character which terminates all transmitted messages.  This

character should be selected as either an ETX ($03), or an EOT

($04).

Honeywell VIP7801            - 2 -             Terminal Emulation

The emulation option CR/LF Delimiters determines if a CR/LF

(carriage return/line feed) delimiter is sent at the end of each

line in Text mode.  Yes selects CR/LF delimiters for lines

transmitted in Text mode.  Selecting No will cause no delimiter to

be sent.  This option is ignored if the Space Suppress option is

set to Yes.

The emulation option Operating Mode determines which mode Softerm

will enter when initiating the online mode from terminal setup or

when a Reset to Initial State command is received.  Text selects

the Text mode of operation, while Character selects the Character

mode of operation.

The emulation option Space Suppress determines if trailing spaces

on a line are to be suppressed on a data transmission to the host.

Selecting Yes for this option will cause trailing spaces to be

suppressed, while selecting No will cause trailing spaces to be

transmitted.

The emulation option Full Field Delimiter W/Space Supress Set

determines the delimiter that will be sent after a full

unprotected field is transmitted in forms mode.  This option may

be toggled to either US ($1F) or HT ($09).  The US character is

the default which should be transmitted under normal conditions.

The HT character normally is sent only after a partially filled

unprotected field is transmitted.

The emulation option Block Transmit Mode determines if data is

transmitted to the host as a series of blocks or as a single

transmission.  If this option is specified as Yes, the terminal

transmits messages in blocks of 256 characters.  If this option is

specified as No, the terminal transmits the entire message in one

transmission.

The emulation option Transmit on Return determines if the Enter

key functions as a transmit key or normally when operating in the

Text mode.  If this option is specified as Yes, the Enter key will

initiate a transmit operation in the Text mode.  If this option is

specified as No, the Enter key will operate normally.

The emulation option Transmit Delay is used to prevent multiple

transmits from being processed.  After pressing the transmit key,

the operator will not be able to transmit again until the delay

has passed.  The delay may be set to any value from 0 to 99 tenths

of a second.

The emulation option Echo Mode determines if keyboard data is

displayed or acted upon when operating in the Character mode.  If

this option is specified as Yes, keyboard data is sent to the host

and not displayed or acted upon until processed and returned from

the host.  If this option is specified as No, keyboard data is

displayed or acted upon as it is keyed.

The emulation option Number Of Rows determines the amount of

display memory available.  Selecting 24 will allocate one page of

display memory; selecting 72 will allocate three pages of display

memory.  The Next Segment, Previous Segment, Scroll Up, and Scroll

Down sequences are valid only while operating in 72 line mode.

Honeywell VIP7801            - 3 -             Terminal Emulation

The emulation option Automatic Tabbing determines the cursor

action while leaving a field in Forms Mode.  It may be set to

either:

Delayed - When the last position of an unprotected field is

filled the cursor will move to the first position of the next

protected field.  On the next keystroke or recieved character

the cursor will move to the first position of the next

unprotected field and display the character, or sound an

alarm if no unprotected field exists.

Immediate - When the last position of an unprotected field is

filled the cursor will move to the first position of the next

unprotected field.  An alarm will sound if no unprotected

field exists.

The emulation option Roll Mode allows or prevents rolling of data

when a LF (line feed) is received on the last line of the display.

Selecting Yes allows rolling of screen data while selecting No

prevents rolling of the screen.

The emulation option Print All determines the extent of the data

printed when a Print Data Space buffered printer adapter command

is received.  If this option is specified as Yes, both protected

and unprotected data is printed.  If this option is specified as

No, only unprotected data is printed.

The emulation option Print Control Characters designates the

character combination to be sent to the printer at the Start and

End of printing when a Print Data Space buffered printer adapter

command is received.  The choices for Start and End control

characters are None, CR, CR/-LF, CR/VT, and CR/FF.  If the

character code is specified as None, no characters are sent to the

printer.

The emulation option Send Key Definition allows you to define the

key which will initiate a transmit operation.  If Enter is

selected, then pressing the Enter key will initiate a transmit

operation and pressing Alt Enter will generate a CR, CR-LF, or LF

sequence.  If Alt Enter is selected, then pressing the Enter key

will generate a CR, CR/LF, or LF sequence, and pressing the Alt

Enter key will initiate a transmit operation.  The operation of

the Enter key is also affected by the Transmit on Return option

described previously.

If the User Defined Function Keys option is set to Yes, pressing

the Alt Enter key will cause the following additional screen to be

displayed:

Honeywell VIP7801            - 4 -             Terminal Emulation

��� Honeywell VIP7801 Function Key Definitions ��Ŀ

Unshifted              Shifted

Mode Definition       Mode Definition

F1    1  �ESC�0�cC�         1  �ESC�1�cC�

F2    1  �ESC�2�cC�         1  �ESC�5�cC�

F3    1  �ESC�6�cC�         1  �ESC�7�cC�

F4    1  �ESC�8�cC�         1  �ESC�9�cC�

F5    1  �ESC�:�cC�         1  �ESC�;�cC�

F6    1  �ESC�<�cC�         1  �ESC�=�cC�

F7    1  �ESC�>�cC�         1  �ESC�?�cC�

F8    1  �ESC�P�cC�         1  �ESC�Q�cC�

F9    1  �ESC�R�cC�         1  �ESC�S�cC�

F10   1  �ESC�T�cC�         1  �ESC�V�cC�

F11   1  �ESC�\�cC�         1  �ESC�]�cC�

F12   1  �ESC�^�cC�         1  �ESC�_�cC�

Transmit Mode  1=Remote  2=Local  3=Remote+Local

4=Operating Mode Dependent

The fields displayed for each function key definition include the

Mode and Definition which define a communication mode and a

message of up to 10 characters to be transmitted by that function.

The Mode field defines a communication mode for the function key

according to the following values:

Mode Effect

1    Send message to computer

2    Send message to terminal

3    Send message to both computer and terminal

4    Operating Mode Dependent

If Mode 4 is selected, and the current operating mode is Text or

Form, the message will be sent to the terminal only.  Otherwise,

the message will be sent to both the computer and terminal.

The Definition is the message to be transmitted by that function

key.  The message can contain any combination of up to 10

alphanumeric characters, control codes, or escape sequences.  The

default values for the function keys match the standard default

character codes of the VIP7801.

If the User Defined Function Keys option is set to Yes, characters

transmitted by the unshifted and shifted function keys are defined

by the Function Key Definitions screen.  If the Ctrl key is

pressed in combination with an unshifted or shifted function key,

the function key will operate as a standard VIP7801 function key.

Operator Status Line Display

The Honeywell VIP7801 terminal includes a Status Line (25th line

on the display screen) which is operationally separate from the 24

lines of screen data.  It can be used as a host message line, a

host conversation line, or it can be used to inform the operator

of status conditions produced automatically by the terminal.

Honeywell VIP7801            - 5 -             Terminal Emulation

Included on the status line are the READY indicator, the modes of

operation, the error messages, and the current row and column

position of the cursor.

Softerm displays the status information on row 25 whenever the Alt

T key is pressed.  The status information displayed includes

Insert mode, Graph mode, Text, Form, or Char mode, Echo mode, Tx-

RET mode, Roll mode, Auto-LF mode, Local mode, and the current Row

and Col.

The status line also allows certain modes of operation to be

toggled by pressing specific keys while the status line is

displayed.  Online terminal operation is suspended and input is

enabled on the status line whenever a blinking Right Arrow appears

in the first column.  To return to the online mode after

displaying the status, enter Alt T or Alt Esc.  If you exit using

the Alt T key, the status line will remain on the screen during

online operation and will be automatically updated if required.

If you exit from the status line display using the Alt Esc key,

the status line will be cleared.

The status line is displayed automatically when an error condition

occurs.  Status messages displayed by Softerm include DATA

OVERFLOW, INVALID COMMAND, FILL REQUIRED, ENTRY REQUIRED, ALPHA,

DIGITS, NUMERIC, ALPHA/-DIGITS, ALPHA/NUMERIC, and READY.  In

addition, the PRINTER INVALID CMD message may be displayed if an

error condition occurs with a VAF7821 Buffered Print Adapter

command.  If a field entry error condition, such as attempting to

enter alphabetic data in a numeric field occurs, the alarm is

sounded and the appropriate error message will be displayed on the

status line.  No additional entry is permitted until the error

condition is cleared by pressing the PgUp (RESET) key.

After entering Alt T to display the status line, the Auto-LF mode

setting can be toggled by pressing the A key.  The Local mode

setting can be toggled while the status is displayed by pressing

the L key.  Setting the terminal for Local operation will not

cause a disconnect and allows local editing to be performed in the

Char mode of operation.

Honeywell VIP7801 Keyboard Functions

The following defines the standard keyboard functions for the

Honeywell VIP7801 emulation:

Keyboard		Function

Enter			CR, CR/LF, or TRANSMIT

Alt Enter		TRANSMIT

Ctrl Enter		CTRL TRANSMIT

Ctrl Break		Soft Reset

Ctrl NumLock		Stop Display

Shift PrtSc		PRINT

Alt Ctrl PrtSc		PRINT RESET

End			ERASE EOF

Honeywell VIP7801            - 6 -             Terminal Emulation

Alt A			ATTRB

Alt B			Break

Alt C			CHAR

Alt D			DELETE ATTRB

Alt F			FORM

Alt T			Display VIP7801 Status Line

Alt X			TEXT

Right Arrow		Cursor Right

Alt Right Arrow	TAB SET

Left Arrow		Cursor Left

Alt Left Arrow		CLEAR TAB

Up Arrow		Cursor Up

Alt Up Arrow		SCROLL UP

Down Arrow		Cursor Down

Alt Down Arrow		SCROLL DOWN

Backspace		BS or DEL

Shift Backspace	DEL or BS

Tab			Tab

Shift Tab		Back Tab

Alt Ctrl Tab		TAB INITIALIZE

Home			HOME

Ctrl Home		CTRL HOME

PgDn			ERASE EOP

Alt PgDn		NEXT SEGMENT

PgUp			RESET

Alt PgUp		PREV SEGMENT

Ctrl PgUp		CLEAR

Alt Ctrl PgUp		INIT

Del			DEL CHAR

Alt Del		DEL LINE

Ins			INS MODE

Alt Ins		INS LINE

Ctrl Ins		INS MODE RESET

F1	F1		Shift F1	Shift F1

F2	F2		Shift F2	Shift F2

F3	F3		Shift F3	Shift F3

F4	F4		Shift F4	Shift F4

F5	F5		Shift F5	Shift F5

F6	F6		Shift F6	Shift F6

F7	F7		Shift F7	Shift F7

F8	F8		Shift F8	Shift F8

F9	F9		Shift F9	Shift F9

F10	F10		Shift F10	Shift F10

Alt F1	F11		Alt Shift F1	Shift F11

Alt F2	F12		Alt Shift F2	Shift F12

Honeywell VIP7801            - 7 -             Terminal Emulation