Lear Siegler ADM-11 Video Display Terminal

Description

The general capabilities of the Softerm emulation of the Lear

Siegler ADM-11 Video Display Terminal include the full 128-

character ASCII set with 80 X 24 display; a host-writable status

line; visual attributes; full keyboard emulation including cursor

control and function keys; and gated extension port printer

support.

Features

The display format of the ADM-11 is 24 lines by 80 characters with

the capability to display 260 characters including the 128 ASCII

character set, and 132 Mosaic and Line Drawing characters.

Softerm supports display of the standard ASCII character set, 5

Mosaic characters, and 13 Line Drawing characters.  The ADM-11

also displays a 25th line, called the Status Line, which is used

to as a message line separate from the data space to provide a

one-line window for dialog from the host to the operator.  Softerm

provides full support for the status line capabilities.

Setup options available on the ADM-11 and supported by equivalent

Softerm options include Online, Cursor Blink, Status, Wrap,

Newline, Baud Rate, Bits, Bit 8, PTY Enable, Parity, Duplex,

CHRS/FNC, FNC Keys, SO/SI, Handshake, XON/XOFF, Busy, and

Answerback.

Softerm supports all setup options available on the ADM-11 except

Key Click.

Softerm provides support for all keyboard functions and host-

initiated ADM-11 operational commands.

Lear Siegler ADM-11 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

��� Lear Siegler ADM-11 Emulation Options ��Ŀ

Status Line Mode: REV

Function Key Buffer Size: 8

Enable Non-Volatile

Memory For Function Keys: No

SO/SI Mode: KBD LOCK

Answerback Mode: No

Answerback Message:

The emulation option Status Line Mode determines the video

attribute of the status line.  The choices are Reverse, Blink,

Blank, and Normal.

The emulation option Function Key Buffer Size sets the maximum

length of a user-defined function key string to either 4 or 8

characters.

Lear Siegler ADM-11          - 1 -             Terminal Emulation

The emulation option Enable Non-Volatile Memory For Function Keys

determines if user or default function keys will be sent.

The emulation option SO/SI Mode determines whether the keyboard

lock or gated auxiliary port will react to an SO or SI character.

Receiving a $0E character will enable either the Gated Auxiliary

Mode or the Keyboard Lock.  Receiving a $0F character will disable

either the Gated Auxiliary Mode or the Keyboard Lock.

The emulation option Answerback Mode enables the user-defined

answerback message (next option) to be sent if set to Yes.

The Answerback Message can be any sequence of up to 32 characters

sent to the host when requested by the host.  The sequence may be

sent automatically or manually and is often used to transmit a

password.

After the Emulation Options screen has been filled and if the

Memory for Function Keys option has been set to Yes, the following

screen will appear:

This menu allows the operator to define the eight function keys.

The sequence consists of up to 4 or 8 characters, depending on the

setting of the Function Key Buffer Size emulation option, that

will be transmitted when the matching Fkey is pressed.

For instance, function key F1 could be assigned the sequence Esc

H, which would set the Alignment Pattern.

Lear Siegler ADM-11 Operator Status Line Display

The Lear Siegler ADM-11 terminal emulation includes a Status Line,

operationally separate from the 24 lines of screen data, which may

be displayed on the 25th line by pressing the Alt T key.

The status information displayed includes Conversation mode (full

or half duplex), Data Carrier Detect and Clear To Send.

The status line allows certain modes of operation to be toggled by

pressing specific keys while the status line is displayed.  Online

terminal operation is suspended and input is enabled on the status

line whenever a blinking Right Arrow appears in the first column.

Pressing the letter C will toggle the Conversation mode between

full and half duplex.

Pressing the letter K will lock the keyboard.  The keyboard can be

re-enabled only by the host.

Pressing the letter S will toggle the Status Line between solid

(reverse video), blinking, normal and blank displays.

Pressing the letter U will allow a direct dump of the serial input

to the screen.  All Ctrl and Esc characters (those with a hex

value of $20 or less) will be graphically represented.  This is an

echoed display only; no action will be taken.

Lear Siegler ADM-11          - 2 -             Terminal Emulation

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key, the status

line will remain on the screen during online operation and will be

automatically updated if required.  If you exit from the status

line display using the Alt Esc key, the status line will be

cleared.

Lear Siegler ADM-11 Keyboard Functions

The following table defines the standard keyboard functions for

the Lear Siegler ADM-11 terminal emulation:

Keyboard		Function

Enter			CR/LF

Ctrl_			CR/LF

Home			HOME

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Backspace		Backspace

Del			Delete

Tab			Tab

Shift Tab		Back Tab

Alt A			Answerback

Ctrl G			Sound Bell

Ctrl O			Keyboard Lock

Ctrl Z			Home/Erase to Spaces

Shift Esc		Local Escape

Function Keys:

F1 - F8

Volatile		Programmed Functions

Non-Volatile		Default function

Screen Printing Functions:

PrtSc			Print Line

Shift PrtSc		Print Page Formatted

Alt PrtSc		Print Page Unformatted

Screen Clearing Functions:

PgUp			Clear Page to Spaces

Ctrl PgUp		Clear Page to Nulls

PgDn			Erase to End of Page with Spaces

Ctrl PgDn		Erase to End of Page with Nulls

End			Clear to End of Line with Spaces

Ctrl End		Clear to End of Line with Nulls

Lear Siegler ADM-11          - 3 -             Terminal Emulation