Qume QVT102A

Introduction

The Qume QVT102A terminal can be emulated using Softerm PC by

selecting Qume 102A for the terminal emulation when defining a

configuration from the Setup Options menu.  Softerm supports all

the general capabilities of the QVT102A model including the full

128 character set and 80 X 24 display, Special Line Drawing

Character Set, user definable answerback message, Auto XON/XOFF,

Line/Local, New Line, Tabs, Cursor Mode Functions, Program

Function Keys, Block Mode, End of Message Character, and the print

and host editing features provided by the Printer Port Option.

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����������� Qume QVT102A Emulation Options ����������Ŀ

Operating Mode: Block

E.O.M. Char:    CR

�������������������Function Keys���������������������

PF1: �cA�@�cM     SHIFT-PF1: �cA�D�cM

PF2: �cA�A�cM     SHIFT-PF2: �cA�E�cM

PF3: �cA�B�cM     SHIFT-PF3: �cA�F�cM

PF4: �cA�C�cM     SHIFT-PF4: �cA�G�cM

The emulation option Operating Mode can be toggled to Block or

Character mode.  Characters entered from the keyboard in block

mode are not transmitted as they are entered.  A complete line or

screen may be entered and then transmitted.  In the character

mode, a character is immediately transmitted when a keyboard key

is pressed.

The emulation option E.O.M. Character selects the character which

terminates all messages transmitted in block mode.  The choices

are: CR ($0DH), EOT ($04H), ETX ($03H), and NULL ($00H).

The emulation option Function Keys allow you to edit what the PF

keys will transmit.

Qume QVT102A                 - 1 -             Terminal Emulation

Operator Status Line Display

The Qume 102A terminal emulation includes a status line which may

be displayed on the 25th line by pressing Alt-T:

��������������������������������������ͻ

�         QUME QVT102A Status          �

�                                      �

�    Press Character to Toggle Mode    �

� Character  Mode                      �

�                                      �

�     R      Margin Bell               �

�     E      EOM: NUL/ETX/EOT/CR       �

�     G      Graphics Off/On           �

�     B      Block/Line/Character      �

�     H      Half Intensity            �

�     P      Protect                   �

�     K      Keyboard Lock             �

�     M      Monitor Mode              �

�                                      �

�   Alt T    Resume, Leave Status Line �

�   Alt Esc  Resume, Clear Status Line �

��������������������������������������ͼ

The status information displayed includes: cursor position; Margin

Bell On or Off; Graphics On or Off; End-of-Message character;

Block, Line, or Character Mode; Half Intensity On or Off; Protect

On or Off; Keyboard Lock On or Off, and Monitor Mode On or Off.

While the Status Line menu is displayed, you can press any of the

indicated keys to toggle the states.

If you exit the menu by pressing Alt-T again, the Qume 102A status

line will remain.  If you exit the menu by pressing Alt-Esc, the

Qume 102A status line will be cleared.

Qume QVT102A Keyboard Functions

Key			Function

Enter			CR

Alt Enter		Transmit

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

BackSpc		BACK SPACE

Shift BackSpc		FORWARD SPACE

Tab			TAB

Shift Tab		BACK TAB

Home			HOME

PgUp			SHIFT CLEAR

PrtSc			Print (Full Page)

Shift PrtSc		Print (Top To Cursor)

Ctrl PrtSc		Print (Cursor To End)

Qume QVT102A                 - 2 -             Terminal Emulation

Use the function keys (F1 - F4) for the following functions :

F1 - F4		PF1 - PF4

Ctrl F1 - F4		Ctrl PF1 - PF4

Shift F1 - F4		Shift PF1 - PF4

Qume QVT102A                 - 3 -             Terminal Emulation