TeleVideo 910

Introduction

The TeleVideo Model 910 terminal can be emulated using Softerm by

selecting TeleVideo 910 for the terminal emulation filename when

generating a configuration in terminal setup.  Softerm supports

all the general capabilities of the TeleVideo 910 including the

full 128 character set and 80 X 24 display, video attributes, Tab

and Back Tab, editing features, cursor controls, conversational

full or half duplex transmission, and the simultaneous or

transparent print modes.

Features

The display format of the TeleVideo 910 is 24 lines by 80

characters with the ability to display all 128 ASCII character

codes including 32 control codes in the Monitor Mode.

Softerm supports all available keyboard functions.  The SHIFT

CLEAR key which clears the screen to nulls is provided as a

separate key in Softerm but it is processed identically to a

CLEAR.

Softerm processes all functional commands and recognizes but

ignores Display Control Character and Self Test On.

Softerm includes support for the Printer Port if the terminal

configuration includes a printer.  If either an Enable Printer

Port or Enable Transparent Print remote command is received, all

subsequent data is sent to the printer until a Disable Printer

Port command is received.  If a Disable Transparent Print command

is received while the printer port is enabled, the printer will

remain enabled and received data will be displayed as well as

printed.  In the transparent mode, data is sent only to the

printer and not displayed, otherwise the received data is both

displayed and printed.  The printer port may also be enabled by

the Ctrl PrtSc keyboard function which provides the PRINT key

capability of the TeleVideo 910.

TeleVideo 910 Keyboard Functions

The following table defines the standard keyboard functions for

the TeleVideo 910 terminal emulation:

Keyboard		Function

Enter			CR, LF, or CR/LF

Ctrl Break		Soft Reset

Ctrl NumLock		No Scroll

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

Alt A			Send Answerback Message

Alt B			Break

Alt M			Monitor Mode On/Off

Backspace		BS

TeleVideo 910                - 1 -             Terminal Emulation

Shift Backspace	DEL

Tab			TAB

Shift Tab		BACKTAB

Home			Home Cursor

PgUp			CLEAR

Ctrl PgUp		SHIFT CLEAR

PgDn			Erase to End of Page

End			Erase to End of Line

Ctrl PrtSc		Enable/Disable Printer Port

The following functions are entered on the numeric keypad:

Alt 0		FUNCT/0		Alt 5		FUNCT/5

Alt 1		FUNCT/1		Alt 6		FUNCT/6

Alt 2		FUNCT/2		Alt 7		FUNCT/7

Alt 3		FUNCT/3		Alt 8		FUNCT/8

Alt 4		FUNCT/4		Alt 9		FUNCT/9

TeleVideo 910                - 2 -             Terminal Emulation