TeleVideo 912 Video Display Terminal

Introduction

The TeleVideo 912 Terminal can be emulated using Softerm by

selecting TeleVideo 912 for the terminal emulation when generating

a configuration in terminal setup.  The TeleVideo 912 general

capabilities supported by Softerm include the full 128-character

ASCII set with 80 X 24 display; asynchronous character

transmission mode; visual and logical attributes; full keyboard

emulation including function keys; and auxiliary port printer

support.

Features

Softerm recognizes and processes all TeleVideo 912 operational

commands with the exception of Self Test Mode.  The TeleVideo 912

Terminal Emulation also supplies a set up option to allow

emulation of the TeleVideo 920.  This option can be selected from

the Terminal Setup Options screen.

After the last standard screen in the Terminal Parameters mode in

Terminal Setup, the following screen will be displayed:

���� TeleVideo 912/920 Emulation Options ��Ŀ

Compatability Mode: 912

Mode: CONVERSATION

Alternate Page: Yes

The emulation option Compatibility Mode can be toggled to 912 or

920.  If set to 912, the emulation performs Line and Page Erase

functions by clearing the screen to Spaces.  If set to 920, the

emulation recognizes and processes the Line and Page Erase clear

to nulls command if received from the host.  Line and Page Erase

functions performed locally result in a clear to Spaces action.

The emulation option Mode can be toggled to CONVERSATION or BLOCK.

If set to CONVERSATION, characters are transmitted to the host at

the same time as they are typed from the keyboard.  If set to

BLOCK, data is transmitted and received as complete blocks.  In

Block Mode, up to a full screen of data can be entered and edited.

Transmission does not occur until a Send command is transmitted to

or received from the host.

The emulation option Alternate Page can be toggled to Yes or No.

If set to Yes, an additional page of memory space (1920

characters) is available and may be accessed by pressing Ctrl-

Numeric Keypad + or Esc K.  This is equivalent to installing the

Second Page Memory Option.  If set to No, the second page is not

available for use.

TeleVideo 912                - 1 -             Terminal Emulation

Operator Status Line Display

The TeleVideo 912 terminal emulation includes a Status Line which

may be displayed on the 25th line by pressing Alt T.

The information includes the current display page, cursor row and

column, Conversation or Block mode, duplex setting (either Full

Duplex or Half Duplex), Protect Mode (if set), Write Protect (if

set), Auto Page (equivalent to Auto Flip), Keyboard Lock (if set),

and the Compatibility Mode setting (either 912 or 920).  Note:

Although the user may set Keyboard Lock On, the host must issue

the command to turn Keyboard Lock Off.

TeleVideo 912 Keyboard Functions

The following table defines the standard keyboard functions for

the TeleVideo 912 terminal emulation:

Keyboard		Function

Enter			Return

Backspace		BS

Shift Enter		Send Line

Alt Shift Enter	Shift Send Line

Ctrl Enter		Send Page

Ctrl Shift Enter	Shift Send Page

Home			Home

Ins			Char Insert

Alt Ins		Line Insert

Del			Char Delete

Alt Del		Line Delete

End			Line Erase

Ctrl End		Shift Line Erase

Alt Num+		Page Newline

Ctrl Num+		Shift Page Newline

PgDn			Page Erase

Ctrl PgDn		Shift Page Erase

PgUp			Clear Space

PrtSc			Print

Tab			Tab

Ctrl Tab		Back Tab

Alt Tab		Tab Set

Alt F			Funct

Alt End		Prot Mode

Alt PgDn		Shift Prot Mode

Alt Ctrl Ins		Shift Block Conv

Alt Ctrl End		Ctrl Block Conv

Right Arrow		Cursor Right

Left Arrow		Cursor Left

Up Arrow		Cursor Up

Down Arrow		Cursor Down

F1-F10			Function Keys 1-10

Shift F1-F10		Shift Function Keys 1-10

Alt F1			Function Key 11

Alt Shift F1		Shift Function Key 11

Ctrl Break		Soft Reset

Ctrl Numlock		Stop Display

Alt T			Display 912 Status

TeleVideo 912                - 2 -             Terminal Emulation