TYMNET78

Introduction

The TYMNET78 display terminal can be emulated using Softerm by

selecting TYMNET78 for the terminal emulation when generating a

configuration in terminal setup.  Softerm supports all the general

capabilities of the TYMNET78 including the full 128 character set

and 80 X 24 display, conversational half or full duplex

transmission, buffered transmission in Page, Message, Forms, and

Modify operating modes, all addressable cursor modes, auxiliary

port operation with a printer, visual attributes, line drawing,

function keys, editing, and special terminal command sequences.

Features

The display format of the TYMNET78 is 24 lines by 80 characters

with the ability to display all 128 ASCII character codes

including 32 control codes in the Monitor Mode.  The TYMNET78 also

incorporates a 25th line used as a Status Line which is supported

by Softerm.

Softerm supports all available keyboard functions including

BACKSPACE, TAB, BACK-TAB, DEL, NEW LINE, E ALL, EOF, ESC, PRINT,

RESET, HOME, CURSOR FORWARD, CURSOR BACK, CURSOR UP, CURSOR DOWN,

INS C, DEL C, RESET, CURSOR SELECT, SYSTEM REQUEST, SOFT

DISCONNECT, HARD DISCONNECT, REDISPLAY, PF keys 1-3, and function

keys F1-F24.

Remote terminal commands supported by Softerm include Cursor Home,

Cursor Forward, Audible Tone, Cursor Back, Tab, Cursor Down,

Vertical Address, Erase All, Carriage Return, Horizontal Address,

Printer On, Printer Off, Cursor Up, Set Video Attribute, Keyboard

Lock, Keyboard Unlock, Delete Character, Insert Character, Back

Tab, Conversational Mode, Newline, Print All, Status Line Enable

and Status Line Disable.

Softerm includes support for the Auxiliary Port if the terminal

configuration includes a printer.  If either a Printer ON or

Trans-parent Mode Enable remote command is received, all

subsequent data is sent to the printer until a Printer OFF command

is received.  If a Transparent Mode Disable command is received,

the Auxiliary Port will be returned to the state it was in before

the Transparent Mode Enable command was received.  In the

transparent mode data is sent only to the printer and not

displayed, otherwise the received data is both displayed and

printed.  The Auxiliary port may also be enabled by the Ctrl PrtSc

Softerm function which emulates the AUX Key capability of the

TYMNET78.

Tymnet78                     - 1 -             Terminal Emulation

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

������������� TYMNET78 Emulation Options ������������Ŀ

Version 1.04

Line Terminator Code: CR

Quick Display Mode: ON

Display Mode: Color

The emulation option Line Terminator Code allows the character

code(s) which terminate locally initiated function key sequences

as well as the Read Status and Transmit remote functions to be

defined.  Selecting CR will cause a carriage return ($0D) to

become the terminator.  Selecting a CR/ETX will cause a carriage

re-turn and end of text sequence to be the terminator and Softerm

will send $0D $03 after each function key sequence, status

message, or transmit function.  Selecting a CR/EOT will cause a

carriage return and end of transmission sequence to be the

terminator and Softerm will send $0D $04 after each function key

sequence, status message, or transmit function.

The emulation option Quick Display Mode may be toggled to On or

Off.  When set to On, display screens are created in the

background and then "popped" to the foreground display.  The

"Updating" indicator on the status line will flash while a new

screen is being created.  When set to Off, displays are created

normally in the foreground.

The emulation option Display Mode will only be available if the

machine contains a color graphics adapter.  This option determines

if the emulation will use the color or monochrome colors as

defined by TYMNET.

Operator Status Display

The status line can be displayed by pressing Alt T during online

terminal operation.  To return to the online mode after displaying

the status, enter Alt T or Alt Esc.  If you exit using the Alt T

key, the status line will remain on the screen during online

operation and will be automatically updated if required.  If you

exit from the status line display using the Alt Esc key, the

status line will be cleared.

TYMNET78 Keyboard Functions

The following table defines the standard keyboard functions for

the TYMNET78 terminal emulation.  The function key terminator code

is set by the emulation option Line Terminator Code described

previously.

Tymnet78                     - 2 -             Terminal Emulation

Keyboard		Function

Enter			CR, LF, or CR/LF

Shift Enter		Newline

Rt. arrow		Cursor Forward

Left arrow		Cursor Left

Up arrow		Cursor Up

Down arrow		Cursor Down

Backspace		BS or DEL

Shift Backspace	DEL or BS

Tab			Tab

Shift Tab		Back Tab

Home			Home Cursor

PgUp			Clear

End			Erase to End of Line/Field

Alt End		Erase all Input

Del			Delete Character (Line/Field)

Ins			Insert Character (Line/Field)

Shift PrtSc		Print ALL

Alt C			Cursor Select

Alt H			System Request

Alt R			Reset

Alt T			Display TYNET78 Status

Alt D			Soft Disconnect

Alt Ctrl D		Hard Disconnect

Alt Ctrl R		Redisplay

10 Function keys:

F1-F10		F1-F10

Shift F1-F10	F11-F20

Alt F1-F4	F21-F24

12 Function keys:

F1-F12		F1-F12

Shift F1-F12	F13-F24

Ctrl F1 - F3		PA1 - PA3

Tymnet78                     - 3 -             Terminal Emulation