Wyse WY-60

Description

The Softerm emulation of the Wyse WY-60 terminal supports all

general capabilities of the actual terminal including the full

128-character set, international character sets, 80 X 24 or 80 X

132 display area, reverse screen, 16 shifted/unshifted host/user

definable function keys, transparent/aux print, split screen, host

message area, shifted/unshifted labeling areas, host-defined

display area attributes, and programmable edit keys.

Features

The display format for the Wyse 60 is host or user selectable to

80 or 132 columns and 24, 25, 42, or 43 lines per page using

normal or reverse video.  If it is not configured for the Extended

Text option, Softerm supports the 132-column format using

horizontal scrolling.

Softerm supports all standard keys on the Wyse 60.

Commands not supported by Softerm are Screen Display On/Off,

Steady Block Cursor, and Steady Line Cursor.

Additional commands performed locally by Softerm are View Wyse 60

status line, View host message area, View Unshifted/Shifted

function key label areas, Select 80 or 132 column mode (normal or

reverse), Enable Block/Conversational modes, and enable

Full/Half/Local modes.

Wyse WY-60 Emulation Options

After the last standard screen in the Terminal Options mode in

Terminal Setup, the following screen will be displayed:

����������������������� Wyse 60 Emulation Options ����������������͸

�                                                                  �

�    Screen Width: 80 COL.                    Personality: WY 60   �

�    Lines / page: 24                     Local Edit Mode: Off     �

�     Page Length: 1 x Lines                 Received CR : CR      �

�     Status Line: Standard              End of Line Wrap: Off     �

�    Scroll Speed: Jump Scroll                Auto Scroll: On      �

�      Background: Dark                         Auto Page: Off     �

�        Send ACK: On                  Width Change Clear: Off     �

�       Block End: US/CR                         Language: US      �

�  WPRT Intensity: Dim                          Enter Key: CR      �

�    WPRT Reverse: Off                        Margin Bell: Off     �

�  WPRT Underline: Off                          Page Edit: Off     �

�                                                                  �

������������������������������� Tab Stops ������������������������Ĵ

�Columns                                                           �

�1-65   .......................................................... �

�       12345678901234..901234567890123456789012345678901234567890 �

�66-132 ...........................................................�

�       67890123456789..4567890123456789012345678901234567890123452�

�Use left/right arrow, enter t or T at tab stop, or space to clear �

������������������������������������������������������������������;

Wyse 60                      - 1 -             Terminal Emulation

The first three options work together and are summarized in the

table.

The Screen Width option may be toggled to 80 COL., 80 ECONOMY, or

132 COL.

The Lines / Page option may be toggled to 24, 25, 42 or 43 and

determines the number of data lines which will be displayed per

page.

The Page Length Option may be toggled to 1 x Lines, 2 x Lines, or

*.

�����������������������������������������������������������������͸

�                                                                 �

� Valid Page Configurations:                                      �

�                                                                 �

�			80/132 Column		Economy 80 Column �

� Data			Lines/	No. of		Lines/	No. of    �

� Lines	Multiplier	Page	Pages		Page	Pages     �

�                                                                 �

� 24	1 x Lines	24	  2		24	  3       �

�	2 x Lines	48	  1		48	  1       �

�	*		24 and	  2		24 and	  2       �

�			24			56                �

�                                                                 �

� 25	1 x Lines	25	  1		25	  3       �

�	2 x Lines	N/A			50	  1       �

�	*		N/A			25 and	  2       �

�						55                �

�                                                                 �

� 42	1 x Lines	42	  1		42	  1       �

�                                                                 �

� 43	1 x Lines	43	  1		43	  1       �

�����������������������������������������������������������������;

The Status Line option may be toggled to Standard, None or

Softerm.  If set to Standard, the terminal display will default to

using the Wyse 60 status line (described later).  If set to None,

the terminal screen will default to showing no status line.  If

set to Softerm, the terminal emulation will default to showing the

standard Softerm status line on row 25.  Note: If you have set the

Lines / Page option to 25 or 43, the status lines can be viewed by

pressing Alt T or Alt V, but they will be replaced by the last

display row as soon as a new character is received from the host.

The Scroll Speed option allows selection of one of five scrolling

types: Jump (default); 1 Row/Second; 2 Rows/Second; 4 Rows/Second;

and 8 Rows/Second.

The Background option may be toggled to Dark or Light.  If set to

Light, the display will have dark characters on a light

background.

The Send ACK option may be toggled to On or Off.  If set to On, an

ASCII ACK character is sent to the host after executing certain

commands.

Wyse 60                      - 2 -             Terminal Emulation

The Block End option determines the type of terminators to be used

when the terminal executes a Send command.  The options are US/CR

(default) or CRLF/ETX.  These terminators are sent to the host

system when: 1) An entire line of data has been sent and there is

still more to transmit.  Then either a CRLF or VS is sent; 2) An

entire line of data is not sent or data transmission is complete.

Then a CR or ETX is transmitted to indicate to the host that data

transmission is complete.

The WPRT Intensity option may be toggled to Dim, Normal or

Invisible and determines how write protected characters will

appear.  If set to Invisible, write-protected characters will not

appear on the display.

The WPRT Reverse option may be toggled to Off or On.  If set to

On, write-protected characters will appear in inverse video.

The WPRT Underline option may be toggled to Off or On.  If set to

On, write-protected characters will be underlined.

The Personality option may be toggled to WY 60 or PC TERM.  When

set to PC TERM, keys send scan codes rather than ASCII characters.

In addition, each key sends one code when it is pressed and

another (the down code with the high bit set) when it is released.

Note: The Personality may be toggled from the Online terminal mode

by pressing the Shift Alt Tab key combination.

The Local Edit Mode option may be toggled to On or Off If set to

On, the emulation acts in a block mode, whereby characters are

bufffered in display memory and are not sent to the host computer

until the Send key is pressed or the host requests data.  If set

to Off, the terminal sends data to the host character-by-

character.  This is also known as Conversational mode.

The Received CR option may be toggled to CR or CR,LF.  If set to

CR, the cursor will move to the beginning of the current line when

an ASCII CR character is received.  If set to CR,LF, the cursor

will move to the beginning of the next line.

The End of Line Wrap option may be toggled to Off or On.  When set

to On, the cursor will wrap to the next line when it reaches the

end of the line.  When set to Off, characters at the cursor

position are replaced when the cursor reaches the last display

column.

The Auto Scroll option determines whether or not the screen should

scroll up one line when a character is placed at the last column

of the last row of the active text segment.  If enabled, the

screen is scrolled up one line.  Otherwise, the cursor is

repositioned to the top of the same page.

The Auto Page option may be toggled to Off or On.  If set to Off,

when the cursor reaches the top or bottom of the page, it will

wrap on the page or the page will scroll, depending on the Auto

Scroll setting.  If set to On, a new page of memory will move onto

the screen.

Wyse 60                      - 3 -             Terminal Emulation

The Width Change Clear option may be toggled to Off or On.  If set

to Off, the screen is not cleared when executing a command to

change the number of columns (except when entering or leaving

ECONOMY 80 mode).  If set to on, the screen is cleared.

The Language option may be toggled to US, UK, German, French,

Spanish, or Danish.  You should select the language which matches

that of your keyboard.

The Enter Key option may be toggled to CR, CRLF or Tab and

determines the ASCII character(s) which will be transmitted when

the Enter key is pressed.

The Margin Bell option may be toggled to Off or On.  When set to

Off, the bell doesn't ring.  When set to On, the bell will ring

when the cursor reaches column 72 (80 column mode) or column 124

(132 column mode).

The Page Edit option may be toggled to Off or On.  When set to

Off, the editing functions affect only the cursor line.  When set

to On, the editing functions affect the entire page.

Softerm allows default tab stops to be defined which are set at

initialization, when Ctrl Break is pressed, or any control

sequence is received which would return the terminal to its

initial state.  The tab format consists of 2 string entry fields,

for columns 1-65, and 66-132.  Entering any

non-blank character, such as a T, will result in a tab stop being

set at the current location.

After the preceding options have been specified, press the Alt

Enter key and the following additional screen is displayed:

���������������� Wyse 60 Function Key Set Up �����������������͸

�                                                              �

�  Key  Direction  Function                     Label          �

�                                                              �

�  F1   Remote                                                 �

�  F2   Remote                                                 �

�  .                                                           �

�  .                                                           �

� F15   Remote                                                 �

� F16   Remote                                                 �

��������������������������������������������������������������;

Press Alt Enter again to display the following screen:

������������� Wyse 60 Shifted Function Key Set Up ������������͸

�                                                              �

�  Key  Direction  Function                     Label          �

�                                                              �

�  sF1  Remote                                                 �

�  sF2  Remote                                                 �

�  .                                                           �

�  .                                                           �

� sF15  Remote                                                 �

� sF16  Remote                                                 �

��������������������������������������������������������������;

Wyse 60                      - 4 -             Terminal Emulation

Softerm supports 16 unshifted and 16 shifted function keys, for a

total of 32 function keys.  Each function key field can accept up

to an 8-character definition.  If function keys are not redefined,

the default code values are transmitted.

To define a function key, press Enter or Tab to move the

highlighted cursor bar to the desired key.  Press the Space

bar to toggle the Direction field to either Remote, Local or

Normal.  If set to Remote, the function key definition will

be transmitted only to the host computer and will not appear

on your display (unless echoed back by the host).  If set to

Local, the function key definition will be sent only to your

local display.  If set to Normal, the function key definition

will be sent in a manner consistent with the terminal

setting.

Press Enter to move to the Function field.  You may enter as

many as 80 characters in this field.  The standard editing

keys can be used to edit the string.  Note: To enter certain

codes, such as Carriage Return or Line Feed, use the Control

key equivalents.  e.g.: Ctrl-M for Carriage Return and Ctrl-J

for Line Feed.

When finished entering the Function, press Enter to move to

the Label field.  You can enter up to 8 characters which will

be displayed on the last display row if you execute the U or

S options available from the Operator Status Line Display,

discussed below.

Operator Status Line Display

The Wyse 60 terminal emulation includes a status line which may be

displayed on the 25th line.

The status information displayed includes duplex status, 80 or 132

Column Mode, and Insert mode.

Pressing B toggles between Block and Character modes.  Pressing P

replaces the status line with the current contents of the Host

Message Area.  Pressing U replaces the current contents of Line 25

with the unshifted function key labels.  Pressing S replaces the

current contents of Line 25 with the shifted function key labels,

providing their access has not been disabled by the host program.

To return to the online mode after displaying the status, enter

Alt T or Alt Esc.  If you exit using the Alt T key combination,

and you are in 24 or 42 lines / page mode, the status line will

remain on the screen during online operation and will be updated

automatically if required.  If you exit from the status line

display using the Alt Esc key combination, the status line will be

cleared.

Wyse WY-60 Keyboard Functions

The following table defines the standard keyboard functions for

the Wyse WY-60 terminal emulation:

Wyse 60                      - 5 -             Terminal Emulation

Keyboard		Function

Enter			CR

Alt Enter		Send

Right Arrow		Cursor Right

Up Arrow		Cursor Up

Left Arrow		Cursor Left

Down Arrow		Cursor Down

End			Clear line

Alt End		Clear Screen

BackSpc		Back Space

Shift BackSpc		Del

Alt Shift Del		Delete Line

Del			Delete Character

Shift Tab		Back Tab

PgDn			Next Page

Ins			Insert Mode Off

Alt Ins		Insert Mode On

Alt Shift Ins		Insert Line

PgUp			Previous Page

Alt S			Display Shifted Labels

Alt F			Funct Key

Ctrl PrtSc		Aux Print On/Off

Alt T			View W60 Status Line

Alt V			View Softerm Status Ln

Alt Shift Up Arrow	Scroll Faster

Alt Shift Down Arrow	Scroll Slower

Local Scroll Keys:

Alt Ctrl Left or Right Arrow	Scroll Left or Right

Alt Ctrl Up Arrow		Pan Up In Page

Alt Ctrl Down Arrow		Pan Down in Page

Function Keys:

F1 -F10		Function Keys 1-10

Alt F1 -F6		Function Keys 11-16

Edit keys located on the keypad cannot be shifted.  Press Alt key

to shift these keys.  Other edit keys (Tab, BackSp, Esc) can be

shifted normally.  Function Keys can be shifted.

PC Compatible Mode Functions

Shift Alt Tab		Toggle PC mode

or Wyse 60 mode

Shift Ctrl Tab		Send SYSTEM REQ

Shift Ctrl 1		Utility Functions Menu

Shift Ctrl 2		SOFTERM Functions Menu

Shift Ctrl B		Send BREAK

Alt   Ctrl B		Send LONG BREAK

In PC mode, emulation sends keyboard scan codes to host.  In Wyse

60 mode, emulation sends ASCII characters.

All other functions are disabled in PC mode.

Wyse 60                      - 6 -             Terminal Emulation