******************************************************************

DESIGN CENTER - 6.1 PRODUCTION RELEASE

******************************************************************

This file describes enhancements and modifications made to the 6.1

release of the Design Center programs.  Any changes that have not made

it into the Design Center 6.1 documentation can also be found in this

file.

Hard copy documentation of major new features has also been provided for

some platforms and configurations.  Be sure to refer to that

information, as well, paying particular attention to any items that are

marked with a change bar.

******************************************************************

CONTENTS

******************************************************************

PART 1.  NEW FEATURES	3

PART 2.  INSTALLATION CONSIDERATIONS	4

2.1.  SOLARIS INSTALLATIONS	4

2.2.  WINDOWS INSTALLATIONS	4

2.3. WIN32S SETUP	5

2.4.  DISABLING WIN32S	5

2.5.  EXTENDED DOS VERSIONS	6

2.6.  NETWORK INSTALLATIONS - EXTENDED DOS AND WINDOWS	6

2.7. HP9000/700 INSTALLATION - INSTALLING FROM A CD-ROM	6

PART 3.  PSPICE AND PLOGIC	7

3.1. ALGORITHM CHANGE	7

3.2. PSPICE COMMAND LINE OPTION	7

3.3. CHANGES TO .FUNC COMMAND	7

3.4. NEW .VECTOR COMMAND	8

3.5. NEW SIMULATION FUNCTIONS	8

3.6. NEW E DEVICE PARAMETERS	8

3.7. I AND V DEVICE CHANGES	8

3.8. STIMULUS PARAMETER	8

3.9. BSIM3 MODEL SUPPORTED	8

3.9.1.  Model Level 5	9

3.10. T DEVICE	9

3.11.  INITIALIZING CAPACITORS AND INDUCTORS	9

3.11.1.  Schematic Version	9

3.11.2.  Classic Version	10

PART 4.  PROBE	10

4.1.   WINDOWS PROBE	10

4.1.1.   Single Point Data Sections	10

4.2.   NEW FEATURE:	11

4.3.   NEW FEATURE:	11

4.4. SUPERVGA 1280X1024 SUPPORT (EXTENDED DOS PROBE)	11

4.5. SUN/HP PROBE	12

4.6. WINDOWS PROBE ON THE SUN AND HP (WITHOUT SCHEMATICS)	12

4.6.1. Tracking Simulation Errors	12

PART 5.  SCHEMATICS	12

5.1.  USER INTERFACE CHANGES (WINDOWS AND MOTIF)	12

5.1.1.   New Motif Shortcut Keys for the Symbol Editor	12

5.2.   SCHEMATICS DOCUMENT WINDOWS	13

5.2.1.   Individual Document Window Commands	13

5.2.2.   Global Window Controls	13

5.2.3.  VDC and VAC Symbols	14

5.2.4.  Vectorpoints	14

5.2.5.  OPTPARAM Symbol	14

5.3.  SYMBOL EDITOR	14

5.3.1.  The Symbol Editor Document Window	14

5.3.2.  Invoking the Symbol Editor	15

5.4.  NEW PSEUDOCOMPONENT SYMBOLS - SPECIAL.SLB	15

5.4.1.  OPTPARAM	15

5.4.2.  VECTOR	15

5.5.  NEW MENU COMMANDS	15

5.5.1.  Using Adobe Type Manager with Schematics	15

5.6.  NEW MSIM.INI PARAMETER FOR STMED	16

5.7.  SCHEMATIC EDITOR KNOWN PROBLEMS WITH VERSION 6.1	16

5.8. FILESTIM (SOURCE.SLB)	16

5.9. KBREAK INDUCTOR COUPLING SYMBOL (MAGNETIC.SLB, KBREAK.SLB)	17

PART 6.  CONTROL SHELL FOR EXTENDED DOS VERSIONS	18

PART 7.  DEVICE EQUATIONS OPTION	18

7.1.  CHANGES IN REQUIREMENTS	18

7.2.  WINDOWS	19

7.3.  SUN	19

PART 8.  POLARIS SIGNAL INTEGRITY	19

8.1.  INITIALIZING POLARIS - EXTERNAL MODE	19

8.2.  USING POLARIS IN EXTERNAL MODE	19

PART 9.  NEW LIBRARIES AND DEVICES	20

9.1.  NEW DIGITAL LIBRARY NAMES	20

PART 10.  PARTS PROGRAM	21

10.1.  NEW WINDOWS PARTS	21

10.2.  PARAMETERIZED MODELS	21

10.2.1.  Converting Model Files to MDT Format	21

10.2.2.  Menu Structure	22

PART 11.  FILTER DESIGNER	22

PART 12.  MISCELLANEOUS	22

12.1.  SYSTEM REQUIREMENT CHANGES	22

12.2.  PSPICE BOOK LIST	22

12.3.  GASFET LEVEL 3 MODEL PARAMETER EXTRACTION	22

******************************************************************

Part 1.  New Features

******************************************************************

DESIGN CENTER

* Full Win32S release to run under both Windows 3.1 and Windows NT.

* Cadence integration for Sun systems released.

* Expanded libraries now with over 9,900 devices, now including

parts from:

* F. W. Bell Hall-Effect generators (fwbell.lib)

* Sandia National Laboratories analog models including:

SCRs (thyristor.lib)

HEX Schmitt-Trigger Inverters (misc.lib)

Voltage Regulators (linear.lib, opamp.slb, .plb)

diodes & Rectifiers (diode.lib)

Zener Diodes (diode.lib)

Pin Diodes (diode.lib)

Voltage References (linear.lib, opamp.slb, .plb)

Power MOSFETs (pwrmos.lib)

operational Amplifiers (linear.lib, opamp.slb, .plb)

JFETs (jfet.lib)

Voltage Comparators (linear.lib, opamp.slb, .plb)

Power Bipolar Transistors (pwrbjt.lib)

Bipolar Transistors (bipolar.lib)

Switch-Mode Regulators (swit_reg.lib)

For a complete list of the devices, see our BBS.

* New Paragon product for Analog Performance Optimization.

* New Internet connection to Technical Support; address is

Tech.Support@MicroSim.com

SCHEMATICS (Microsoft Windows and Sun OpenWindows platforms)

* Multiple Document Interface for Schematics.

* New OPTPARAM symbol for interface to Paragon.

* Motif interface for Sun users.

* TrueType font support under Microsoft Windows.

* Font customization through the msim.ini file under Motif.

* Integration to Windows Parts for graphical model editing on supported

part types.

* The special.slb symbol library contains a new symbol called

VECTOR, which can be attached to wires or buses in order to

save their values to a file during simulation.

PSPICE/PLOGIC

* Level 5 modeling using the SPICE3 BSIM3 model added to PSpice.

* Analog behavioral modeling now supports the use of an IF-THEN-ELSE

construct.  .FUNC capability has also been made more flexible.

* The .VECTOR statement is used to create files containing digital

simulation results.  The file contains time and state values for

the circuit nodes specified in the statement.

* New digital stimulus syntax to allow repeat loops in FILE= construct.

CONTROL SHELL

* Support for user-customized menu commands.

PROBE

* Sun and HP MOTIF ports.

* MOTIF Probe enhancements: multiple plot windows and error messaging.

* Now there is the capability to view DC and AC sweep data that only

has one step.  If there are multiple sections, Probe automatically

initializes Performance Analysis.

* In goal functions, the start point, range, and value arguments for

level and x value can now be an expression of marked points.

STIMULUS EDITOR

* Windows stimulus editor has many new features including: the

addition of a clock device, the storing of the current stimuli

display information to be brought up automatically in the next edit

session, auto-scaling and ranging improvements, etc.

PARTS

* New native Windows version.

PLSYN

* Design Center/AMD introduced.

POLARIS

* Polaris now offers two operation modes, giving flexibility in the

design process.  Native Mode allows a design to be entered using

Schematics and the External Mode allows layouts to be imported

into the Design Center for verification.

******************************************************************

Part 2.  Installation Considerations

******************************************************************

2.1.  Solaris Installations

The tar command given in the Installation Guide for Solaris

installations is incorrect.  It should be the following:

tar xvo

2.2.  Windows Installations

Note: Installations of the Windows version of the Design Center requires

that you install the MicroSim security plug onto you computer's

parallel port.  This applies if you are installing the Design Center

for the first time.  If you are updating an older version of the

Design Center, you can continue to use your existing security plug.

For all Windows packages, you must install the WIN32s system files

before installing the MicroSim Design Center software.  To install the

WIN32s files, do the following:

1.     Insert the Microsoft WIN32s Installation Disk into your drive.

2.     From the system prompt, type A:setup (or B:setup) to run the

setup program from the disk.  (This can also be done from the Windows

File/Run command.)

3.     Follow the instructions on your screen to complete the

installation.

After you have completed installation of the WIN32s files, continue with

the installation of the Design Center files as documented in the

Installation Guide.

IMPORTANT NOTE: The Windows Network version of the Design Center is not

compatible with WindowsNT.

2.3. Win32s Setup

The Win32s Setup program creates\updates a win32.ini file in the

\windows\system directory with the following information:

[Win32s]

Setup=1

Version=1.1.0.0

[Nls]

AnsiCP=1252

[FreeCell]

Setup=1

Version=1.1.0.0

If [Win32s] indicates that Setup=1 then Win32s is fully installed.  The

[Win32s] section also lists the current installed version of Win32s.

The code page to be used for Win32s translation is identified in the

[Nls] information. The code page 1252 is used by default if the [Nls]

section is not present.

The win32s.ini file can be used by Win32 to indicate which Win32

applications were installed, and which Win32s version was installed when

the application was installed.  The [FreeCell] example indicates that

Win32s version 1.1 was installed.

2.4.  Disabling Win32s

The Win32s .dll files are loaded only when a Win32 application is

executed.  The Win32s VxD is loaded when Windows starts, but has little

memory overhead.  It should not be necessary to disable Win32s.  But, if

you must disable Win32s, or wish to do a clean reinstallation of Win32s,

take the following actions:

1. Remove the Win32s VxD line from the Windows system.ini file in the

[Enh386] section:

device=c:\windows\system\win32s\w32s.386

2. Delete the w32sys.dll and the win32s16.dll files from the

<windows>\system directory, and all files in the

<windows>\system\win32s subdirectory.

Note: <windows> is the Windows installation directory such as

C:\windows.

3. Restart Windows.

2.5.  Extended DOS Versions

The environment variable PSPICELIB used in previous releases has been

replaced by the environment variable SIMLIBPATH in this release.  This

variable can be set from within the Control Shell from the

Analysis/Analysis Options dialog.

Running the Control Shell under Microsoft Windows (as a DOS shell

application) is NOT recommended.

Note: If you already have a MicroSim security plug, and you are installing

a non-network version, you will continue to use the same serial port

security plug, as long as it has the correct ID number.  If you are

installing for the first time, you need to install the MicroSim security

plug onto your serial port.

2.6.  Network Installations - Extended DOS and Windows

If you are installing multiple, networked versions, each security plug

must be installed on a different license server.  Network installations

of the Extended DOS and Windows versions of the Design Center require

that you install the MicroSim security plug onto your computer's parallel

port.  This applies only if you are installing the Design Center for

the first time.  User's who are updating old versions of the Design

Center can continue to use their existing plug.

If you are installing different simulator versions (such as a combination of

PSpice and PLogic simulators) then each package must be installed in a

different directory.

For Extended DOS network installations, each user's msim.cfg file should

be located somewhere along the path of a local drive, and not on a

network drive, since the file is updated each time a user exits the

Control Shell.  When the Control Shell is invoked, it searches for the

msim.cfg file first in the current working directory, then along the

current path, then in the home directory (where ps.exe resides).

For Windows network installations, each user should invoke the simulator

(PSpice or PLogic) using the -wTEMP command line option to specify a

TEMP or TMP file on a local drive.  This ensures that temporary files

created during simulation do not interfere with the temporary files

created by other users on the network.

2.7. HP9000/700 Installation - Installing from a CD-ROM

1.  Create a temporary directory to load the installation script from

the CD-ROM.

mkdir <msim_temp>

2.  Navigate to the temporary directory you created with the previous

step.

cd <path_name_of_msim_temp>

3.  Insert the CD into the CD-ROM device.

4.  If necessary, create a mount point for the CD-ROM device,  for

example  /cdrom.  You may need to login as a super user to make this

directory.

mkdir /cdrom

5.  Mount the CD-ROM device.  You need root privileges to mount the

device.

/etc/mount -r -t cdfs /dev/dsk/c201d4s0 /cdrom

where X is the SCSI address of the CD-ROM device.

6.  Retrieve the install scripts and documentation by running the

following command:

tar xvfp /cdrom/INSTALL.HPX

7.  Run the installation script retrieved from the CD in the previous

step by typing :

./install

******************************************************************

Part 3.  PSpice and PLogic

******************************************************************

3.1. Algorithm Change

In release 6.1, the algorithm to determine the minimum internal time

step for transient analysis has been changed.  It does not affect the

validity of the discussion on "The Dynamic Range of TIME" in Appendix B

of the Circuit Analysis Reference Manual.  However, this change may

cause minor numerical differences in your simulation results.  For users

comparing Probe traces, the differences are generally smaller than

comparing .PRINT output values.  This is dues to the generally much

smaller time intervals in Probe data and the interpolation used for

.PRINT output.

3.2. PSpice Command Line Option

-wTEMP=<path>     allows you to specify a directory where PSpice can

write temporary files to.

3.3. Changes to .FUNC Command

In many applications it is convenient to have functions which are local

to a subcircuit.  Local functions are specified using the .FUNC command

within the body of a subcircuit definition (just like .PARAM).  See the

.FUNC command in the Commands chapter of the Circuit Analysis Reference

Manual for details on the general form.

3.4. New .VECTOR Command

The .VECTOR statement is used to create files containing digital

simulation results.  The file contains time and state values for the

circuit nodes specified in the statement.  The file format is identical

to that used by the digital file stimulus device (FSTIM).  Thus, the

results of one simulation can be used to drive inputs of a subsequent

simulation.  See the Commands chapter of the Circuit Analysis Reference

for more details.

3.5. New Simulation Functions

Many new functions have been added for use in PSpice expressions.  See

the Circuit Analysis Users Guide for a summary of functions and

operators.  Reference page varies according to platform as follows:

- Windows or Sun (with Schematics), refer to page 2-19.

- Sun or HP9000/700 (without Schematics), refer to page 4-13.

- IBM-PC Extended DOS, refer to page 5.14.

3.6. New E Device Parameters

The voltage-controlled voltage source E device has a new parameter

for frequency tables:

E<name> <(+) node> <(-) node> FREQ { <expression> } = [KEYWORD]

+ < <frequency value>,<magnitude value>,<phase value> >*

+ [DELAY = <delay value>]

See the E device specifications in the Analog Devices chapter of the

Circuit Analysis Reference Manual.

3.7. I and V Device Changes

The general form for the I and V devices has a STIMULUS parameter

which allows you to specify a stimulus name.  See the I and V devices in

the Analog Devices chapter of the Circuit Analysis Reference Manual for

more details.

3.8. STIMULUS parameter

The STIM device has a STIMULUS parameter and the command format

allows for REPEAT loops.  See the Digital Devices chapter in the

Circuit Analysis Reference Manual for more details.

3.9. BSIM3 Model Supported

3.9.1.  Model Level 5

The LEVEL=5 (BSIM3) model is a physical model with extensive built-in

dependencies of important dimensional and processing parameters.

It includes the major effects that are important to modeling deep-

submicrometer MOSFETs such as threshold voltage reduction, non-uniform

doping, mobility reduction due to the vertical field, bulk charge

effect, carrier velocity saturation, drain-induced barrier lowering

(DIBL), channel length modulation (CLM), hot-carrier-induced output

resistance reduction, subthreshold, conduction, source/drain parasitic

resistance, drain voltage reduction in LDD structure, etc.  More

detailed model information is available in the Analog Devices chapter of

the Circuit Analysis Reference Manual.

3.10. T Device

The transmission line device has an initial condition setting, IC.  See

the T device in the Analog Devices chapter of the Circuit Analysis

Reference Manual for more details.

3.11.  Initializing Capacitors and Inductors

3.11.1.  Schematic Version

The IC attribute allows initial conditions to be set on capacitors and

inductors for a transient analysis.  For these conditions to apply, you

must enable the Use Init. Conditions item in the

Analysis/Setup/Transient dialog.  This will skip the bias point

calculation and go directly to the transient analysis at TIME=0.  All

devices which do not have the IC attribute specified are assumed to have

an initial voltage or current of 0.

There is, however, one major caveat.  Because enabling Use Init.

Conditions on the transient analysis skips the bias point calculation,

convergence problems can occur.  If the pseudocomponents IC1 and IC2 are

used instead, this drawback is avoided since Use Init. Conditions should

be left disabled.  This method is recommended for capacitors.  In

particular, the IC2 pseudocomponent can be used to specify a voltage

across a capacitor (a PSpice extension to standard SPICE).

However, the IC1 and IC2 pseudocomponents do not support setting current

through an inductor.  When this is required, the IC attribute of the

inductor must be set and Use Init. Conditions must be enabled.  There

are, however, some work arounds that may apply to your design:

*  If there is a resistor in series with the inductor, the IC2

pseudocomponent can be used to set the voltage difference between the

two ends of the R-L series branch.  Since the inductor is a virtual

short circuit at DC, the initial current through it is V/R.

*  Place a piecewise linear current source (VPWL) in parallel with the

inductor to set its current at time=0, and switch it to zero current

immediately after the beginning of the analysis.  For instance, the VPWL

part instance might have its time-value attribute pairs set as follows:

T1 = 0            V1 = 30ma

T2 = 1ns          V2 = 0

3.11.2.  Classic Version

Capacitors and inductors may have initial conditions set on each element

for the transient analysis.  Capacitors may have an initial voltage, and

inductors may have an initial current.

To use these initial conditions, you must also use the UIC (Use Initial

Conditions) option on the .TRAN statement.  The UIC option will skip the

bias point calculation and go directly to the transient analysis at

TIME=0.  All devices which do not have an IC= specification are assumed

to have an initial value of 0.

There is, however, one major caveat.  Because enabling UIC on the

transient analysis skips the bias point calculation, convergence

problems can occur.  If the .IC statement is used, this drawback is

avoided since UIC should be left unspecified.  This method is

recommended for capacitors.  In particular, the .IC statement can be

used to specify a voltage across a capacitor (a PSpice extension to

standard SPICE).

However, .IC does not support setting current through an inductor.  When

this is required, the IC= attribute of the inductor must be set and UIC

must be specified.  There are, however, some work-arounds that may apply

to your design.

*  If there is a resistor in series with the inductor, the .IC statement

can be used to set the voltage difference between the two ends of the R-

L series branch.  Since the inductor is a virtual short circuit at DC,

the initial current through it is V/R.  For example, the following

circuit fragment is equivalent to defining 10\x11mA of current through

inductor L3 if nothing else is connected to node 8:

L3   5   8   20mH

R3   8   9   1k

.IC  V(5,9)=10

*  Place a piecewise linear current source in parallel with the inductor

to set its current at time=0, and switch it to zero current immediately

after the beginning of the analysis.  For instance, the following

circuit fragment defines an independent current source whose value is 30

mA at time 0 and 0 mA one nsec later:

L3   5   8   20mH

IL3  5   8   PWL  (0,30ma 1ns,0)

******************************************************************

Part 4.  Probe

******************************************************************

4.1.   Windows Probe

4.1.1.   Single Point Data Sections

Sections with only one X value (such as AC or DC sweep) can be

read into Probe.  The single-point data is displayed using a special

marker, similar to an inverted T.  When Tools/Options/Use Symbols is

enabled, the symbol is placed adjacent to the marker.

This "one point" data is displayed as a horizontal line 2 characters

wide (1 character width on each side of the point), with a vertical line

of 1/2 character height extending up from the point.

When displayed, symbols are drawn to the left or right of the

horizontal line.

The cursors can be turned on to display the exact value of the point.

If all the selected sections have one point data at the same x value,

then the x axis has only one tick mark, and one "value."

If all the sections selected from the .dat file have only one x value,

and that x value is the same for all sections, then Performance Analysis

is automatically started.  Additionally, once in Performance Analysis

under these conditions, it is not necessary to use goal functions (only

the trace name is required.  e.g., v(4)).

4.2.   New feature:

Digital trace names can be right or left justified.  The old way was

left justified only.  The new default is right justified.

To override the default and cause the digital names to be left

justified:

For DOS:

Set the environment variable DGTLNAMELEFTJUSTIFY=ON

For Windows:

Add DGTLNAMELEFTJUSTIFY=ON in the Probe section of the .ini file

There is no way to change this at run time (while the program is

running).

4.3.   New feature:

In Goal Functions, an expression of marked points can now be used as:

1. The startpoint of a search command.

2. Any of the arguments of the range specifier for a search command.

3. The value argument for the search commands LEVEL or XVALUE

See the Goal Functions chapter of Circuit Analysis Reference Manual for

more details.

4.4. SuperVGA 1280x1024 Support (Extended DOS Probe)

A new superVGA display resolution of 1280x1024 is now supported under

Extended DOS.  Use the SETUPDEV utility to set Display=VGA1280.  You

must have a display adapter and monitor that support this mode; you must

also install a VESA BIOS extension prior to running Probe.

4.5. Sun/HP Probe

Close Message Window (Sun and HP only) closes the Simulation Error

Message window.

4.6. Windows Probe on the Sun and HP (without Schematics)

On the Sun and HP platforms, Probe now supports multi-windowing and

simulation error tracking.

4.6.1. Tracking Simulation Errors

Probe provides an interactive message facility which allows you to

automatically associate errors that occurred during simulation with

their corresponding waveforms.  Error analysis can be initiated in one

of two ways:

1.     from the Simulation Message Summary dialog, or

2.     directly from the waveform display.

These methods are described in the next two sections. See the Waveform

Analysis chapter of the Circuit Analysis Users Guide for more details.

******************************************************************

Part 5.  Schematics

******************************************************************

5.1.  User Interface Changes (Windows and Motif)

The following keys are now supported under Schematics:

Windows     Motif        Action

<Del>       <Delete>     Edit/Delete

<Ctrl V>    <Paste>      Edit/Paste

<Ctrl X>    <Cut>        Edit/Cut

5.1.1.   New Motif Shortcut Keys for the Symbol Editor

<Control A>     View/Area

<Copy>          Edit/Copy

<Control D>     Part/Definition

<Control E>     Packaging/Edit

<Control F>     Edit/Flip

<Control G>     Part/Get

<Control H>     Edit/Change

<Control  I>    View/In

<Control L>     View/Redraw

<Control N>     View/Fit

<Control O>     View/Out

<Control P>     Part/Pin List

<Control R>     Edit/Rotate

<Control S>     File/Save

<Control T>     Edit/Pin Type

<Delete>        Edit/Delete

<Control U>     Edit/Undelete

<Paste>         Edit/Paste

<Cut>           Edit/Cut

5.2.   Schematics Document Windows

When you invoke Schematics, you will see a new, empty document window.

You can use this window to start creating a new schematic file.  If you

use File/Open (or if you select a recently used file from the File pull-

down menu) before you make any changes to the new file window, the file

will be opened in the current window.

If you have only one document window open, it is always the active

window.  Every action you perform within Schematics is performed on that

schematic file.  If you have more than one document window open,

however, one (and only one) window is the active window at any time.

Always make sure that you have activated the correct document window

before you start making changes from the keyboard.

When you are using multiple document windows to display different

portions of the same schematic file, all changes made in one window are

automatically made to the other window(s), as well.  For example, as a

wire is drawn, only the current window will show the intermediate

rubberbands.  However, once the wire is completed, any other window that

shows that portion of the schematic will be redrawn.

All of the document windows share a common paste buffer.  This allows

you to cut or copy from one window, and paste into another window,

whether or not it is from the same schematic file.  The Edit/Delete

command is used to delete symbols, wires, or selected areas without

changing the contents of the paste buffer.

5.2.1.   Individual Document Window Commands

The following commands and display controls apply only to the active

document window:

- zoom factor (set by the View commands)

- repeat commands and  buffer

- Analysis options (such as Analysis/Setup, and Analysis/Probe

Setup)

- Commands to run or invoke other programs, such as Probe,

Polaris, or Paragon

5.2.2.   Global Window Controls

The following options (selected through the Options Menu) control

settings for all open document windows:

Display options such as grid size and display

Auto Repeat

Auto Naming

Set Display Level

Editor Configuration settings

5.2.3.  VDC and VAC Symbols

New independent voltage source symbols in "source.slb".

Symbol Name          Attribute          Description

-----------          -------------     ----------------------

VDC                  DC                DC voltage value

VAC                  DC                DC voltage value

ACMAG             AC magnitude value

ACPHASE           AC phase value in degrees

5.2.4.  Vectorpoints

Pseudocomponents that write digital simulation results to a vector file

are called vectorpoints.  The vector file uses the same format as the

file stimulus device  (for syntax, see Stimulus Devices in the "Digital

Devices" chapter in the Circuit Analysis Reference Manual).  Thus, the

vector file can be used directly to drive inputs for another simulation.

Each vectorpoint is defined by placing a VECTOR symbol and attaching it

to a wire or bus at the output of a digital part instance.  For each

instance, one or more columns of state values are written to the vector

file.  Whenever any net with an attached VECTOR symbol changes state, a

line of time-value data is written to the vector file.

See the Schematic Capture Users Guide for more details on the VECTOR

symbol.

5.2.5.  OPTPARAM Symbol

The OPTPARAM pseudocomponent can also be used to define global

parameters, though its primary use is to define design parameters for

use with the Paragon analog optimizer option.  If you choose to use

OPTPARAM, one parameter can be defined by specifying the Name and

Initial Value fields.

5.3.  Symbol Editor

5.3.1.  The Symbol Editor Document Window

The Symbol Editor can be maximized to the full size of the Schematics

main window, or displayed in one of the document windows.  You can only

display one instance of the Symbol Editor at a time, and cannot edit

multiple symbols within the same window.

5.3.2.  Invoking the Symbol Editor

You can invoke the Symbol Editor using any of the following methods:

* From the Schematic Editor File Menu, select Edit Library.   You may

select a library file to edit with File/Open, or create a new one.

* You can select a part on a schematic to be edited and use Edit/Symbol

to begin editing the symbol.  This symbol becomes the current part, and

its library file becomes the current library file within the Symbol

Editor.

* You can invoke the Symbol Editor automatically when Schematics is

loaded by using the -sym option on the Schematics command line.

A new Symbol Editor document window will be created if one does not

exist already.  If you already have a Symbol Editor window open, you

will be prompted to save any unsaved changes before the new library is

loaded. When a symbol library is saved, each schematic document window

is updated to reflect any changes to the symbols that it uses.

5.4.  New Pseudocomponent Symbols - special.slb

The following symbols have been added:

5.4.1.  OPTPARAM

Defines simulation parameter values for optimization with the Paragon

Analog Optimizer.  Up to eight parameters can be specified per

optimization file. The symbol allows entry of the initial value, current

value, upper limit, lower limit, and tolerance for each parameter.

5.4.2.  VECTOR

Creates a file containing digital simulation results for the specified

circuit nodes.  The resulting file can then be used as digital input

through the FSTIM device.

The VECTOR symbol adds the following to the circuit file:

.VECTOR <number of nodes>

+ <node>*

+ <optional parameters>*

5.5.  New Menu Commands

See the Symbol Editor Reference chapter in the Schematic Capture Users

Guide for details on all of the new Symbol Editor commands.

5.5.1.  Using Adobe Type Manager with Schematics

If you have Adobe Type Manager installed and are using Schematics, you

may encounter problems with how rotated text is displayed.  We recommend

that you check the ATM.ini file and comment out the line

Modern=Helvetica

in the [ALIASES] section.  If this is not done, rotated text may be

positioned differently on the display versus on hard copies.  We also do

not recommend using the FONTTYPE=ADOBE setting in your msim.ini file at

this time as it has the same problem.

5.6.  New msim.ini Parameter for StmEd

A new parameter ALLOWAD has been added to msim.ini.  ALLOWAD may be set

to:

ALLOWAD=ANALOG

ALLOWAD=DIGITAL

ALLOWAD=BOTH 	(the default)

It also accepts  ANA, A, DIG, AD, A/D, and B.

This parameter limits the choices in the New Stimulus Dialog to the

specified category.  If the [STIMULUS EDITOR] section of msim.ini

contained the line ALLOWAD=Digital, the user would be limited to signal,

clock, and bus when creating new stimuli.  Also, the Stimulus/Copy

dialog would only display digital stimuli.

This .ini file directive is put into msim.ini automatically by the

install programs for our analog only and digital only packages.  It

prevents the user from seeing inappropriate dialog choices.  The user

should never need to modify it.

5.7.  Schematic Editor Known Problems with version 6.1

* Options/Editor Configuration/Fonts: Changing the font from Arial to

Arial Narrow does not affect the display.

* Edit/Attributes: The layer of a system attribute change does not get

saved.

* View/In: When using multiple windows, clicking in another window

leaves the cursor a '+' instead of an arrow.

5.8. FILESTIM (source.slb)

The FILESTIM symbol has been changed to have a variable width pin which

allows you connect a FILESTIM to a bus or wire.  You will need to

specify one or more signal names for the SIGNAL attribute (space

separated) corresponding to the width of the bus.

Due to this change, if you have existing schematics which use the

FILESTIM symbol, you will get a "Warning: invalid connections occurred"

message when you first load the schematic using the new source.slb

library.  In the errors list, you will see a message

"Pin not found:  pin 1 on symbol FILESTIM"

This is just a warning due to the fact the pin name was changed on the

symbol.  Simply save your schematic and the next time you reload the

warning should not occur.

5.9. KBREAK Inductor Coupling Symbol (magnetic.slb, breakout.slb)

The new inductor coupling symbols can be used to couple up to six

independent inductors on a schematic.

5.9.1  magnetic.slb

The Symbol Library file, magnetic.slb, contains one symbol for each

nonlinear magnetic core model in the model library file magnetic.lib.

To Use these Symbols:

1. Draw the schematic and assign the desired names (reference

designators) to all of the parts. (Reference designators are assigned

automatically, but can be changed by double-clicking on them.)

2. Select the coupling symbol for the desired CORE model from

magnetic.slb, and place one coupling symbol, anywhere on the schematic,

for each group of coupled inductors. (See section 5.9.3, below.)

These symbols have no pins; they are represented by the letter K

enclosed in a box.

3. Double-click on each coupling symbol (on the K-in-a-box, not the

attributes) and enter the reference designators for the coupled

inductors as the values for Li (i=1,2,...,6).

4. Set the value of the COUPLING attribute to the value of the coupling

factor, K.

5.9.2  breakout.slb (Kbreak)

A generic symbol, Kbreak, is provided in breakout.slb.  Kbreak does not

have a pre-assigned model attribute, and its corresponding model in

breakout.lib has no parameters.

To use this symbol, follow steps 1 through 4 in section 5.9.1 (in step 2,

select the symbol Kbreak from breakout.slb). If no other steps are taken,

the symbol will effect linear coupling between the inductors.

- To reference a CORE model in a configured model library file:

1. Single click on the symbol (K-in-a-box) to select it.

2. Select the Edit/Model/Edit Model Reference command.

3. Specify the desired model name.

- To define your own CORE model parameters:

1. Change the reference to KBREAK.

2. Select Edit/Model/Edit Instance Model (Text).

5.9.3 Important Notes

The "dot" convention for the coupling is related to the direction in

which the inductors are connected. The dot is always next to the first

pin to be netlisted. When the inductor symbol, L, which is shipped with

Schematics is placed without rotation, the "dotted" pin is the left one.

Edit/Rotate (<Ctrl R>) rotates the inductor +90deg, which makes this pin

the one at the bottom, etc.

Certain rules must be followed when setting the attributes for coupling

symbols and the inductors they affect.  Nonlinear CORE models may be

applied to one or more inductors, so:

* The L1 attribute must have a value (name); the other Li may be blank.

* The MODEL attribute must reference a CORE model.

* The VALUE attributes of the affected inductor symbol(s) must be set

to the number of windings (turns).

Linear coupling must be applied two or more inductors (Kbreak only):

* The L1 and at least one other Li attribute must have values (names);

the rest may be left blank.

* The MODEL attribute must be left blank.

* The values assigned to the inductor symbols must be in Henries.

******************************************************************

Part 6.  Control Shell for Extended DOS Versions

******************************************************************

You can specify up to three custom menu commands that can be used to

invoke DOS commands and programs from within the Control Shell.  When

configured, these commands appear in the pull-down menu for the Options

command.

There are five codes that can be used within command strings as path and

filename variables.  See the Control Shell chapter of the Circuit

Analysis Reference Manual for more details.

******************************************************************

Part 7.  Device Equations Option

******************************************************************

7.1.  Changes in Requirements

Device Equations, for Extended DOS PSpice, requires the WATCOM 9.5b

C/C++ compiler and the DOS/4GW extender version 1.95.

7.2.  Windows

The object and source files necessary to make the Windows version of

PSpice (PSPICE.EXE) are installed in a directory called DEVEQU.  A make

file, "MAKEFILE," is included to compile and link the program.

The object code assumes that you are using the Microsoft Visual C++

Version 1.5 compiler and that you have Microsoft's make utility, NMAKE.

NMAKE is a DOS program.

To create a new PSPICE.EXE, simply type NMAKE while in your development

directory.  This directory should also contain the file "msim.iff."

Once PSPICE.EXE is built, simply add it as an icon to the Design Center

program group.

For information on how to obtain the Microsoft compiler, call or write:

Microsoft Corporation

One Microsoft Way

Redmond, WA  98052-6399

Telephone:  (800) 426-9400

7.3.  Sun

The object files supplied can be linked using the make utility that

comes with the operating system.  Simply type make in the directory

where all the PSpice object files are located.  The object code,

however, assumes that you are using the SunPro SPARCompiler C 3.0.  For

information on how to obtain the SPARCompiler, contact Sun at:

Sun Microsystems, Inc.

2550 Garcia Avenue

Mountain View

CA 94043

(415) 960-1300

******************************************************************

Part 8.  Polaris Signal Integrity

******************************************************************

8.1.  Initializing Polaris - External Mode

If the External Mode is enabled when you select the Tools/Polaris

command Schematics will check to see if setup information from a prior

extraction is present in the schematic.  If the setup information is

present the Extract and Simulate w/ Parasitics buttons will be enabled.

In addition to the setup information, you must have set up a simulation

analysis using the Analysis Setup option in the main Polaris dialog, or

by selecting the Analysis/Setup command.  For more information, see the

Signal Integrity Users Guide.

8.2.  Using Polaris in External Mode

While External Mode operation does allow you to analyze layouts that did

not originate in the Design Center, there are some restrictions when

using the Polaris in this manner.  In the Native Mode operation Polaris

merges parasitic information extracted from the PCB layout with

information about devices extracted from the schematic.  This allows you

to perform functional simulations of your design with board parasitics

included for signals of interest.  Such operation is not possible (or

feasible) when External Mode operation is enabled.  For more

information, see the Signal Integrity Users Guide.

******************************************************************

Part 9.  New Libraries and Devices

******************************************************************

9.1.  New Digital Library Names

* dig_1 thru dig_4 (.slb, .plb, and .lib) are now separated by family.

Each file name has an associated symbol (.slb), package (.plb), and

model (.lib) library.

File Name                      Contents of Library

--------------                 ----------------------------------------------------------

7400           7400-series TTL

74AC           Advanced CMOS

74ACT          TTL-compatible, Advanced CMOS

74ALS          Advanced Low-Power Schottky TTL

74AS           Advanced Schottky TTL

74F            FAST

74H            High-Speed TTL

74HCT          TTL-compatible, High-Speed CMOS

74HC           High-Speed CMOS

74L            Low-Power TTL

74LS           Low-Power Schottky TTL

74S            Schottky TTL

* dig_5.lib has been renamed to cd4000.lib, but there is no change to

the models.

* One new vendor library containing 30 models has been added:

fwbell.lib and fwbell.slb

This library contains models of Hall-effect devices made by F. W.

Bell.  These devices can produce a voltage proportional to the

surrounding magnetic field and the current flowing through them.

* There are over 800 new models in this release, including:

AMP connectors; opamps from Analog Devices, Burr-Brown, Elantec,

and National Semiconductor; power MOSFETs from Harris; RF MOSFETs

from Polyfet RF Devices; and Hall-effect generators from F. W.

Bell.

******************************************************************

Part 10.  Parts Program

******************************************************************

10.1.  New Windows Parts

The Parts program is now available on the Windows platform.  See the

Programs and Operations chapter of the Circuit Analysis Reference

Manual for details on how to start Windows Parts.  Also see the Parts

chapter in the Circuit Analysis Reference Manual for more details on

running the program.

10.2.  Parameterized Models

When Windows Parts creates a new model, it gives parameterized values a

value of -1.  For example if alpha = {test}, then Parts will generate a

value of alpha = -1.  The user needs to manually edit the parameter

value.

10.2.1.  Converting Model Files to MDT Format

The MODTOMDT utility in version 6.0 no longer exists, but the

functionality has been incorporated into the Parts program.

Model files (.mod or .lib) can be converted to the .mdt format.  Thus,

existing device model definitions can be pulled into Parts in order to

*     view performance curves,

*     run what-if analyses, and

*     create new model derivatives for use in simulation.

Conversions are supported for diodes, BJTs, JFETs, and level 3 MOSFETs.

To run the conversion:

1.    Invoke Parts by typing the following at the shell tool prompt:

parts

This will bring up the menu list of part types that can be modeled

and an entry to run the conversion.

2.    Select the Model to MDT file converter entry by typing A at the

Select prompt.

3.    At the prompt, enter the name of the .mod model file to be

converted (exclude the .mod extension because it is automatically

appended to the file name).  This file must reside in the current

working directory.

Note:  The converter expects only one device model per ".mod"

file.  If multiple model definitions are contained in the file,

only the first definition is converted.

10.2.2.  Menu Structure

See the Parts chapter in the Circuit Analysis Reference Manual for

details on all the new menu commands for Parts.

******************************************************************

Part 11.  Filter Designer

******************************************************************

Filter Designer has no modifications for this release.

******************************************************************

Part 12.  Miscellaneous

******************************************************************

12.1.  System Requirement Changes

The minimum memory requirements for our Windows Design Center is now 8

Mb.

12.2.  PSpice Book List

The following are new books added to the PSpice book list since 6.0.

* Walter Banzhaf, Computer-Aided Circuit Analysis Using SPICE, Prentice-

Hall, ISBN 0-130162579-9.

* Andrei Vladimirescu, The Spice Book, John Wiley & Sons, Inc., 1994

ISBN 0-471-60926-9.

* Martin Santen, PSpice Design Center Arbeitsbuch zur

Schaltungssimulation (PSpice Design Center, Workbook for Circuit

Simulation), HOSCHAR Systemelektronik GmbH, 1992.

* Hidehiko, Aoki, Introduction To Functional Circuit Designs in Analog

ICs, CQ Publishing, Co., Ltd., 1992, ISBN 4-7898-3291-0.

* Michio Okamura, Designing Transistor Circuits by SPICE, CQ Publishing

Co., Ltd., 1991, ISBN 4-7898-3292-9.

* Michio Okamura, Opamp Circuit Designs by SPICE, CQ Publishing Co.,

Ltd., 1992 ISBN 4-7898-3290-2.

12.3.  GASFET LEVEL 3 Model Parameter Extraction

We have recently received some information which indicated that BML -

Barnard Microsystems Limited has developed a parameter extraction

program for use in the extraction of the GASFET LEVEL 3 model in PSpice.

If you would like to contact them for information, they are located at:

134 Crouch Hill

London N8 9DX

Telephone: +44 (0)81 341 0566

Fax: +44 (0)81 341 0583