DXF3DS

Autodesk 3D Studio DXF Conversion Utility

Copyright 1991 Dan Silva

For distribution among 3D Studio users.  NOT FOR SALE.

Produced Exclusively for Autodesk, Inc.

by

Yost Group, Inc.

7-15-91

Converts DXF files to 3DS, SHP, or LFT files, and converts FLM

files to 3DS files.  This program addresses many of the

limitations of the built-in DXF translation that was included

with 3D Studio release 1.0.  The major features of DXF3DS include

the conversion of AutoCAD Blocks and Extruded lines, automatic

welding/unifying/smoothing of the resulting objects, better

handling of face normals, and the ability to convert AutoCAD 3D

polylines to Lofter path (.LFT) files.

Usage:

dxf3ds froon.dxf  ..Switches..

or

dxf3ds baz.flm  ..Switches..

Switches

/T  "to" file type: ( default to .3DS )

/TS       output .SHP file containing Shape.

/TL       output .LFT file containing Path.

/T3       output .3DS file containing Objects.

Note:     The resulting file will get written to

the same directory as the source DXF

file.  After conversion, you will want

to put this file in the appropriate 3D

Studio directory:  MESHES, SHAPES, or

LOFTS.  CAUTION -- this utility will

overwrite an existing file with the same

name.

The following switches only have meaning when creating

a .3DS file:

/O   object definition: ( default to "by layer" )

Note:  This only works with .dxf files...

It's invalid for .flm files (filmrolls

only convert by color).

/OE       by entity

/OC       by color

/OL       by layer

/Wnnn   auto-weld        Automatically welds coincident

vertices.

nnn = optional weld-threshold

in multiples of .00001 units.

Defaults to 1.

Note:  Welding (and subsequent

Unifying) can only take place

within a given 3DS "Object"

group.  Welding can not happen

between objects.  Remember

this when you create your

objects in AutoCAD, and

subsequently when you convert

the .dxf files into .3ds files

(by layer, color, or entity),

if you want them to be welded

and unified.

/U     auto-unify        Note:  Only welded objects may

have their surface normals

unified.  And only 3D entities

can be unified (ie, a 2D plane

floating in space cannot be

unified).  This is important

to note when you convert .dxf

files into .3ds files and make

the decision to bring them in

either by layer, by color, or

by entity.  Plus, although the

current unification code is

better than the 3DS 1.0 code,

it still requires you to

follow the golden rule:  the

object's topology must have

integrity. This means that

every edge must be shared by

two faces, except for edges at

the object's perimeter.

/Snn   auto-smooth       Automatically sets smoothing

groups for the objects based

on the angle between non-

coplanar edges.

nn =  optional edge angle

threshold.

Defaults to 30 degrees.

/Ann   arc-smoothness    Arc step size for polyline

conversion.

nn = step angle in degrees.

Defaults to 10 degrees.

(Note that this only needs to

be lowered if you want

smoother arcs as they're

converted from Acad polylines

into 3DS objects.  It'll most

likely never need to be raised

above 10-degrees, since that

will make arcs coarser.)

--------

Examples:

To convert from FROON.DXF to FROON.3DS, defining objects by

Entity, welding with a threshold of .001, auto-unifying surface

normals, smoothing with an angle threshold of 40 degrees, and

arc-smoothing with an angle threshold of 3 degrees:

dxf3ds froon.dxf /OE /W100 /U /S40 /A3

To convert from DWEEB.FLM to DWEEB.3DS, welding with a

default threshold of .00001, auto-unifying surface normals,

smoothing with a default angle threshold of 30 degrees, and arc-

smoothing with the default threshold of 10 degrees (arc smoothing

is done automatically, so no switch is needed to accept the

default):

dxf3ds dweeb.flm /w /u /s

To convert from DWEEB.DXF to DWEEB.SHP:  (note:  will only

convert 2D polylines)

dxf3ds dweeb.dxf /ts

To convert from DWEEB.DXF to DWEEB.LFT:  (note:  will only

convert the last found 3D polyline)

dxf3ds dweeb.dxf /tl

A sample batch file "DXF3DSL.BAT" is included to show you

how you might automate your conversion with preferred switches.

To convert DWEEB.DXF to DWEEB.3DS defining objects by layer,

with an auto-smoothing angle threshold of 35 degrees, autoweld

on, and auto-unification of surface normals, type:

dxf3dsl dweeb

dxf3dsl.bat contains:

dxf3ds %1.dxf /Ol /W /U /S35

You may want to modify this to customize it for your

particular needs.

_____________

Installation:

Two versions of this utility are included:

DXF3DS.EXE    -     INTEL version

DXF3DSW.EXE   -     WEITEK version for PC's that are

configured with only a WEITEK math

coprocessor.

______________

Helpful Notes:

1)   Some of the techniques as outlined in Appendix B of the 3D

Studio reference manual for working with DXF files no longer

apply.

-    You no longer have to explode AutoCAD AME meshes prior

to making the DXF file.

-    Welding and smoothing is now done automatically.

-    You no longer have to use Filmrolls if your AutoCAD

drawing contains Blocks and Extruded lines.

2)   DXF files produced by GENERIC CADD's 3DD are accepted by

this utility.

3)   Recommended procedure for Using 3D Studio to create

walkthroughs of architectural models produced in AutoCAD.

-    While in AutoCAD, create a 3D Polyline that defines a

camera path through your building.  If you have more

than one 3D Polyline in your model, turn off or freeze

the layers that contain the other polylines.

-    Create a DXF file.

-    Convert the DXF file to a 3DS file using DXF3DS.

-    Convert the DXF file to a .LFT file using DXF3DS /TL.

-    In the 3D Editor, assign materials and add lights.

-    In the Keyframer, use Get Path and select the .LFT file

that contains the converted 3D Polyline.  You may want

to assign a dummy to this path and link your camera and

target to the dummy.  (Note, if you want the camera to

track the target, you'll have to slide the camera keys

back in time so the camera is following the dummy.)

4)   After converting a DXF file to a 3DS file,  you may want to

break down the resulting 3D Studio objects into smaller

objects.  A technique for doing this is:

-    Use Select Face to select faces that you want to

convert into separate objects.

-    Use Create/Face/Detach  with the "SELECTED" button on.

This will prompt you for the name of the selected

objects.

5)   The following AutoCAD Entities in DXF files get converted to

3D Studio Objects:

-    3D faces and polygon meshes

-    Circles, solids, traces, doughnuts, polylines with

width, closed arcs, closed 2D polylines, and extruded

polylines

-    Any extruded entities

-    Polyface meshes from AutoCAD Release 11

-    AutoCAD blocks containing any of these entities

-    Entities on frozen or turned-off layers are ignored

6)   AutoCAD Advanced Modeling Extension (AME) note:  Remember

while still in AutoCAD to use SOLMESH on AME models prior to

exporting a DXF file for conversion to 3D Studio.

7)   As noted above, the successful unification of face normals

depends on the welding of coincident vertices.  Sometimes,

depending upon the precision of the model as it was created

in AutoCAD, the vertices may not be "coincident enough" for

proper face normal unification.  In this case, you may have

to increase the weld-threshold to account for any floating-

point round-off errors that occur when the converter looks

for coincident vertices.  This is especially true when

converting models produced in AutoCAD release 11 with the

AME (Advanced Modeling Extension) version 1.0.   This is

because AME 1.0, like 3D Studio, uses single precision

floating point calculations.

Example:  A DXF file was produced from an AutoCAD AME model

of a skateboard.  This DXF file was converted to a

3DS file.  The first time the default weld-

threshold was used, which resulted in some

incomplete surface-normal unification.   This DXF

file was converted again, but this time /W100 was

used -- which resulted in correct surface-normal

unification.

8)   Converting DXF files using the "By Layer" Object Definition

method can result in objects consisting of many elements.

In certain cases, some of these elements may have all of

their face normals flipped the wrong way.  This is

detectable in 3D Studio by selecting Backface in the

Display/Geometry command menu.  Use Surface/Normals/Element

Flip to correct this.  Note:  If you would rather not pay

attention to face normals when using 3D Studio to Render

AutoCAD-produced models, use 2-Sided Materials.

9)   If you are loading a large scene containing thousands of

entities (such as 3D faces) and have chosen to bring objects

in "by entity," it can take a tremendous amount of time to

create the file.  It will also create huge numbers of

objects which you'll have to wade through in 3D Studio.  We

recommend that you organize your DXF file in AutoCAD so that

these kinds of entities are grouped by layer, and that you

make the conversion into 3D Studio "by layer."

10)  If you have created a DXF file that doesn't successfully

load into 3D Studio after being converted by this utility,

please send us the file on a floppy disk with a document

describing how you made the file, (both the technique and

with what version of AutoCAD or other program), and what

exactly the nature of the problem is.  Send it to:

Autodesk 3D Studio DXF3DS tech support

Autodesk, Inc.

2320 Marinship Way

Sausalito, CA  94965

[end]