.              DS/CMD - by Bruce A. Tankleff...      FUNCTION -.              Initializes the TRS/LS-DOS operating system for double.              sided disks, allowing the FORMAT utility to query as to.              the type of drive being used for a format (1 or 2 sides),.              and obtain it's stepping rate.  DS/CMD also will (optionally).              patch FORMAT to make these two questions mandatory...      HARDWARE REQUIRED -.              Tandy TRS-80 Model 4, nlt 64k memory, nlt 1 DS disk drive..      SOFTWARE REQUIRED -.              TRS/LS-DOS version 06.0x.0x.      OTHER REQUIREMENTS -.              none..      COMMAND SYNTAX -.              -------------------------------------------------------.              :                    DS <*>                           :.              :                                                     :.              :       * = Optional parameter to patch FORMAT/CMD    :.              :                                                     :.              -------------------------------------------------------..      INSTRUCTIONS -.              By itself, DS acts as a toggle to switch back and forth.              between allowing the FORMAT queries and "normal" operations...              At DOS Ready, type DS <ENTER>.  The system should print the.              program's title, and then indicate that DS' condition.              is either ON or OFF.  Another DS will reverse it..              Either condition can be SYSGENed...              It should be noted that while double sided disks can be.              formatted without DS, the operator has to supply the.              SIDES and (in some cases) stepping rate parameters...              If you wish to patch FORMAT to ask for these each time,.              type DS * <ENTER>.  Both PATCH and FORMAT must be on line.              in the system, but not necessarily on drive 0..              ===> ONLY USE THE * PARAMETER ON A BACKUP!!!!!!! <===..              NOTE: Radio Shack does NOT currently support the.              operation of double sided diskettes...              This software is intended for the public domain, and may.              be freely distributed and copied.  The author is not.              responsible for this program's use, misuse, or abuse...              In case of questions, contact Bruce A. Tankleff on.              FORUM-80 of El Paso (915-755-1000, 300/1200bd),.              or via CompuServe (70765,1474)...11th Nov 1984 - 15.59