(Editor's note:  This is the original modification mentioned inHDBT63/DOC in issue #3 of TRSLINK.  Please refer to it in using       the instructions given for the LS-DOS 6.3 version).                        --------------                          HDBOOT/DOC     This file  describes  modifications  to  TRSDOS  6.2.1  toenable a TRS-80  Model 4P to  boot directly from  a Radio Shack5Meg hard disk drive.  You will need a file or sector editor tomodify your hard disk.                            NOTICE     Changing any operating  system,  especially at this level,is a dangerous  operation.  If  anything  in  this  file is notabsolutely clear to  you,  do NOT attempt  any modifications byyourself. The method in this file appears to work as described.However, I do not accept responsibility for any consequences ofusing the information in this file,  and any modifications mustbe made at your own risk.                      SYSTEM REQUIREMENTS  1) TRS-80 Model 4P ONLY, not Model 4 or 4D.  2) TRSDOS 6.2.1.  3) Radio Shack 5-meg hard  disk drive.  This method might     work  on  other   drives  with  a   WD1000  or  WD1010     controller, but has not been tested.     Any  hard   drive   used   must   meet  the  following     conditions:  TRSDOS logical drive :0 must lie entirely     under the first head  (head 1) on  the hard drive.  On     this drive,  logical cylinder  and sector numbers must     be identical to physical track and sector numbers.  4) Use of  Radio  Shack  hard  disk  drivers when running     under  TRSDOS  6.2.1.  Other  drivers  have  not  been     tested.                         INSTRUCTIONSAll references to drive  :0  refer  to  the  first logical (andphysical) drive on your hard disk.  1) Boot  the  hard  disk  system  with  your  usual  boot     diskette.  If there is  an AUTO  command on  your boot     diskette, write it down.  2) Backup all files from the hard disk onto floppies.  3) Use SYSGEN  to  create  a  CONFIG/SYS  file on logical     drive :0.  4) Fill in  the  values  marked  "xx,"  "yy,"  and "zz zz     zz..." in the table of changes to BOOT/SYS.  5) Using a  file or sector  editor,  CAREFULLY change the     files SYS0/SYS and  BOOT/SYS  on  logical  drive :0 as     specified in the tables below.     To restore the  original BOOT/SYS  and SYS0/SYS files,     either while changing them  or at any time afterwards,     reboot using  your boot  diskette and  backup SYS0/SYS     from the boot diskette to :0.  6) Exit  from  your  file  or  sector  editor  to "TRSDOS     Ready."  If there was an  AUTO  command  on  your boot     diskette, store it on drive :0.  7) Remove all  floppy  disks  from  the  drives and press     RESET.  In a  few seconds, you should see  he familiar     Tandy logo.  If so,  you  are  finished modifying your     system.  8) If your  system does  not boot  properly,  reboot with     your boot  diskette  (hold down  <F2>  while  pressing     RESET) and restore SYS0/SYS and BOOT/SYS from the boot     diskette to the hard disk (see step 5 above).  You may     either abandon the entire project or attempt it again.When using the modifications in  this file,  a few restrictionsapply:  1) The system will attempt to boot from the hard drive if     it is turned on.  To boot from a floppy, hold down the     <F2> key while pressing RESET.  2) When booting,  if the hard  drive is not completely up     to speed you may see either  a screenful of garbage or     "The floppy disk drive is not ready." Press RESET, and     the system should boot properly.  3) The CONFIG/SYS  file must be  loaded when booting from     the hard drive, and cannot be suppressed. If it cannot     be loaded, the system will not boot.     In that case,  you  must boot with  the boot diskette,     and use SYSGEN  to create  a valid  CONFIG/SYS file on     the hard disk's drive :0.     You may change your system  configuration at any time,     as long  as it  meets the  requirements above.  Merely     boot the system from the hard drive, establish the new     configuration, and re-SYSGEN.  4) When  booting from  the hard  drive,  you cannot enter     DEBUG by holding  down  the  <D>  key,  as  this would     prevent CONFIG/SYS from  being loaded.  To enter DEBUG     in this manner, boot from a floppy.     Of course,  once you are at the "TRSDOS Ready" prompt,     DEBUG may be used as desired.  5) An AUTO command  may be  installed on  the hard disk's     drive :0.  To fix a runaway AUTO command,  reboot with     your boot diskette.  6) Do not move  the modified SYS0/SYS  from hard drive :0     onto another  disk.  SYS0/SYS  and  BOOT/SYS have been     modified for hard  drive boot,  and  BACKUP will alter     the non-existent  bootstrap  step  rate  of  BOOT/SYS,     rendering  the  copy  unusable  for  booting  either a     floppy or hard disk system.                 CHANGES TO SYS0/SYS.LSIDOSRecord,Byte     From        To-----------     ----        --0C,B0           31 80 03    00 00 000C,FE           00 00 00    C3 86 1E0E,7A           C2 A0 19    00 00 000E,C0           C0          000E,F2           0C          FE                 CHANGES TO BOOT/SYS.LSIDOSRecord,Byte     From             To-----------     ----             --00,4D           12 FD...01 87    xx 00 00 00 00 00 00 0000,5A           06               yy00,9F           70 04            C3 4301,41           1E 00...B6 77    18 0D F9 43 zz zz zz zz                                 zz zz 00 00 00 00 0001,6F           C3 38 02         18 2C 0001,7C           96               C301,80           E6 1C            00 0001,F9           00 00...00 00    3E 09 90 28 AD AF C901,96 thru 01,DE        From:   01 F4 81 ... 18 10 FE        To:     00 00 00 00 00 00 00 21 43 43 11 71 04 01 08 00                ED B0 C3 38 02 F3 CD C3 43 3A 78 00 D3 84 FB 3E                05 C0 3A 79 04 92 3E 06 28 01 AF A7 C9 E5 D5 3E                85 D3 84 D3 9C 4B 5A 16 00 06 0C 3E 02 CF F5 AF                D3 9C 3E 86 D3 84 F1 D1 E1For  "xx," "yy," and "zz zz zz...,"  substitute the appropriatevalues for drive :0 of your hard disk system.xx = Sectors per cylinder, in hexadecimal.     example, thirty-two sectors per cylinder would be 20 hex.yy = Sectors per granule, in hexadecimal.     example, sixteen sectors per granule would be 10 hex.zz zz zz... = values of DCT+3 through DCT+8 for HD drive :0.Boot  the  hard disk system  with  your boot diskette,  and useDEBUG to examine the Drive Code table for drive :0.                          Adam Rubin                       CIS [71320,1052]                      September 27, 1986