How  to Modify  your      A Hardware Mod         TRS-80  Model 3 or 4      for your TRS-80       to accept   a double                           Sided  Floppy   Disk        Drive.   A  hardware      modification written      by David Arcand        I  was  at  a  local  computer  show in  the Hartford  /  Springfield  area and happened  to notice  these double sided  40 track  floppy  drives that they sell for IBM systems.  Now  I  knew that almost  all IBM  floppy drives would  work in my  TRS-80 Model 4  so I paid my $55.00 (plus  7% Conn sales tax)  and  walked away  with  a  new double  sided,  fourty  track,  floppy  disk  drive.    However,   upon  taking  it   home  I  discovered that it  would not work double  sided  like  my 4P  would.   Herein  lies  the  problem,   even  with   a   drive  physically  capable of  operating  double sided,  the  system  just  would  not operate  on the  second side  of the  floppy  disk.         Needless to  say  I found the problem, fixed it, and now  am reading  and  writing  double sided  floppy  disks  on  my  original Model 4.  This fix will also work on a Model 3.         So how  to do this you may ask ?  Simple, I just moved a  pin from a ground wire (pin 31) to the  side select wire (pin  32) on the floppy drive cable connector for the drive.         Now  the  technical types  have  probably already turned  off their  computer, torn off the cover, and removed most EMI  shields on  their  TRS-80 by  now.  But  for  the  benefit of  those folks who are  not so gifted, I'll go over  it  step by  step for you.         Before you even start,  you better have  a double  sided  floppy drive. Otherwise there is  really  no  point  in  even  doing this  modification.  If you do have it ready, then here  we go.   TOOLS NEEDED: First, you are going to need some tools:                 A number 2 phillips screwdriver.                A medium sized needle nose pliers.                A large set of VISE-GRIP pliers.   OPENING YOUR TRS-80: Remove the top of your TRS-80.                 Turn off your computer. (very important)                 Unplug your computer. (also very important)                 Remove the black screw  at the  back/top/middle                of the box.                 Turn your TRS-80 on its  back. (you may need to                unplug  any cables you might  have  pluged into                the bottom of your system)                 Remove  three black screws under  keyboard near                front.                 Remove  two long screws at middle of right/left                edges of bottom.                 Remove 5 screws near rear/bottom of TRS-80.                Turn your TRS-80 so it sits normally.                 Make room to the left of your  TRS-80  to place                top of computer.                 Left  top  of TRS-80 straight  up  *AND*  while                doing so,  look in through the top floppy drive                hole at  the back of the VIDIO tube. Make  sure                that  it does not  hang up  on the  wires while                lifting the  top  of  half of your TRS-80. When                the  top  half  of your TRS-80 is  clear of the                bottom,  rest it to  the left of the  system on                its left side.                 Remove the six screws holding on the  rear  EMI                shield.            Vola...           Your TRS-80 is sitting with its guts exposed !   REMOVE ORIGINAL FLOPPY DRIVES                           On the right side of  the drive tower are  four           screws  that hold  the  original floppy drives down.           Remove them.                           On the  left side  of the  drive tower adjacent           to  the power  supply  there  are  two other screws.           Remove  them.  (not  the  screws  holding the  power           supply  to  the  drive  tower  but  the  two  screws           between the front of  the  drive tower and the power           supply)                           Disconnect  the grey flat ribbon cable from the           back  of each drive.  Disconnect the four pin  power           cable   from  each   drive.  With   both  of   these           connectors  removed  from the floppy disk drive, you           may  remove both drives. (you will only need to take           out the top drive)   REMOVE FLOPPY DRIVE CONTROLLER CABLE                           The flat  grey  ribbon  cable  that pluged into           the  drives is  now  only  fastened by  being pluged           into   the  Floppy  Drive  Controller  (FDC)  module           behind the motherboard.  It  may  be  simply removed           by gently lifting  the  cable  and  rocking it  from           side to side until the cable is free.            NOTE:                     Be careful not  to rough up the flat white                cable from the motherboard  to the FDC as to do                so will require  that  it be  reconnected which                may require that the motherboard be  removed to                access the FDC.   EXAMINE THE DRIVE-CONTROLLER CABLE                 The  controller  cable  has three  connectors and is      twelve  inches long. Examine the middle connector and you      will  find  that it has three pins pulled and that it has      a slot in it between pin numbers four and five.           Connector facing toward you.         3432302826242220181614121008060402             3331292725232119171513110907050301     One hole type connector.              Two hole type connector.            The pulled pins  on the  middle  connector  will  be      pins 32,  14, and  10.  The two connectors on the end are      similar except that  one  of the FDC connector  does  not      have  the  slot  nor  the  pulled  pins while  the  other      DRIVE-0 connector has pin 10 intact but  pin  12 has been      pulled.                 The  odd numbered  pins in the  cable are used  as a      common ground  and are  all  tied  together  on both  the      floppy  drive and the  FDC  so  there is  a great deal of      duplication here.                 The even numbered pin have the following use:                      2> <unused>                      4> <unused>                      6> Drive 4 select. (IBM)                      8> Index Sector hole detect.                     10> Drive 1 select.                     12> Drive 2 select.                     14> Drive 3 select.                     16> Motor On.                     18> Direction select In/Out.                     20> Step In/Out.                     22> Write data.                     24> Write gate.                     26> Track 0 detect.                     28> Write Protect.                     30> Read Data.                     32> Head Select (IBM)/ Drive 4 select. (TRS-80)                     34> Enable Drive. (not always used)   MAKING THE MODIFICATION                           The  modification   is  only   done  to   those           connectors  which have  the  space in them and  have           pin number 32 missing.  The one connector on the end           with  all of  its pin and without the insert between           pins four and five is not to be  touched as this end           plugs into the FDC.                       This plug does not have the           slot filled in between pin             ||||||||||||||||   numbers four and five.              ||||||||||||||||   This is the FDC connector.               Flat Grey Cable                 ||||||||||||||||                 ||||||||||||||||<= This is the red stripe.             ||||||||||||||||                ||||||||||||||||                 ||||||||||||||||                 ||||||||||||||||              This connector has pins           10, 14, and 32 missing.             ||||||||||||||||   This is the drive number              ||||||||||||||||   one connector.              ||||||||||||||||                 ||||||||||||||||                 ||||||||||||||||                ||||||||||||||||                ||||||||||||||||                 ||||||||||||||||                 ||||||||||||||||   This connector has pins           12, 14, and 32 missing.           This is for drive zero.                 Pick which  drive position you wish to make the           modification  to.    Drive  0 is at one end  of  the           connector,  and drive 1 is the middle connector.  It           is  important that you  only do one  connector at  a           time.                           Pick one side of  the  connector to work on and           position  the vise-grips pliers  to hold on  to  the           drive  side of the connector near that side. Be sure           when  you attach  the  vice  grip  pliers  to  leave           enough room to  access  the corner  of the connector           where the  hole(s) is(are).   (Say around pin  2  or           pin 32)  Our intent  is  to  remove  the back of the           cable  strap to the  connector so you don't  want to           position the vise-grips on either side  of the cable           but  on  one side  only to grip the main part of the           connector. The  tension/pressure  of  the  vise-grip           pliers  should  be secure,  but not enough to  crush           the connector.                          Cable                    Vise-Grips                                              Connector                                                                                                                                                                        Using the point  of  the  needle  nose  pliers,           push at the  hole(s)  near  the  front  of the  vise           grips and you will  push out the cable strap  of the           connector.   Remove   the  cable  strap   from   the           connector and save it.                           You can now  gently remove the cable  from  the           connector.   Start at one  end and  of the connector           and slowly  lift one side  in  one continuous motion           so that the cable is removed from the connector.                           Save the cable in a  safe place and examine the           connector.  Notice  that  there are two pins removed           on one side of the connector,  and  another (pin 32)           removed on the other side.                           What you  will  do is to take your  needle nose           pliers  and pull the pin (#31) from  the rear of the           connector,  which  is  immediately below the  single           missing  pin, rotate it, and stick it into the empty           single pin position (#32).                           Now  turn the connector so  that  you may  face           the pins and  examine  the  pin number 32 which  you           just  installed.  Usually I  need  to  push the  pin           into position from  the  front of the connector with           the  needle nose  pliers which is usually  enough to           make it work fine.                           If  you are  changing  the drive # 1  connector           then you will replace  the cable  such  that  it  is           positioned closer to the drive # 0 connector.                           If you  are  changing the drive #  0  connector           then you  will  replace the cable such  that  it  is           positioned  slightly  closer  to  the   drive   #  1           connector.                           The  reason for  altering the  position of  the           replaced connector  is  that I  have had  unreliable           connections  when  replacing  the  connector in  its           original  holes.   On one  occasion, one or  more of           the wires within  the  flat grey  cable were cut and           this prevented the cable from operating properly.                           Once  the cable has been positioned properly in           the  top  strap,  position  the top  strap  (holding           tightly  to  the cable) onto the connector.  Push it           stiffly together so that the cable is  not likely to           move.                           Using the Vise-Grips  pliers, squeeze  the  top           strap  onto  the  connector.  You will  need  to  be           careful and apply  even pressure with the vise-grips           along the length  of  the connector.  If you fail to           apply even  pressure, you will snap the top strap of           the connector.   This  is  not  fatal, but  is  will           cause problems should  you ever need  to remove  the           top strap of the connector again.                                         Vise-Grips                                    Cable                                                                         To squeze the top             Connecter  strap back onto              the connector and             puncture the cable            to make proper a             connection.                                                           Now  turn the connector so  that  you  may face           the  pins and examine  the pin  number  32 which you           just  installed.   Usually  I need  to push the  pin           into position  from the front of the  connector with           the needle nose  pliers which  is  usually enough to           make it work fine.                           The  connector  has been modified.  Repeat this           section  for the other  connector on  the  cable  if           desired.                           When  done  making  all  modifications  to  the           cable, re-insert the  FDC end of the cable back onto           the FDC so that the cable flops over into the  drive           tower and  the  connectors  face  onto  the  drives.           This will only fit one way.   PUT THE SYSTEM BACK TOGETHER                           Re-attach  the rear EMI shield  to  the back of           the motherboard. Be sure that  you  do not pinch any           wires or cables.                           Fasten  down  the  floppy  drives  with   their           screws. (except if the drive is a  half height drive           with a twist lever to open it.)                           Lift the top of  the TRS-80  Model 3/4 case and           rotate  it  carefully  so  that you  are holding  it           above the  bottom half of the  case.   Look  through           the top drive hole in  the top half  of the  case to           view the rear of  the vidio tube and  make sure that           it does not bind up on the  cables  nor collide with           the motherboard shield.   Lower the top half  of the           case onto the bottom half.            NOTE:                If you have replaced  the  original drives with                a double sided  half height  drive which has  a                twist  handle,  then  you will need to lift the                front of the drive up  for a moment  while  you                lower the top  half down.  Failure to do so may                prevent  the case  from sliding past the  front                of the drive  due  to the length  of the handle                on the floppy drive.                           Lift the front  of  the case so that the TRS-80           Model 3/4 is sitting on its back.                           Insert  the three long black screws  into their           place under the  front  edge  near the keyboard near           the feet.                           Insert  one of the  two  long silver  screws on           each side of  the bottom of the  case in the  middle           holes   under   the  power  switch  and  the   vidio           adjustment knobs.                           Insert the five  medium  length screws into the           five holes  at the rear of the bottom  in the  screw           holes there.                           Turn the TRS-80 back right-side-up.                           Insert the  short  black  screw in the  hole at           the back of the case near the middle top.   TESTING THE MODIFICATIONS                           Plug in the TRS-80 computer.                           Turn  on  the  TRS-80  computer   and  BOOT  up           normally.                           Try to perform  a  directory of a disk in  each           drive  in the system.  At this  time,  you  are only           interested in checking that everything still works.                           To  check a  double sided drive  1,  2,  or  3,           insert  a  blank  floppy  disk  in  that  drive  and           attempt  to format it  for two sides.   A successful           format probably  indicates a good modification.  Now           set VERIFY (ON) and  attampt to copy a large file to           the  double  sided disk.  If  successful,  then  you           have  a good  modification.  Failure  of either  the           format  or the copy  indicates problems with  either           the modification or the drive.                           To check a  double  sided  drive  0, perform  a           SYSRES  of  overlay modules 1, 2, 3, 4, 10, and  12.           Perform  a  FORMAT  :0  and  when  the   LDOS/LS-DOS           questions appear about  density, sides, and  tracks,           insert a  blank floppy in drive  0.  If  the  floppy           formats properly,  then all is  well  with the drive           and the modification.                           Be aware that  some  special deal floppy drives           are  not the deal that they appear to be.  When  you           buy a floppy drive,  be sure to get  a guarantee.  I           have  bought many  floppy drives As-Is at a  *great*           price but  they did  not work  properly.  Some would           only  work  single sided,  some  only worked  for  a           short while, and most  didn't work at all.   You get           the floppy drive that you pay for and  I've replaced           my   original   tandy   drives   with   typical  IBM           replacements  for  only  $55  from   one  of   these           traveling  computer shows and sales.  So far  I have           three and intend to get three more.                                                                                     Make Mine                              a TRS-80 !