Modem Noise Killer     With this  circuit diagram,  some basic  tools including asoldering iron,  and four  or five components from Radio Shack,you should  be able  to cut  the noise/garbage  that appears onyour computer's screen.     I started  this project  out of  frustration at using a USRobotics 2400 baud modem and getting a fair amount of junk whenconnecting at  that speed.   Knowing  that capacitors make goodnoise filters, I threw this together.     This is  very easy  to build,  however conditions  may  bedifferent due  to modem  type, amount of line noise, old or newswitching equipment  (Bell's equipment),  and on and on.  So itmay not  work as  well for you in every case.  If it does work,or if  you've managed  to tweak it to your computer/modem setupI' d like to hear from you.     I'd also  appreciate any  of you  electronic  wizards  outthere wanting  to offer any improvements.  Let's make this workfor everyone.     Please read  this entire message and see if you understandit before you begin.     OK, what you'll need from Radio Shack:       1  #279-374 Modular  line cord if you don't already have          one.   You won't need one if your phone has a modular          plug in its base.  $4.95       1  Modular surface mount jack (4 or 6 conductor) $4.49       1  #271-1720 Potentiometer.   This  is a  5k audio taper          variable resistor.  $1.09       1  #272-1055 Capacitor.  Any non-polarized 1.0 to 1.5 uf          cap should  do.   Paper, Mylar,  or metal  film  caps          should be  used, although  #272-996 may  work as well          (272-996 is a non-polarized electrolytic cap) $.79       1  100 ohm resistor - quarter or half watt.  $.19     1    #279-357 Y-type  or duplex  modular connector.  Don't          buy this  until you've read the section on connecting          the Noise Killer below.  (A, B, or C) $4.95     First off,  open the modular block.  You normally just prythem open  with a  screwdriver.   Inside you'll  find up  to  6wires.  Very carefully cut out all but the green and red wires.The ones you'll be removing should be black, yellow, white, andblue.   These wires  won't be needed and may be in the way.  Socut them  as close  to where  they enter  the plug as possible.The other end of these wires have a spade lug connector that isscrewed into  the plastic.   Unscrew and remove that end of thewires as well.  Now, you should have two wires left.  Green andred.   Solder one  end of  the capacitor  to  the  green  wire.Solder the  other end of the capacitor to the center lug of thepotentiometer (there  are three  lugs on this critter).  Solderone end  of the  resistor to  the red  wire.   You may  want toshorten the  leads of the resistor first.  Solder the other endof the  resistor to either one of the remaining outside lugs ofthe potentiometer.   Doesn't  matter which.  Now to wrap it up,make a  hole in  the lid of the mod block to stick the shaft ofthe potentiometer through.  Don't make this hole dead center asthe other  parts may  not fit into the body of the mod block ifyou do.   See  how things  will fit  in order to find where thehole will  go.   Well, now that you've got it built you'll needto test  it.   First twist the shaft on the potentiometer untilit stops.  You won't know which way to turn it until later.  Itdoesn't matter which way now.  You also need to determine whereto plug  the Noise  Killer onto  the telephone line.  It can bedone by one of several ways:       A  If your  modem has two modular plugs in back, connect          the Noise  Killer into  one of them using a line cord          (a line cord is a straight cord that connects a phone          to the wall outlet; usually silver in color).       B  If your  phone is  modular, you  can unplug  the cord          from the back of it after you're on-line and plug the          cord into the Noise Killer.       C  You may  have to  buy a Y-type modular adaptor.  Plug          the adaptor  into a  wall outlet, plug the modem into          one side and the Noise Killer into the other.  Call a          BBS that  has known  noise problems.    After  you've          connected and  garbage begins  to  appear,  plug  the          Noise Killer  into the phone line as described above.          If you have turned the shaft on the potentiometer the          wrong way  you'll find out now.  You may get a lot of          garbage or  even disconnected.  If this happens, turn          the shaft the other way until it stops and try again.          If you don't notice much difference when you plug the          Noise Killer  in, that may be a good sign.  Type in a          few commands  and look  for garbage characters on the          screen.   If there  still is,  turn the  shaft slowly          until most of it is gone.  If nothing seems to happen          at all,  turn the  shaft slowly  from one side to the          other.     You  should   get  plenty  of  garbage  or          disconnected at  some point.   If  you don't,  reread          this message to make sure you've connected it right.                     End of original file      ---------------------------------------------------   ADDITION TO ORIGINAL FILE - 2/29/88 - Mike McCauley - CIS                          71505,1173     First, a  personal recommendation.   THIS  WORKS!   I havebeen plagued  with noise  at 2400  for some time.  I went roundand round  with Ma  Bell on it, and after they sent out several"repair persons"  who were,  to be kind, of limited help in thematter, I  threw in  the towel.   I saw this file on a board upeast a  few days  ago, and  thought I'd  bite.  Threw the gizmotogether in  about 10  minutes, took another five to adjust thepot for  best results  on my  worst connection, and guess what?No more worst connection! A few pointers:       1) The pot need not be either 5K or audio taper.  I used          a 10K  15 turn  trim pot.   Suggest  you use  what is          handy.       2) I  used   2MFD's  of   capacitance  (two   1MFD's  in          parallel).  0   Two  R.S   p/n  272-1055  work  fine.          Remember that about 90 Volts will appear across red &          green at ring, so the caps should be rated at 100 VDC          or more.       3) I ended  up with a final series resistance value (100          ohm +  pot) of  2.75K.   I speculate  that one  could          probably use  2MFD and  a fixed  2.7K resistor and do          the job  90% of  the time.  The adjustment of the pot          is not  very critical.  Changes of +/- 1K made little          difference in the performance of the circuit.     Hope it works as well for you as it did for me.                         Mike McCauley      ---------------------------------------------------      ADDITION TO ORIGINAL FILE - 1/17/1990 - VICTOR WONG     After I built the noise filter as described (I tried usingboth one capacitor and two capacitors), I found that while somenoise were  filtered out, there were still enough line noise toprevent me from using my modem at 2400 baud.     I decided  to see  what would  happen if  I had  used  tworesistors.  I hooked the two transistors (100 ohms and 1/2 watteach) in serial and found that this setup worked for me.     I couldn't  guarantee that  this setup  would solve  everyline noise  problem, since  I  built  only  one  filter.    Butresistors are  cheap, so  why not  give it  a try  and see whathappens.   I hope  this will  help some  people with their linenoise problem.  Good Luck!                          Victor Wong      --------------------------------------------------- ADDITION TO ORIGINAL FILE - 1/27/1990 - KARL FLICK - PRODIGY:                            SJHV74A     I CAN  HEAR...  I CAN HEAR!!!  This really DOES work.  Andvery well  too...  the noise was so bad here, I was usually cutoff if  I tried  anything but  local BBS.  I've tried 3 problemboards (long distance) since I built this thing and have had NONOISE AT ALL!  It took a little tweaking here and there: I useda 10k  resistor (#271-215a),  and could  not find  the  surfacemount jack  (#279-420) so instead I used a line cord (#279-391)and mounted  the whole  thing in  a project box.  It looks goodand works even better.     NOTE:   The quickest way to "tune" the thing is to plug itin, pick  up a phone, and listen.  Turn the knob until the dialtone becomes a little more quiet.  Continue turning the knob inthe same  direction until  it stops.  Next, call a BBS.  When Ifirst started,  so much  "noise" was  filtered, that  my  modemwouldn't even connect.       1  Turn the knob "up" a little.       2  Call the BBS again.     Repeat steps  1 and  2 until  there is a connect.  At thispoint there will probably be little or no noise at all.     Good Luck, and make sure to send this file far and wide sothat others may benefit!  Long live the Noise Buster!