Page -1-                                       ARC4 & ARC31                        Archive Marker, Header Type Code                               and Archive Header                                   DESCRIPTION          This  file describes  the  format  of  an  archive  file  header.  This     information was requested by Guy Omer and while  it is available in the     published source  code for MS-DOS  ARC, it would  take some  effort  to     locate  and  extract the information. ARC4  and ARC31 utilize the exact     same file  header format as MS-DOS ARC. The  information provided is of     limited usefulness even to programmers. In  the unlikely event that the     header information is damaged, very little repair  work would  actually     be possible.          Each entry in an  archive begins with a 1 byte archive  marker which is     set to 26. This marker is  followed by a 1 byte archive type code  from     zero to 9.               Archive Type Code     Meaning          -----------------     -------                   0            End marker, no more data should be read from                                the archive.                   1            Old style archive. Indicates no compression                                was used. (Obsolete)                   2            New style archive. Indicates no compression                                was used.                   3            Compression of repeated characters only -                                Packing.                   4            Compression of repeated characters plus                                Huffman squeezing.                   5            Lempel-Zev packing of repeated strings (old                                style). (12 bit static without non-repeat)                   6            Lempel-Zev packing of repeated strings (new                                style). (12 bit static with non-repeat)                   7            Lempel-Zev Williams packing with improved                                has function. (Internal to ARC)                   8            Dynamic Lempel-Zev packing with adaptive                                reset. The initial code  size is 9-bits with                                a maximum code size of 12 bits  (note 1). An                                adaptive reset is used on the  table when it                                becomes full. Non-repeat packing is used.                   9            Dynamic Lempel-Zev packing with adaptive                                reset. The  initial code size is 9-bits with                                a  maximum code size of 13 bits (note 2). An                                adaptive reset is  used on the table when it                                becomes  full.  Non-repeat  packing  is  not                                used.          Note 1:          The first byte  of the data area following the  header  is used to          indicate  the  maximum  code  size, however,  only  a  value of 12          (decimal) is currently used or accepted by existing ARC programs.                                                Page -2-                                       ARC4 & ARC31                        Archive Marker, Header Type Code                               and Archive Header                                   DESCRIPTION               Note 2:          Unlike  type 8 files, the first byte following  the file header is          actual data. No maximum code size is stored.          If the  header type code is in the range 2 to 9, then it is followed by     a standard archive header.  If the header type code is  one, then it is     followed by an older format archive header. The older format header did     not contain the true length of the file.          The standard archive header is composed as follows:          Mneomonic             Bytes            Description     ---------             -----            -----------     Name                  13 bytes         file name     Size                   4 bytes         current file size     date                   2 bytes         creation date     time                   2 bytes         creation time     crc                    2 bytes         cyclic redundancy check     length                 4 bytes         true file length               The general format for an ARC file is:          ((archive  mark + header version  + file header  +  file  data)....)  +     archive mark + end of archive mark.          Please refer any technical questions to:                                      David Huelsmann                               9112 Dempsey Dr NE                              Albuquerque, NM 87109                                           or to                                     SSE BBS                                  (714)963-7864                              1200-2400 bps anytime                           300 bps after 2400 Pacific                                           or to                               CIS PPN 71525,1363                                         ARC4 & ARC31                        Archive