-------- M O D E M - 8 0   D O C U M E N T A T I O N --------  MODEM-80 is a "Public Domain" program that enables fast anderror-free disk file transmissions between two TRS-80 Model Idisk systems and based on the Ward Christensen CP/M MODEM.COMprogram. It has been substantially rewritten by Stan Misel tooperate with the TRSDOS family of operating systems and it isNOT compatible with the CP/M version.------------------ OPERATING INSTRUCTIONS -------------------  MODEM-80 sets all of its options during program initializa-tion by use of a command string.  Enter the command string atDOS Ready prompt by typing MODEM, space, and then primary andsecondary options.Primary Options:  S - sends a file  R - receives a file  T - acts as a terminal  E - acts as a terminal with echo  H - lists abbreviated set of instructions  X - lists examplesSecondary Options:  T - enter terminal mode after file transfer  E - enter echo mode after file transfer  W - pause execution to allow user to switch disks  . - set baud rate  A - perform multiple file transfers without exiting program  Q - quiet mode - does not screen print status messages  R - show all data received  S - show all data transmitted  V - view file sent or received----------------------- FILE TRANSFER -----------------------  To transfer a file via MODEM-80 the person sending the filetypes "MODEM S FILENAME/EXT".   MODEM-80 automatically loads,initializes the RS-232 at 300 baud, and prepares to send file"FILENAME/EXT" to the receiver.  To receive a file via MODEM80 the person receiving the filetypes "MODEM R FILENAME/EXT". At this point the two computersare able to communicate with one another and the file will betransferred a sector at a time.  If a sector has been garbledduring transmission, it will be resent automatically.  If the receiver enters "MODEM RV FILENAME/EXT", the file isalso displayed on the screen while it is received.  If the receiver enters "MODEM RR FILENAME/EXT", then statusmessages as well as header and checksum info are displayed.  If a printer is linked to the display it might be desirablenot have status messages displayed during a file transfer. Ifthis is the case, use the "Q" suboption.-------------------------------------------------------------  MODEM-80 also has a "dumb terminal" mode that is entered bytyping "MODEM T".  In this mode MODEM-80 allows communicationwith BBSs and timeshare computers. Communication with anotherTRS-80 also using MODEM-80, requires one of them to be in theecho mode which is invoked by typing "MODEM E".-------------------------------------------------------------  Terminal and echo modes may also be entered upon completionof a program transfer by using suboption "T" or "E". This canbe especially handy when usIng MODEM-80 with a BBS.-------------------------------------------------------------  If you have only one drive and the file you wish to send isnot on the same disk as MODEM-80 enter suboption "W" to allowswitching disks before MODEM-80 opens the disk files.                    *   W A R N I N G   *              When utilizing suboption W ensure              disk in Drive 0 contains same DOS              otherwise system will most likely              bomb and possibly damage the disk-------------------------------------------------------------  Suboption "A" allows multiple file transfers without returnto DOS after each transfer. After the first file transfer thesystem prompts for a new filespec.  Press <ENTER> in responseto "new filespec" to abort.-------------------------------------------------------------  The TRS-80 RS-232 is normally set at 300 baud. MODEM-80 canmodify this value with suboption "."      The variations are:  .F - 1200 baud operation  .S - 110 baud operation  .0 - Skip previously established RS-232 settings-------------------------------------------------------------To abort MODEM-80 hold down CLEAR - all files will be closed!-------------------------------------------------------------A cassette-port-driven amplifier will prompt when MODEM-80requires your attention!-------------------------------------------------------------###################