PDRAW INSTRUCTIONS  B. N. Briggs - 14 Allan Berry Avenue, Napier - New Zealand.PDRAW is based on a clever program written by Jim Clark, andpublished in 80 MICRO July 1984.  The program as it stood waswritten for the RADIO SHACK series of printers, which are arare beast in this part of the world.  At the request of afriend in another town, I rewrote it to work on any of the STARGEMINI 10X.  However it should work on any of the EPSONcompatable series of Printers.  The program has beenconsiderably enhanced with a lot of extra features.  It is nowlargely self prompting as instructions are built in, howeversome brief explanations may help you get started.One point that should be mentioned before someone asks.  Youwill note when a board is printed, that some hoizontal Linesare thicker than others.  This is because tha original programwas written for the Radio Shack Printers which have a 1/100"advance.  The EPSON uses a main advance of 1/72" and this madeit necessary to 'juggle' line feeds to retain the correct I/Cpin spacing.USING PDRAW:-When the program has loaded, it will show a screen full ofGarbage.  This is random bytes in the two Buffers.  Press '0'(Zero) to display the Instructions.  Then Press 'CL'  and<ENTER> which will clear the Buffer. This will place you in theDRAW mode. The 4 Arrow keys control the (Non Destructive)cursor movement.  You will note the border of '1's around thescreen.  This is to enable you to tell which Buffer (PrintedCircuit Side) is in being used.  Also some 'crosshairs' havebeen added to aid in the placement of components in theBuffers.  These will 'NOT' show in the final printout.To DRAW lines, hold the <ENTER> key and the Arrow key for thedirection you wish to Draw.To ERASE lines, hold the <CLEAR> key and any Arrow key.PDRAW COMMAND MODES:-NOTE:- Most Commands are NOT EXECUTED until the key that waspressed is released. (This does NOT apply to the <SHIFT> key.)<SHIFT>........Toggle between the 2 Buffers.  Buffer One has               a border of '1's.  Buffer Two will have '2's.               To produce a double sided PCB, place a socket               at any location.  Press <SHIFT> and place the               same one. Repeat this for each Socket in turn.               This will duplicate the placement of all major               components in each Buffer."P" 1,2 or A...Will Print side 1,2 or Both Sides."CL"...........Will clear both print buffers."S" 1 - 9".....Change the speed of the cursor's movement over               the screen. This also controls the speed at               which some of the other commands operate."X"............Return to the Drawing Mode."E" ...........And <ENTER> - Return to DOS."DW"...........Save 1,2 or both Print Buffers to Disk."DR"...........Load previously save file to Original Buffer.DRAW MODE COMMANDS :-"C"............Copy and store the screen in a buffer.               This enables you to repeat the present screen               pattern at other locations in the buffer."D"............Restore saved Pattern."R"............Reverse the displayed side and create a Mirror               Image.  When this is done the Border of '1's               become 'A's and the '2's become 'B's."A"............Print a 10 pin edge connector at CursorLocation."B"............Print a 40 pin edge connector at CursorLocation.SOCKETS :-"E"............Print an 8 pin socket at Cursor Location."1"............Print a 14 pin socket at Cursor Location."2"............Print a 16 pin socket at Cursor Location."3"............Print a 20 pin socket at Cursor Location."4"............Print a 24 pin socket at Cursor Location."5"............Print a 40 pin socket at Cursor Location."6"............Print a single Pad at Cursor location."7"............Remove last change from the screen.The screen reversal feature is to allow you to draw a 2 sidedPrinted board oriented as you would visualise it, but reverseit before Printing to ease the production of a contact negativefrom the Printout.