TRS-80 SmartWatch Hardware and Software Installation and Use The three programs CLK1/CMD, CLK3/CMD and CLK4/CMD are copyrighted by Duane M. Saylor, 854 Eads Court, Newport News, VA.  As such, they may not be sold as a separate item or included with another software package or hardware device. The programs are being distributed as "freeware".  That is, they may be copied and distributed to others freely.  If a user of these utilities finds them useful, he is asked to make a contribution to the author of $10.00 or more as his conscience dictates. Since I do not have unlimited funds for Compuserve access, I suggest you direct your questions to me on my BBS system known as the VolksBoard at 804-220 0003.  I will answer some questions on Compuserve but if it begins to require too much online time my support will be limited to questions on the VolksBoard. Additions and modifications for Model I installation and creation of CLK1/CMD by Michael Keller (71620,447).  Please address any Model I-related questions to me on CIS or on my BBS IKE at 918/744-9456. Gate-array Model 4 information provided by Michael Phillips (73347,765). INSTALLATION INSTRUCTIONS FOR SMARTWATCH IN A MODEL I 1.  Connect a jumper from pin 26 to pin 28 on the SmartWatch.  This provides 5 volts for pin 28 of the SmartWatch. 2.  For Model I's with two-chip Level II on the main board, remove the 4K ROM chip located at Z34.  This chip should have a part number suffixed with 32; the 8K chip to the left (away from the Z80) should be suffixed with 64. For Model I's with a satellite Level II board (whether two- or three-chip) skip step 4. 3.  Insert SmartWatch into ROM socket Z34 with pins 1,2,27 & 28 hanging over the end of the 24 pin socket.  Pin 3 of the SmartWatch plugs into pin 1 of the ROM socket. 4.  Insert ROM into SmartWatch Socket such that it is aligned with the ROM socket on the main circuit board.  Note that pin 1 of the 4K ROM will be in pin 3 of the SmartWatch socket which in turn is in pin 1 of the socket on the main circuit board. 5.  This completes the installation of the SmartWatch in a Model I. INSTALLATION INSTRUCTIONS FOR SMARTWATCH IN A MODEL III 1.  Connect a jumper from pin 26 to pin 28 on the SmartWatch.  This provides 5 volts for pin 28 of the SmartWatch. 2.  Remove ROM C from the Model III.  This should be U106 and is the rightmost ROM of the 3 ROM's in the bottom right corner of the main circuit board when viewed from the rear.  The ROM should be a Motorola MCM68A316. 3.  Insert SmartWatch into ROM socket with pins 1,2,27 & 28 hanging over the end of the 24 pin socket.  Pin 3 of the SmartWatch plugs into pin 1 of the ROM socket. 4.  Insert ROM into SmartWatch Socket such that it is aligned with the ROM socket on the main circuit board.  Note that pin 1 of the Boot ROM will be in pin 3 of the SmartWatch socket which in turn is in pin 1 of the socket on the main circuit board. 5.  This completes the installation of the SmartWatch in a Model III. INSTALLATION INSTRUCTIONS FOR SMARTWATCH IN A MODEL 4 1.  Connect a jumper from pin 26 to pin 28 on the SmartWatch.  This provides 5 volts for pin 28 of the SmartWatch. 2.  Remove ROM C from the Model 4.  This should be U70 on the non-gate array version and is the rightmost ROM of the 3 ROM's below the Z-80 chip.  The ROM should be a Motorola MCM68A316. The gate-array Model 4 ROM C should be U4, in the upper left portion of the board. This chip is actually ROM B and ROM C embedded on one chip. 3.  Insert SmartWatch into ROM socket with pins 1,2,27 & 28 hanging over the end of the 24 pin socket.  Pin 3 of the SmartWatch plugs into pin 1 of the ROM socket. 4.  Insert ROM into SmartWatch Socket such that it is aligned with the ROM socket on the main circuit board.  Note that pin 1 of the ROM will be in pin 3 of the SmartWatch socket which in turn is in pin 1 of the socket on the main circuit board. 5.  This completes the installation of the SmartWatch in a Model 4. INSTALLATION INSTRUCTIONS FOR SMARTWATCH IN A MODEL 4P 1.  Connect a jumper from pin 26 to pin 28 on the SmartWatch.  This provides 5 volts for pin 28 of the SmartWatch. 2.  Bend pin 20 of SmartWatch so it will not go in pin 18 of ROM socket. 3.  Jumper pin 20 to pin 14 of SmartWatch.   This provides a ground for pin 20 of the SmartWatch. 4.  Remove Boot ROM from 4P.  This will be U69 on the gate array version and U70 on the nongate array version.  The ROM should be a Motorola MCM68A332. The Tandy part number on the gate array version is 8075332.  I do not know what the part number is for the nongate array version. 5.  Insert SmartWatch into ROM socket with pins 1,2,27 & 28 hanging over the end of the 24 pin socket.  Pin 3 of the SmartWatch plugs into pin 1 of the ROM socket. 6.  Bend pin 18 of the Boot ROM so it will not insert into pin 20 of the SmartWatch.  Insert ROM into SmartWatch Socket such that it is aligned with the ROM socket on the main circuit board.  Note that pin 1 of the Boot ROM will be in pin 3 of the SmartWatch socket which in turn is in pin 1 of the socket on the main circuit board. 7.  If you are installing the SmartWatch in a gate array 4P, connect a short piece of wire from pin 18 (The pin you bent out in step 6 above) of the Boot ROM to pin 18 of U27.  U27 is a 74LS244 Octal Buffer.  This connects A11 (Address line 11) to the Boot ROM. 8.  If you are installing the SmartWatch in a nongate array 4P, connect a short piece of wire from pin 18 (The pin you bent out in step 6 above) of the Boot ROM to pin 18 of U26.  U26 is a 74LS244 Octal Buffer.   This connects A11 (Address line 11) to the Boot ROM. 9.  This completes the installation of the SmartWatch in a Model 4P. SOFTWARE UTILITY FOR THE SMARTWATCH IN A TRS-80. The names of the 3 SmartWatch utilities are as follows: CLK1    -  For Model I under LDOS 5.x.x CLK3    -  For Model III, Model 4 and 4P under LDOS 5.x.x CLK4    -  For Model 4 and 4P under TRSDOS 6.x and LS-DOS 6.x CLK will be used as the name of the SmartWatch utility used in the remaining discussion.  Substitute the correct name as listed above for your machine and operating system. The first order of business after you install the SmartWatch in your computer is to set the time into the SmartWatch.  This is done with the command: CLK S hhmmssMMDDYYdm Where: hh = hours (0-23) mm = minutes (0-59) ss = seconds (0-59) MM = month (1-12) DD = date (1-31) YY = year (0-99) d  = day of week (1=Sunday, 7=Saturday) m  = mode (A=AM, P=PM, omit for 24 hour format) Since the TRS-80 operating systems use 24 hour time you should omit the 'm' parameter when setting the time in the SmartWatch. Once you have set the time you need to disable the system DATE and TIME prompts.  If you are operating in a hard disk or memdisk environment you must first make the boot floppy your drive 0.  The issue the command "SYSTEM (DATE=NO)" and "SYSTEM (TIME=NO)".  Then you can "AUTO CLK T".  At this point you can turn off the computer, then turn it back on and boot.  The SmartWatch time will be displayed and your system time will be set.  Alternately if you normally start with a /JCL file of some sort, just add the command "CLK T" to the beginning of your startup /JCL file. Another available command is "CLK O" which will turn off the SmartWatch. This will save the battery if you are going to be keeping the computer idle for a year or so!  To restart it you will have to use the "CLK s hhmmssMMDDYYd" command. The last command is "CLK".  This will display a list of available commands. If you issue any command with the SmartWatch not installed or installed improperly you will get a "No SmartWatch Present" message. If you turn the SmartWatch off and then issue a command you will get a "The SmartWatch is not Set" message. SUPPORT OF "ALIEN" OPERATING SYSTEMS Using CLK1/CMD and CLK3/CMD with NEWDOS 80 Version 2.0: CLK will run fine under NEWDOS 80.  To turn off the prompt for Date and Time just issue the command "SYSTEM :0 AY=N AZ=N".  Then do an "AUTO CLK T" to force automatic running of the time setting utility. Using CLK1/CMD and CLK3/CMD with DOSPLUS: CLK will run fine under DOSPLUS 3.3, 3.4 and 3.5.  Just do an "AUTO CLK T" to force the automatic running of the time setting utility.  If you are using DOSPLUS 3.5 you will also have to do a "SYSTEM (DATE=NO,TIME=NO) to turn off the prompts for Date and Time. Using CLK3/CMD with TRSDOS 1.2 and 1.3 For those of you that still run TRSDOS 1.2 or 1.3 on occasion for some unknown reason, I have also done the necessary research to permit use of CLK3/CMD with both TRSDOS 1.2 and 1.3. TRSDOS 1.2 must be patched as follows to turn off the prompt for Date and Time during a cold boot: PATCH *00 (ADD=4EAB,FIND=CA154FED53B542,CHG=ED53B542C3204F) TRSDOS 1.3 must be patched as follows to turn off the prompt for Date and Time during a cold boot: PATCH *00 (ADD=4EA9,FIND=CA2E4FED53B542,CHG=ED53B542C3394F) CLK3 must also be patched to use a different system call for display of strings.  This can be accomplished with: PATCH CLK3/CMD (ADD=524B,FIND=6744,CHG=1B02)