NOTES ABOUT VOICE DEMO DISK      THIS DISK IS TO DEMO THE VOICE OUTPUT PROGRAM ANDTO SHOW HOW VOICE CAN ADD TO YOUR PROGRAMS. WITH VOICE PROMPTSIN YOUR PROGRAMS IT IS NOT NECESSARY TO ALWAYS WATCH THE TUBE.     THE HEXDEMO PROGRAM SHOWS THAT SOME COMPUTER OPERATIONSCOULD BE DONE EVEN WITHOUT A MONITOR; OR BY A BLIND PERSON.GAMES WOULD ALSO BE SPICED UP WITH REAL VOICE PROMPTS. OR HOWABOUT A TALKING CLOCK THAT REMINDS YOU WHEN YOUR FAVORITE TVPROGRAM IS ABOUT TO START. ALL IT TAKES IS A BIT OF MEMORY ANDSOME PROGRAMING SKILL. THE VOICE COMES OUT THE CASSETTE WRITEPORT AND MAY BE CONNECTED TO ANY SIMPLE AMPLIFIER.     THIS DEMO DISK IS DESIGNED TO RUN IN A 32 K SYSTEM AND HAS22 DIFFERENT WORDS WHICH TAKE ABOUT 9.62 K OF MEMORY. THE SMALLMACHINE LANGUAGE DRIVER USES ANOTHER 54 BYTES AND IS AT THESTART OF THE WORD FILE (HEX/CMD) . TO LOAD THE WORDS JUST TYPE'HEX' FORM 'DOS READY'. WHEN YOU GO TO BASIC SET MEMORY AT39225. THEN RUN 'HEXDEMO/BAS' OR 'DECIDUMP/BAS' TO SEE WHAT ACHANGE VOICE CAN MAKE TO A PROGRAM.     THE ENTRY POINT FOR THE USR ROUTINE IS AT 9840H AND THEARGUMENT YOU PASS IS THE ADDRESS OF THE WORD YOU WANT TO VOICE.THE WORD ENTRY POINTS ARE IN THE DATA STATEMENTS AT THE END OFTHE PROGRAMS. HEXDEMO HAS SOME DUPLICATES IN THE DATA LINES TOMAKE IT EASIER TO DECODE SOME OF THE KEYBOARD ENTRIES.     ALSO NOTE THAT YOU MAY MOVE THE WORD FILE OR THE DRIVERFOR DIFFERENT MEMORY SIZES. JUST REMEMBER TO ADJUST THE USRADDRESS AND ARGUMENT TO MATCH THE NEW LOCATION.     THIS DISK CONTAINS THE NUMBERS 0 TO 9 AND THE FOLLOWINGWORDS-- 'AARGH' 'PLUS' 'MINUS' 'POINT' 'ENTER' AND 'ERROR'.      HAVE FUN WITH YOUR NEW TOOL, DICK BARKER