Graf/DCT is a utility that allows a Model III, or a Model 4 in the Model III mode to use the memory on a Hi-Res Graphics board as a RAM disk simulation (GrafDISK).

The /HEX file is an ASCII hex file, and needs to be converted (BINHEX) before using.  The /SRC file was created using the Radio Shack ALDS package, and needs to be assembled.  In either case, the resulting file should be given the extension of /DCT.

The Graf/DCT is installed using:  SYSTEM (DRIVE=n,DRIVER="GRAF").  From this point, the driver will prompt for all necessary data.

Originally this utility was put on the LSI SIG as GrafDISK in October of 1984.  Since then, one user pointed out some inconsistencies in the structure of GrafDISK as tested by DIRCHECK from LSI.  Starting with the changes necessary to allow GrafDISK to pass the DIRCHECK test, the capabilities of Graf/DCT were expanded to allow the Graf/DCT to be used as the system drive, and to allow images of the GrafDISK to be saved and restored.

Graf/DCT now has 5 options available:

0)  Disable GrafDISK

(the drive specified must be the GrafDISK)

1)  Format GrafDISK (16 Cyl, 1 Sec/Gran, 8 Gran/Cyl)

(the drive specified must be disabled)

(no active drive can be the GrafDISK)

2)  Format GrafDISK (16 Cyl, 2 Sec/Gran, 4 Gran/Cyl)

(the drive specified must be disabled)

(no active drive can be the GrafDISK)

3)  Load GrafDISK Image

(if the drive specified is actinve, it must be the GrafDISK)

(if the drive specified is disabled, no active drive can be the GrafDISK)

(if no extension is specified, the default is /CIM)

4)  Save GrafDISK Image

(the drive specified must be the GrafDISK)

(if no extension is specified, the default is /CIM)

By changing the size of the tracks, the full 32K of the Graphics RAM is utilized, where previously, only 30K could be used.  It is possible to configure the GrafDISK to be used as the system drive.  There is not enough room on the drive for all the /SYS files, and there are not enough sectors in a track to properly install SYS4/SYS, SYS5/SYS, and SYS12/SYS.  The /SYS files are put in reserved locations in the directory, and these reserved HIT locations are not valid directory entry locations.  SYS0/SYS is not needed, and without SYS5/SYS, SYS9/SYS cannot be used.  I have successfully been using a system disk consisting of: SYS1/SYS, SYS2/SYS, SYS3/SYS, SYS6/SYS, SYS7/SYS, SYS8/SYS, SYS10/SYS, and SYS11/SYS.  I have SYS4/SYS resident, and when DEBUG is necessary, I can always change the system disk.  This arrangement works especially well on my Model 4P, where I have only the two floppy drives.

When the GrafDISK image is saved, the DCT associated with it is also saved.  In this way, it is not necessary to remember how the GrafDISK was formatted to ensure proper "image load" operation, as the DCT is "restored" at the same time.  If a previously saved GrafDISK image is being loaded, it is not necessary to format the GrafDISK first.  If the GrafDISK is not already active, it will be formated before the image file is loaded.  I have also added a dummy BOOT/SYS entry to the directory of GrafDISK, as this seems to be necessary if GrafDISK is to be used as a system drive.  The BOOT/SYS file does not actually exist, but is mapped onto track 0 (in nonexistant memory).  The only times that BOOT/SYS are actually used are upon boot (which GrafDISK cannot do), and if the directory is not where the DCT thinks it is (which should not happen either), so a nonexistant BOOT/SYS should cause no trouble.

David Hall [71056,422]