MODEL 4/4D/4P RAMDISK PROGRAM PLUS MODEL 4P MEMORY              MODIFICATION FOR GATE ARRAY BOARDS                        by Dave Huffman         1345 Williams Street, Des Moines, Iowa  50317  Telephone:  (515) 266-1759  (7-10 P.M. Central time only!)     [EDITOR'S NOTE:  This article really covers two relatedtopics.  One is a RAMDISK program that can be used withNEWDOS/80 on any Model 4/4D/4P that has more than 64K of memory(even if it's just a stock 128K machine).  The other is ahardware modification to the Model 4P with gate array typecircuit board.  Although this modification is not extremelycomplicated, we recommend that you attempt it ONLY if you areexperienced in making this type of hardware modification.  Wehave not tested this modification, and because of this itshould be considered experimental.  The author of this articleand/or The Alternate Source will NOT be responsible for damagesof ANY kind that may occur as a result of your attempting tobuild and use this modification.]     How about expanding your Model 4P to 512K RAM or even to 2Meg?  Sound expensive?  Well, for the 4P we will need ten ICchips, a roll of wire wrap wire and tool, and 120ns 256K RAMs(Yeah, I know 150ns will work just as well, but... if you wantto speed up the machine faster that 4 Mhz then get the 120ns).I use NEC RAMs and have had good luck.  The last set I boughtwere $3.54 each, so a 512K system should cost you $75.     This modification is for the GATE ARRAY board ONLY.  Ifyou do not have a gate array board but have schematics for theboard that you have, the logic that is used here will alsoapply, however, the chip numbers will change.  Actually, theoriginal memory modification was first done on my old Model 4,which has 1 Meg installed.     We will need to add a new port and it was nice of Tandy toleave two decoded address on U28.  I used U28-10 (94H) as theRAM control port.     The way this modification works is simple.  In the Model4/4P the alternate 64K RAM is accessed using port 84H, and thiswill not change after the modification is installed.  Port 94Hwill select which 64K group within the 2048K (maximum) RAM willbe used as the alternate 64K RAM bank that is accessed by port84H.  So we have five bits in port 94H (D0-D4).  This will giveus one to 31 pages of 64K RAM that we can select for port 84Hto access (the upper three bits of port 94H are used by otherhardware modifications designed by the author, primarily whenthe Z-80 CPU is replaced by a Hitachi HD64180 [see NORTHERNBYTES Volume 7, Number 3, page 6].  Bit D5 is used for speedselection when the CPU is run at more than 4 MHz, and Bit D7selects whether memory is to be mapped in 64K or 256K segments.These additional modifications are not discussed here, but theadditional bit usage of Port 94H is mentioned here in the hopethat possible software compatibility problems can beforestalled).     To be fully compatible with all Model 4 software, port 94Hmust have at least one of the lower five bits set.  Forexample, if you set 94H to a 1 then this will enable the secondgroup of 64K addressing in the first 256K RAM to be used forthe alternate 64K RAM bank (32K high & 32K low).  When youfirst power up the machine, you can go to BASIC and issue aPOKE &H94,1 command if you are going to run a program thatrequires 128K of memory.  You could also AUTO a short machinelanguage program that contains a  LD A,1  instruction followedby an  OUT (94H),A  instruction, or it might be possible topatch the boot sector of your DOS to automatically issue thiscommand (the really ambitious hardware hacker could probablycopy the boot ROM to an EPROM and add some code to flip a bitof port 94H, but that's beyond the scope of this article).     To get more than 512K RAM we will need to stack RAM chipsand bend up pin 15 of the stacked chips.  This is the CAS*control line and will be connected to the 74S08 chip in thedrawings.  In the 4P, however, you will need to cut away someof the metal shield so the RAMs will clear.  You can get onemeg in a Model 4 without cutting any metal.  You MAY be able toget one meg in a 4P without any cutting, if you keep everythingnice and tight and are careful that no pins touch the metalshield (it might not hurt to stick a strip or two of insulatingtape on the shield at the point where it might otherwisecontact the IC pins).  If you do cut any part of the shieldaway, it may permit Radio Frequency Interference leakage inviolation of Federal Communications Commission rules andregulations, so you should try to fashion some sort of metalbox-like enclosure to fit over the cut-away part in order tokeep the shielding effect intact.     I don't own a Model 4P (don't get worried!) so JerryJohnson's 4P was the first guinea pig.  Jerry says it nice tohave 352 Grans (512k) for a system disk.     The installation for the 4P is as follows (refer to thediagrams):     1. Open up the machine and remove the main CPU board.     2. Mount the following IC chips on the board.  We willpiggyback them, and ONLY solder the pins that we don't bend up:      #1 74LS32 - U105P - bend all pins except 7&14      #2 7486   - U24P  - bend all pins except 7&14      #3 74S08  - U129P - bend all pins except 7&14      #4 74LS32 - U87P  - bend all pins except 7&14      #5 74S08  - U114P - bend all pins except 7&14      #6 74157  - U111P - bend all pins except 8,15&16      #7 74157  - U110P - bend all pins except 1,8,15&16      #8 74LS244- U125P - bend all pins except                               3,5,7,9,10,12,14,16,18,20      #10 7442  - U116P - bend all pins except 8,16      (#9 is not mounted until step 4!)     3. Connect the following:                      U85-16 to U24P-2                      U85-12 to U105P-9                      U85-15 to U105P-10     4. Place the 74LS273 (#9) on U85 with all pins bent upexcept 1,3,4,7,8,10,13,14,17,18,20.     5. Make all connections shown in Figure 1 (only!) exceptU114P-3&6 and U129P-6 (also don't add any circuitry associatedwith over 512K of memory yet).  Then power up the board in themachine and check for operation.  At this point the only changenoticed should be a new port 148 (94H).  Set and reset the bitsand read the port to verify operation.  NOTE:  If you haveremoved the drives from the case, the 4P may not boot becausethe drives are not properly terminated.  In this case, tryholding down the . (period) key while pressing RESET.  Thiswill run the built-in memory test of the boot ROM.  If thisruns correctly, everything is probably okay.     6. Then remove the Main CPU board and do the following:a. Pull all RAMs.b. Wire all pin 1 of the RAM sockets together and connect toU129P-6.     7. Remove R21.     8. RAS MOD - Lift one end of R23 that goes to U115-11 (seedrawing).  Add jumper from R23 on board side to pad on R21opposite to U115 (see drawing).  Connect lifted end of R23 toU114P-11.  Connect remaining pads of R21 & R23 to U114P-12&13.MAKE SURE IT IS AS SHOWN IN THE DRAWING!!!!!!     9. CAS MOD - Find the feedthrough next to RP1 between pins15 & 16.  This should be the CAS line for the RAMs.  MAKE SURE- it should connect to pin 15 of the RAMS (use digital ohmmeteronly or LOOK).  Turn board over to the non-component side(bottom) and follow the trace to another feedthrough about thesame distance from the RAMs as the first feedthrough.  Cut thetrace that connect the two feedthroughs.  The feedthroughs willnow be the RAM CAS connections.  U133-140 will be "CAS1" &U153-160 will be "CAS2".  Now follow the trace from the firstfeedthrough on the bottom of the board to U136 & U156-15.  Cutthe trace that connects U136-15 & U156-15.  Follow the trace toa feedthrough next to pin 16 of U136.  Cut this trace near thefeedthrough.  Now we should have separated the two rows of RAMson pin 15.  Check with a digital ohmmeter that the two rows areseparated.  Make SURE you understand which traces to cut!!!!     10. Now connect U114P-3 to a 47 Ohm resistor and place theother end in the feedthrough "CAS1".  Connect U114P-6 to a 47Ohm resistor and place the other end in the feedthrough "CAS2."     11. Refresh is found on pin 5 of U87.  Connect it (you canrun the wire through any available feedthrough) to U114P-2&4.     12. Replace the main board and power up.  If you get trashon the screen, check the wiring - you GOOFED.  If it runs butis unreliable, you will need a good scope and will have to lookat the RAS line.  Check for ringing.  Add resistance to theseries resistor if you have ringing.  If the transitions are agood 4.75V then reduce the resistance.  Check the CAS line also- 47 Ohms may be too much.  On my Model 4 I have no resistorsin the RAS or CAS.     Now, to add more RAM you will have to piggyback one or twomore 74S08 IC's as U96P and/or U117P (bend up all pins except 7and 14 before piggybacking).  One input of each of the ANDgates (example: pins 1,4,9,12) must be connected to the refreshline (pin 5 of U87).  The other input for each of the AND gates(example: pins 2,5,10,13) should be connected to one of theCASnA lines (3 through 8) at U116P.  You must then stack theadded RAM and bend pin 15 out and connect all pin 15's per rowof RAMs to a 47 Ohm resistor and to one of the CAS lines (3through 8) at the output of one of the added AND gates(example: pins 3,6,8,11).  See the detail in the lower leftcorner of Figure 1.  Walla - 2 Meg.  I recommend you run 512Kfor a month before you stack RAMs.     I have modifications for the Model 4 and have one Meg inmine, but the design incorporates the use of a HD64180.  Thismakes the modification a little complicated.  The Model 4 boardversion that I have is one of the older non-Green Screen typeand I had to redo the RAS/MUX/CAS circuits in order to run theone Meg RAMs.

However, if there is some interest for the Model4's I will send in the modifications I have done.                        RAMDISK PROGRAM     If you use NEWDOS/80 then I have a RAMDISK program thatwill add a 5th drive and use the memory we just added.  TheRAMDISK uses the fourth PDRIVE, so if you have 512K RAM thenset PDRIVE #4 as follows:                       PDRIVE 0 4=0                       PDRIVE 0 4 tc=99     Then type:    RAMDISK 4   'I renamed it to RM so it would be " RM 4 "     If the NEWDOS/80 4P system has two directories then weneed two steps (I calculate the directory starting and endingpoints so I can return Directory Protected Status):                  COPY 0 4,,FMT CBF NDMW /SYS                  COPY 0 4,,NFMT CBF USR NDMWIf, on the other hand, you made the MODELA/III a SYS file thena full disk copy will work:                    COPY 0 4,,FMT CBF NDMWThen, to mount as drive 0:                           RAMDISK 0That's it (yes, Jack, we CAN chain through this version).     The RAMDISK program will support a standard Model 4 or 4Pwith just 128K if you set the switches (at the start of theassembly language program) accordingly.     [Editor's note:  The assembly language program was writtenusing the ZEUS editor-assembler, however, on this public domaindisk it has been saved using the standard EDTASM format for thebenefit of those who may be using another editor-assemblerprogram (and are willing to make the necessary changes insyntax).  To load the source code file into ZEUS, use the L*command (adding the asterisk to indicate an EDTASM formatfile).]