MOUSE/CMD                            (c) 1988 Scott McBurney            INTRODUCTION                Welcome to the wonderful world of mice!  I congratulate          you on your choice  of mouse  software (mine)  and offer you          these instructions  and suggestions  to help  you in writing          software for the mouse.            THE MOUSE                The driver software  is  written  specifically  for the          Tandy Serial Mouse.   (catalog  #25-1040,  $49.95)  You will          also need to purchase their 9  pin to 25 pin adapter to plug          the mouse in (or make your own).  I think the price of it is          $7.95.   The software  *should* work  with any  serial mouse          that is Microsoft Serial  Mouse compatible!   Other mice may          use different data  packet formats,  and  therefore will not          work with this driver.                The serial  mouse communicates  to its  host in  a very          simple way.   Whenever  something  occurs (movement,  button          pushed,  or button released) it sends a 3 byte packet to the          host.  The first byte is a status byte.   The second byte is          the amount  of movement  in the  X direction,  the  third is          movement in the Y direction.   I won't go into the specifics          of each bit in each byte here,  because it would take up too          much room.            THE SOFTWARE                The driver  included  with  this  documentation  is the          result of  about 30  to 40  hours of  work.   It has several          components:    Status  byte  handler  and  synchronizer,   X          coordinate  adjuster,   Y  coordinate   adjuster,   and  two          supervisor calls.                The interrupt driven  routine  will  keep  track of the          mouse within a 1024 by  1024 area.   The programmer needs to          do nothing to  update  the  current  cursor  position of the          mouse since this is done with interrupts.                Two SVC's are  provided  to  read  information from the          mouse.  They are as follows:              SVC 120 - Read mouse position and status              On entry:                  A = 120             On exit:                 HL = X coordinate                 DE = Y coordinate                  B = Button status when last pushed (bits 4 & 5)                  C = Current button status (bits 4 & 5)              * Bit 4 is for the right button, bit 5 is for the left.                When a button is  pushed,  the current button status is          saved and can be recalled  in register B.   This status byte          is NOT reset by the interrupt routine,  but must be reset by          the programmer (using  SVC  121).   The  status  returned in          register C is the last status byte that was read.                You can find out if a button has been pushed, and if it          is still being held down by compairing B and C.  If bit 4 in          B is set,  then the right button has been pushed.   If bit 4          is set in C, then the right button is still being held down,          otherwise it has been released.              SVC 121 - Change Coordinates and button status              On entry:                  A = 121                 HL = X coordinate                 DE = Y coordinate                  B = What you want the button status to be.           In most cases, you will probably want to set B to 0 to clear          the status of the buttons.                You now  have a  way to  write programs  using a mouse.          The routine is very  accurate,  but I have  noticed that the          mouse can sometimes make unusual jumps.           THE DRIVER                The routine  to monitor  the mouse  is located  in high          memory  from  FF00  to  FFFF  (hex).   No  other  drivers or          routines should be loaded  in high memory  when you load the          mouse driver!   It is NOT a relocatable driver.   You should          NOT load any drivers dealing with  the RS-232 port with this          driver  installed.    This  includes  COM/DVR  and  terminal          programs such  as XT4.   If  you want  to use  any of these,          please re-boot first.                I have also noticed that several programs will not work          once this driver has been loaded.  AllWrite is one of these,          it must do something will the RS-232 interrupts.                Another note:   Do  not load  the driver  after exiting          XT4!   I believe that  XT4 forgets  to reset  some interrupt          addresses when it finishes executing, and your computer will          crash easily!          If you have any questions or comments, please E-mail them to          me.                                         Scott McBurney                                        Genie ID:  S.MCBURNEY