**** MOD ON CONVERTING MODEL I SCREEN TO INVERSE VIDEO ****This is the version that works with just a switch to selectmodes. The theory behind this one is simple, there are twocharacter generators in a TRS-80/SYSTEM-80, one makes thecharacters and the other one makes the graphics. The hardwaredecides whether it is about to draw a character or a graphicsymbol and lets the appropiate character generator know, itthen has a look at memory to see which character/graphicsymbol it has to draw, and tells which ever generator isneeded, which sends out the first row of dots in parallel(ie. 8 at a time). They are grabbed by a shift register andmade to go in single file - (serial). These serial bits gothrough an OR gate so if there is a dot to be lit up from thegraphics generator OR there is a dot to be lit up from thecharacter generator then the gate sends a dot off to the syncsection.     If there IS a dot to be lit up, it is sent off to anothersection where it is mixed in with horizontal and verticalsync and sent of to the screen. What we are going to do isreverse all these dots BEFORE the get mixed in with the syncsignals. (ie. turn all black dots-white and all white dots-black). To do this we are going to use an I.C. called an"inverter". This nifty little I.C. has SIX inverters in it(hex=six), we are only going to use one, the other five canbe used for other mods. You will also need a switch to selectmodes. So run off down to your local electronics store andbuy yourself the following :- 1 x 74LS04 I.C. and a SINGLEPOLE,DOUBLE THROW switch. (The switch should have threeterminals and the I.C. should have 14 pins.). Once again thisarticle is only for the AMERICAN TRS-80. If you are not surewhat type yours is then open it up and have a look at theprocessor socket and see what number it is if it is Z40 thenyour computer is AMERICAN, any other number and it's JAPANESE     Now open up your case and carefully remove the keyboardand the circuit board being careful not to flex the cablejoining them too much. Lay it out on the table with the sidewith all the I.C.'s on it facing up. Prepare the 74LS04 bybending all pins except pins 7 & 14 (bottom left & top right)so that they stick out sideways. now cut the thin parts offso that you are left with a neat little solder pad.     Find the I.C. labelled Z6 on the board and tin the OUTSIDE of pins 7 & 14 on Z6. Tin the INSIDE of pins 7 & 14on our piggy back 74LS04 and sit it on top of Z6 so that allthe pins line up correctly. Push pins 7 of the top and bottomI.C.'s together with the soldering iron so that they join anddo the same for pin 14.  If they don't join properly thenremove the I.C. , add some more solder and reapply. Oncesoldered on, check that there are no shorts.     Find the track leading between Z30 pin 1 and Z41 pin 6.(it may be underneath the I.C.!!). If you are not sure thencheck with a multimeter. When you have found it then use asharp razor blade and cut it neatly and carefully. Drill ahole somewhere in your case and mount the switch somewhereconvenient. Connect a wire (nice,thin stuff, NOT 240 voltwire) between Z30 pin 1 and our piggyback 74LS04 pin 1.     Connect the center pin of the switch to Z41 pin 6.Connect another wire from either side of the switch to pin 2of our 74LS02. Join the side of the switch that you haventused yet to our 74LS04 pin 1 (that should make 2 wires tothis pin). The mod is complete. *****TRIPLE CHECK***** everything reassemble the system andpower up. If everything doesn't work properley then powerdown and recheck your work. If it does work then the computercould power up in either mode depending on which way theswitch is set. Flick the switch and the other mode shouldcome up.**** NOTE **** - some monitors will NOT display inversevideo, some of these are RITRONICS and BMC monitors. Alltandy monitors and all dick smith monitors WILL work andmost others will as well. in all cases an increase of thebrightness control will be needed. it is perfectly all rightto switch modes while the computer is running.#�#####>###6#

M#####P#################################################################################################################################################################################################################