MXP DOCUMENTATION     MXP is a routine which allows you to directly controlmany of the features of the MX-80/100 printer equipped withGRAFTRAX PLUS roms when used with a TRS-80 model 1 or 3operating under NEWDOS 2.0.  It consists of a high memorydriver which, once loaded, can be accessed from DOS orMINIDOS or BASIC via the NEWDOS '*' vector.     Initialize "MXP/CMD" from "DOS" like any other "CMD"file -- i.e., form "NEWDOS 80 READY" type:   "MXP<enter>".     Successful initialization will give the message "MX-80utility...".  Two error messages are possible: "FILE ALREADYEXISTS" -- this error means MX is already loaded.  The othererror is "HIGH MEMORY DRIVER ALREADY IN PLACE -- REBOOTSYSTEM".  If either of these errors occur, it is best toreboot and start again.     Once initialized, MX can be activated from "DOS" or"MINI-DOS" or from BASIC's CMD"doscmd" by the NEWDOS "*"vector.  The snyntx is:     "*MX <param><X><,><param><X><,>...<enter>".     Without parameters, (i.e., "*MX<enter>", all acceptable*MX parameters will be listed to the screen.  (This listing,with a appropriate error message, will also be given if animproper parameter is tried.)     Allowable parameters are: B BX - Bidirectional printing on/off (powerup=on) C CX - Condensed printing on/off (powerup=switch) D DX - Double printing on/off (powerup=off) E EX - Emphasized printing on/off (powerup=off) I IX - Italic printing on/off (powerup=off) W WX - Wide printing on/off (powerup=off) U UX - Underline on/off (powerup=off) O OX - Out of paper transfer on/off (powerup=off) Y YX - Superscript on/off (powerup=off) Z ZX - Subscript on/off (powerup=off) L n - Set forms length to n lines, 1<n<127 (powerup=66) S n - Set skip over perf to n lines, 1<n<Ln (powerup=0) H n - Set linespace to n/72 per inch 1<n<255 (powerup=8) Q n - Set max chars to n/line 1<n<132 (powerup=80/132) F - Execute form feed R - Reset all parameters to powerup state K - Kill *MX routine and remove from que     Command parsing allows multiple commands on one line --e.g., '*MX WX,E,S,6,F,F<enter>' will do the following: 1) Turn OFF wide print (if it was ON, otherwise NOP). 2) Turn ON emphasized print. 3) Set skip over perf to 6 lines. 4) Do 2 form feeds.(Spaces may be used in place of commas in the command line.)    Attempting to execute a non-existent parameter will givethe message 'Improper/non-existant parameter error'.Attempting to input a numeric variable which exceeds thepermitted limit or failure to include a numeric variablewhere needed will create a 'Numeric parameter error'.Multiple command strings are pre-tested for correctness -- ifa multiple command string contains an error, no commands willbe performed (except for the 'K' parameter which will beexecuted regardless of any other errors).