TWO JCL FILES FOR LDOS USERS                         by Gary Bryce     [This article is reprinted from the SYDTRUG NEWS, P.O. Box297, Padstow, New South Wales 2211, AUSTRALIA.]     The first JCL (MIRROR/JCL) presented here forms thesolution to a problem which has existed for some time.  Thoseof you using double density/sided drives on the Model I arequite familiar with the fact that BACKUP refuses to create a"MIRROR IMAGE" of the original diskette (giving a DATA SECTORNOT FOUND DURING READ error).     When BACKUP is required to do a mirror-image copy itcalculates the number of sectors per cylinder.  With a doublesided, double density disk there are 36 sectors per cylinder.The SOLEd disk has 36 sectors per cylinder on every cylinderEXCEPT the first (0), where there are only 10 on side 0 (0 to 9in single density) and 18 on side 1 (0 to 17 in doubledensity).     The SECTOR NOT FOUND error happens because BACKUP hascomputed that there are 36 sectors per cylinder on the disk,and when sector 10 of cylinder 0 is passed to the driver (PDUBLor RDUBL), the driver has already switched to single densityand has automatically updated the Drive Control Table (DCT) toshow that sector 9 is the highest numbered sector on this side.Sector 10 is readdressed to sector 0 of side 1 - BUT IN SINGLEDENSITY!  When the driver tries to read this sector, it gets arecord not found error because side 1 is in double density.The driver then switches density to double (part of ADR) whichnow updates the (DCT) to show 17 as the highest numberedsector; it now invokes the SEEK routine which calculates thatsector 10 is on side 0!  Therefore, the second attempt stillresults in a sector not found error.  The driver switches tosingle density and repeats the process until it times out afterten retries.  Sectors 10 to 17 of cylinder 0 cannot be backedup because they do not exist!     As you can see, the main cause of the problem is thatBACKUP was not designed to deal with a dual density diskette.This can be avoided by locking out cylinder 0 (effectively whatis done in NEWDOS/80 with TI=CK).  BACKUP bypasses the lockedout cylinder and proceeds with a normal "MIRROR IMAGE" backup.The only remaining problem is to update the information aboutthe default drive types and the state of the SYSTEM (SYSGEN)configuration parameter, all of which is stored within the fileBOOT/SYS, which is of course on cylinder 0.  The correct methodto update this information is to backup SYS0/SYS from thesource to destination diskette; the required information istransferred from source to destination in the process.     Rather than patch SOLE1/CMD (as there seems to be a coupleof revisions of the programme), the JCL sets all bits in thecylinder lockout table for cylinder 0, by "PATCHING" thedirectory.  As the PATCH utility rewrites the sector with astandard data address mark, the REPAIR utility is used tocorrect it to the one normally used for the directory.     NOTE: MIRROR/JCL cannot be used to perform a backup wherecylinder 0 of the source diskette is not "Locked Out".     The following JCL (FMBK/JCL) is an expanded version of aJCL written by Frank Marten of the Adelaide Micro Users Group.I have made additions and modifications to allow selection ofSource and Destination drives and Cylinder count, as well asusing the same procedure to lock out cylinder 0 as was used inthe previous JCL.     The normal BACKUP (reconstruct or by class) results in apoor system disk, with LDOS 5.1.4 the system files end upclustered at the beginning of the disk, as far from thedirectory as is possible, increasing the time to load anysystem overlays.     An optimum system disk is created by this JCL byperforming several patches to SYS8/SYS, and by using anorganised approach to the files to be copied, resulting in adisk with the system files placed as evenly as possible oneither side of the directory.  After creating the disk it maybe subsequently backed up using the MIRROR/JCL file.References :-LDOS Manual - BACKUP UTILITY - Backup by class & Backup        reconstruct.LDOS Quarterly - Vol.2 No.3 - LDOS: HOW IT WORKS - Non-Radio        Shack disk drives - by Joseph J.  Kyle-DiPietropaoloNOTES FROM MISOSYS - Iss.2 page 55 - by Roy SoltoffADELAIDE MICRO USER NEWS - June 1985 - A Double Sided Booting        Disk under LDOS - by Frank Marten