# MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

Table of Contents

General Information  . . . . . . . . . . . . . . . . . .  1-2

Distribution Disks . . . . . . . . . . . . . . . . . . .  1-2

MRAS Macro Assembler

MRAS Invocation  . . . . . . . . . . . . . . . . . . .  2-1

Symbolic Names . . . . . . . . . . . . . . . . . . . .  2-7

Operands . . . . . . . . . . . . . . . . . . . . . . .  2-9

Expressions  . . . . . . . . . . . . . . . . . . . . . 2-10

Pseudo-Ops . . . . . . . . . . . . . . . . . . . . . . 2-16

Assembler Directives . . . . . . . . . . . . . . . . . 2-40

Using Macros . . . . . . . . . . . . . . . . . . . . . 2-45

Error Messages . . . . . . . . . . . . . . . . . . . . 2-57

MLINK Linker

Invoking MLINK . . . . . . . . . . . . . . . . . . . .  3-1

Command File Generation  . . . . . . . . . . . . . . .  3-7

Overlay Processing . . . . . . . . . . . . . . . . . .  3-8

Error Messages . . . . . . . . . . . . . . . . . . . . 3-10

MLIB Librarian

Operating MLIB Interactively . . . . . . . . . . . . .  4-1

Re-entering MLIB . . . . . . . . . . . . . . . . . . .  4-2

MLIB commands  . . . . . . . . . . . . . . . . . . . .  4-3

Operating MLIB in batch mode . . . . . . . . . . . . .  4-6

Error Messages . . . . . . . . . . . . . . . . . . . .  4-7

SAID full screen text editor

Invoking SAID  . . . . . . . . . . . . . . . . . . . .  5-1

Editing Functions  . . . . . . . . . . . . . . . . . .  5-3

Calculator . . . . . . . . . . . . . . . . . . . . . .  5-7

Installing SAID  . . . . . . . . . . . . . . . . . . .  5-8

Cross Reference Utility

Invoking XREF  . . . . . . . . . . . . . . . . . . . .  6-1

Cross Reference Listing  . . . . . . . . . . . . . . .  6-3

Technical Information

Tips for programming relocatable modules . . . . . . .  7-1

Microsoft compatible "REL" format  . . . . . . . . . .  7-1

Reproduction of this manual  in any manner, electronic, mechanical,  magnetic,

optical, chemical, or otherwise, without written permission, is prohibited.

The MISOSYS Relocating Macro Assembler product is published  by: MISOSYS,

Inc., P. O. Box 239, Sterling, Virginia 22170-0239 [703-450-4181]

FIXUP/CMD - Copyright 1985 MISOSYS, Inc., All rights reserved.

MLIB/CMD - Copyright 1983/85 Richard N. Deglin, All rights reserved.

MLINK/CMD - Copyright 1985 MISOSYS, Inc., All rights reserved.

MRAS/CMD - Copyright 1985 MISOSYS, Inc., All rights reserved.

SAID/CMD - Copyright 1984 Karl A. Hessinger, All rights reserved.

SAIDINS/CMD - Copyright 1984 Karl A. Hessinger, All rights reserved.

XREF/CMD - Copyright 1983/84 MISOSYS, Inc., All rights reserved.

LDOS is a trademark of Logical Systems, Inc.

MICROSOFT is a trademark of the Microsoft Corp.

TRSDOS is a trademark of Tandy Corp.

## General Information

1 – 1

MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Important Note

Certain  documentation pertaining to this package may be available  after

the user  manual has  gone  to press. Consult the file entitled README/TXT for

details on additional support material and errata.

# General Information

The MISOSYS Relocating Macro  Assembler (MRAS)  is a disk assembler which

generates a relocatable (REL)  module from  one or more source files. The  REL

module generated by MRAS is a bit-stream  compatible with Microsoft (TM)  M-80

generated files.  Multiple REL modules are then linked via  the MISOSYS linker

(MLINK) to produce an  executable  object  code file  (CMD). The assembler  is

also capable of  directly  generating  a  CMD  file when  the  source  file(s)

contain  no references  to relocatable segments. Source files may  be  created

and  edited  with the full  screen text editor (SAID) provided.  Libraries  of

relocatable modules are organized with the librarian (MLIB).

MRAS was  designed to provide the maximum in assembly  power. As such, it

is an  advanced tool  which is not  recommended  for  the novice Z-80 assembly

language programmer. This user manual is not  a "learning" manual - it details

the use  of MRAS and its companion utilities - and in no way attempts to teach

you how to program in the Z-80 assembly language. You  should have available a

standard reference handbook on the Z-80 code. Many texts are available.

The MISOSYS Relocatable Macro Assembler Development System includes:

FIXUP/CMD   - a utility to convert from/to line-numbered source files

MLIB/CMD    - a relocatable module librarian

MLINK/CMD   - a relocatable module linker

MRAS/CMD    - a macro assembler generating relocatable modules

OVERLAY/REL - a module which supports overlay handling

README/TXT  - a LISTable text file containing errata

SAID/CMD    - a full-screen text editor for source code preparation

SAIDINS/CMD - SAID installation program

XREF/CMD    - a symbol cross-reference listing generator

All  source  text  to  MRAS  must have a  Control-Z  (1AH)  as  the  last

character of the  text.  This byte  must immediately follow  a CARRIAGE RETURN

(0DH). If you are  using an  editor  other  than SAID  to  prepare your source

text, and that editor does not terminate  the text file with  a CONTROL-Z, you

may  have difficulty in  using the file with  the  assembler.  If such is  the

case,  load the file into  SAID  using the ASM parameter and resave it  (after

ensuring that the last character in the file is a carriage return).

Source  files  commonly  used with  other assemblers  take one  of  three

forms; a pure  ASCII text  file, a line-numbered text file, or a line numbered

file which includes a header. MRAS  will automatically accept any of the three

types for its  input provided all files included in one source stream use  the

same  convention.  On the other hand, headered and numbered source files would

be  found unworkable with the  SAID text editor. Thus, a  utility called FIXUP

has been  provided.  FIXUP allows  you to  change from  one form to any of the

General Information

1 – 2

## MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

other forms. FIXUP requires a properly terminated file. Its syntax is:

FIXUP filespec {(*/strip/header/number}

where  the parameter "strip"  is  used to eliminate headers and  line numbers,

"number is used to add line numbers, and header  is used to add both a  header

and  line  numbers.  The '*' is used to  rewrite the  file  left in  the FIXUP

buffer. FIXUP defaults to "strip";  reads its input from and writes its output

to the file identified as "filespec".

# Distribution Disks

The  TRSDOS  6.x MRAS  Development  System  is  distributed on a 40 track

double density data diskette.

The Model I/III  MRAS  Development system works  on  both the Model I and

Model  III under  LDOS 5.x, DOSPLUS 3.5, TRSDOS 2.3,  and  TRSDOS  1.3. It  is

released on  a  35 track single  density data diskette.  TRSDOS 1.3 users must

use the CONVERT utility and a two-drive system  to transfer the files from the

master disk to a working system disk. Model I  TRSDOS 2.3 users need  to first

modify  their  TRSDOS  system via a one-byte patch prior to  transferring  the

files from  the master  disk to a working system disk (see below).  The master

disk is readable by LDOS and DOSPLUS.  Model I  or  III use under a  DOS other

than LDOS may require patches to one or more of the supplied programs.

# Model I TRSDOS 2.3 Patch

Model I  TRSDOS users  will  find difficulty  in reading the distribution

disk due to  the data address  mark used for the directory.  Therefore, before

making a BACKUP or copying  MRAS files  from  the  diskette,  you will need to

change one  byte of  the TRSDOS  2.3 disk driver using either of the following

two methods. This change should not affect the operation of your TRSDOS.

Method  (1)  directly  modifies the  system  diskette with  a  patch.  To

prepare for this patch,  obtain  a fresh  BACKUP of your TRSDOS 2.3 to use for

this operation. Then enter the following BASIC program  and RUN  it. After you

RUN the program, re-BOOT your TRSDOS diskette to correct the byte in memory.

10 OPEN"R",1,"SYS0/SYS.WKIA:0"

20 FIELD 1,171 AS R1$, 1 AS RS$, 84 AS R2$

30 GET 1,3: LSET RS$="<": PUT 1,3: CLOSE: END

Method  (2)  uses  a POKE  from BASIC to change  the  value  directly  in

memory. This procedure is as follows:

1. Enter BASIC (files = 0, protect no memory)

2. Type POKE &H46B0,60 followed by <ENTER>.

3. Type CMD"S followed by <ENTER>.

Now,  after  using either method noted above,  COPY the  MRAS  files  from the

master diskette to your TRSDOS system diskette.

# General Information

1 - 3

# The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Invoking MRAS

MRAS is a macro assembler used to assemble a  source disk file(s) into  a

relocatable  object  code  module.  MRAS  provides  a  command  line  rich  in

features. The syntax:

________________________________________________________________

|                                                                |

| MRAS source/ASM {+L=listing/PRN +O=object/CMD +X=reference/REF |

|      +S=symbol/SYM +I=include/ASM } {assembler switches}       |

|      {(p1=value1,p2=value2,p3=value3,p4=value4,LINES=n)}       |

|                                                                |

| +L=listing/PRN   - send listing to spec in lieu of *DO.        |

| +L=:d              Use -LP for printer (or +L=*PR if DOS       |

|                    supported). Will inhibit -NL and -LP.       |

|                                                                |

| +O=object/REL    - send object to spec in lieu of "source/REL".|

| +O=:d              Will inhibit -NO.                           |

|                                                                |

| +X=reference/REF - send cross reference data to spec in lieu   |

| +X=:d              of "source/REF" if -XR switch invoked.      |

|                    Will invoke -XR.                            |

|                                                                |

| +S=symbol/SYM    - send symbol table to spec in lieu of *DO or |

| +S=:d              *PR depending on setting of -WS and -LP     |

|                    switches. Will invoke -WS.                  |

|                                                                |

| +I=include/ASM   - use spec for "*INCLUDE" assembler directive |

|                    which is similar to "*GET".                 |

|                                                                |

| Switches:          -CI -FE -GC -LP -MF -NC -NE -NH             |

|                    -NL -NM -NO -SL -WE -WS -XR     (see text)  |

|                                                                |

| Parms:                                                         |

| Pn               - Set internal symbols            (see text)  |

| Lines=n          - set printed lines per page to n (abbrev=L). |

|                                                                |

| Note: Default file extensions are shown capitalized in the     |

|       file option filespecs.                                   |

|________________________________________________________________|

# File options

File options are  denoted with  a plus  sign  prefix and are used  to re-

direct one  or more output streams of the  assembler. They can accept a syntax

of "+s=filename:d" or "+s=:d". The "s" refers to any of the file switches:  O,

I, L,  S, X. The latter  will re-use the  source  filename for the  file being

switched and the extension  appropriate  for  the  switch. File switches  must

precede the assembler switches.

## MRAS Invocation

2 - 1

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Assembler switches

These switches control various aspects of the assembler.  They are always

prefixed with a minus sign.

# Switch -CI

The "-CI" switch  is used  to generate  a "core-image" object code  file.

Executable command  files in  the DOS are constructed with address information

that  the system loader  uses  when loading and executing  your  command file.

Also, a  header record is  usually found in  a  load module  object code file.

When the "-CI" switch is  specified,  a  number of changes take place in MRAS.

First, the object code file default extension  is changed to "/CIM". Next, the

header  record  and  the  transfer address  record  are  suppressed.  Any  COM

pseudo-OP statement  is,  likewise,  suppressed. A core-image  file  needs  to

contain  contiguous address sequential code. Since MRAS reserves  only storage

locations  when assembling the DS/DEFS pseudo-OPs,  DS instructions will auto-

matically be converted to  their corresponding  "DC"  statements  with a  zero

value for operand2. The "-GC" switch will also be turned on.

# Switch -FE

The  normal operation of MRAS will suppress  the  output of  linkage data

for  symbols  declared  EXTRN but  never  referenced  within the  source  code

stream.  This  switch forces chain external  linkage data to be  generated for

external symbols declared via EXTRN where no reference is  made in the module.

This can be used to force a loading of the extrn'd module  from a library even

though no  other reference is made  in  the  module with the EXTRN. Its use is

generally  associated with  the inclusion of desired library modules  into the

ROOT segment  of an overlayed  program.  If  -FE is not specified,  any symbol

listed in the argument of the EXTRN statement which has no other  reference in

the module  will  not generate a "chain external" and will not be searched for

in a library search.

# Switch -GC

This switch tells  the assembler to directly output a  CMD file. The nor-

mal object file output is  a relocatable  module (/REL).  Do not specify  this

switch if your  source  contains any  CSEGs, DSEGs, or COMMONs. The -GC switch

is automatically turned on by switch -CI. The  assembler will default to  ASEG

if this switch is specified.

# Switch -LP

The -LP switch  is  used  to send  the  assembler  listing, error messages

occurring  during the assembly  of  your  source code,  and  the  symbol table

listing (if specified by means of the "-WS"  switch)  to  a line printer. MRAS

assembler listings print 56  lines  per page and send a form  feed at the con-

clusion of the 56  lines. If you are  generating a listing output and a  prop-

MRAS Invocation

2 - 2

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

erly paged display  is  desired,  it is  suggested  that you set your paper to

begin  printing  at the sixth  line from the top  of  the page  (which assumes

paging parameters  set at 56 print  lines and 66 lines  page length - the  de-

fault). This will provide five  blank  lines for a top margin, and  five blank

lines for a bottom margin.

If you are using other  than 11" form  paper, use the  LINES parameter to

alter the paging parameters to  suit the specifications of your  printer. Note

that MRAS does not count characters per line!

# Switch -MF

The assembler normally  searches  the  OP code  table prior  to the macro

table. If  you want to redefine the code generation of Z-80 OP code mnemonics,

you can  specify  the -MF  switch. It causes the assembler to search the macro

table before the OP code table.

# Switch -NC

Conditional  assembly (see  the  section  on  ASSEMBLER  PSEUDO-OPS)  can

greatly  ease  the maintenance of programs designed to work with multiple con-

figurations of  hardware.  However, it  is  unnecessary  to  "see" the  source

statements  within  conditional  clauses that  are logically "false". This -NC

switch is provided to have no "false"  conditionals  appear in  your listings.

If  a conditional is suppressed, neither  the "IF"  statement nor  the "ENDIF"

statement of the "false" clause will be listed.

# Switch -NE

Various  data declaration pseudo-OPs create  a structured format for  the

listing  of  code generated after the  first  byte of the statement. These are

the DB/DEFB, DM/DEFM, DW/DEFW, and  the  DC pseudo-OP  statements. If you want

to inhibit the expansion  from the listing  only  (the code will still  be ex-

panded  for  assembly  of  object code), then specify the "no expansion", -NE,

switch.

# Switch -NH

Object code files  usually start off with  a header record of X'05 06  xx

xx  xx xx xx  xx'. The x's would be replaced  with the first six characters of

the object code filename (buffered with spaces). MRAS automatically  generates

this  record when writing an object code CMD  file.  The  DOS  loader  has  no

problem with this record. If you would like your object  code files to contain

this record,  then do absolutely nothing.  If  you do  not want to  have  this

header record generated, then specify the "no header", -NH, switch.

MRAS Invocation

2 - 3

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Switch -NL

The second  phase of the  assembly process  generates the assembler list-

ing. If  you  do not  want  to see  a  listing,  then  you  may  enter the "no

listing",  -NL, switch. This will completely suppress phase  two and shift the

assembler to  phase  three. If you are interested  in  listing statements con-

taining  errors, then you must not  suppress the  second phase. Note that  the

lines  containing only assembly errors can be  listed by specifying the "*LIST

OFF"  assembler  directive. See  the  section  on  "ASSEMBLER  DIRECTIVES" for

further details.

The cross-reference data  file is written during phase  two.  In order to

guarantee that the second phase  is available, a cross-reference specification

will automatically override any entry of the -NL switch.

# Switch -NM

The macro model code  is repeated whenever you invoke the macro. Once you

become familiar with what the  macro does,  you really don't need to  see  its

expansion in your  listings every  time the  macro is invoked.  Switch -NM has

been provided to inhibit the listing  of such  expansions.  If you specify  no

macro expansions,  only  the statements invoking the macros  will be listed  -

the listing  of the  expansions will be inhibited.  In  the case  of a  nested

macro invocation, only the highest level macro call will be listed.

# Switch -NO

MRAS will generate  an  object code  output  file  unless you  tell it to

suppress this generation via the -NO switch.

# Switch -SL

If you  specify  -SL, then any  label  starting with  a dollar sign, "$",

will be suppressed  from the symbol table listing and from any cross-reference

data  file.  Therefore, by using a "$"  as the first character of local labels

and specifying -SL  will result in keeping your symbol table listings  unclut-

tered with local labels.

# Switch -WE

In a long assembly, you  may want the assembler  to pause the  listing if

it detects an assembly error  (you're bound to get some of them). The "wait on

error"  switch,  -WE, is available for  that purpose. If specified,  each time

the assembler comes  to an error during phase two,  it will pause the listing.

Any character entered from the  keyboard will  continue the assembly and list-

ing. If you  choose to  enter  the  character  "C" or "c",  then the phase two

process will continue without further  interruption -  even  though additional

errors may  be detected. The  listing may  also  be paused  at any time by de-

pressing the <SHIFT-@> key, momentarily.

## MRAS Invocation

2 - 4

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Switch -WS

A  complete symbol table  cross-reference listing  of a  single  assembly

stream is  available via  the  -XR  switch and  subsequent processing  by  the

XREF/CMD program. An abbreviated printout that contains only a sorted  listing

of  symbols and their value is available at assembly  time by invoking the -WS

switch.  The symbol  table  listing would normally  be displayed  on the video

display. If the -LP  switch  was specified,  the listing would be  directed to

the Line Printer. The  symbol table  can also be invoked via the "+S=filespec"

file option.

# Switch -XR

This  is  the  switch option to use if you  want  to generate a  complete

symbolic  cross reference  listing of  the assembly  stream.  Switch -XR  will

invoke the generation  of a reference data  file used by the  XREF/CMD utility

(see  the chapter on  CROSS REFERENCE UTILITY).  The  reference  data  file is

generated  during  the  listing  pass  (phase two).  If the  XREF  filespec is

entered via "+X=filespec",  this switch  is assumed to  have  been entered. If

the  XREF  filespec is not entered via  "+X=", the filespec  of the  reference

file will be generated from the source filename.

# Parameters: Pn=val

This  parameter  provides  the power  of  entering  symbol table  equates

directly from the  MRAS command line. "Pn" is actually four  parameters as "n"

can  range  from <1-4>. Thus, you specify the parameter as either  P1, P2, P3,

or  P4. These parameters are entered in MRAS as absolute DEFL  values added to

the symbol table. By passing parameter values with  these  on the MRAS command

line, you can alter four symbol table  entries.  Thus,  you can  use  these to

control EQUate  options, pass values  to symbols, etc.  The  format  usable is

dependent on that supported by your DOS and may include:

_____________________________________________________________

|                                                             |

|  Pn          sets @@n to TRUE.                              |

|                                                             |

|  Pn=ddd      sets @@n to decimal value ddd.                 |

|                                                             |

|  Pn=X'hhhh'  sets @@n to hexadecimal value hhhh.            |

|_____________________________________________________________|

The  actual labels added  to the symbol table as  DEFLs are  "@@n", where

"n" is the same as the "n" of "Pn". This is depicted as follows:

_____________________________________________________________

|                                                             |

|     P1 == @@1     P2 == @@2     P3 == @@3     P4 == @@4     |

|_____________________________________________________________|

MRAS Invocation

2 - 5

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

The four symbols initially have a value of zero (logical  FALSE). You can

use these to externally set  flags for use in conditional  assembly. For exam-

ple, say you  have a program that uses two conditional symbols, MOD1 and MOD3.

If your program has the statements:

MOD1     EQU   @@1

MOD3     EQU   @@3

then an MRAS command line including   (P1)  will set  "@@1" to TRUE, "@@3" was

defaulted to  FALSE, and thus "MOD1"  would be TRUE  and "MOD3" would be FALSE

since the  two  conditional  symbols you  are using  are equated to  the "@@n"

parameters.

# Assembler listing

During the first  pass, the name of each file included  or searched  will

be  displayed as an informative message. During the listing  pass,  MRAS keeps

track of  each  statement's logical line number within its source file and the

logical  line number of  the assembly output stream. Stream  line  numbers are

output  in a sequential  order incremented by one  for  each line  of  logical

output. Lines  suppressed from display use up  one  line number for  each line

omitted [i.e. from  *LIST  OFF to *LIST  ON;  -NC statements; -NM statements].

Lines containing errors will be prefixed with the name of the  file containing

the line, the line number within the  file,  and the error message. The state-

ment itself will display the stream line number.

The "+"  indicator  denoting  a  macro expansion will  appear  after  the

stream  line number. The address will be suffixed with a  mode indicator which

indicates the current mode of the assembly source. The  16-bit operand will be

suffixed with a mode indicator which indicates  the  mode of  the operand. The

symbol table  will  include  a  mode indicator following  the  value  of  each

symbol. The indicators are as follows:

blank - absolute

'  - code relative

"  - data relative

!  - common relative

C  - named common

*  - extern symbol

At  the conclusion of the listing pass, the  free  space remaining in the

buffer  pool will be displayed  as, "ddddd Free space". This can be used as an

indicator of how dangerously huge your program is getting.

# Error totals

At  the conclusion of  pass  three, the total  number  of errors  will be

listed.  An "Unclosed conditional" error is also included in the  ERROR TOTALS

count. This error total  will be displayed after the conclusion  of phase  two

if object code  is  not generated. If you place a "*LIST OFF" pseudo-OP at the

beginning of your code, lines containing errors will be listed.

MRAS Invocation

2 - 6

# The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

# Syntax

The basic format of  an  assembly language statement consists  of  up  to

four fields of information. These fields, in order, are:

_____________________________________________________________

|                                                             |

|  {LABEL}  {OPCODE}  {OPERAND{S}}    {;COMMENT}              |

|                                                             |

|  LABEL       is a symbolic name assigned the address value  |

|              of the first byte of the object instruction.   |

|                                                             |

|  OPCODE      is the mnemonic of a specific Z-80 assembler   |

|              instruction or pseudo-OPeration code.          |

|                                                             |

|  OPERANDS    are arguments of the OPCODE.                   |

|                                                             |

|  ;COMMENT    is an informative notation that is ignored by  |

|              the assembler but aids in documenting the      |

|              source code.                                   |

|                                                             |

|  Note: Fields are separated by a tab or spaces.             |

|_____________________________________________________________|

As can  be noted  from the format  box, none of the fields  are required;

however, each line should contain  at least one field. If you want the comment

field to  occupy  the  entire line, start  the line  with a semi-colon  in the

first  character position  of  the line - then, no  other field is  needed.  A

symbolic label can  exist by itself on a line.  There are some  Z-80 operation

codes that have no arguments; thus, an OPCODE could exist  by itself on a line

(in  field  2). You will  never have an  argument  by itself  as  an  argument

relates to an OPCODE.

The statement line is considered to  be freely formatted. That means that

there are no columnar restrictions. Fields  are separated  by one or more tabs

or  spaces. If a  tab  is used, it makes  for  neater listings.  Tabs are also

retained as tabs and thus will keep  source files  smaller than using multiple

spaces. A statement line must not exceed  128 characters in length; thus, if a

carriage return is  not detected by the  129th character,  a "Load file format

error" will be generated.

# Symbolic names

A  label is a  symbolic name  of  a  line  of  code.  Labels  are  always

optional. A label is a  string of characters of any length; however,  only the

first 15 characters will be significant. A symbol is defined as:

name{{:}:}                 Defines "name"

A  terminating single  colon is  optional.  A double colon defines  "name"  as

PUBLIC. If "name" is used as  a reference suffixed with "##",  then  "name" is

declared  extern. Labels  designated  as  PUBLIC  or EXTRN which  exceed seven

MRAS Assembly Language Syntax

2 - 7

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

characters  in length will  be  automatically  truncated to seven  during  the

generation of the  /REL  file  without  warning.  If two  or more labels  with

identical  first seven characters are  so truncated,  the linker  will flag  a

multiple definition  error. The  first character must be a letter (A-Z) or one

of the special characters:  the  underline,  "_"; the dollar sign, "$"; or the

at  sign, "@".  It is recommended that  you reserve  use of  "$" as the  first

character  of "local" labels since they  can be suppressed from a symbol table

output via the "-SL" assemble switch

A label may  contain,  within  character positions  2-15,  letters (A-Z),

decimal digits  (0-9), or  certain special  characters:  the at sign, "@"; the

underline, "_"; the  question  mark, "?"; or the dollar  sign, "$". The dollar

sign "$", appearing  by itself,  is reserved  for the  value of the  reference

counter of the  current  instruction. It  cannot be used as a single character

symbol.

A symbol appearing by itself  in  the  LABEL  field of  a line,  will  be

interpreted as  being  equated  to the current  value  of the program counter.

Thus, the following two LABEL examples are completely equivalent:

ALLALONE

ALLALONE   EQU $

Certain  labels are reserved  by  the  assembler for  use in referring to

registers.  Others are reserved for branching conditions (condition codes) and

may  not be  used for labels. (these conditions apply  to  status  flags). The

following labels are reserved and may not be used for other purposes:

_____________________________________________________________

|                      Reserved Labels                        |

|                                                             |

|                 A, B, C, D, E, H, L, I, R,                  |

|                 IX, IY, SP, AF, BC, DE, HL, ON              |

|                 C, NC, Z, NZ, M, P, PE, PO, OFF             |

|_____________________________________________________________|

Examples of labels:

ENTRY       @OPEN       BUFFER$      BYTE_POINTER     WHAT?

SELECT_CODE $$CORE      @            CARRIAGE_RETURN  @EXIT

A special symbol is  "$MEMRY".  If this symbol  name is  declared PUBLIC,

the  linker will  store  the  address of the  first  available memory location

which follows your program  into the  word defined as  "$MEMRY". Thus,  if you

choose to  use  this capability, $MEMRY must be defined via a DW statement. or

equivalent.

# Opcodes

The   OPCODES   for  the  MRAS  assembler  correspond  to  those  in  the

"Z-80-Assembly Language Programming Manual", 3.0 D.S., REL 2.1, FEB 1977.

MRAS Assembly Language Syntax

2 - 8

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

# Operands

Operands  are  always  one  or  two  values  separated  by  commas.  Some

instructions may have no operands at all.

A value in parentheses "()" specifies indirect addressing when  used with

registers, or "contents of" otherwise.

Constants are  data  declarations of fixed value. They are constructed as

a sequence of one or more  digits  and  an optional radix  specification char-

acter.  The  digits  must be valid  for the  radix used.  The  following table

denotes aceptable constant composition:

_____________________________________________________________

|                                                             |

|   Data Type   Radix Char  Digits       Examples             |

|  -----------  ---------  --------     --------------------  |

|  hexadecimal       H      <0-9,A-F>    1AH, 0ABH, 0FFH      |

|                                                             |

|  decimal           D      <0-9>        107D, 107, 15384     |

|                                                             |

|  octal          O or Q    <0-7>        166Q, 166O           |

|                                                             |

|  binary            B      <0-1>        01101110B            |

|                                                             |

|  Note: Decimal is assumed if the radix character is omitted |

|        unless *RADIX is used to change the default radix.   |

|_____________________________________________________________|

A constant not followed by one of the  radix characters  is assumed to be

decimal. This assumption can  be changed via the *RADIX assembler directive. A

constant must  begin with a  decimal digit. Thus "FFH" is not permitted, while

"0FFH" is valid.

Operands may  also  be constructed  as complicated  expressions  using the

mathematical  and logical  operators.  These are  described in the section  on

"Expressions".

# Comments

All comments must begin  with a semicolon ";". If a source statement line

starts  with  a  semicolon in  the first character position of the  line,  the

entire line is a comment.

MRAS Assembly Language Syntax

2 - 9

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

# Expressions

A  value of an operand may be an expression consisting of  multiple terms

(labels  and  data  constants)  connected with mathematical  operators.  These

expressions are evaluated  in strictly LEFT to RIGHT order. No parentheses are

allowed.  MRAS  does not  support  operator  precedence.  Most  operators  are

binary, which means  that  they require  two arguments. Both "+"  and "-" have

unary uses also. The following operators are supported:

_____________________________________________________________

|                                                             |

|  Operator    Function                  Example              |

|                                                             |

|     +        Addition                   value1 + value2     |

|     -        Subtraction                value1 - value2     |

|     *        Multiplication             value1 * value2     |

|     /        Division                   value1 / value2     |

|   .MOD.      Modulo Division            value1 .MOD. value2 |

|     <        Shift Left or Right        value1 < -value2    |

|  .AND. or &  Logical Bitwise AND        value1 .AND. value2 |

|  .OR. or !   Logical Bitwise OR         value1 .OR. value2  |

|   .XOR.      Logical Exclusive OR       value1 .XOR. value2 |

|   .NOT.      Logical 1's Complement    FALSE EQU .NOT. TRUE |

|    .NE.      Logical Binary Not Equal   value1 .NE. value2  |

|    .EQ.      Logical Binary Equal       value1 .EQ. value2  |

|    .GE.      greater than or equal to   value1.GE.value2    |

|    .GT.      greater than               value1.GT.value2    |

|    .LE.      less than or equal to      value1.LE.value2    |

|    .LT.      less than                  value1.LT.value2    |

|   .SHL.      shift value1 left          value1.SHL.value2   |

|   .SHR.      shift value1 right         value1.SHR.value2   |

|  .HIGH.      obtain high order byte    .HIGH.value          |

|   .LOW.      obtain low order byte     .LOW.value           |

|     %        Length of MACRO            %#LABEL or %%       |

|     %&       MACRO label concatenation  #NAME%&L            |

|_____________________________________________________________|

Addition (+)

The  addition operator  will  add two constants and/or  symbolic  values.

When used as a unary operator, it simply echoes the value.

001E    CON30 EQU    30

0010    CON16 EQU    +10H

0003    CON3  EQU    3

002E    A2    EQU    CON30+CON16

MRAS Assembly Language Expressions

2 - 10

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

Subtraction (-)

The  minus  operator  will subtract two constants and/or symbolic values.

Unary minus produces a 2's complement.

000E    A2    EQU    CON30-CON16

FFF2    A4    EQU    -A2

Multiplication (*)

The multiplication operator  will perform an integer  multiplication of a

16-bit multiplicand by a 16-bit multiplier.  Overflow  of the resulting 16-bit

value is not flagged as an error.

01E0    A5    EQU    CON30*CON16

BF20    A6    EQU    60000*3   ;this overflows

Division (/)

The  division  operator  will perform  an  integer division  of  a 16-bit

dividend by a 16-bit divisor.

0002    A7    EQU    5/2

1B4D    A8    EQU    48928/7

Modulo (.MOD.)

The  modulo  operator  calculates  the remainder  of  the  above  integer

division.

0001    A9    EQU    5.MOD.2

0005    A10   EQU    48928.MOD.7

Shift (<)

This operator can be used to shift a value left or right. The form is:

____________________________________________________________

|                                                            |

|              VALUE     <    {-}AMOUNT                      |

|____________________________________________________________|

If  AMOUNT is positive, VALUE is shifted left.  If  AMOUNT  is  negative,

VALUE is shifted  right.  The magnitude of  the shift  is determined from  the

numeric value of AMOUNT.

0057    HIORD EQU    5739H<-8

C000    A1    EQU    3C00H<4

MRAS Assembly Language Expressions

2 - 11

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

03C0    A2    EQU    3C00H<-4

BBFF    A3    EQU    3CBBH<8+255

03C0    A3    EQU    15+3C00H<-4

Logical AND (.AND. or &)

The  logical  AND operator bitwise  ANDS  two  constants and/or  symbolic

values. Each bit position of the 16-bit resultant value  is a "1" only if both

arguments have a "1"  in the corresponding position,  or a "0" if either argu-

ment has a "0".

3C00    A1    EQU    3C00H&0FFH

0000    A2    EQU    0&15

0000    A3    EQU    0AAAAH.AND.5555H

Logical OR (.OR. or !)

The logical  OR  operator  bitwise  "ORS"  two  constants and/or symbolic

values.  Each bit position of the 16-bit  resultant  value is  a "1" if either

argument has a "1" in the  corresponding  position, or a "0" if neither  argu-

ment has a "1".

3CFF    A1    EQU    3C00H!0FFH

000F    A2    EQU    0.OR.15

FFFF    A3    EQU    0AAAAH.OR.5555H

Logical XOR (.XOR.)

The logical  XOR  operator performs a bitwise  exclusive  OR on two  con-

stants  and/or  symbolic values. Each bit  position  of  the  16-bit resultant

value is a "1"  only if both arguments have reversed bits in the corresponding

position (i.e. one must have a "1" while the other  must  have a "0"). The XOR

operation is considered a modulo two addition.

3CF8    A1    EQU    3C07H.XOR.0FFH

0007    A2    EQU    8.XOR.15

FFFF    A3    EQU    0AAAAH.XOR.5555H

Logical NOT (.NOT.)

This  is a unary  operator. It performs a one's complement on the term it

precedes. Observe the following examples:

FFFE    T1    EQU    .NOT.1

FFFF    T2    EQU    .NOT.0

0000    T3    EQU    .NOT.-1

MRAS Assembly Language Expressions

2 - 12

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

Logical NOT-EQUAL (.NE.)

This operator is a binary operator that compares two adjacent terms.  The

resultant  value  is TRUE if  the terms  are  not  equal.  A FALSE  result  is

returned if the two terms are equal. Observe the following examples:

0000    T1    EQU    1000.NE.1000

FFFF    T2    EQU    1000.NE.10

FFFF    T3    EQU    1.NE.-1

0000    T4    EQU    .NOT.0.NE.-1

Logical EQUAL (.EQ.)

This operator is a binary  operator that compares two adjacent terms. The

resultant value is TRUE if the terms are  equal. A FALSE result is returned if

the two terms are not equal. Observe the following examples:

FFFF    T1    EQU    1000.EQ.1000

0000    T2    EQU    1000.EQ.10

0000    T3    EQU    1.EQ.-1

FFFF    T4    EQU    .NOT.0.EQ.-1

Logical GREATER-THAN-OR-EQUAL-TO (.GE.)

This  is a binary operator that  compares two adjacent terms. The result-

ant value is TRUE if the left term is equal to or larger then the right term.

0000    T1    EQU    1.GE.2

FFFF    T2    EQU    2.GE.2

Logical GREATER-THAN (.GT.)

This  is  a  binary  operator  that  compares  two  adjacent  terms.  The

resultant value is TRUE if the left term is larger than the right term.

0000    T1    EQU    1.GT.2

0000    T2    EQU    2.GT.2

Logical LESS-THAN-OR-EQUAL-TO (.LE.)

This  is  a binary operator that compares two adjacent terms. The result-

ant value  is TRUE  if the left  term  is smaller  than  or equal to the right

term.

FFFF    T1    EQU    1.LE.2

FFFF    T2    EQU    2.LE.2

MRAS Assembly Language Expressions

2 - 13

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

Logical LESS-THAN (.LT.)

This  is  a  binary  operator  that  compares  two  adjacent  terms.  The

resultant value is TRUE if the left term is smaller than the right term.

FFFF    T1    EQU    1.LT.2

0000    T2    EQU    2.LT.2

Logical SHIFT LEFT (.SHL.)

This  is  a  binary operator  that  shifts the left term a number of bits

left according to the right term. It is the same as "value1<value2".

2340    T1    EQU    1234H.SHL.4

Logical SHIFT RIGHT (.SHR.)

This  is a binary  operator  that  shifts the left  term a number of bits

right according to the right term. It is the same as "value1<-value2".

0123    T1    EQU    1234H.SHR.4

Obtain HIGH-ORDER byte (.HIGH.)

This is a unary operator that provides a low-order result  which is equal

to the high order value. It is the same as "value.SHR.8".

0012    T1    EQU    .HIGH.1234H

Obtain LOW-ORDER byte (.LOW.)

This is a unary operator that provides  a low-order result which is equal

to the low order value. It is the same as "value.AND.0FFH".

0034    T1    EQU    .LOW.1234H

Macro Length Operator (%)

The length operator is  applicable only with  MACRO usage. Therefore, its

use will be discussed in the section on MACRO PROCESSING.

MRAS Assembly Language Expressions

2 - 14

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS., Inc. All rights reserved

# Evaluation of expressions - limitations of mode/class

Symbols  have both a mode and a class. The modes are absolute, code  rel-

ative,  data relative, and  common relative (which  is common  specific,  i.e.

coupled to common  relative  is the specific common which the symbol is a part

of). The class  is either  extern  or not extern. The following rules apply to

all expressions:

A. Addition:

1. One term must be absolute.

2. The resulting mode is: absolute + <mode> = <mode>

3. Either term may be extern but not both.

4. If one term is of class extern, the other must be absolute.

B. Subtraction:

1. <mode> - absolute = <mode>

2. <mode> - <mode> = absolute; both modes must be the same.

3. <extern> - absolute = extern; the result is extern

4. The second term cannot be of class extern.

C. All other binary operators require absolute terms. All unary

operators except unary minus require an absolute term. Unary

minus follows the rules of subtraction with the left term assumed

to be absolute.

Additionally, all expressions which resolve to a byte value (in contrast

to 16-bit word value) must resolve to absolute mode, class not-extern.

MRAS Assembly Language Expressions

2 - 15

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OPs

There  are many pseudo-OPs  which  MRAS will recognize.  These  assembler

operations, although written much  like processor  instructions, interface  to

the  assembler instead  of the Z-80  processor. They direct  the assembler  to

perform specific  tasks during the assembly process but have no meaning to the

Z-80  processor. Some of these  pseudo-OPs generate  data values used  by your

program  and are  called "data declaration" pseudo-OPs. Others control  paging

operations and may be  best termed, "listing"  pseudo-OPs. A  broad  range  of

operations to  invoke the assembly of code clauses based  on conditional eval-

uations are supported through  many "conditional"  pseudo-OPs. These assembler

pseudo-OPs are:

________________________________________________________________

|                    Constant Declarations                       |

|                                                                |

|  DATE   Assembles system date as 8-character string, MM/DD/YY. |

|                                                                |

|  DB     Specifies a data byte or string of bytes. Also         |

|         equivalent to DEFB, DEFM, and DM.                      |

|                                                                |

|  DC     Specifies a multiple of byte constants.                |

|                                                                |

|  DS     Reserves a region of storage for program use.          |

|         Equivalent to DEFS.                                    |

|                                                                |

|  DSYM   Assembles "label" as an n-character string. (Similar   |

|         to the construct, DB '&#label', in a macro.            |

|                                                                |

|  DW     Specifies a word (16-bit data value) or a sequence of  |

|         words. Also equivalent to DEFW.                        |

|                                                                |

|  DX     Assembles "expression" as a 4-hexadecimal digit string.|

|                                                                |

|  TIME   Assembles system time as 8-character string, HH:MM:SS. |

|________________________________________________________________|

MRAS Pseudo-OPs - General

2 - 16

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

________________________________________________________________

|                     Origins and Values                         |

|                                                                |

|  ASEG   Sets the program counter to the absolute segment       |

|                                                                |

|  COMMON Sets the program counter to a common relative segment. |

|                                                                |

|  CSEG   Sets the program counter to the code relative segment. |

|         This is the default mode of the assembler.             |

|                                                                |

|  DEFL   Establishes a value for a label which can be altered   |

|         during the assembly.                                   |

|                                                                |

|  DSEG   Sets the program counter to the data relative segment. |

|                                                                |

|  END    Signifies the end of a *GET, *INCLUDE, or *SEARCH      |

|         member. Supplies the  execution transfer address.      |

|                                                                |

|  ENTRY  Synonomous with GLOBAL.                                |

|                                                                |

|  EQU    Estalishes a constant value for a label.               |

|                                                                |

|  EXT    Synonomous with EXTRN.                                 |

|                                                                |

|  EXTRN  Specifies the symbols in the name list as external.    |

|                                                                |

|  GLOBAL Specifies the symbols in the name list as entries.     |

|                                                                |

|  LORG   Establishes a load origin for executable object code   |

|         files. LORG is unusable for /REL output.               |

|                                                                |

|  NAME   Specifies the module's name for the /REL file. This    |

|         defaults to the object code filename.                  |

|                                                                |

|  ORG    Establishes an origin for executable object code files |

|         or relative code segments.                             |

|                                                                |

|  PUBLIC Synonomous with GLOBAL.                                |

|                                                                |

|  Note:  An ORG can follow ASEG, CSEG, DSEG, or COMMON //; but  |

|         not a named common. A maximum of seven named commons   |

|         are permitted in one module. The "name" of a common    |

|         cannot be the same as any symbol.                      |

|________________________________________________________________|

MRAS Pseudo-OPs - General

2 - 17

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

________________________________________________________________

|                         Conditionals                           |

|                                                                |

|  ELSE   Alternate clause to be assembled if the prior clause   |

|         has evaluated FALSE.                                   |

|                                                                |

|  ENDIF  Signifies the end of a conditional block.              |

|                                                                |

|  IF     Conditional evaluation of expression.                  |

|                                                                |

|  IF1    Logically TRUE if the assembler is on the first pass.  |

|                                                                |

|  IF2    Logically TRUE if the assembler is on the second pass. |

|                                                                |

|  IF3    Logically TRUE if the assembler is on the third pass.  |

|                                                                |

|  IFABS  Logically TRUE if "name" is absolute.                  |

|                                                                |

|  IFDEF  Logically TRUE if "name" has been defined prior to     |

|         this statement or if "name" is extern, else FALSE.     |

|                                                                |

|  IFEQ   Logically TRUE if expression1 = expression2.           |

|                                                                |

|  IFEQ$  Logically TRUE if string1 = string2.                   |

|                                                                |

|  IFEXT  Logically TRUE if "name" is extern.                    |

|                                                                |

|  IFLT   Logically TRUE if expression1 < expression2.           |

|                                                                |

|  IFLT$  Logically TRUE if string1 < string2.                   |

|                                                                |

|  IFGT   Logically TRUE if expression1 > expression2.           |

|                                                                |

|  IFGT$  Logically TRUE if string1 > string2.                   |

|                                                                |

|  IFNDEF Logically TRUE if "name" has not been defined prior to |

|         the statement or if "name" is not extern, else FALSE.  |

|                                                                |

|  IFNEXT Logically TRUE if "name" is not extern.                |

|                                                                |

|  IFNE   Logically TRUE if expression1 <> expression2.          |

|                                                                |

|  IFNE$  Logically TRUE if string1 <> string2.                  |

|                                                                |

|  IFREF  Logically TRUE if "label" has been referenced but not  |

|         defined prior to the statement, else FALSE.            |

|                                                                |

|  IFREL  Logically TRUE if "name" is relative.                  |

|                                                                |

|  Note: "IFxx$" denotes alternate macro string comparison.      |

|________________________________________________________________|

MRAS Pseudo-OPs - General

2 - 18

# The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

________________________________________________________________

|                        Miscellaneous                           |

|                                                                |

|  COM    Generates a CMD object code file comment record.       |

|                                                                |

|  ENDM   Designates the end of a MACRO model. [**]              |

|                                                                |

|  ERR    Forces an assembly error.                              |

|                                                                |

|  EXITM  Can be used to prematurely exit from a MACRO expansion.|

|         This is normally used within a conditional. [**]       |

|                                                                |

|  IRP    The statements within IRP-ENDM are repeated for as     |

|         many items are in the argument list with "dummy" being |

|         replaced by each argument in turn. [**]                |

|                                                                |

|  IRPC   The statements within IRPC-ENDM are repeated for each  |

|         character in the character-list while the "identifier" |

|         is replaced, in turn, from the identifier list. [**]   |

|                                                                |

|  MACRO  Designates the prototype of a MACRO model. [**]        |

|                                                                |

|  OPTION This permits the altering of any of the permissable    |

|         assembler switches from within the source code during  |

|         the first pass of the assembler.                       |

|                                                                |

|  PAGE   Outputs a form feed during a listing.                  |

|                                                                |

|  REF    Forces a reference to the symbols identified in the    |

|         argument list.                                         |

|                                                                |

|  REPT   The statements within REPT-ENDM are repeated according |

|         to the result of "expression". [**]                    |

|                                                                |

|  SPACE  Generates extra line feeds during a listing.           |

|                                                                |

|  SUBTTL Invokes a heading sub-title for listings.              |

|                                                                |

|  TITLE  Invokes a heading title for listings.                  |

|                                                                |

|  [**]   Details are in the section on USING MACROS             |

|________________________________________________________________|

## MRAS Pseudo-OPs - General

2 - 19

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP DB

The "DB" pseudo-OP is used to define a data byte or series  of bytes. Its

syntax is:

_____________________________________________________________

|                                                             |

|  DB    n{,n}{,'c'}{,s}{,expression}                         |

|                                                             |

|  n           defines the contents of a byte at the current  |

|              reference counter to be "n".                   |

|                                                             |

|  'c'         defines the content of one byte of memory to   |

|              be the ASCII representation of character "c".  |

|                                                             |

|  's'         defines the contents of n bytes of memory to   |

|              be the ASCII representation of string "s",     |

|              where "n" is the length of "s".                |

|                                                             |

|  expression  is a mathematical expression which evaluates   |

|              to a number in the range <0-255>.              |

|_____________________________________________________________|

The constant declaration  "DB"  permits  the concatenation  of  its  data

arguments using the comma  "," as  an  argument separator. Data values are de-

noted according to  the  specifications in the section  on  ASSEMBLY  LANGUAGE

INFORMATION.

The  pseudo-OPs DM, DEFB, and DEFM can be  used in  lieu of  "DB" and are

completely equivalent.

"DB" string arguments  permit  two  connected single-quotes to indicate a

single-quote value PROVIDED  that two  or more  characters precede the 2-quote

appearance in the string. For example:

DB    'AB''C'

will produce  the character string: 41 42 27 43. This may have been coded as a

complex  declaration such as,  "'AB',27H,'C'", but  the extensive  declaration

support in MRAS provides the easier specification.

The following are valid declaration statements:

DB    1,2,'buckle your shoe',3,4,'close the door'

DB    'This is a tes','t'!80H,CR

The hexadecimal  expansions  of the constant will  appear in listings  as

rows  of  eight bytes per row. The  expansions  may be  suppressed  from  your

listings by using the assembler switch, -NE.

Pseudo-OPs - Data Declarations

2 - 20

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP DC

This pseudo-OP defines a repetitive constant. Its syntax is:

_____________________________________________________________

|                                                             |

|  DC quantity,value                                          |

|                                                             |

|  quantity    specifies how many times that "value" is to be |

|              repeated as a data byte. It can be defined as  |

|              any other data definition: n, expression, 'c'. |

|                                                             |

|  value       is the constant to be repeated. As in a "DB"   |

|              data declaration, the value can be specified   |

|              as a character, 'c', a numeric value, n, or an |

|              expression evaluated to a number in the        |

|              range <0-255>.                                 |

|_____________________________________________________________|

The  pseudo-OP, "DC", will define a repetitive constant and eliminate the

necessity of defining a  series of identical data values by  long  DB specifi-

cations. For example, the following two statements are equivalent:

DB 0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

DC 16,0

The latter is much shorter, easier to enter as text, more  readable, and takes

up less space in its source form.

The  "quantity" must range from 1 to 65535 (a  zero  value will result in

65536). The "value" must be less than 256.  With this  pseudo-OP, you can gen-

erate repetitions of a single constant. For  example, say you want  to set 100

storage locations to a zero value during the assembly. Insert the statement,

DC 100,0

and it  will be done. A  character  constant  can also  be used for "value" as

illustrated in the following example:

DC 256,'A'

which will set the next 256 storage locations to the letter, "A".

The expansions of the  constant  will  appear in listings just as they do

in the DB  expansion. The expansions may  be suppressed from your  listings by

using the assembler switch, -NE.

Pseudo-OPs - Data Declarations

2 – 21

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP DS

This pseudo-OP is  used  to  reserve a quantity of  storage locations for

use by your program. Its syntax is:

_____________________________________________________________

|                                                             |

|  DS nn                                                      |

|                                                             |

|  nn          reserves "nn" bytes of memory starting at the  |

|              current value of the reference counter.        |

|_____________________________________________________________|

The DS pseudo-OP can also be entered as "DEFS".

The quantity, "nn", can be a data value or an expression. Note  that "DS"

does not  define  data values. "DS" adds the quantity of storage locations re-

served to the  current program counter (PC) to calculate  a new PC value. When

generating a CMD object code file, this action will  cause the next  assembled

byte to create  a  new  load  record. When generating a REL object code  file,

this action will generate a Set Location Counter special link item.

The statement,

FCB   DS   32

will  define a  32-byte region  for later use  as  a  File Control  Block. Its

origin can then be referenced as "FCB". The statement,

TABLE DS   TABLE_LENGTH * TABLE_WIDTH

will  reserve  a  quantity of storage locations  equal to the result of multi-

plying the two terms, TABLE_LENGTH and TABLE_WIDTH.

If  your source  code  is  being assembled  with the  "-CI" switch,  MRAS

automatically converts  all  "DS"  declarations into equivalent  "DC" declara-

tions  using  a value equal to zero. The previous two examples would therefore

be translated to the following:

FCB   DC   32,0

TABLE DC   TABLE_LENGTH * TABLE_WIDTH,0

Pseudo-OPs - Data Declarations

2 - 22

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP DW

This declaration specifies a 16-bit data value. Its syntax is:

_____________________________________________________________

|                                                             |

|  DW nn{,'cc'}{,nn}                                          |

|                                                             |

|  nn          defines the contents of a 2-byte word to be    |

|              the value, "nn".                               |

|                                                             |

|  'cc'        defines the contents of a 2-byte word to be    |

|              the characters, 'cc'                           |

|_____________________________________________________________|

The DW pseudo-OP can also be entered as "DEFW".

In the expansion of the data  word, its least significant byte is located

at the current program  reference counter while  the  most significant byte is

located at the reference  counter plus  one.  The data  word can  be a numeric

constant, an  expression  that  evaluates  to a 16-bit  value, or a  character

constant of  one or two characters.  The following examples illustrate various

forms of "DW" data declarations.

DW    10000,1000,100,10,1

DW    'ab'

DW    'R','o','y'

Note that if a single character is defined as a character constant  word,  the

low-order  byte  of  the  word  will  contain  the  character  value  and  the

high-order byte of the word will be set to zero.

# Pseudo-OP DATE

The DATE pseudo-OP is used to assemble the  system date as an 8-character

string, MM/DD/YY. It's syntax is:

_____________________________________________________________

|                                                             |

|  DATE                                                       |

|_____________________________________________________________|

This actual date is established when you power  up your computer and  re-

spond to the DOS's  date entry  query or by using  the DOS's DATE library com-

mand. The  date string  can  be useful  to  place an ASCII date  stamp in your

object program for the purpose  of identification as to when it was assembled.

See example 1 for an illustration of DATE.

## Pseudo-OPs - Data Declarations

2 - 23

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP DSYM

DSYM is usually used within a macro to assemble the  "symbol" argument as

if it were a DB character string. It's syntax is:

_____________________________________________________________

|                                                             |

|  label  DSYM  symbol                                        |

|                                                             |

|  label       An optional statement label.                   |

|                                                             |

|  symbol      A defined symbolic label.                      |

|_____________________________________________________________|

When used  in  a macro  environment, "symbol" will have the "#" indicator pre-

fixed to designate the symbol as a  macro  dummy argument name. An alternative

method is  to  use  the ampersand  escape function  within a  standard  quoted

character  string  such as "DB '&#symbol'"  which also assembles to  the  same

thing in a macro. See example 1 for an illustration of DSYM.

# Pseudo-OP DX expression

DX assembles "expression" as a  4-hexadecimal digit character string. Its

syntax is:

_____________________________________________________________

|                                                             |

|  label  DX  expression                                      |

|                                                             |

|  label       An optional statement label.                   |

|                                                             |

|  expression  An expression operand.                         |

|_____________________________________________________________|

The  expression can be a simple  symbol or a  complicated collection of terms.

The expression is evaluated to a 16-bit  value  and output as four hexadecimal

digits. See example 1 for an illustration of DX.

# Pseudo-OP TIME

The TIME pseudo is  used  to  assemble the  system time as an 8-character

string, HH:MM:SS. It's syntax is:

_____________________________________________________________

|                                                             |

|  TIME                                                       |

|_____________________________________________________________|

This actual  time  is established when  you  power up your  computer and

respond to the  DOS's  time  entry  query  or by using the DOS's  TIME library

command.  The TIME string can be useful to place an ASCII  TIME stamp in  your

Pseudo-OPs - Data Declarations

2 - 24

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

object program for the purpose of identification as to when  it was assembled.

See example 1 for an illustration of TIME.

Example 1

3000          00001         ORG     3000H

3000          00002 NAME    MACRO   #SYM

3000          00003         DSYM    #SYM

3000          00004         DX      #SYM

3000          00005         ENDM

3000          00006         ENTRY   BEGIN

3000 210730   00007 BEGIN   LD      HL,MSG$

3003 3E0A     00008         LD      A,10

3005 EF       00009         RST     40

3006 C9       00010         RET

3007          00011 MSG$    NAME    BEGIN

3007+42       00012         DSYM    BEGIN

45 47 49 4E

300C+33       00013         DX      BEGIN

30 30 30

3010 0D       00014         DB      13

3011 31       00015         DATE

32 2F 33 31 2F 38 34

3019 30       00016         TIME

39 3A 31 31 3A 33 36

0000          00017         END

Pseudo-OPs - Data Declarations

2 - 25

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP ASEG

This pseudo-OP is used  to set the program counter  to the absolute  seg-

ment. The syntax of "ASEG" is:

_____________________________________________________________

|                                                             |

|       ASEG                                                  |

|       ORG  expression                (optional)             |

|                                                             |

|  expression  When evaluated, "expression" will be the       |

|              origin of the segment. Expression must         |

|              evaluate to an absolute value.                 |

|_____________________________________________________________|

It is not necessary for an  ORG to follow an ASEG.  An ASEG will  set the

absolute program  counter to the last encountered ASEG value, or to zero if no

previous ASEG had been specified.

# Pseudo-OP COMMON

This pseudo-OP  is  used to set the  program counter to a common relative

segment. The syntax of "COMMON" is:

_____________________________________________________________

|                                                             |

|       COMMON  /{name}/                                      |

|                                                             |

|  name        An optional name which specifies a name        |

|              for the common segment.                        |

|_____________________________________________________________|

This pseudo-OP  sets the PC to a common relative segment: the slashes are

required. If "name" is  omitted, blank  common is assumed. A maximum of  seven

named commons are permitted in any one  module. The "name" of a  common cannot

be the same as any symbol.

It would be unusual for an ORG to  follow a COMMON. An  ORG cannot follow

a  named  common.  A  COMMON will set  the specified common  relative  program

counter to the beginning of the common segment (i.e. to zero relative).

MRAS Pseudo-OPs - Origins and Values

2 - 26

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP CSEG

This pseudo-OP is  used to set  the program counter to  the code relative

segment. The syntax of "CSEG" is:

_____________________________________________________________

|                                                             |

|       CSEG                                                  |

|       ORG  expression                (optional)             |

|                                                             |

|  expression  When evaluated, "expression" will be the       |

|              origin of the segment. Expression must         |

|              evaluate to an absolute value.                 |

|_____________________________________________________________|

It is not necessary for an  ORG  to  follow  a CSEG. A  CSEG will set the

code relative program counter to  the last encountered  CSEG value, or to zero

if no previous CSEG had been specified.

The assembler defaults to CSEG if  no  other segment  pseudo-OP is speci-

fied; however,  if MRAS is  invoked  with  the -GC switch,  it will default to

ASEG.

# Pseudo-OP DEFL

The "DEFL" pseudo-OP assigns  a value  to a label. The value is permitted

to be changed during the assembly. The "DEFL" syntax is:

_____________________________________________________________

|                                                             |

|  label  DEFL  nn                                            |

|  label  DEFL  expression                                    |

|                                                             |

|  nn          sets the value of "label" to the quantity "nn" |

|                                                             |

|  expression  sets the value of "label" to the evaluated     |

|              result of "expression".                        |

|_____________________________________________________________|

This  declaration is similar  to the "EQU"  declaration except  that  the

label value is permitted to  change during the  course of the assembly without

producing phase  errors (which are  generally observed  as  numerous  MULTIPLY

DEFINED SYMBOL errors). If  the value of "label" is declared by a  "DEFL", the

declaration can be  repeated in the program with different values for the same

label.

Labels  defined as "DEFL" will be  carried  as "DEFL" in the  EQUate file

generation  of the Cross-Reference utility. They will also  be notated  in the

cross-reference listing by a plus sign, "+", prefix to the label name.

MRAS Pseudo-OPs - Origins and Values

2 - 27

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP DSEG

This pseudo-OP is used  to set the program  counter  to the data relative

segment. The syntax of "DSEG" is:

_____________________________________________________________

|                                                             |

|       DSEG                                                  |

|       ORG  expression                (optional)             |

|                                                             |

|  expression  When evaluated, "expression" will be the       |

|              origin of the segment. Expression must         |

|              evaluate to an absolute value.                 |

|_____________________________________________________________|

It  is not necessary for an ORG to  follow  a DSEG. A  DSEG will  set the

code relative program counter to  the last encountered  DSEG value, or to zero

if no previous DSEG had been specified.

# Pseudo-OP END

The  "END" pseudo is  used  to  denote the exit  of a *GET, *INCLUDE,  or

*SEARCH process.  If the  END  statement of  the  source  file has  a non-zero

operand, it will denote the transfer address of the module. Its syntax is:

_____________________________________________________________

|                                                             |

|       END {nn}                                              |

|       END {label}                                           |

|                                                             |

|  nn          Specifies an execution transfer address branch |

|              that will be used by the system loader.        |

|                                                             |

|  label       Specifies an execution transfer address branch |

|              to be the value of "label".                    |

|_____________________________________________________________|

The "END"  statement is used to indicate  to the assembler, when the last

source code  statement  is reached  so that  any following  statements are ig-

nored.  If no "END"  statement is found, processing stops when  the end of the

file is reached. The END statement can  specify a  transfer address (i.e.  END

LABEL or  END  6000H).  Only  one  transfer address should  be  specified  per

assembly stream; however,  if more than one  non-absolute-zero END operand  is

detected,  only  the first one  will be retained. The transfer address is used

by the  DOS program execution to  transfer control to the address specified in

the  END statement.  Note  that the END statement cannot have a label  in  the

label field of the statement).

## MRAS Pseudo-OPs - Origins and Values

2 - 28

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OPs ENTRY, GLOBAL, and PUBLIC

Any one  of these may be used to specify that the names of the symbols in

the name list are  symbols  global to linkage  when REL modules  are linked by

the linker (MLINK). The syntax is:

_____________________________________________________________

|                                                             |

|       ENTRY   name{,name}...                                |

|       GLOBAL  name{,name}...                                |

|       PUBLIC  name{,name}...                                |

|                                                             |

|  name        A symbol to be defined global.                 |

|_____________________________________________________________|

MRAS treats  GLOBAL,  ENTRY,  and PUBLIC totally  equivalent  in order to

provide compatibility with  other  relocating assemblers. A  symbol classified

as such  is known to other  separately  assembled modules which specify "name"

as  EXTRN.  All symbols not specified as GLOBAL, PUBLIC,  or  ENTRY  are known

only to the module currently being assembled.

A  symbol can also be implicitly declared PUBLIC by appending two  colons

to the "name" where  the symbol is  defined. Thus,  the  following two methods

both declare the symbol, TRUST, as PUBLIC:

--------method 1-------              --------method 2-------

PUBLIC TRUST

TRUST   LD     HL,VALUE              TRUST:: LD     HL,VALUE

Symbols declared PUBLIC in  one module  that  need  to  be referenced  by

another module must  be declared EXTRN  in all modules other  than  the module

where the symbol is defined.

# Pseudo-OP EQU

This pseudo-OP assigns a constant value to a label. Its syntax is:

_____________________________________________________________

|                                                             |

|  label  EQU  nn                                             |

|  label  EQU  expression                                     |

|                                                             |

|  nn          Sets the value of label to nn.                 |

|                                                             |

|  expression  Sets the value of label to the calculated      |

|              value of "expression"                          |

|_____________________________________________________________|

The "EQU"  (equate) pseudo-OP is  the generally  accepted  way  to define

constant  values for use in  your program. This declaration serves a different

purpose than  the data declarations  such as DB, DC, and DW. Data declarations

specify storage  locations that contain the values declared. The "EQU" assigns

MRAS Pseudo-OPs - Origins and Values

2 - 29

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

the  value to the label;  thus, anywhere the label is used, the assigned value

is utilized. Your programs  will  be more  readable, and easier to maintain if

the values need to be altered in a program revision.

An  "EQU" can  occur  only  once  for any  label.  A  multiple "EQU" with

different values will result in the MULTIPLY DEFINED SYMBOL error.

# Pseudo-OP EXTRN

The  EXTRN pseudo-OP  is used to declare a PUBLIC symbol which is defined

in some other module. EXT is synonomous with EXTRN. The syntax of "EXTRN" is:

_____________________________________________________________

|                                                             |

|       EXTRN  name{,name}...                                 |

|       EXT    name{,name}...                                 |

|                                                             |

|  name        A symbol defined external to the current       |

|              module.                                        |

|_____________________________________________________________|

When your program is made up of  more than one REL module linked together

by  the linker,  symbols  which  are referenced  in a  module but  defined  in

another  must  be  declared EXTRN in  all modules which reference  the  symbol

other than the module which defines it.

# Pseudo-OP LORG

The  "LORG" pseudo-OP is used  to  establish a CMD object code  file  (or

part of  one) that loads at an address different  from  where it will execute.

The syntax of "LORG" is:

_____________________________________________________________

|                                                             |

|       LORG  nn            Turn on LORG                      |

|       LORG  expression    Turn on LORG                      |

|       LORG  $             Turn off LORG                     |

|                                                             |

|  nn          Is the address to start loading the object     |

|              file (or part of the file).                    |

|                                                             |

|  expression  When evaluated, "expression" will be treated   |

|              the same as "nn".                              |

|_____________________________________________________________|

A load-origin assembler directive, "LORG",  is provided to cause the load

addresses  of the  object file to  be based on the LORG operand while the exe-

cution code address references will still be based on the  "ORG" operand. This

is  useful  to construct  a module (or part of a  module) that will load at an

address  different from  its execution address. Such is the  case  when  using

MRAS to generate a  PROMable module  to  be  used  on  an  external  processor

MRAS Pseudo-OPs - Origins and Values

2 - 30

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

origined at zero. For example:

ASEG

ORG   0000H

LORG  7000H

will  assemble code so  that  absolute  address references and  the  execution

addresses  are referenced from X'0000';  however,  the object  code file  will

start loading  at X'7000'.  Any  subsequent  "ORG"  will maintain  the  offset

difference   established  at  the  previous  "ORG"  until  another  "LORG"  is

detected.

If  you  want  to  switch off the offsetting operation of  LORG,  add the

statement:

LORG  $

to follow  the last statement of the offset block  of code. The assembler will

specifically test for the case, LORG  $, so  that it forces a new  load  block

where one is required.

LORG is usable only  when  generating CMD files directly from  the assem-

bler via the -GC switch. LORG cannot be used when generating REL output.

# Pseudo-OP NAME

This is  used to specify the  module name of the generated  REL file. The

syntax of "NAME" is:

_____________________________________________________________

|                                                             |

|       NAME modname                                          |

|       NAME ('modname')               (equivalent)           |

|                                                             |

|  modname     Specifies the module name for the REL file.    |

|                                                             |

|_____________________________________________________________|

If NAME is not specified in  the  source stream of an assembly which gen-

erates a REL  object code file, a special link item module name record will be

generated using as a default, the first seven non-blank characters of the  REL

file's name. The second format is supported for compatability with M-80.

# Pseudo-OP ORG

The "ORG" pseudo-OP is  used to  establish  an  address  for the  program

counter  so that the address references  within a  program  are designated  to

origin from other than address 0000H. The syntax of "ORG" is:

## MRAS Pseudo-OPs - Origins and Values

2 - 31

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

_____________________________________________________________

|                                                             |

|       ORG  nn                                               |

|       ORG  expression                                       |

|                                                             |

|  nn          sets the address reference counter to the      |

|              value "nn".                                    |

|                                                             |

|  expression  when evaluated, "expression" will be treated   |

|              the same as "nn". Terms of "expression" must   |

|              be defined prior to the "ORG" statement.       |

|_____________________________________________________________|

The "ORG" statement  is  used to tell the assembler  at  what address  to

begin generating  the object code for statements which  follow.  The assembler

will  generate  object code  starting at  the  address  specified by  "nn"  or

"expression",  automatically advancing  the program  counter by the length  of

each  instruction  or  data  declaration assembled.  The "DS" data declaration

advances the program counter by the amount of storage locations reserved.

A  program can have more than one "ORG" statement. If multiple "ORGs" are

used,  and one or more inadvertently  will cause the overwrite of a previously

assembled module of  code, no warning message  of any kind will be issued.  It

is  left up  to  the programmer, to  protect against such  events  by  use  of

conditional tests (using conditional pseudo-OPs) and the "ERR" pseudo-OP.

An  ORG  can follow  an ASEG, CSEG, DSEG,  or COMMON //; but not a  named

common.  When  ORG follows  a  relative  segment  specification,  the  program

counter will be set relative  to the beginning of the segment, an amount equal

to the operand  of the  ORG.  The  operand  of  the ORG  must evaluate  to  an

absolute value.

MRAS Pseudo-OPs - Origins and Values

2 - 32

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Conditional Pseudo-OPs

The "conditional" pseudo-OPs provide  a powerful way to maintain  a  pro-

gram that is  slightly different  when assembled  to  run on different machine

configurations. Instead  of having to maintain multiple copies  of a  program,

with each  having  some routines and modifications  to make a "custom" version

of the  program, by using the conditional pseudo-OPs, you can maintain one set

of source code that has conditional  clauses that perform the "customization".

It  is  very easy to specify  which  clauses are  to  be  assembled  during  a

particular assembly. The structure of a conditional clause is as follows:

_____________________________________________________________

|                                                             |

|       IFxx  operand_of_IF                                   |

|       .                                                     |

|       clause                                                |

|       .                                                     |

|       ENDIF                                                 |

|                                                             |

|       THE OPERAND OF THE CONDITIONAL MUST BE DEFINED        |

|       PRIOR TO THE EVALUATION OF THE "IF" STATEMENT!        |

|_____________________________________________________________|

The operand  of the "IF"  takes  on different formats  depending  on  the

particular "IF"  pseudo-OP. It can be an  expression,  a label, or two expres-

sions separated by commas. If the operand evaluates  to  a non-zero  value, it

is  interpreted as a  logical TRUE  condition. If  the argument evaluates to a

zero value, it is interpreted as  a  logical  FALSE condition. When the condi-

tion  is TRUE, the conditional clause between the "IF"  and the "ENDIF" is as-

sembled. If the  evaluation is to a zero value then  the conditional clause is

not assembled,  For the sake of uniformity, use the  value of "-1" for a logi-

cal TRUE  and  "0" for  a logical  FALSE so  that, "FALSE EQU .NOT.TRUE"  is a

valid statment. The values can be set in program as follows:

TRUE     EQU     -1

FALSE    EQU      0

MOD1     EQU      TRUE

MOD2     EQU      FALSE

MOD3     EQU      FALSE

Conditional clauses  can  also be nested,  in  case  complicated  logical

constructs are  needed or in  case a  conditional clause itself has  a  condi-

tional sub-clause. For example:

IF    expression1

IF   expression2

ENDIF

ENDIF

is a two-level conditional. Conditional clauses  can be nested to sixteen (16)

levels although you will rarely find a need for more than three.

MRAS Pseudo-OPs - Conditionals

2 - 33

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

The conditional construct of IF-ELSE-ENDIF may be used. It is coded as:

IF    expression

clause_1.

ELSE

clause_2.

ENDIF

which implies that if  expression is TRUE,  clause_1 assembles. If  expression

is  FALSE, then clause_2 will be assembled. The ELSE construct is not required

in a conditional but may be used  where you have alternative  clauses that can

be based on one switch.

As  mentioned earlier, the IF argument can take  one of  three forms. The

conditional structures of these are as follows:

_____________________________________________________________

|                                                             |

|  ---Type I---     -----Type II------        --Type III--    |

|  IF[x] exp        IFxx[$]  exp1,exp2        IFyy    name    |

|  .                .                         .               |

|  clause           clause                    clause          |

|  .                .                         .               |

|  ENDIF            ENDIF                     ENDIF           |

|                                                             |

|  [x]        Optional entry of 1, 2, or 3 to evaluate based  |

|             on the assembler phase during the assembly      |

|                                                             |

|  xx         Can be "LT", "EQ", "GT", or "NE" meaning less   |

|             than, equal to, greater than, or not equal to   |

|             respectively when comparing "exp1" to "exp2".   |

|                                                             |

|  [$]        The "$" is specified in macro comparisons with  |

|             the expressions treated as strings (see the     |

|             section on USING MACROS).                       |

|                                                             |

|  yy         Can be "DEF", "NDEF", or "REF" representing     |

|             whether <name> has been defined, undefined,     |

|             or referenced but undefined; or ABS, REL, EXT,  |

|             or NEXT representing a test of the mode or      |

|             class of the symbol.                            |

|_____________________________________________________________|

# Pseudo-OPs IFx - Type I

The IF1, IF2, and  IF3 conditional pseudo-OPs evaluate  TRUE when the as-

sembler is on pass 1, 2, and 3 respectively. Pass 1 is the  first pass used to

evaluate the  value of all symbols.  Pass  2  generates the  listing and cross

reference data file. Pass 2  will  be omitted if -NL is TRUE and -XR is FALSE.

Pass  3  generates  the object code. Macros  must  be  read in  on each  pass.

EQUates  must  be  read in on each  pass if they are  the  object of  an IFDEF

pseudo-OP,  otherwise, they  can be  read in  on  the first pass  only. In the

latter case, surround the *GET which gets the equate file with an IF1-ENDIF.

## MRAS Pseudo-OPs - Conditionals

2 - 34

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OPs IFxx - Type II

Among the Type II constructs, using  "IFLT", if the value of expression_1

is less than the value of expression_2,  then the  conditional clause will  be

assembled. Using "IFEQ",  the  conditional  clause  will be assembled only  if

expression_1 and expression_2  have  equal values.  The "IFGT"  pseudo-OP will

assemble the conditional  clause  (i.e.  result in a  TRUE  condition) only if

expression_1 has a value exceeding  that of expression_2. The last possibility

is  "IFNE", which  will  cause the assembly  of the conditional clause if  the

expressions are not of equal value.

If,  for  instance,  you want to ensure  that a program does not assemble

code  past a particular address, then the  ERR pseudo-op could be used in con-

junction with IFGT to force an assembly error as follows:

IFGT  $,MAXADDRESS

ERR    Program is too long!

ENDIF

which compares the  current  value of the  program  counter (PC) to  some pre-

viously  specified maximum address.  Once the  PC exceeds  this maximum value,

the condition  evaluates  TRUE  resulting  in an assembly of the segment.  The

"ERR" pseudo-OP is used to force an assembly error.

# Pseudo-OPs IFyy - Type III

Among the Type III constructs, "IFDEF  name" will evaluate TRUE if "name"

has been defined prior to the evaluation of the IFDEF on each  assembler  pass

or if  name has  been  declared EXTRN. "IFNDEF  name"  will evaluate  TRUE  if

"name"  has  NOT been  defined prior to  the evaluation of  the IFNDEF on each

assembler pass nor has  it  been declared  EXTRN. "IFREF  name"  will evaluate

TRUE if "name" has been referenced but NOT defined  prior to the evaluation of

the IFREF on each assembler pass.

The  "IFEXT  name"  pseudo-OP  will  evaluate TRUE  if  "name"  has  been

declared EXTRN.  "IFNEXT  name" will evaluate  TRUE  if "name" is not declared

extern. "IFABS  name" will evaluate TRUE  if "name" is defined  in an absolute

segment whereas  "IFREL  name" will  evaluate TRUE if "name" is defined in one

of the relative segment types (code, data, common).

The Type III constructs will find greater  use  when working  with source

libraries of code. For instance, if a clause  is a routine  that is surrounded

with an IFREF-ENDIF conditional, the routine will only  be  assembled if prior

to the clause, the "name"  has  been referenced but not yet defined. If "name"

is the entry point symbol  to the routine,  then the routine will be assembled

if it is needed. Similarly, you may  have  a library routine that is always to

be placed in your program  unless its "name"  has already been defined in some

alternate  routine.  Surrounding  it  with the  IFDEF-ENDIF  conditional  will

inhibit its assembly if your program has defined that "name".

MRAS Pseudo-OPs - Conditionals

2 - 35

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Suppressing FALSE Conditionals

If during the listing pass, you  want to  suppress the listing of certain

conditional  clauses that  are  not  assembled  (i.e.  they  are  evaluated as

FALSE), use the following sequence of operators:

*LIST OFF

IF    expression

*LIST ON

clause

*LIST OFF

ENDIF

*LIST ON

With this sequence, the "IF" and "ENDIF" lines will always be suppressed.  The

conditional clause will  only be  listed  if the condition  being evaluated is

logically TRUE. If no FALSE conditional segment is to  be listed, then you may

use the assembler -NC switch which  inhibits the listing  of all FALSE  condi-

tionals - including the IF-ENDIF statements.

# Pseudo-OP ENDIF

Each  IF statement  must be matched  up with  a corresponding  ENDIF. The

ENDIF is needed to define the scope of the conditional clause.

# Pseudo-OP COM

This pseudo-OP is used  to generate a  comment record  in the object code

file of a directly generated CMD file. Its syntax is:

_____________________________________________________________

|                                                             |

|  COM   <string>                                             |

|                                                             |

|  <string>   is the information to be placed as a comment.   |

|_____________________________________________________________|

An object deck  comment block can be generated within the  executable ob-

ject code  file directly by using the COM  pseudo-OP.  The comment string must

have a length less than 128  characters. As  can be noted, the comment  string

must be enclosed in angle  brackets.  The closing bracket may be  omitted.  If

lower case characters are desired,  then single quotes must surround the angle

brackets.  Neither the quotes nor  the  angle brackets will be  a  part of the

comment record.

The  COM  pseudo-OP will  generate a comment block in  the object file of

the  format  X'1F'  followed  by  the string  length, followed by  the  string

itself. A  typical use  would be to place a non-loading copyright statement in

an executable object code file. For example:

COM   '<Copyright (c) 1985 by MISOSYS, Inc.>'

MRAS Pseudo-OPs - Miscellaneous

2 - 36

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

will  produce the  comment  record which would  be viewed  if  the  file  were

listed.

The generation of the COM  object  code record will be inhibited  if  the

assembly is  performed using the -CI switch. A binary core-image  file can not

have a non-loadable record.

# Pseudo-OP ERR

The ERR pseudo-OP is used to force an assembly error. Its syntax is:

_____________________________________________________________

|                                                             |

|  ERR {message}                                              |

|                                                             |

|  message    is an optional message to inform what is wrong. |

|_____________________________________________________________|

This  pseudo-OP  forces  an  immediate  warning error  and  displays  the

optional message.  It  is  commonly  used  in a  conditional clause for  error

trapping.

# Pseudo-OP OPTION

This  pseudo-OP  is  used  to  alter the  state  of  any of the assembler

switches entered on the command line invoking the assembly. Its syntax is:

_____________________________________________________________

|                                                             |

|  OPTION  {-/+}switch{,-/+switch},...                        |

|                                                             |

|  -/+     An optional prefix to turn the switch OFF or ON    |

|                                                             |

|  switch  Any of the permissable assembler switches.         |

|_____________________________________________________________|

Prefix each switch  with  "-"  to turn OFF,  or "+" to turn  ON (i.e. +NL

suppresses  the listing - sets  the  NO  LISTING switch  to  TRUE). If "+"  is

omitted, it is assumed. The COMMA separator is  mandatory if you omit the "+".

OPTION switches over-ride command line switches.

The OPTION pseudo-OP is only processed during the first pass;  therefore,

you cannot use  it to dynamically switch options ON  and OFF during an  assem-

bly.  It  is  used to conveniently  set options specific to a source stream to

eliminate the need for their entry on the assembler command line.

MRAS Pseudo-OPs - Miscellaneous

2 - 37

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Pseudo-OP REF

REF may  be  used to force a reference to the symbol(s) identified in the

argument list. Its syntax is:

_____________________________________________________________

|                                                             |

|     REF  symbol1{,symbol2},...                              |

|                                                             |

|  symboln    A "name" to be force-referenced.                |

|_____________________________________________________________|

This function  may  be useful  to force references to macros so that they

may be loaded via a '*SEARCH' operation.

# Listing Pseudo-OPs

Four pseudo-OPs are available to  control the  assembler  listings. These

are: PAGE, SPACE, SUBTTL, and TITLE. Their syntax is:

_____________________________________________________________

|                                                             |

|  PAGE                                                       |

|                                                             |

|  SPACE   n                                                  |

|                                                             |

|  SUBTTL  {<string>{                                         |

|                                                             |

|  TITLE   <string>                                           |

|                                                             |

|  n          Specifies how many line feeds to generate.      |

|                                                             |

|  <string>   Is the title or sub-title string to appear in   |

|             the listing headings.                           |

|_____________________________________________________________|

A new page can be  forced to  provide  separation of routines, functions,

etc.  by  using  the PAGE pseudo-op.  This pseudo-OP  will be  ignored  if  it

appears  between *LIST  OFF and  *LIST ON.  PAGE statements are  automatically

suppressed  from the listing.  PAGE  will output  a FORM FEED  character  only

during the listing pass.

"SPACE n" performs line  spacing whenever the "SPACE"  pseudo-OP is used.

When  assembled, "n"  is  the number of  lines to space and is interpreted  as

modulo  256. The  line containing the SPACE  pseudo-op is  not displayed. This

pseudo-op also will be ignored if it appears between *LIST OFF and *LIST ON.

A sub-title  to a  heading  is permitted with the "SUBTTL" pseudo-OP. The

subtitle  string  length can be from zero (0) to  80 characters in  length.  A

zero length indicates that sub-titling is disengaged.  The SUBTTL string  does

not need  to be  enclosed in  angle brackets; they  are  optional. SUBTTL also

automatically invokes a PAGE.

MRAS Pseudo-OPs - Miscellaneous

2 - 38

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

Lower case strings can be maintained by  the  use of  single quotes which

surround the angle brackets. You may change the subtitle  by  using additional

SUBTTL pseudo-OPs throughout  the  text.  Subtitles will appear  on  the first

page  following  the SUBTTL pseudo-op. If the SUBTTL text string  is null  (of

zero length),  then subtitling will cease on the subsequent  page. A line will

also be skipped between the subtitle and first printed text  line on the page.

Where  many  *GETs are being used, you  may want to establish  a sub-title for

each to provide a visual indication on the listing.

The  TITLE  pseudo-OP automatically invokes a page heading  and adds  the

title to the headings of assembler listings. The title  string  is limited  to

28 characters and only one  TITLE  is  accepted. The  angle  brackets must  be

entered but are not output in the  listing  - they serve only  to delimit your

title string. The title line will include the MRAS version, the  date and time

retrieved from the system, your title string, and a page number  [page  number

is limited to the range <1-255> and  will wrap around to zero if more than 255

pages  are printed]. For this reason,  if you use  a title, it is advisable to

set DATE  and TIME prior to  executing the assembler.  A line  will be skipped

between the title and  start of printed text (or subtitle if used). Lower case

titles  will  be maintained by  surrounding  the  angle  brackets with  single

quotes as in:

TITLE   '<This is an UC/lc title>'

The first TITLE pseudo-OP found  in the text  will be used  for  titling.

All other TITLE pseudo-ops will be ignored.

MRAS Pseudo-OPs - Miscellaneous

2 - 39

The MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Assembler Directives

MRAS supports seven assembler  directives. In contrast to  source  state-

ments  which  are  translated  to  machine  language,  these   directives  are

"conversation" to  the assembler. Each  directs the assembler  to behave  in a

particular manner  or perform a specific function.  The directives do not gen-

erate any  machine language code by themselves, they  merely act as "commands"

to the assembler. Each "command" must  start in column one of a source  state-

ment line, and must start with  either an asterisk  "*"  or a period  ".". The

entire directive word may be entered,  or it may be abbreviated to its minimum

unique character string. The assembler directives are:

_____________________________________________________________

|                                                             |

|  *Get file   Causes the assembler to begin reading source   |

|              code from the "file".                          |

|                                                             |

|  *Inc file   Causes the assembler to begin reading source   |

|              code from the file identified on the command   |

|              line via "+I=filespec". Treated as *GET if no  |

|              "+I=filespec" was specified.                   |

|                                                             |

|  *List OFF   Causes the assembler listing to be suspended,  |

|              starting with the next line.                   |

|                                                             |

|  *List ON    Causes assembler listing to resume, starting   |

|              with this line.                                |

|                                                             |

|  *Mod exp    Advances the "module" character substitution   |

|              string.                                        |

|                                                             |

|  *RAdix exp  Changes the default radix to expression which  |

|              must evaluate to the range <1-16>.             |

|                                                             |

|  *REquest    Generates a Special Link Item to request a     |

|              search by the linker of the library file       |

|              identified in the *REQUEST directive.          |

|                                                             |

|  *Search lib Invokes an automatic search of the Partitioned |

|              Data Set (PaDS) "lib" to resolve any undefined |

|              references capable of being resolved by PaDS   |

|              assembler source member modules.               |

|_____________________________________________________________|

## MRAS Assembler Directives

2 - 40

The MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

*GET filespec

This directive invokes assembly from a source disk file. Its syntax is:

_____________________________________________________________

|                                                             |

|  *Get filespec/ASM                                          |

|                                                             |

|  filespec    Causes the assembler to begin reading source   |

|              code from the file, "filespec".                |

|_____________________________________________________________|

This directive tells the assembler to temporarily switch  its  source as-

sembly to  the  file identified as  "filespec",  and  use it  to continue  the

assembly. A default file  extension of  "ASM" will be used if none is provided

in  the directive statement. The file itself can be  headered and/or numbered,

as MRAS will  automatically detect its  type and adjust  accordingly; however,

all  source  files  must  be  similarly structured.  When  the  end-of-file is

reached,  or an assembly  language "END"  statement is  read, assembly resumes

from the next statement following the statement which invoked the "*GET".

"*GETs" can  be nested to four (4) levels. That is, a statement can GET a

file which GETs a  file which GETs a file file which GETs a file. This  assem-

bler directive is  extremely powerful. It can be used to  provide the capabil-

ity  of assembling  large programs  which  are stored on disk  in a series  of

source files as one assembly stream.

*INCLUDE filespec

This directive invokes assembly from a source disk file. Its syntax is:

_____________________________________________________________

|                                                             |

|  *Include filespec/ASM                                      |

|                                                             |

|  filespec    Causes the assembler to begin reading source   |

|              code from the file identified on the command   |

|              line via "+I=include"                          |

|_____________________________________________________________|

This directive tells the  assembler to temporarily switch its source  as-

sembly to the file identified  on  the MRAS command line via the  "+I=include"

file  switch.  If  no "+I=" file switch  was entered, the *Include  is treated

exactly as if it were a "*Get filespec."

MRAS Assembler Directives

2 - 41

The MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# LIST ON/OFF

This is used to suppress the listing of blocks of code. Its syntax is:

_____________________________________________________________

|                                                             |

|  *List off/on                                               |

|                                                             |

|  OFF         Causes the assembler listing to be suspended,  |

|              starting with the next statement.              |

|                                                             |

|  ON          Causes assembler listing to resume, starting   |

|              with this statement.                           |

|_____________________________________________________________|

The pair  of directives, "*LIST OFF" and "LIST ON",  can be used to  sup-

press the  listing of a block of  code.  All statements  which follow a "*LIST

OFF" will be suppressed  during  the  listing pass. The "*LIST ON" will resume

standard  listing.  An  exception to the suppression  is  that  any  assembler

source statement containing an  assembly  error will be listed along  with its

appropriate error message. In this  manner, you can  use an "*LIST OFF" direc-

tive at the beginning  of your  assembly  source (to suppress all listing) and

lines containing errors will be forced to be displayed.

*MOD

This directive  increments  a  character  substitution string to simulate

local labels in blocks within one module. Its syntax is:

_____________________________________________________________

|                                                             |

|  *Mod                                                       |

|                                                             |

|              Advances the "module" character substitution   |

|              string.                                        |

|_____________________________________________________________|

The  *MOD directive  will  increment a string  replacement variable  each

time the directive is  executed. The string will replace  the  question  mark,

"?", character  in labels and label references found in any statement. Its use

is essentially applicable to subroutine libraries where duplication  of labels

could occur. By specifying  the *MOD directive  as the first statement of each

module  of  code and by  using  a  question mark in  labels, you can construct

source  subroutine libraries for  use in your programs without having to worry

about  duplicate labels occuring. Unless at least one *MOD statement  is spec-

ified, the question mark will not be translated.

Labels  such  as $?001 will have  the  "?" replaced with  the current MOD

string value. Thus,  a *MOD directive preceding  each module will  force $?001

labels  in each  module to  be distinctly  named by  having  the question mark

replaced with the substitution  string.  The MOD string value cycles from A-Z,

then  from  AA-AZ, BA-BZ, ...,  ZA-ZZ, then from AAA-AAZ,  BAA-BAZ,  ..., ZZZ.

MRAS Assembler Directives

2 - 42

The MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

This will  allow for  a simulation of "local"  labels. Remember, the "?"  sub-

stitutions will only be made if *MOD was specified.

*RADIX expression

This directive sets the default radix for  all numeric  terms  except for

"*RADIX expressions" which always default to 10. Its syntax is:

_____________________________________________________________

|                                                             |

|  *RAdix expression                                          |

|                                                             |

|  expression  Is evaluated and becomes the new default radix |

|              for all numeric terms. The value of expression |

|              must be in the range <1-16>.                   |

|_____________________________________________________________|

Note that in the evaluation of the expression  for  the *RADIX directive,

the assembler  will always use  a radix default of 10. The assembler  defaults

to a radix of 10 unless overridden by a *RADIX directive.

*REQUEST lib1{,lib2},...

This  is used to convey information  to the  linker.  It will  generate a

"Request Library Search" special link item for use by MLINK. The syntax is:

_____________________________________________________________

|                                                             |

|  *REquest lib1{,lib2},...                                   |

|                                                             |

|  libn        The 1-7 character name of the REL library to   |

|              be searched by the linker.                     |

|_____________________________________________________________|

*REQUEST will generate the  link item to the linker for each library name

identifed in the argument list.

*SEARCH filespec

This directive  is used  to  invoke an  automatic search of a Partitioned

Data  Set (PaDS)  source library,  "filename/LIB",  for all members  that will

resolve  undefined references  in the source stream.  This  provides  a source

library structure.  *SEARCH  will  require  two (2)  levels of "*GET" nesting.

Also, a *SEARCH member cannot use a  *GET  directive or another *SEARCH direc-

tive. The default  file extension for searched files is "LIB".  The  syntax of

*SEARCH filespec is:

## MRAS Assembler Directives

2 - 43

The MISOSYS Relocating Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

_____________________________________________________________

|                                                             |

|  *Search filespec/LIB                                       |

|                                                             |

|  filespec    Invokes an automatic search of the PaDS        |

|              "filespec/LIB" to resolve any undefined        |

|              references capable of being resolved by        |

|              PaDS assembler source member modules.          |

|_____________________________________________________________|

The  PaDS  source  library constitutes members  composed of one  or  more

routines.  Each  routine should have its  routine name (the label field entry)

in  the PaDS member directory. This  is accomplished by naming the source file

to  be appended  to the library the same name as  the  routine or by appending

using a  MAP. Details  on  constructing  and  using  Partitioned Data  Sets is

included with PaDS documentation. The PaDS utility is available separately.

MRAS  will search the PaDS library and locate a member name  that matches

up  with  a  symbol  table entry. If that symbol  is  currently undefined, the

member will be  accessed and read just as if it  were  the target of  a  *GET.

MRAS will verify that the member just accessed  did in fact define  the symbol

invoking its  access. If a  member  is  accessed and there exists  no  defined

symbol in the member that has  the same  name as the  member name,  MRAS  will

abort the assembly and advise of a library error by displaying the message:

Member definition error: filespec(member)

After  the member's source code is read, MRAS will continue to search the

PaDS library until it  exhausts all members. There are  no restrictions on the

order  of  members.  Routines in one member can reference  other  members with

complete disregard as to any ordering of entries in the PaDS.

Where more  than one routine is in a  member,  each should be surrounded

by  IFREF/ENDIF and each  should have  an entry  in the member directory  (you

must  use the MAP option of  PaDS  to provide multiple entries to  a  member).

This will benefit by not  having needless routines appear in  your object code

output. For example, the following depicts two routines stored as one member.

; Entry for routine entitled "MOVE"

IFREF  MOVE

MOVE    .            ;Routine of code

.

ENDIF

; Entry for routine entitled "SHIFT"

IFREF  SHIFT

SHIFT   .            ;Routine of code

.

ENDIF

If  your  source  code  references  "SHIFT"  but  not "MOVE", as  long as both

"SHIFT" and "MOVE" are member  entries in the PaDS library, a *SEARCH  of  the

library will access the member and assemble only the SHIFT routine.

MRAS Assembler Directives

2 - 44

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# What is a MACRO?

In virtually all programs,  you find particular sequences  of  code  that

are repeated. These sequences may be termed  routines.  They could be so short

that the overhead needed to set them up  as CALLable  routines is ineffective.

Or, they  could be longer routines that just cannot be constructed as CALLable

segments.  You  may even want  a code  sequence to be  an  in-line assembly in

contrast to  a  CALLable routine for the purpose  of fast execution. The  most

useful function is  to be  able to have  parameterized  routines -  algorithms

that operate on different values each time the algorithm is invoked.

There are a few ways  to deal with routines  that are  repeated in a pro-

gram.  You  could block copy  it from the  first  appearance to  wherever  you

needed the routine. Or you could establish  the routine as a macro. The  first

method could  take  up more source storage than is desirable. Also, if you de-

cide to change  the routine's algorithm, having many  copies in a  program can

be cumbersome to update.

The second method mentioned is the use  of macros. Consider the following

commonplace sequence of code:

LD     HL,VALUE

LD     (MEMORY),HL

How many  times is this little sequence repeated in  your programs? Five? Ten?

If we set up a macro near  the beginning  of our program that looked something

like this:

STOR MACRO  #VAL,#MEM     ;Macro to store "VAL" into memory

LD     HL,#VAL       ;Get value into HL

LD     (#MEM),HL     ;Load value into memory

ENDM                 ;End of the macro

we could perform the above two statements with one macro call as follows:

STOR   VALUE,MEMORY  ;Invoke the macro

The first part of  the example, defines a macro called  "STOR".  This is  done

exactly once per program! If  we save our macros in a macro  source file, each

of our programs could "*GET MACROS";  thus, we would not have to even manually

enter the macro into each program.

We invoke the statements  defined in the  macro  by specifying  the macro

name  AS IF IT WERE AN OPCODE. Using the macro invocation method,  we can save

storage  space and introduce structured techniques to our coding.  Notice that

we  have used some fictitious  names when the  STOR  macro  was defined. These

names  are called "dummy" parameters. They serve to  provide  a means to  pass

actual parameters  when the macro is  invoked.  Through  the dummy parameters,

the real  power  of the macro  is  utilized.  During the macro invocation, the

model  statements are  expanded with substitutions for  the  dummy  parameters

that are provided in the macro call.

MRAS - Using Macros

2 - 45

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# MACRO Definition

The format for a macro definition is illustrated as:

_____________________________________________________________

|                                                             |

|     MOVE     MACRO   #parm1,#parm2=dflt2,#parm3             |

|              LD      HL,#parm1                              |

|              LD      DE,#parm2                              |

|              LD      BC,#parm3                              |

|              LDIR                                           |

|              ENDM                                           |

|_____________________________________________________________|

The macro definition consists of  three parts: a macro prototype, a macro

model,  and  the ENDM statement. The  prototype is used to  specify  the macro

name and  the  dummy parameter names used  in the model. Default substitutions

may be  specified  in the prototype  to be used if the corresponding parameter

is  not  passed in the macro  invocation. The macro model contains all of  the

assembler statements  to be generated when the macro  is invoked. The model is

sometimes  called  the  macro skeleton or template. The dummy parameter  names

occupy the positions where the  actual parameters will  be placed by the macro

processor  in  MRAS. The third  part, the  ENDM statement, is used to indicate

the end of the macro model.

When a macro  is  defined,  it is  not  assembled into your  program. The

macro prototype is parsed and analyzed.  The  macro  definition is then stored

in a compressed format  within the macro storage area. Comments appearing with

the  macro definition are  not  stored  if  the comment  starts with  a double

semi-colon in lieu  of  a single one.  Comments  with a single  semi-colon are

thus carried through a macro expansion to the listing.

Macro definitions may be nested. The  inner macro will not become defined

until  the outer macro is expanded during an invocation. However, since macros

cannot be redefined, the outer macro should be invoked only once!

# Macro Prototype

Macros  are  named just like symbolic labels. The same  rules  apply. The

number sign  "#" is  used to denote a  parameter in the macro  prototype; how-

ever, its use is optional. It is still required  in  the macro model to  indi-

cate the start of a parameter name.  The length of  macro names can range from

<1-15>. Special characters  <@, $, _> may  be  used in the name construct.  Do

not use the question mark in macro names as  it would conflict with the symbol

substitution string use made of "?".

MRAS - Using Macros

2 - 46

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

The MACRO  pseudo-OP is  used  to define  the prototype of a macro model.

Its syntax is:

_____________________________________________________________

|                                                             |

|  mname  MACRO  {#parm1}{=dflt1}{,#parm2{=dflt2}}{,...}      |

|                                                             |

|  mname       is the macro name used to invoke the macro.    |

|                                                             |

|  #parmn      are dummy parameters of the macro which will   |

|              be replaced by actual parameters during the    |

|              macro invocation. "#" is an optional prefix.   |

|                                                             |

|  dfltn       are optional default strings to be used for    |

|              the dummy parameters when a parameter is not   |

|              provided in the macro invocation.              |

|_____________________________________________________________|

The  upper limit on the  number of macro parameters  is 127; however, you

can  not exceed the length of a standard assembler source statement. Thus, the

statement length  becomes  the limiting  factor.  As  is  the case  with macro

names, the  rules  for naming  dummy parameters are identical to the rules for

labels. If a macro parameter is enclosed  in angle brackets, the entire string

which  is enclosed within brackets will be treated as one parameter  - even if

it  contains  separator  characters. Neither the macro  names nor the  "dummy"

names  are  included  in the symbol table  generated by MRAS, thus there is no

restriction  on reusing the same name  as  a "dummy"  for a label; however, to

avoid confusion, it is  recommended that you  avoid  using dummy names as sym-

bolic label names.

Default strings can contain  any  character  except  the  comma, ",". The

comma is used  as a field delimiter. There is no limit to the length  of a de-

fault string other than the limiting factor of the statement length.

Macros  must  be  defined prior to use but can  be defined in  a separate

disk file accessed via a "*GET filespec".

MACRO parameters are acceptable within a quoted string if prefixed by  an

ampersand. i.e. TEST DB '&#NAME'. See the following example.

5200          00002 FEED    MACRO   #STRING

5200          00003 $?1     JR      $?2

5200          00004 LABEL?  IRPC    XX,#STRING

5200          00005 LABXX   DB      '&XX'

5200          00006         IFGT    $-LABEL?,3

5200          00007         EXITM

5200          00008         ENDIF

5200          00009         ENDM

5200          00010 $?2     LD      HL,LABEL?

5200          00011         ENDM

5200          00012         FEED    012345

5200+1806     00013 $A1     JR      $A2

00014 LABELA  IRPC    XX,012345

MRAS - Using Macros

2 - 47

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

5202+         00015 LABXX   DB      '&XX'

5202+         00016         IFGT    $-LABELA,2

5202+         00017         EXITM

5202+         00018         ENDIF

5202+         00019         ENDM

5202+30       00020 LAB0    DB      '0'

00021         IFGT    $-LABELA,2

00022         EXITM

00023         ENDIF

5203+31       00024 LAB1    DB      '1'

00025         IFGT    $-LABELA,2

00026         EXITM

00027         ENDIF

5204+32       00028 LAB2    DB      '2'

00029         IFGT    $-LABELA,2

00030         EXITM

00031         ENDIF

5205+210252   00036 $A2     LD      HL,LABELA

0000          00037         END

# Macro Model

Any valid Z-80 statement, MRAS pseudo-OP,  or assembler directive (except

*GET or *SEARCH) is valid in the macro model.

# ENDM pseudo-OP

This pseudo-OP is used to specify the scope of a macro model.  It is used

much like ENDIF. Its syntax is:

_____________________________________________________________

|                                                             |

|  mname  MACRO  parms                                        |

|         model statements                                    |

|         ENDM                                                |

|_____________________________________________________________|

The ENDM pseudo-OP must be used  to let the macro processor know what  is

the last macro model statement. If macros are nested, each must have an ENDM.

# EXITM Pseudo-OP

This pseudo-OP  can be used  to prematurely exit from  a MACRO expansion.

This is normally used  within a conditional  clause. One  level of conditional

nesting will be removed (if any are present). See the example for IRP.

MRAS - Using Macros

2 - 48

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Macro Definition Examples

This macro will  move a block of memory from one  location to another. If

the "length" parameter is omitted, then a value of "255" will be used:

MOVBLK  MACRO  #FM,#TO,#LEN=255

LD     HL,#FM

LD     DE,#TO

LD     BC,#LEN

LDIR

ENDM

This is  a macro to clear a region of memory (i.e. set to 0). This  macro

will invoke the MOVBLK macro in a nested invocation:

CLRMEM  MACRO  #BUF,#LEN=255

LD     HL,#BUF

LD     (HL),0

MOVBLK #BUF,#BUF+1,#LEN

ENDM

This macro will add the 8-bit register "A" to 16-bit register pair "HL":

ADDHLA  MACRO

ADD    A,L

LD     L,A

ADC    A,H

SUB    L

LD     H,A

ENDM

A macro  is not required to contain  dummy parameters as is  evidenced  by the

last example.

# Incorporating Conditionals

Conditional  pseudo-OPs can be specified in  macro  models. For instance,

say you want the MOVBLK  macro to be able to perform a non-destructive move (a

destructive move would be where  the destination is an address  between "from"

and  "from+length-1").  You  can  insert  conditional pseudo-OPs  to test  the

parameters during the assembly of the expansion. Don't  forget that the actual

labels  substituted for parameters  must  be  defined  prior  to  invoking the

MACRO! Then,  only certain segments of the macro  will be  assembled according

to the result of the evaluation. Analyze the following example:

MOVBLK MACRO   #FM,#TO,#LEN=255

IFNE    #FM,#TO         ;Don't expand if #FM=#TO

LD      BC,#LEN         ;Establish the length

IFGT    #FM,#TO         ;Do we LDIR or LDDR?

LD      HL,#FM          ;#FM > #TO => LDIR

LD      DE,#TO

LDIR

MRAS - Using Macros

2 - 49

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

ELSE

LD      HL,#FM+#LEN-1   ;#TO > #FM => LDDR

LD      DE,#TO+#LEN-1

LDDR

ENDIF

ENDIF

ENDM

# MACRO Nesting

The CLRMEM example depicts a macro  that nests a macro invocation. Macros

may be nested to seven (7)  levels. That is, at any time, macro expansions for

7 macros called in a chain can be pending. For example:

ABC   MACRO    #PARMS,...

(model statements)

MOVE     parm,parm   ;call macro "MOVE"

(model statements)

ENDM

MOVE  MACRO    #parm1,#parm2,#parm3

(model statements)

ENDM

is perfectly legal. The expansion of the "MOVE" macro is not  performed during

the definition of the "ABC" macro but rather during the invocation of "ABC".

Macro  definitions also  may be nested. The inner  macro  will not be de-

fined until the outer macro is expanded. For instance:

ABC   MACRO     #PARM

(model statements)

XYZ   MACRO     #PARMs,...

(model statements)

ENDM

ENDM

is a  legal  macro definition. The inner macro (XYZ) will not be defined until

the outer macro (ABC) is invoked. Note the two ENDM statements.

If  macro A  "calls"  another macro,  say B, any  dummy  parameter in the

macro call  of  B that  matches a dummy in macro A, will be considered part of

macro  A  and the parameter  substitution  will  be invoked by  the  parameter

passed when the user calls macro A.

# MACRO Invocation

The invocation of a macro is termed a macro  "call". The macro  processor

then  proceeds to replace  the call with  the  model statements specified when

the macro was  defined.  The replacement  of the macro call by the macro model

statements is termed the macro "expansion".

MRAS - Using Macros

2 - 50

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

During the expansion, the  "actual" parameters passed in the call  state-

ment  are substituted for the "dummy" parameters  which  appear in  the  macro

model  and which  are designated in  the prototype of the macro. Note that the

actual parameter values  are character strings and can be labels, expressions,

or data  constants. An actual  parameter can even be a  quoted string data de-

claration if its use is designed into the macro model.

The entire expanded macro model is listed during  the listing pass (phase

two). Macro  expansions in the  listing will be so noted by the appendage of a

plus  sign immediately following the line number displayed. You  may find that

you don't  really want  to see  these expansions  since the  macro  definition

contains the entire illustration of the macro.  An assembler switch,  "-NM" is

provided  to suppress listing of macro expansions. In the case of nested macro

calls (i.e. a macro is defined which calls  another macro which was separately

defined), only the primary macro call will  be listed if the "suppress" switch

is invoked.

The substitution  of  the actual  character  string  parameters  for  the

dummys occurs during the  macro expansion  when the  macro is  called. Since a

macro  can have more than one parameter, it is necessary  to have a  procedure

that  specifies which  actual parameter corresponds to each  dummy  parameter.

There are  two  methods  supported in  MRAS. Parameters  can be passed  to the

macro expansion when calling by either position or keyword.

# Positional Parameters

"Positional"  parameters are correlated by  the position they  appear  in

the macro call. For example, if the "MOVBLK" macro was called with:

MOVBLK  VIDEO,CRT_BUFFER,CRT_SIZE

then the substitution string "VIDEO" would replace every appearance of  "#FM",

the  string   "CRT_BUFFER"  would  replace  every  appearance  of  "#TO",  and

"CRT_SIZE"  would  replace  the  dummy parameter,  "#LEN".  Note  that  actual

strings are positionally  correlated with the  positions  of  the dummy  para-

meters in the macro prototype.

If you wish to  omit an actual  parameter in a macro call, then you  must

supply the comma to denote its place. For instance:

SHIFT  4200H,,100H

omits  the  middle of three parameters. Generally,  a default would  have been

provided in the macro definition.

# Keyword Parameters

If  the  number of parameters is large, it is sometimes burdensome to re-

member  the order  of  the  parameters, or  to provide the  correct number  of

commas if a series of parameters are  omitted. These drawbacks are remedied by

the use of "keyword" parameters. The macro call  parameter  list can  identify

MRAS - Using Macros

2 - 51

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

the actual parameters by using the name of the dummy  parameter  as  well. The

keyword syntax is:

_____________________________________________________________

|                                                             |

|                #dummy=actual parameter                      |

|                                                             |

|         mname  #parm2=actual2,#parm3=actual3                |

|_____________________________________________________________|

If  the previous  macro  call was invoked by  keyword parameter  specifi-

cation, it could look something like this:

SHIFT  #LEN=100H,#FM=4200H

# Mixing Positional and Keyword Parameters

A single macro  invocation  can  intermix  both  positional  and  keyword

parameters.  The point that needs clarification,  is  what positions are actu-

ally denoted in the  parameter list. It  is  simply treated. In a  mixed para-

meter  list, keyword parameters  are ignored when considering place positions.

For example, in the following macro call:

SHIFT  #LEN=100,BLOCK,BUF_START

even though  the length parameter appeared first in the  parameter list, since

it was  designated as a keyword,  it is ignored from the  positional count and

"BLOCK" is the first parameter with "BUF_START" second. In a similar manner:

COMP   PARM1,#P6=2,,PARM3,#P8=38,PARM4

"PARM1" is in  position one, the second parameter is omitted  (the double com-

ma), "PARM3" and  PARM4" are in the third and  fourth positions  respectively.

The sixth and eighth parameters have been entered by keyword.

Note that the  parameter  list contains five parameters. Thus if you were

to  use the "%%" operator which returns the number  of parameters passed in  a

macro call ("%%" is described later), it would return a value of five.

# Local Labels

So far, all of the examples have shown macro  models without labels. What

would happen if we had a macro defined as follows:

FILL MACRO  #CHAR,#NUM

LD     B,#NUM

FLP  LD     (HL),#CHAR

INC    HL

DJNZ   FLP

ENDM

MRAS - Using Macros

2 - 52

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

We would  have a problem because every time the  macro was  called, the label,

"FLP", would be  used.  If "FILL"  was  invoked  more than once, the assembler

would  generate MULTIPLY DEFINED SYMBOL errors on each expansion.  We have  to

be able to use labels, but we  need to find a way to be able to make  "unique"

labels on each macro expansion.

MRAS provides  a facility for doing this by keeping a substitution string

which  is  changed each  time a macro  is expanded.  The  string replaces  the

question  mark  character, "?", during a  macro expansion  whenever it appears

outside of single quotes in  a  macro model  statement. Each  time a macro  is

expanded, the string  will  be  changed. The  string starts  with  the  single

letter  "A",  changes  to  "B",  ..., "Z",  then increments  to the two-letter

strings,  "AA", "AB", ..., "ZZ", then to three  letter  strings, AAA-ZZZ  each

time  a  macro  call is  made.  By  using the question  mark  as  one  of  the

characters in symbols of a macro  model statement,  it will  uniquely identify

labels  local  to a  macro. You may  want  to standardize  the  way you create

labels to  ensure  that  uniqueness is  maintained. For example,  you may  use

macro  labels of the  form,  "$$?1",  "$$?2", ...  You  can repeat the use  of

"$$?1",  "$$?2", ... in another macro  since  the  substituted  string will be

uinique for each macro expansion.

The  substitution  string will be different from the *MOD directive  sub-

stitution  but  is similarly used. Macro expansion substitution  of "?"  takes

precedence over *MOD  substitution. In the  case  of nested macros, each  nest

level will have its own unique substitution.

By using  the question mark string substitution specifier,  the  previous

macro would be defined like this:

FILL MACRO  #CHAR,#NUM

LD     B,#NUM

$$?1 LD     (HL),#CHAR

INC    HL

DJNZ   $$?1

ENDM

# String Comparisons

It is  sometimes desirable to be  able to test  within a macro model, the

exact string  passed as  a parameter.  Four  conditional pseudo-OPs  have been

added strictly for string comparisons within macro processing. These are:

_____________________________________________________________

|                                                             |

|  IFLT$    string1,string2     TRUE if string1 < string2     |

|                                                             |

|  IFEQ$    string1,string2     TRUE if string1 = string2     |

|                                                             |

|  IFGT$    string1,string2     TRUE if string1 > string2     |

|                                                             |

|  IFNE$    string1,string2     TRUE if string1 <> string2    |

|_____________________________________________________________|

MRAS - Using Macros

2 - 53

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

These  pseudo-OPs  provide  TRUE/FALSE  evaluation  in the comparison  of

string1 to  string2 (like the non-"$" pseudo-OPs  do with mathematical expres-

sions). Obviously,  hard encoding of both  string1  and string2 would  be non-

sense!  Aha, he said... If we use a macro dummy  parameter, it will be substi-

tuted by the actual parameter string passed in  the macro call expansion. This

means that  the macro itself can test  the parameter string in a limited  man-

ner. For example:

IFNE$   #TO,(DE)

LD      DE,#TO

ENDIF

as part of a macro model, will have  the "#TO" replaced during  the expansion.

The  test becomes  dynamic!  The  dummy  parameter can  be either  string1  or

string2 - it doesn't matter.

These string conditional pseudo-OPs can only be useful  in macros. That's

because the evaluation, to make sense, has to be dynamic.

# Testing String Lengths

Another feature available in the  macro processor  is  the  per cent sign

"%" operator. This  operator  is used to  recover the  length  of  the  passed

parameter string and the number of  parameters passed  in the macro call. Note

that the limitation for the use of the "%" operator, is that  it is acceptable

only for  parameters of the current macro expansion. That means that you can't

test for lengths outside of  the current macro  if you are nesting macro calls

(macros cannot be recursive!). The operator can be used like these examples:

LD   B,%#PARM          ;loads B with the length of #PARM

IFGT %#PARM1,6         ;Restricts parm1 to a length <1-6>

ERR  Parm too long!

ENDIF

IFLT  %%,4             ;This macro requires 4 actual parms

ERR   Missing required parameters!

ENDIF

The "%%" operator will return the  number  of parameters passed in the current

Macro call.  When a  dummy parameter name  (including the  "#" prefix) follows

the per cent operator, the length of the parameter string is returned.

These values can be  tested arithmetically to produce a TRUE/FALSE result

(as was just  demonstrated), or they  can be used  directly to represent logic

TRUE/FALSE conditions.  Realizing that if  a parameter  was not  passed in the

parameter list of the macro call, its length  would be zero. A  zero is also a

logical FALSE. MRAS will accept as TRUE, any non-zero  value (in normal use of

TRUE/FALSE  specifications,  "-1" is recommended for  TRUE to  maintain proper

evaluation  of  the ".NOT."  operation). Thus, the string lengths can be mini-

mally  used to  test  if the  parameter was not passed (%#parm=0=FALSE) or the

parameter was passed (%#parm<>0=TRUE).

MRAS - Using Macros

2 - 54

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Concatenating MACRO Labels

You can concatenate a string to a dummy parameter  name by  connecting it

with the concatenation operator, "%&". For instance, the model statement:

IFREF    #NAME%&L

will have the "#NAME" replaced by  the MACRO call substitution string appended

with the letter "L".

# Special in-line MACROs

MRAS supports  the  standard  INTEL macro operations of  REPT, IRPC,  and

IRP. These macro operations immediately  expand the model statements according

to specifications  in  the  macro  prototype statement.  They may  also  be an

interior macro of a nested macro definition.

# Macro REPT

The statements within REPT-ENDM are  repeated according  to the result of

"expression". The syntax of this macro is:

label  REPT <expression>

statements

ENDM

In the  prototype  statement, the angle  brackets are not  required.  See  the

following example  which generates values from 1  through n  where "n" is con-

trolled by the value passed as "#COUNT" in the DOIT invocation.

5200          00002 DOIT    MACRO   #COUNT

5200          00003 T       DEFL    0

5200          00004         REPT    #COUNT

5200          00005 T       DEFL    T+1

5200          00006         DB      T

5200          00007         ENDM

5200          00008         ENDM

5200          00009         DOIT    3

0000+         00010 T       DEFL    0

00011         REPT    3

5200+         00012 T       DEFL    T+1

5200+         00013         DB      T

5200+         00014         ENDM

0001+         00015 T       DEFL    T+1

5200+01       00016         DB      T

0002+         00017 T       DEFL    T+1

5201+02       00018         DB      T

0003+         00019 T       DEFL    T+1

5202+03       00020         DB      T

MRAS - Using Macros

2 - 55

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Macro IRPC

The statements  within  IRPC-ENDM are repeated for  each character in the

character list while the "identifier" is  replaced with each character in turn

from  the identifier list. The identifier  can  be  a  multi-character  string

which is not a reserved word. This macro's syntax is:

label  IRPC identifier,character-list

statements

ENDM

See the following example which generates values from 1 to 3.

00002         IRPC    X,123

5200          00003         DB      X

5200+         00004         ENDM

5200+01       00005         DB      1

5201+02       00006         DB      2

5202+03       00007         DB      3

# Macro IRP

The  statements within IRP-ENDM are  repeated for as many items as are in

the  argument list with "dummy" being replaced  by each argument in  turn. The

angle brackets surrounding the argument list are mandatory. Its syntax is:

label  IRP <dummy>,<arg1,arg2,...,argn>

statements

ENDM

where label is  an optional  statement label. See the following  example which

generates values from 1 to 3 and makes use of the EXITM escape.

00003 LABEL   IRP     XX,<1,2,3,4,5>

5200          00004 LABXX   DB      XX

5200          00005         IFGT    $-LABEL,3

5200          00006         EXITM

5200          00007         ENDIF

5200+         00008         ENDM

5200+01       00009 LAB1    DB      1

00010         IFGT    $-LABEL,3

00011         EXITM

00012         ENDIF

5201+02       00013 LAB2    DB      2

00014         IFGT    $-LABEL,3

00015         EXITM

00016         ENDIF

5202+03       00017 LAB3    DB      3

00018         IFGT    $-LABEL,3

00019         EXITM

00020         ENDIF

MRAS - Using Macros

2 - 56

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# General

MRAS recognizes two types of errors. These are:

_____________________________________________________________

|                                                             |

|  DOS         This is an operating system disk I/O error.    |

|              The error message is displayed and control is  |

|              returned to DOS.                               |

|                                                             |

|  Assembler   These errors may occur while executing an      |

|             Assemble command. There are three types:        |

|             terminal, fatal, and warning.                   |

|_____________________________________________________________|

Disk I/O errors can  be received during an assembly.  When  an I/O  error

occurs, the assembly will be aborted and control will be returned to DOS.

Three different types of assembler errors  can occur. The types relate to

the severity of the error. These types are:

_____________________________________________________________

|                                                             |

|  Terminal   Assembly is terminated and control is returned  |

|             to command mode.                                |

|                                                             |

|  Fatal      Processing of the line containing the error is  |

|             immediately stopped and no object code is       |

|             generated for that line. Assembly proceeds      |

|             with the next statement.                        |

|                                                             |

|  Warning    The error message is displayed and assembly of  |

|             the line containing the warning continues. The  |

|             resulting object code may not be what the       |

|             programmer intended.                            |

|_____________________________________________________________|

Following is a list of all error messages and an explanation of each.

# DOS Errors

The standard DOS error messages will be displayed  if the  DOS returns an

error code  after  return from any disk  operation. Consult your DOS operating

manual for  explanations of those errors.  If an  I/O error is detected during

an  assembly,  the long form of  the  error message  will  be displayed.  This

provides an observance as to which file was affected by the I/O error.

Any  attempt to  load or *GET  a  file that has  a  line longer than  128

characters will result in "Load file format error".

If you attempt to assemble a  file that is not a valid source code  file,

the message, "Bad parameter(s)" may be displayed.

MRAS Error Messages

2 - 57

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

Terminal Errors:

# Symbol table overflow

There is not enough  memory for the assembler to  generate your program's

symbol table  or macro storage. You have one  option: divide your program into

two or  more  relocatable modules and  assemble each separately then link with

the linker.

*GET or *SEARCH error

A "*GET filespec" or "*SEARCH  library" assembler directive  was found in

a  library  member.  A  searched  library   cannot   have  "*GETS"  or  nested

"*SEARCHes".

Member definition error: filespec(member)

This  is a result  of  a  fetched *SEARCH member not resolving the symbol

reference invoking its fetch.

Fatal Errors:

# Bad label

The character string  found in  the label  field of  the source statement

does not match the criteria specified under SYMBOLIC NAMES.

# Expression error

The operand field contains an ill-formed expression.

# Illegal addressing mode

The operand field does  not specify  an addressing  mode which  is  legal

with the specified OPCODE.

# Illegal opcode

The character string found  in the opcode  field of the source  statement

is not a recognized instruction mnemonic, assembler pseudo-op, or MACRO name.

# Missing information

Information vital to  the  correct assembly  of the  source  line was not

provided. The OPCODE is missing or the operands are not completely specified.

# Too many nested *GETS

*GET filespec nesting exceeds  the number of  levels supported. The  *GET

will be ignored.

MRAS Error Messages

2 – 58

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Unclosed conditional

The  "END"  statement or end of  source  was  reached  and  an open  "IF"

conditional  block was still  pending.  Your program is  missing  the  closing

"ENDIF".

# ENDIF without IF

An "ENDIF" pseudo-op was  detected without  a  corresponding  conditional

"IF" or "Ifxx" in effect. The "ENDIF" will be ignored.

# ELSE without IF

An  "ELSE"  statement was detected  without a preceding "IF"  conditional

segment.

# Filespec required

A  *GET  or *SEARCH  directive was  detected but  the statement  did  not

contain the required file specification. The *GET or *SEARCH will be ignored.

Bad parameter(s)

When  output preceding  a  MACRO definition, it  implies  an error in the

parameters of a MACRO.

# Nested MACRO ignored

A macro definition statement was nested in the model of another macro.

# Missing MACRO name

The  name  field of  the macro definition statement  did not  contain the

macro name. The macro will not be defined.

# ENDM without MACRO

An ENDM pseudo-OP was detected  while not in a macro definition phase. It

will be ignored.

# Too many parameters

In  a  macro call, the number  of parameters passed  exceeded  the number

defined for the macro. The macro call will not be expanded.

# Too many nested MACROs

The  number of pending nested macro calls exceeds  the current nest level

supported. The macro call will not be expanded.

MRAS Error Messages

2 - 59

# The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# MACRO forward reference

A macro  call  was detected  prior  to  the definition of the  macro. The

macro call will not be expanded since gross phase errors would result.

# Multiply defined MACRO

A  macro definition statement was detected  for a macro  already defined.

The subsequent definition will be ignored.

Warnings:

# Branch out of range

The  destination  of a relative jump  instruction  (JR  or  DJNZ)  is not

within the proper range for that instruction. The instruction is  assembled as

a branch to itself by forcing the offset to hex X'FE'.

# Field overflow

A number  or expression result  specified  in  the operand  field  is too

large  for  the specified instruction operand.  The result is truncated to the

largest  allowable number  of  bits. This error would also be output during  a

global change if a resultant line would exceed 128 characters.

# Multiply defined symbol

The  operand  field  contains a reference  to  the  symbol which has been

defined  in another  line.  The  first  definition of  the  symbol  is used to

assemble the line.

# Multiple definition

The source  line  is  attempting  to  illegally  redefine  a symbol.  The

original definition of  the symbol is retained.  Symbols may only be redefined

by the DEFL pseudo-OP and only if they were originally defined by DEFL.

# Undefined symbol

The  operand  field  contains a reference to a symbol which has  not been

defined. A value of zero is used for the undefined symbol.

MRAS Error Messages

2 – 60

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

# Invoking MLINK

MLINK is used to  link one or more relocatable  modules  to  generate  an

executable program file. MLINK is easily  invoked interactively or from a  Job

Control Language (JCL) by the syntax:

________________________________________________________________

|                                                                |

| MLINK {filespec}{,filespec}... {switch}{switch}...             |

|                                                                |

| filespec       - One or more relocatable modules to load.      |

|                                                                |

| switch         - any valid MLINK switch as follows:            |

|                                                                |

| -A={y/N}       - Specify that MLINK is to abort or not abort   |

|                  upon an error in the command processing.      |

|                  Default=N.                                    |

|                                                                |

| -C=address     - Specify the hexadecimal origin of the common  |

|                  blocks for succeeding modules loaded.         |

|                                                                |

| -D=address     - Specify the hexadecimal origin of the data    |

|                  segment for succeeding modules loaded.        |

|                                                                |

| -E             - Exit to DOS; Save CMD file if modules loaded. |

| -E=symbol      - Same as -E but use symbol as entry point.     |

|                                                                |

| -H={y/N}       - Generate 05 record header from 1st module's   |

|                  name. Default=N.                              |

|                                                                |

| -I={y/n}       - Generate a core-image '/CIM' file. Default=N. |

|                                                                |

| -L=address     - Specify the hexadecimal link origin of all    |

|                  blocks not switched by -P, -D, or -C          |

|                                                                |

| -M             - List all defined symbols (3 per line).        |

| -M=filespec    - List defined symbols to filespec (1 per line).|

|                                                                |

| -N=filespec    - Specify /CMD file generation.                 |

| -N=:d          - Specify /CMD file generation. Use default     |

|                  filename for filespec.                        |

|                                                                |

| -O             - Handling for overlays - See overlay section.  |

|                                                                |

| -P=address     - Specify the hexadecimal origin of the program |

|                  segment for succeeding modules loaded.        |

|                                                                |

| -Q={wxyz}      - Specify generation order of segment classes   |

|                  in the /CMD file generated [A,P,D,C].         |

| -Q             - Reset segment classes to link origin.         |

|                                                                |

| -R             - Reset and clear all tables.                   |

|________________________________________________________________|

MLINK linker

3 - 1

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

________________________________________________________________

|                                                                |

| -S=filespec    - Search referenced library to resolve any      |

|                  undefined globals.                            |

|                                                                |

| -U             - List undefined symbols.                       |

|                                                                |

| -V=filespec    - Use virtual memory file for stream buffering. |

| -V=:d          - ditto; filename generated is MLINK/VMF:d.     |

|                                                                |

| -X             - Equivalent to -E                              |

|                                                                |

| -Y=<text>      - Used to specify the text for a 1F load module |

|                  record (a comment record).                    |

|                                                                |

| -Z={y/N}       - Generate hex zeroes for DS regions in DSEGs   |

|                  and COMMONs. This switch defaults to N.       |

|________________________________________________________________|

# Command input

Filespecs and switches can be  intermixed on  a single line by separating

filespecs  with either a switch  prefix, a  comma, or a space. A plus sign '+'

or a minus  sign  '-' can  be used as the switch prefix.  Any character string

not starting with a switch prefix will  be interpreted  as a file  which  con-

tains a relocatable  module  or  modules to  load into  the linker.  The  /REL

extension  will be automatically assumed. If  a file contains  more  than  one

module, each  module will be  processed and loaded. If  any symbol is detected

as being defined at more than  one address during the loading of a module, you

will  be  notified  of the symbol name and processing will continue.  This may

occur when one  or more symbols of more than seven  characters have been trun-

cated to a length of seven and are thus no longer unique.

At the  conclusion of processing a complete command  input line, you will

be  prompted via '?' to enter another command  line. A <BREAK> is an immediate

exit  to  DOS. Command line entry  uses the KEYIN handler and is  thus  usable

from Job Control Language.

If any error is  detected in an  input, it will result in an  appropriate

error  message  and any  remaining input fragment(s)  from that entry will  be

ignored. File I/O errors may result in MLINK aborting.

# Status messages

At the initiation of the '?' prompt  for  more input, a status line  will

list the number of free bytes remaining in the buffer area in the format:

ddddd free space (ddddd in decimal)

The segment origin address, end address,  and length (all in hexadecimal) will

also be listed for any segment chains loaded.  Segment  chains are either Pro-

MLINK linker

3 - 2

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

gram (P), Data (D),  or Common (C) for  segments loaded after a -P, -D,  or -C

switch respectively. The  "Program" segment results from  a CSEG  or code seg-

ment. "P" is used in  the  linker so  as to be able to  differentiate from the

single  letter  "C" used  to designate the COMMON segment. All  other segments

loaded without a respective switch  will  be stored classified as being in the

Link  Origin chain and will  be designated by the letter,  "L". The  format of

the status will be:

a <ssss-eeee llll>

where  "a" = L, P,  D, or C;  "ssss" = the chain's lowest origin; "eeee" = the

chain's highest  load  address;  "llll"  = the chain's length  calculated from

"llll=eeee-ssss+1".

The '-E' switch will also display this status prior  to generation of any

specified output file.

MLINK switches:

MLINK  switches  control various aspects  of  the  linker.  Some  of  the

switches include a parameter  which may  be  optional. A  switch parameter  is

connected to its switch by either the '=' or ':' separator.

# Switch -A={y,N}

Upon detecting an  error,  MLINK will normally recycle to the '?' command

prompt  to await a  new command.  If you wish an  immediate exit on a detected

error, then this  switch  can be used to tell MLINK to abort or not abort upon

an error in the command processing. Its use is recommended  for JCL invocation

of the linker. The switch defaults to NO.

# Switch -C=address

MLINK  provides the capability of  separating  the code, data, and common

segments to origins of your choosing.  Any not switched are origined according

to the link  origin  (see the -L switch).  Use this -C  switch to specify  the

hexadecimal origin of  the common  blocks for  succeeding modules loaded. Note

that the switch does not take effect on any module already loaded.

# Switch -D=address

MLINK  provides  the  capability of separating the code, data, and common

segments to origins of your choosing. Any not  switched are origined according

to the  link origin  (see the -L switch).  Use this -D switch  to specify  the

hexadecimal origin  of the data  segment for  succeeding  modules loaded. Note

that the switch does not take effect on any module already loaded.

MLINK linker

3 - 3

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

# Switch -E{=symbol}

This switch is used  to generate  the executable  program  file (/CMD) if

any modules had been loaded then exit to DOS. If any symbol is  left undefined

(or some other error is  detected), MLINK will  terminate  the  -E switch pro-

cessing and recycle to  the '?' prompt. You may exit via <BREAK> or "-R-E". If

you wish  to designate  one  of the  PUBLIC  symbols as the entry point to the

program, use the '-E=symbol'  syntax. This will override any entry point which

was designated via an END statement.

# Switch -H={y/N}

This  switch  is used  to  have MLINK  generate a 05 object  file  record

header when the executable program  is output. The header name is derived from

the first loaded module's name. Switch -H defaults to "N".

# Switch -I={y/N}

This  switch is  used to specify that MLINK should generate  a core-image

'/CIM' object file in lieu of a load format program file. -I defaults to N.

# Switch -L=address

MLINK  provides the capability of separating the code, data,  and  common

segments to origins of your choosing. Any not switched are origined  according

to  the  link  origin. The link origin  defaults  to 3000H for TRSDOS 6.x  and

5200H for Model  I/III operation. If you wish to designate another link origin

for all blocks not  switched by -P,  -D,  or -C, specify the  hexadecimal link

origin  via this -L switch.  Note that  the switch does not take effect on any

module already loaded.

# Switch -M{=filespec}

This switch is used  to list all defined symbols at  three  per line. Any

symbol defined  at  more than one address (multiply defined) will be indicated

by the appendage of an asterisk.  If you enter  a filespec parameter, the list

of defined symbols will be written to the designated file at one per line.

# Switch -N=filespec {-N=:d}

This switch is  used to request the  /CMD  file  generation.  It  may  be

entered anytime during the  entry of link commands.  If you use the  syntax of

"-N=:d", the generated /CMD file  will use  the default filename for  filespec

which  will be  the same name as  the first  REL  module  loaded.  This syntax

requires that at least one REL module is loaded prior to entry of the -N=:d.

MLINK linker

3 - 4

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

# Switch -O

This switch  schedules the  series of modules  loaded  since the last  -O

switch for  generation as an overlay file. The first series of  modules loaded

will be the root program. See the section on overlays for more information.

# Switch -P=address

MLINK  provides the  capability of separating the code, data,  and common

segments to origins of your choosing. Any  not switched are origined according

to the  link origin. Use this switch  to specify the hexadecimal origin of the

program segment for succeeding modules  loaded.  Note that the switch does not

take effect on any module already loaded.

# Switch -Q={wxyz}

During the generation of  the  output  object  file,  the byte stream  is

output  by module segments in  the  order of the modules  loaded.  If you have

switched  any of the  segment origins via the -P, -D, or -C switches,  you may

want to redesignate  the specific order of  the segments being output. Specify

the generation  order of segment classes in the output file generated via this

-Q switch. Denote the segment class via: A, P, D, or  C for absolute, program,

data, and common  respectively. Any segment class not  sequenced by your entry

will  automatically be output in the  sequence  APDC. Note  that a  maximum of

four entries are permitted and no two may be duplicated.

The action  of the -Q  switch  may  be restored  to the  default mode  of

module segment order by entering the -Q switch without a parameter [-Q].

# Switch -R

This switch  is used  to  reset and clear all tables MLINK had internally

developed.  Any open VM file will be deleted. Use  -R  when you wish to recon-

struct a set of modules  using different origins. If you wish to terminate the

link  session  after you have loaded modules, you can reset the linker with -R

then issue the -E switch..

# Switch -S=filespec

You can manually  request  the  linker to search  a  library  of  modules

created  with the MLIB librarian so as  to resolve undefined  globals.  Do NOT

enter a file  extension with the  filespec.  The  library search  facility  of

MLINK is capable of searching either a /REL library or an /IRL library.  MLINK

will first assume a RELocatable structured  (REL) library. If a REL library is

not found, then an Indexed  ReLocatable  (IRL)  library will be assumed. If an

IRL library is  not found,  the  "File not found" error will be issued. The -S

switch processing must assign its  own extension to be able to properly select

REL searching or IRL indexed searching!

MLINK linker

3 - 5

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

# Switch -U

This switch will cause the  listing of  all symbols currently  undefined.

Unlike the -M switch, the  list of undefined symbols cannot be redirected to a

disk file.

# Switch -V=filespec {-V=:d}

As relocatable modules  are loaded, their code, data, and common segments

are  stored in memory (the available buffer pool starts  immediately after the

linker and  extends to  the memory  address noted  as HIGH$ by the  DOS).  The

linker also maintains a set  of linkage tables  which  contain information  on

how to interconnect the modules which  make up the output file. A symbol table

is  also maintained in memory. In order  to be  able to handle the  linkage of

programs which are larger than the available  memory, MLINK provides a virtual

memory file facility which  can be  used to provide buffering for the segments

in  each module. The linkage tables and the symbol table are still  maintained

in memory. Operation of the linker via virtual memory  will certainly  degrade

the  speed performance  of MLINK; however, it  will enable you  to  link large

programs. It is beneficial if the VM file is on a RAMdisk.

If you specify the  -V=filespec switch, all  modules  subsequently loaded

will be buffered through the virtual memory  file specified. If you  enter the

switch  with only a  drivespec as  the parameter (as  in  -V=:d), the filename

generated  will  be MLINK/VMF:d. Note that the VM file is deleted during  pro-

cessing of the -E switch.

If you  are using virtual memory buffering and still  get a "Symbol table

overflow" error, attempt to reclaim any high memory used by  filters, drivers,

or  resident  programs so as to increase the size of the  buffer are available

to  the  linker. Once you have  exhausted that  procedure, examine the  memory

usage requirements of the linker and then  attempt  to  "compact" some  of the

modules making up your program.

# Switch -X

This switch is equivalent in operation to switch -E.

# Switch -Y=<text>

This switch is used to specify  the  text for  a  1F load  module record.

Text can be continued onto as many  additional  lines  as needed. A maximum of

255 characters may be entered. The '>' character terminates the text entry.

# Switch -Z={y/N}

The  normal operation  of  the  linker is to  suppress the  generation of

object code for  data  regions which have  been skipped  over  by the  DS/DEFS

pseudo-OP. If you  wish  to generate byte zeroes  (00H) for all  such space in

MLINK linker

3 - 6

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

DSEGs and COMMONs, Specify "-Z=Y". All space skipped over  by DS/DEFS in CSEGs

will be set to zero in the output object file.

The action  of the -Z switch doesn't take effect  until the generation of

output but  will pertain to all  modules. To turn off the -Z switch and revert

to normal MLINK operation, specify "-Z=N". This switch defaults to -Z=N.

Command file generation:

The default origin of the  REL stream will be the link origin  (3000H for

TRSDOS 6; 5200H  for Model I/III  operation). This default value may  be  per-

manently patched at the link  origin  of the linker. You can also override the

default link origin at MLINK's execution by using the  -L switch. All segments

will  be  loaded starting at this origin. You can  alter  the loading  of  the

program, data, and or common segments via the -P, -D, and  -C switches. Once a

switch is in  effect,  all segments  of  the  switched  class will be origined

relative to the address entered.  All unswitched segment  classes continue  to

utilize the link  origin. The command file  will  be generated in the order of

the modules  loaded unless you  specify a different order  via the  -Q switch.

The linker will NOT advise you of any segments which overlap!

An output file is only generated when the -E switch is invoked if  the -N

switch  has been specified. The  -E command will first check for any  multiply

defined symbols and abort the  operation if at least one symbol is  defined at

more than one  address. Any  "Request library search"  requests will  be first

satisfied  by  searching the  referenced libraries.  If  any symbols are  left

undefined, they will be displayed and the  request will be aborted. The linker

will  recycle to request another command. Otherwise,  next, the chain external

requests will be  handled. Finally, the  extern+offset  requests will  be han-

dled. If $MEMRY is defined, it will be loaded  with  the address of the  first

free byte. The loaded modules are now ready for object code generation.

If you  specified "-H=y",  a 05 header record will be generated using the

module name of the first file loaded.  After the header  record, a record type

1F will be generated if you specified the "-Y=<text>" switch.

Note: MLINK handles  the module bit  stream  on  a segment  record basis which

incurs  a great  deal of overhead  in the  linker compared to  a  memory-fixed

linker  such  as L-80. The method was chosen for MLINK so as to  minimize  the

size of the resulting /CMD file. This is important for TRS-80 systems.

Linker memory overhead free space requirements:

Each segment (code, data, common,  absolute)  requires 9 bytes  plus  the

segment length. Data and COMMON segments  require an additional  storage equal

to the  segment length. Code, data, and common segments require no storage for

the segment length if the -V virtual memory switch has been specified.

Each chain external requires 8 bytes of storage.

MLINK linker

3 - 7

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

Each extern+offset requires 8 bytes of storage.

Each entry symbol requires 5 bytes  of  storage  plus  the length  of the

symbol.

Each request library search takes 3 bytes plus the name length.

Each -O switch takes 20 bytes.

# Overlay processing

MLINK supports  the handling of  programs which  desire to use  overlays.

The method is straightforward. A program may  be  partitioned into a root pro-

gram and one or more overlay  programs. No address of an  overlay is  known to

the root program. The  entry  point of the  overlay  is its link origin estab-

lished by the  linker. Proper entry  is performed when  the OVERLAY subroutine

is called by  your  root program.  No address  of an  overlay is known  to any

other overlay.  All addresses of the root program  are known to all  overlays.

The root program and all overlays are generated in a single link session.

When the -O switch is entered, MLINK will perform a fixup of  the current

chain of modules loaded. The fixup process will:

1) Search all libraries specified via a REQUEST assembler

instruction,

2) Fix all chain externals,

3) Fix all extern+offset,

4) Prepare MLINK for a new program chain. This resets the -Y

switch for the next chain (the -H switch is global). This

also establishes the transfer address (entry point) for all

overlays to be the next available LINK address after the

root. That LINK address will be used for the overlay's

entry and not a -P switched origin. It is therefore the

proper procedure to not use the -P, -D, and -C switches

if you want to use the overlay handling scheme of MLINK.

All modules loaded after an -O switch  will  be partitioned into a  new chain.

PUBLIC symbols in  one overlay chain of modules  do NOT interfere  with PUBLIC

symbols  in other overlay  chains;  however, they must  be distinct  from  the

ROOT.  The  terminating -E switch will fixup the last chain loaded and prepare

for the object file generation, if  requested. Prior  to its  generation,  the

linker will store  the address  of  the  first available  byte  following  all

modules into the address of  $MEMRY, if defined. The file specification of the

overlays  will  also  be added to $OVLNAM  if  defined  (this  is part  of the

OVERLAY/REL subroutine which has been provided).

MLINK provides  support for up  to 35  overlays. Each overlay  is automa-

tically  assigned a file specification which consists of the filename assigned

by  the -N  switch and  an  extension of /OVx:  x = 1,2,...,9,A,B,...,Z. MLINK

provides a subroutine to automatically  invoke a desired overlay. The protocol

for its use is as follows:

MLINK linker

3 - 8

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

1) Load reg_HL with any root program data,

2) Load reg_DE with a pointer to a File Control Block (This

is normally a 32-byte space under all systems except

TRSDOS 1.3),

3) Load reg_A with the desired overlay in ASCII

(i,e, LD A,'1'),

4) Invoke the OVERLAY handler via a CALL OVERLAY assembler

instruction.

Note  that  return is  not made  from this overlay handler  routine unless  an

error  is  detected in  loading the  requested  overlay!  However,  the RETurn

address  to OVERLAY is  in  register  DE at the time control is passed  to the

overlay.  You therefore  may  find it  useful  to CALL  a routine which  CALLs

OVERLAY.  This is illustrated in  the  following  set  of  programs  coded for

operation under TRSDOS 6:

;TESTOVER/ASM

CSEG

BEGIN   LD      HL,START$

LD      A,10         ;@DSPLY line handler

RST     40

LD      A,'1'

CALL    GETOVER

LD      A,'2'

CALL    GETOVER

LD      A,'3'

CALL    GETOVER

LD      HL,FIN$

LD      A,10         ;@DSPLY line handler

RST     40

RET

GETOVER LD      DE,FCB

CALL    OVERLAY##    ;OVERLAY doesn't return unless error

OR      80H          ;Set ABORT bit

LD      C,A

LD      A,26         ;@ERROR handler in DOS

RST     40

START$  DB      'This is the root executing',13

FIN$    DB      'End of processing',13

DSEG

FCB     DS      32

END     BEGIN

;TESTO1/ASM

CSEG

LD      HL,MESS$

LD      A,10

RST     40

RET

MESS$   DB      'This is overlay 1',13

END

;TESTO2/ASM

MLINK linker

3 - 9

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

CSEG

LD      HL,MESS$

LD      A,10

RST     40

RET

MESS$   DB      'This is overlay 2',13

END

;TESTO3/ASM

CSEG

LD      HL,MESS$

LD      A,10

RST     40

RET

MESS$   DB      'This is overlay 3',13

END

The following MLINK  session  will create the root  and overlay /CMD files  as

DOOVER/CMD, DOOVER/OV1, DOOVER/OV2, and DOOVER/OV3 respectively:

mlink

testover:1,overlay:2

-o,testo1:1

-o,testo2:1

-o,testo3:1

-n=doover:1 -e

Error and Status messages:

Any of the following error messages may  be displayed  during  processing

of the link session.

2nd COMMON larger /symbol/

The first reference to a common must be the largest.

Bad parameter(s)

The parameter of a switch was invalid for the switch.

# Break detected

You depressed the <BREAK> key.

# File not found

The referenced file was not found.

# Filespec required

The switch required a filespec.

MLINK linker

3 - 10

The MISOSYS Relocatable Macro Assembler Development System

Copyright (c) 1985 MISOSYS, Inc., All rights reserved

# Internal Error

This error  should not  appear with  properly  assembled REL modules.  If

this message  does appear,  save all  of  the REL modules and assembler source

modules loaded and contact MISOSYS.

# Invalid command

The command character is not acceptable.

# Invalid link item

Something is wrong in the REL bit stream.

# Invalid switch

The switch entry is not supported.

# Multiply defined symbol

A symbol has been defined at more than  one  address.  When  this  occurs

while a module is being  loaded, the message will be prefixed with the name of

the symbol.  The  -M switch  appends an  asterisk to the names of multiply de-

fined symbols. Symbols are generally in  this category if two or more  modules

declare them  as PUBLIC, or  if symbol names greater  than seven characters in

length  are truncated to seven characters by  the  assembler and  thus  are no

longer unique.

# Non-contiguous image file

File output generated  with the  -I switch must present a  contiguous se-

quence of load addresses. Thus,  if you use the  -P, -D,  or -C switches, make

sure that the  address ranges of each  segment connect. Use the  -R switch and

start over.

# Record 1F too long

The 1F record text exceeds 255 characters.

# Symbol table overflow

There is no more buffer  room. Use the virtual  memory switch. If you al-

ready are,  you're probably  out of luck. Try combining lots of little modules

into one big module. Read the section on the -V switch.

# Unknown COMMON referenced

A named  common has been referenced without it being defined. Most likely

a REL bit stream error.

MLINK linker

3 - 11

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Operating MLIB in interactive mode

MLIB functions in two  different  modes, interactive and  batch.  In  the

interactive mode you can enter commands one at  a time. MLIB will process  the

command, and the results will  be displayed  when the command is completed. In

batch  mode, MLIB will get  its command input from an existing file containing

the sequence  of commands. Batch mode is functional only under those operating

systems which support the "DO" command.

From "DOS Ready", the librarian is invoked simply by entering:

_______________________________________________________

|                                                       |

|  MLIB (JCL,PAGE=nn)                                   |

|  MLIB *                                               |

|                                                       |

|  JCL     - is used to specify BATCH mode.             |

|                                                       |

|  PAGE=nn - specifies page length for listings and can |

|            range from 30-255 [make entry in decimal]. |

|            If PAGE is omitted, it will default to 66. |

|                                                       |

|  *       - indicates a re-entry to MLIB (see text)    |

|                                                       |

|  Abbreviations: J=JCL, P=PAGE                         |

|_______________________________________________________|

Upon successful loading and execution of  MLIB, a command menu similar to

the following is displayed:

_____   ________________________________________________

___|__   |TM                                                |

|   |  |  |  MLIB 3.1L - Relocatable Object File Librarian   |

| Ri|cl|in|  (C) Copyright 1983 Riclin Computer Products     |

|   |__|__|                                                  |

|______|                                                     |

X Load library              Save Library               |

|       Add module                Module map                 |

|       Purge module              DOS Command                |

|       Replace module            Clear buffer               |

|       Extract module            eXit program               |

|       Insert before module                                 |

|                                                            |

|              Bytes used:  0, free: 29736                   |

|                                                            |

|____________________________________________________________|

To  execute a command, enter the  first letter of  the  command (e.g.  to

select "<L>oad library", enter the letter <L>). The only exception  to this is

the  "e<X>it program" command,  which is selected by  typing  the  letter <X>.

Alternatively, you may find it simpler to move  the  blinking cursor  until it

is  next to the  desired command,  and then  depress <ENTER>  to execute  that

function.  This   is  accomplished   by  using  either  the  <DOWN-ARROW>  and

MLIB Relocatable Module Librarian

4 - 1

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

<UP-ARROW> keys,  or  equivalently,  the  <SPACE> and  <BACKSPACE>  keys.  The

blinking cursor will reappear as  a solid  block next to the selected command,

as a visual indication of which command is executing.

MLIB will  accept either uppercase  or lowercase  for all keyboard input.

Whenever MLIB  asks  you to  enter a  file specification,  you do not normally

have to enter an  extension; it will default to either  "/IRL"  or "/REL", de-

pending  on whether you  request Indexed ReLocatable  modules  or  RELocatable

modules at the "IRL or REL ?" prompt.

Certain options require you to  enter  either a  file specification  or a

module  name. MLIB will fully buffer your keystrokes. Several special function

keys are available:

Key entry               Key function

-------------------     -----------------------------------------

<LEFT-ARROW>            non-destructive backspace

<RIGHT-ARROW>           non-destructive forward space

<SHIFT-CLEAR>           erase from cursor position to end of line

<SHIFT-LEFT-ARROW>      restart from scratch

<SHIFT-RIGHT-ARROW>     move cursor to end of line

<BREAK>                 cancel this command

<ENTER>                 invoke your entry.

If an error has  occurred, MLIB will "beep" the console  speaker  if your

machine is so equipped (on a Model I/III computer, a short  beep  tone will be

directed to the  cassette  port which can  be  audible  if you have  an  audio

amplifier and speaker hooked  up). An error message will appear at the  bottom

of  the screen, and will  remain there until you enter any  keystroke, indica-

ting that you have acknowledged the error.

# Re-entering MLIB

If you have exited MLIB accidentally without saving the buffer to disk, or

your  system has rebooted  itself,  or  you were forced  to reboot due to your

system crashing, MLIB may be re-entered by the command "MLIB *".  All  command

line  parameter  values  will be  retained as per the  previous  invocation of

MLIB. In nearly all cases your buffer  will be  intact. However, it is wise to

immediately  request a detailed  module  map of what is now  in the buffer. If

strange things appear in the  map, then  you  have  lost your data. If  things

look normal,  then  you  should  save the buffer to disk  right away.  Prudent

practice dictates that you keep  backups of your  work, and if your system has

hardware  problems,  save the  buffer  to  disk  frequently to prevent loss of

data. If a  reboot  occurs  or is  necessary,  don't  forget to  hold down the

<ENTER> key  to prevent any AUTO  command  from  being executed  and  possibly

overwriting the MLIB data.

MLIB Relocatable Module Librarian

4 - 2

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# MLIB commands

<A>dd module

This  command  adds a new  module to  the  library in memory from  a disk

file. The new module  will be loaded following what is already in  memory. Use

of  this command is the  primary method of building a new library or adding to

an  existing  one.  It will abort on out of memory, symbol  table overflow, or

disk error.

<C>lear buffer

This command  resets all  internal pointers, thus effectively  destroying

what is currently in the memory buffer. In interactive mode, MLIB gives you  a

chance to cancel this command if you decide not to clear the buffer.

<D>OS command

This  allows you to enter any DOS library command.  You  will be prompted

to enter the command with the message, "Enter DOS command:".

<E>xtract module

This command  extracts a  single  module from the  library in memory  and

writes it to disk as a stand-alone /REL or /IRL  file. It will abort on module

not found, or disk error.

<I>nsert before module

This command  inserts a new module  from disk into the library in memory.

Answer the  first prompt with  the name of the module BEFORE which you wish to

insert the  new  module. This  command will  abort  on module not  found, disk

error, out of memory, or symbol table overflow.

<L>oad library

This  command loads an existing library into memory from a disk  file. It

will abort  on  disk error,  out of memory, symbol table overflow, or data al-

ready in memory.

<M>odule map

This command maps the attributes of  the  library  which is  currently in

memory.  This listing can be in either detailed  or  summary  format. When you

request  the  MODULE MAP command,  you will be asked  if  you  want a  printer

listing with the question:

# Listing to printer (Y,N) ?

MLIB Relocatable Module Librarian

4 - 3

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

You can  print a module map, if desired,  by responding <Y>. Printed output is

paginated  at a  page length of 66  lines unless changed  by  use of the  PAGE

command  line  parameter.  In  either  interactive or batch mode, <BREAK> will

cancel a printout at any time.

In interactive  mode,  with certain exceptions,  MLIB will prompt  you to

set your printer to the  top of a  page - do so and then depress any keystroke

to continue.  If  you are  running  MLIB  under LDOS,  you can use the  system

spooler, or you  can have printer output routed to a  file.  In  either  case,

output  will  begin  with  a  form feed,  and  there  will be  no  prompt  for

top-of-form.

In batch mode, the module  map will always  go to the printer,  and  will

begin with an automatic form feed and no prompt for top-of-form.

The  detailed format option  is available  in both  interactive and batch

modes. The attributes of  each module are displayed as  shown in the following

illustration:

____________________________________________________________

|                                                            |

| Module name   Module size   Program size   Data area size  |

|   DSKDRV         1417          996             76          |

|                                                            |

| Entries:   $FLBUF  $FLCNT  $FLFCB  $FLFLG  $GTFCB  $GTFLG  |

|            0000"   0014"   0032"   0028"   0024"   0018"   |

|            $MEMRY  DSKDRV  OPEN                            |

|            0048"   000A'   033C'                           |

|                                                            |

| Externals: $BF     $BL     $CLSFL  $ERR    $IOERR  $IOINI  |

|            $LUNTB  $REC    $UN                             |

|                                                            |

| Commons:                                                   |

|                                                            |

|       Hit <ENTER> for next screen, Hit <BREAK> to exit     |

|____________________________________________________________|

# Module name

This  is  shown  if it exists; otherwise "------" is  displayed.  If  the

module is written with MRAS, the name is defined by use of the NAME  pseudo-op

or will default to the REL file name.

# Module size

This is  the absolute length, in decimal,  of  the module, including  all

segments,  symbol  definitions, etc. A stand-alone  disk  file  containing the

module would have the same length plus one.

# Program size

This is the length of the module's program segment, in decimal.

## MLIB Relocatable Module Librarian

4 - 4

# The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Data area size

This is the length of the module's data segment, in decimal.

# Entries

Each  entry  point  is defined  by a symbol and  its  location within the

module. A module may have more  than  one entry point. MLINK entry symbols may

be up to  seven characters in length. Locations  are expressed in hexadecimal,

relative from the starting point of the  segment in which the  entry point  is

contained (X'0000'). The entry point type is also displayed as follows:

<space>    absolute segment

'       program relative (code) segment

"       data relative segment

!       COMMON area

# Externals

Any  symbol  which  is not  defined in the module is called  an  external

symbol,  and  is listed here. These are  actually  references to entry  points

defined in other modules.

# Commons

Any COMMON  areas defined in  the module  are listed. The lengths of  the

COMMON areas are also shown, in hexadecimal.

The  summary format option  lists all modules, by name only, in the order

they exist in memory (from left  to right,  top to bottom, on the CRT screen).

Summary format is available only in interactive mode and would look like:

____________________________________________________________

|                                                            |

|  RAN     INT4    DSQRT   DMOD    DSIGN   DABS    DATAN2    |

|  DATAN   DLOG10  DCOS    DSIN    DBLEXP  DEXP    DLOG      |

|  DMIN1   DMAX1   DCOMP   DBLDIV  DBLFLR  DBLPLY  DBLUTL    |

|  DMLDV   DSHR    DBLCON  MFM     AMIN0   MIN0    AMAX0     |

|  TANH    SQRT    MIN1    MAX1    MAX0    IFIX    FLOAT     |

|  EXP     DIM     COSIN   ATAN2   ATAN    AMOD    AMIN1     |

|  AMAX1   AINT    ABS     ALOG10  ALOG    MOD     RIEXP     |

|  NEGF    RREXP   NEG     NEG     EXPB    LOG2    POLY      |

|  FADD    IIEXP   FDIV    STOP    FMUL    ADE     FLR       |

|  FCP     FLT     CMPGTO  IDIV    SIGN    RAT     DAT       |

|  ISIGN   IABS    NEGATE  XAR     IDIM    FAT2    IMUL      |

|  POKE    UNPACK  PSPLAC  ICP     FORMIO  NORM    SHIFT     |

|  RNDOVF  ZAC     IOINIT  LUNTB   IAT2    LODSTR  SAF       |

|                                                            |

|       Hit <ENTER> for next screen, Hit <BREAK> to exit     |

|____________________________________________________________|

MLIB Relocatable Module Librarian

4 - 5

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

<P>urge module

This  command  purges a module from the library in memory.  If the module

has  multiple entry  points,  all will  be deleted. In response to the prompt,

enter  a  module name. The  module  will  then be purged,  and the library  in

memory will be compressed to recover freed  space. This command  will abort if

the module is not found in the symbol table.

<R>eplace module

This  command replaces an  existing module in  memory  with a new version

from a disk file.  All aliases will be replaced or deleted. The new module may

be longer,  shorter, or the same length as the old one -  MLIB  will compress,

overlay in place, or expand the library in memory to fit  the new length. This

command will  abort on module not found, out of memory, symbol table overflow,

or disk error.

<S>ave library

This command saves  the entire  library  in  memory to  a disk file. MLIB

will prompt  you, if the file already  exists, as to whether you wish to over-

write or not. This command will abort on memory empty, or disk error.

e<X>it program

This command exits MLIB and  returns you to  "DOS Ready".  In interactive

mode,  if there  is  data in the buffer which has not yet  been saved to disk,

you have  a chance  to recover  (i.e abort  the eXit). If you  choose to  exit

anyway, your data will be lost, unless you  re-enter MLIB immediately with the

"MLIB *" command, as  explained in OPERATING MLIB  IN INTERACTIVE MODE. If the

data has been saved, MLIB will exit immediately without a prompt.

# Operating MLIB in batch mode

The command which executes MLIB in batch mode is as follows:

_______________________________________________________

|                                                       |

|  MLIB (JCL,PAGE=nn)                                   |

|                                                       |

|  JCL      -  specifies BATCH mode.                    |

|                                                       |

|  PAGE=nn  - as shown earlier.                         |

|                                                       |

|  Abbreviations: J=JCL, P=PAGE                         |

|_______________________________________________________|

In  batch mode,  MLIB  will take  its  commands from  a batch file, whose

extension is usually '/JCL'. Under some systems, the extension may be '/BLD'.

MLIB Relocatable Module Librarian

4 - 6

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

MLIB  will request each  command  with  the  prompt "Enter option:".  The

valid  responses to this prompt  are: Add,  Clear, Extract, Insert, Load, Map,

Purge,  Replace,  Save,  or eXit. Only the first letter  of  each  command  is

significant; the  remaining letters  are ignored and may be  left  out of  the

command line.  Command lines may also  be commented. Each command line must be

terminated with a carriage return.

Prompts may  be  given  to, "Enter filespec:" which  expects  a JCL  line

containing a file specification; and "Enter module name:" which expects a  JCL

line containing the name of a module which is currently in the symbol table.

The  "Module  map" command will  default to a detailed map which  will be

printed. The listing will start with a form feed.

Any  command which writes out a disk file will automatically overwrite an

existing file; you will NOT be given the chance to abort this command.

The  "Clear buffer" and "eXit program" commands will do  just that,  with

no opportunity to abort.

The occurrence  of ANY error is fatal, and will  terminate the batch pro-

cessing via the DOS ABORT error return.

Here is an  example  of a typical  series of commands  for a batch opera-

tion. The comments may be included in the batch file.

Batch command              Comment

-------------              --------------------------------

mlib (jcl)                 execute MLIB in batch mode

load                       load a library...

irl                        designate an "IRL"

funcs2                     this one is FUNCS2/REL

add                        add a new module...

rel                        designate a "REL"

shelsort                   SHELSORT/REL

replace                    replace a module...

index                      module name is INDEX

rel                        designate a "REL"

index                      replace it with INDEX/REL

save                       save the new version of...

irl                        designate an "IRL"

funcs2                     FUNCS2/REL

xit                        and exit to DOS

# Error messages

MLIB  will  recover from  all non-fatal errors;  you will not  lose  your

data.  Standard  disk I/O errors may also occur, and MLIB will  recover if  at

all possible.  Fatal errors  result  in  an  immediate return  to DOS  without

warning. In batch mode all errors become fatal, and will terminate execution.

## MLIB Relocatable Module Librarian

4 - 7

## The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

Non-fatal errors:

# Buffer empty

You cannot save the library, since memory is empty!

# Buffer not empty

You cannot load a library if one is already in memory.

# Invalid file format

You tried to read in a file which was not in the expected /REL format.

# Module not found

A module name was not found; it was not in memory.

# Out of memory

You cannot load a disk  file because  the file length would exceed avail-

able memory. When replacing or inserting a module, there must  be  enough free

memory  so  that  the  new  module  can be read  in before  it  is  eventually

relocated to its correct position in the buffer. If you do run out  of memory,

it's time to consider splitting the library into two smaller libraries.

# Symbol table overflow

MLIB handles up to 200 individual modules; you have exceeded this limit.

Fatal errors:

# Can't - buffer destroyed!

MLIB determined that part of  the memory buffer was destroyed before MLIB

was re-entered with the "*" parameter.

# Invalid option

An invalid command was given to MLIB while in batch mode.

# Parameter error!

An invalid DOS  command  line parameter was entered.  The only acceptable

command  line parameters are  "PAGE" which  requires  a decimal entry  in  the

range 30 through 255, and "JCL", which can be either ON or OFF.

# Unrecoverable error!

If a  file has been read into memory which appears ON THE SURFACE to be a

/REL-format  file,  but  really  isn't, there  is a  high probability that  an

unrecoverable error will occur during symbol table update.

MLIB Relocatable Module Librarian

4 - 8

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Invoking the SAID Editor

SAID is a full-screen  text editor that can be  used  to  edit  assembler

source, C-language source, or other ASCII  text files. When used under  TRSDOS

6.x or equivalent,  SAID provides  you with  up  to seven editing buffers, de-

pending on the availability  of 32K  memory banks, and the added capability of

moving blocks of text from one  buffer to another. Before SAID is  first used,

please install it into your system by  running the  SAIDINS installation  pro-

gram as noted at the end of this chapter. SAID is easily invoked via:

____________________________________________________________

|                                                            |

|  SAID [filespec] (parm1,parm2,...)                         |

|                                                            |

|  SAID *       Used to re-enter SAID immediately after      |

|               exiting so as to reclaim the text buffers.   |

|                                                            |

|  filespec     The name of the file to edit. If filespec is |

|               not found, SAID prompts to create it.        |

|               Command line filespec entry is optional.     |

|                                                            |

|  ASM          Tabs default to 8. File extension defaults   |

|               to "/ASM". X'1A' stripped from end of file   |

|               on read and replaced on write.               |

|                                                            |

|  CCC          Tabs default to 4. File extension defaults   |

|               to "/CCC". X'1A' stripped from end of file   |

|               on read and replaced on write.               |

|                                                            |

|  EXT=string   Sets the default file extension.             |

|                                                            |

|  TAB=nn       Set default tab width.                       |

|                                                            |

|  Abbreviations: A=ASM, C=CCC, E=EXT, T=TAB                 |

|                                                            |

|  Note: EXT parameter not usable under TRSDOS 1.3 and 2.3.  |

|        TRSDOS 1.3 users must enter ASM and CCC parms as    |

|        parm=0FFFF and TAB entry in hexadecimal, 0xx.       |

|____________________________________________________________|

Unless altered  by SAIDINS, SAID  command functions are invoked  with the

keyboard depressions as shown  in the  SAID menu. In the following  text, mul-

tiple key depressions are  shown as connected  sequences of keys  within angle

brackets, i.e.  <CLEAR><4> means  simultaneously depress  the <CLEAR> key  and

the <4> key.

# Editing Status

SAID  can display a great deal of status concerning the text contained in

the  editing  buffer.  You also can  control the optional display of  the SAID

menu of command keys. This information will look like the following:

SAID - Full Screen Text Editor

5 - 1

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

-------+-------+-------+-------+-------+-------+-------+-------+-------+-------

Srch  Repl  Again  All   Rev   Hex  Quote Copy  Move   Cmd  Print Exit

==1== ==2== ==3== ==4== ==5== ==6== ==7== ==8== ==9== ==0== ==:== ==-==

Ins  Line   Del  Word  Block Load  Save  Macro File  Meta  Pg Dn Pg Up

File:aaaaaaaaaaaaaa Len:bbbbb/ccccc Ln:ddddd Col:ee = x'ff' ggg%  Banks: hhhhhhh

Srch:iiiiiiiiiiiiiiiiii Repl:jjjjjjjjjjjjjjjjjj Dir:kkk  Cnt:lll

"Messages and prompts... "                                      SAID Version 1.1

The top line  will be a rule of dashes with a plus  sign "+" denoting each TAB

stop as  established by either the  default  tabbing for the file type [ASM=8,

CCC=4] or that set via the TAB parameter.

The next  three-line  menu is  optional. Its  display mode is established

when SAID  is invoked by the MENU setting during the SAIDINS installation. The

META command also permits  you to toggle the display of  this  menu. It is re-

commended that you keep  the menu displayed  until you get proficient at using

SAID's editing  commands. The menu  displays the  command  function  activated

when the key identified  by the second line is pressed simultaneously with the

<CLEAR> key. The first  line designates shifted  keys and the  third  line de-

signates unshifted keys.

The  next line contains a great  deal  of information. The file  specifi-

cation of the file currently being edited in the context  buffer is identified

by "aaaaaaaaaaaaaa". The current length of the text  is shown as "bbbbb" while

the total length of the editing buffer is shown  as "ccccc". SAID keeps  track

of a logical line number for the text. For line  numbering purposes, a line is

considered  to be all  characters  up  to and including a carriage return. The

number of the  line to which the cursor is positioned to is shown as  "ddddd".

The  video column number to which  the cursor is positioned is  shown as "ee".

This value  will  range from column 00  to column  79  (63 in the case of a 64

column screen).  The  character  which  the cursor is positioned  over has its

value shown in hexadecimal as "ff". This value is  useful for determining  the

text character for undisplayable  character values. SAID also keeps a ratio of

where the cursor is positioned relative to the end of the text. This is  shown

as a percentage by  the "ggg" value. It is accurate only when the text exceeds

99 characters.

The  last field  of  the  status  line shows the  availability of editing

buffers  as  "hhhhhhh". When  SAID  is invoked under  those systems supporting

banked switching,  SAID scans for the  availability  of  up to  seven  editing

buffers.  SAID  will display a  dash for each  buffer that is available. These

are selected by you  with the assignments 1,  2, 3, ...,  7. Anytime that  you

have entered text  into one of these  buffers,  its corresponding dash will be

changed to a plus sign.

The  next  line of status shows the current search  string:  "iiiiii...";

the  current  replacement string:  "jjjjjj..."; the search  direction,  "kkk":

"For=forward", "Rev=reverse"; and the macro repeat count: "lll".

The last line will be used to display  prompting messages or  error  mes-

sages on an as required basis.

SAID - Full Screen Text Editor

5 - 2

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Cursor Movement

Cursor  movement  relates  to the re-positioning  of  the  cursor  on the

screen.  "Up"  movements invoked  while the cursor  is on the  top line of the

screen cause the text  to  be scrolled downward. Conversely, "down"  movements

invoked while the cursor is on the bottom text  line will cause the text to be

scrolled upward.

Left one position             <LEFT ARROW>

Right one position            <RIGHT ARROW>

Up one line                   <UP ARROW>

Down one line                 <DOWN ARROW>

Beginning of next word        <CLEAR><4>

Next Screen Page              <CLEAR><->

Previous Screen Page          <CLEAR><:>

Start of line                 <SHIFT><LEFT ARROW>

End of line                   <SHIFT><RIGHT ARROW>

Start of file                 <CLEAR><UP ARROW>

End of file                   <CLEAR><DOWN ARROW>

Insert a tab                  <CLEAR><RIGHT ARROW>

# Modes

SAID operates  in various modes.  In normal operation, entered text over-

types any  text  beneath the cursor. The  cursor  will appear as  an underline

unless  it is positioned over an underline character at which point the cursor

will be  displayed as a full  block (191D).  When toggled  into "insert" mode,

all text  to the right of the cursor will be pushed down one character as each

character  is entered.  The cursor is also changed to a full block. Although a

TAB character occupies one position in the  text buffer, it is expanded on the

screen  via spacing  to  the next tab stop. In  line insert  mode,  a  line of

spaces  is  inserted into the  text at the cursor  position.  A  new line will

automatically be inserted when  you  attempt to type past the last position of

the opened line. Hex insertion  mode can be invoked regardless of the state of

insert mode. The hex mode allows you to enter all 256  character values by the

entry of two hexadecimal  digits  per  character.  In  quote insert  mode, all

cursor  movement functions are defeated and the character values used for  the

functions are entered into the text when a cursor movement key is pressed.

Insert/overtype mode          <CLEAR><1>

Line insert                   <CLEAR><2> - <Break> to cancel

Hex insert                    <CLEAR><SHIFT><6> - <Break> to cancel

Quote insert                  <CLEAR><SHIFT><7> - <Break> to cancel

SAID - Full Screen Text Editor

5 - 3

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Deletions

This  section  relates  to  the various operations of deleting text. When

you invoke a deletion, it  is  preserved by SAID  and can be restored via  the

Reverse-DELETE function; however, only the LAST deletion performed  is  saved.

Thus, if you delete a block  via "delete-block", the character  removed by the

DELETE key  is lost after  the  "block"  key causes the removal  of the  block

since the "block" delete is  the last  deletion. Fortunately,  you  only would

have to manually enter one character.

Delete character              <CLEAR><3>.

Delete word                   <CLEAR><3> followed by <CLEAR><4>

or just <CLEAR><4> if in delete mode.

Delete line                   <CLEAR><3> followed by <CLEAR><2>

or just <CLEAR><2> if in delete mode.

Delete block                  Mark the block, position cursor inside,

then <CLEAR><3> followed by <CLEAR><5>

Delete to top                 <CLEAR><3> followed by <CLEAR><UP ARROW>

or just <CLEAR><UP ARROW> if in delete mode.

Delete to end                 <CLEAR><3> followed by <CLEAR><DOWN ARROW>

or just <CLEAR><DNARW> if in delete mode.

Delete all                    <CLEAR><3> followed by <CLEAR><SHIFT><4>.

Deletes entire context except macro.

Undelete [oops function]      <CLEAR><SHIFT><5> followed by <CLEAR><3>

which is "reverse" followed by "delete".

# Macro functions

SAID supports a macro  key which can be soft programmed (or reprogrammed)

by you throughout the  operation of SAID.  This  key  can store  up to 64 key-

strokes.  Its use for capturing a series of  key entries  for repetitive entry

will help in speeding up your  editing  and minimize key entry. Note that  in-

voking the  macro will cause it to  repeat according to the repeat  rate count

set  with the Meta function. This repeat count is set to one  when you store a

series of keystrokes for the macro key.

Invoke current macro          <CLEAR><8>

Store a macro                 <CLEAR><6> followed by <CLEAR><8>; The Macro

will be saved until the next <CLEAR><8> or

until 64 characters are entered.

SAID - Full Screen Text Editor

5 - 4

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# I/O functions

This section relates to the loading or saving of text  as  well as print-

ing  out the text  buffer or a portion of  it. Note that when you are  merging

two or more files into the text  buffer, SAID will load the file at the cursor

location -  be it the  beginning of the text, the middle  of the text,  or the

end of the text. When  you  invoke "exit", SAID will prompt  you  to save  any

buffer which contains text. Note that when you  save  a file or exit, you will

be prompted for the filespec. Respond via <ENTER>  to use the current filespec

shown in  the  status  line or enter a new filespec which will  become the new

current one. SAID will abort a printing request if a <BREAK> is detected.

Print a block                 Mark block, then <CLEAR><SHIFT><:>,

followed by <CLEAR><5> followed by <0-9>

Print a file [in memory]      <CLEAR><SHIFT><:> followed by <CLEAR><9>

Load file at cursor position  <CLEAR><6> followed by <CLEAR><9> then the

filespec in response to the prompt.

Save file under current name  <CLEAR><7> followed by <CLEAR><9>

Save block                    Mark block, then <CLEAR><7> followed by

<CLEAR><5> followed by filespec, then <0-9>.

Exit                          <CLEAR><SHIFT><->

Change filespec               <CLEAR><9> followed by the filespec.

# Block functions

SAID  allows you to  designate up to ten distinctly labeled blocks. These

are numbered  from  0-9.  You can have more than one block designated with the

same number; however, in block  copy or block  move operations, the first such

numbered block found in the text when searched from the beginning of the  file

will be used for  the copy or move  operation. Blocks are marked by indicating

a BLOCK START  and  a BLOCK END (the end marker must appear in the  text after

the start marker).

Block start                   <CLEAR><5> followed by <0-9>

Block end                     <CLEAR><5> followed by <CLEAR><DOWN ARROW>

or <CLEAR><5> followed by <E>.

Copy block                    Mark block, position to destination, then

<CLEAR><SHIFT><8> followed by <0-9>. Note

that this command duplicates the contents

of the block at the new position. The

marked block is retained in its marked

position.

Move block                    Mark block, position to destination, then

SAID - Full Screen Text Editor

5 - 5

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

<CLEAR><SHIFT><9> followed by <0-9>. Note

that this command deletes the marked block

after inserting the block text into the

designated position.

Unmark all blocks             <CLEAR><SHIFT><5> followed by <CLEAR><5>.

Use before saving assembler source.

# Search and replace

This section relates  to the facility  for finding character  strings  in

the text and optionally  replacing  them  with another string. The replacement

string may be  null. If  any  character in  the  search string is in uppercase

then the  search will  be case  sensitive  (i.e. "A"  and  "a" are  distinct),

otherwise the search  will be  case  insensitive (i.e.  "A" and "a" are consi-

dered  to be the  same character). The search string  may contain  a  wildcard

character or characters which match  all character values (the  wildcard char-

acter is  specified  during  the installation  via SAIDINS).  The  replacement

string  may also contain a  wildcard character or characters  which  indicates

that the character in that position  in the  search string will be  re-used in

the  replacement string.  Both the search and  replacement strings may contain

hexadecimal values  via entry  of a  per  cent  "%" character followed by  two

hexadecimal digits. "Again" finds  the next matching string  or  replaces  the

next matching string. "All" invokes  the  search or  replace  on  all matching

strings. Note that the  meta command provides an  option to force a  query be-

fore replace which is also installable with SAIDINS.

Search                        <CLEAR><SHIFT><1> followed by the string.

Reverse search                <CLEAR><SHIFT><5> followed by <CLEAR><SHIFT><1>

followed by the search string.

Replace                       Invoke a SEARCH, then <CLEAR><SHIFT><2>

followed by the replacement string.

Again                         <CLEAR><SHIFT><3>

All                           <CLEAR><SHIFT><4>

# Miscellaneous

Invoke a DOS command        <CLEAR><SPACE>

This allows you to enter any DOS command that is acceptable  at DOS Ready

with the exception of any command which alters HIGH$.

Invoke a Meta command       <CLEAR><0> followed by [C,E,G,H,M,O,R,T,7,UP,DN]

This gives  you  access  to  an additional set  of infrequently  accessed

commands. The meta command letter determines the function invoked.

SAID - Full Screen Text Editor

5 - 6

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

C             Calculator

E             External memory [TRSDOS 6.x version only]

S             swap memory bank and full context

C             copy a block from an external memory bank

G             Go to the start of a line via its line number

H             Toggle the help display

M             Set macro repeat count

O             Set SAID options

A             set ASM mode in current buffer

C             set CCC mode in current buffer

T             set default extension in current buffer

R             Replace options: query before replace

T             Set tabs position (i.e. every nth column)

7             Strip bit 7 off all text in buffer

<UP ARROW>    Uppercase next word

<DOWN ARROW>  Lowercase next word

# Calculator

SAID contains a built-in  reverse  polish  notation calculator which sup-

ports the following three types of numbers:

xxxxB - Binary (i.e. 101101)

xxxxD - Decimal (default, i.e. 45)

xxxxH - Hexadecimal (i.e. 2d)

The following functions are supported:

*   Multiplication

/   Division

+   Addition

-   Subtraction (negation is not supported)

&   Logical AND

|   Logical OR

^   Logical XOR

.   Used to denote the previous result

If you wish to output the answer in any base other  than  decimal  then follow

the '='  with a 'B' or  an 'H' to  specify  binary or hexadecimal.  Entering a

period  will cause  the  last  result to  be substituted. Note  the  following

sample calculation which  multiplies  22 base 16 by 1111 base 2,  then adds  2

base 10 and outputs the result in decimal:

22h 1111b * 2 +<ENTER>

To output the same result in binary, specify ". =b".

SAID - Full Screen Text Editor

5 - 7

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Installing SAID [running SAIDINS]

SAID should be installed in your system by invoking the command,

# SAIDINS filespec

where "filespec" should be SAID/CMD - the name of  the  screen editor - unless

you renamed SAID/CMD to some other name. SAID  is  supplied to  support all of

the SAID functions mapped to the  keyboard, with the exceptions  of  functions

34,  35, and 36.  This mapping can be tailored  to your  specifications during

the installation of  SAID  while SAIDINS/CMD  is  running.  This  installation

program must be used first to establish  certain DOS interfacing needed before

SAID can be used.  The following function  codes  are used during the  instal-

lation  of SAID. They  designate the  function  numbers corresponding  to  the

thirty-six separate command functions in SAID.

1 Cursor left            19 Meta

2 Cursor right           20 Previous Page

3 Cursor up              21 Next Page

4 Cursor down            22 Find

5 Beginning of line      23 Replace

6 End of line            24 Again

7 Top of file            25 All

8 End of file            26 Unmark

9 Insert a tab           27 Hex

10 Insert mode toggle     28 Quote

11 Line                   29 Copy block

12 Delete                 30 Move block

13 Word                   31 DOS command

14 Block                  32 Print

15 Load                   33 Exit

16 Save                   34 Delete previous character

17 Macro                  35 Swap buffer with external buffer # 1

18 File                   36 Swap buffer with external buffer # 2

The TRSDOS 6.x  version of SAID uses the DOS  keyboard driver and makes use of

the type-ahead supported by the  DOS. The Model I/III version of SAID contains

a  built in  keyboard driver which supports type-ahead as well  as a  complete

ASCII  keyboard.  The  installation  program  can be  used  to  override  this

built-in keyboard  driver.  For  LDOS users who are using the DOS KI/DVR,  you

must either override the SAID driver or not use the LDOS KI/DVR (meaning  that

type-ahead must  be off). The Model I/III  keyboard  driver  uses various  key

combinations  to produce  the  extra characters  not  available  on the TRS-80

keyboard. These are as follows:

<CLEAR> plus                      <CLEAR><SHIFT> plus

<,>       [ (left bracket)        <,>       { (left brace)

</>       \ (reverse slash)       </>       | (vertical bar)

<.>       ] (right bracket)       <.>       } (right brace)

<;>       ^ (carat)               <;>         (tilde)

<ENTER>   _ (underline)           <ENTER>     (delete)

<SHIFT><DOWN ARROW>    (control - use with A-Z)

SAID - Full Screen Text Editor

5 - 8

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Invoking XREF

The  XREF utility is  used  to generate  a  cross  reference  listing  of

symbols used in your source code of a single assembly stream. Its syntax is:

_____________________________________________________________

|                                                             |

|  XREF filespec/REF {(LEN=val,PAGE=val,LINES=val,EQU,LIMIT)} |

|                                                             |

|  filespec    is the specification of the reference data     |

|              file generated by the -XR switch of MRAS. If   |

|              the file extension is omitted, "REF" is used.  |

|                                                             |

|  LEN         is the length of your print line (the default  |

|              value is 80).                                  |

|                                                             |

|  PAGE        is the maximum number of lines per page (the   |

|              default is 66 for Mod I, 67 for Mod III).      |

|                                                             |

|  LINES       is the number of lines to print on a page (the |

|              default is 56 for Mod I, 57 for Mod III).      |

|                                                             |

|  EQU         is used to generate a file of EQUates instead  |

|              of the cross reference listing.                |

|                                                             |

|  LIMIT       is used to limit the file of EQUates to those  |

|              symbols containing a special character.        |

|                                                             |

| Note: the format of "value" is PARM=ddd or PARM=X'hhhh'.    |

|                                                             |

| PAGE is not supported under TRSDOS 6.x                      |

|                                                             |

| There are no parameter abbreviations.                       |

|_____________________________________________________________|

The XREF/CMD  utility generates a symbolic  cross-reference listing which

includes  a sorted  list of all  defined labels, the  file  of  origin  of the

definition,  the  line number of the  definition, the value of the definition,

and the  line numbers of all statements referencing  the label. XREF will also

identify  the filename of the file  containing the references.  XREF will  not

identify unresolved  labels. Therefore, make sure that  either  all labels are

resolved during the assembly that generates the  XREF data file, or you do not

need the line numbers  of those  unresolved references appearing in the  cross

reference listing.

XREF can also be used to generate  an assembler source file of EQUates of

all symbols used  in the  program being  assembled or a subset of all  symbols

used. The  LIMIT  parameter is used to limit the EQUates to only those symbols

having at least one special character in the symbol name.

XREF  uses,  as  input,  the reference  data  file  which  is  optionally

generated  by the -XR switch during the LISTING  pass of  MRAS (phase 2). XREF

cannot  function  without  this  data  file.  You  need  not  enter  the  file

Cross Reference Utility

6 - 1

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

extension, /REF, as it will be assumed if omitted.

The  XREF command  line parameters  enclosed in parentheses are  entirely

optional. The may be used as follows:

# LEN

This parameter controls the printed line length  during the XREF listing.

If omitted, a value of 80 is  assumed to deal with 80-column line printers. If

you are using a wide-carriage printer (typically  132  columns), then XREF can

use the entire print line by specifying the parameter as:

XREF (LEN=132)

# PAGE

This parameter controls the page  size. A value of 66 lines per  page (67

on the Model III  due  to its line counter starting  from  1 instead of 0)  is

used.  If your paper is shorter or longer, you  can  respecify the page length

from the command line. For instance:

XREF filespec (PAGE=51,LINES=41)

will set the page  length to 51  lines  per  page  and initialize to  print 41

lines.

# LINES

This parameter controls the quantity of lines printed  on a page before a

form feed is generated. LINES defaults to a value of 56.

# EQU

This parameter controls the generation of the  EQUate file. If specified,

then  the cross  reference listing is suppressed and  a source file of symbols

equated  to their value  is generated. The filespec  used to  write the EQUate

file will be constructed using  the  filename  and drive specification of  the

"/REF" file. A file extension  of "/EQU" will be used. Symbols defined  by the

"DEFL" pseudo-OP will be maintained as DEFL's in the EQUate file.

# LIMIT

This parameter  controls  what symbols are written to the EQUate file. If

entered in  addition  to  the  "EQU" parameter, then  the  EQUate file will be

limited  to  those  symbols that  contain at  least  one  special character (a

character other than A-Z, 0-9).

Cross Reference Utility

6 - 2

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Cross-Reference Listing

Three informative messages will be  displayed  prior  to  generating  the

listing. "Building symbols  declared" will  be  displayed  as  XREF creates  a

table of  all symbols declared.  The  message,  "Sorting symbol table" will be

displayed as the  symbols are sorted. A second pass  through the REF data file

will be made  while  the message, "Building symbols referenced"  is displayed.

This pass is used to create a second table of all references to symbols.

The listing will contain  a heading  on each page composed  of the system

DATE  and TIME, the TITLE pseudo-op text, and a page number. The heading needs

a minimum of 74 columns. Thus,  you should not specify a LEN parameter of less

than 74. The reference columns will include:

# Origin

The filename where the  symbol was declared. The  ORIGIN will list either

the source filename or the filename of the "*GET"/"*SEARCH" directive.

# Symbolic Label

This column contains the defined symbol name. If  the  symbol was defined

by a "DEFL" pseudo-OP, a plus sign, "+", will precede the symbol name.

# Value

This  column  contains the  value  of the symbol as determined during the

assembly  process.  If the symbol shows a DEFL definition, the  value  will be

the first defined value.

# Line#

This column lists the line number of the statement defining the symbol.

# Usage

This  column contains the filename of the file containing a reference  to

the label. It will be the filename of the *GET/*SEARCH filespec.

# Line# of References

This field will contain the line number  of all  source  statements which

reference the symbol. All of the  references listed  on  a print  line will be

contained in the  file identified under  the usage column. Whenever  the Usage

file changes, it will cause a new line to be generated in the listing.

# Statistics

The  quantity of symbols defined is  listed along  with  the  quantity of

references associated with those definitions.

Cross Reference Utility

6 - 3

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

# Tips for programming relocatable modules

Make a module's /REL filename the same as its  program NAME and its  main

ENTRY point. This will help you build libraries in an organized way.

Avoid writing  modules  with  multiple  entry points.  Modules written in

this way can often lead to confusion and inefficient programming.

Library sizes are absolutely limited by  the size  of MLIB's  memory buf-

fer.  In practice, however, a  smaller library size of about 10K bytes is most

useful for  three reasons: (a) MLINK library searches will take less time; (b)

the library will fit into MLIB's  buffer even if you have a  lot of things  in

high memory; (c) you will have some room for library growth.

Build special purpose libraries  (e.g. communications,  graphics,  string

processing, disk I/O).

Use MRAS's CSEG  (code segment), DSEG (data segment), and COMMON  pseudo-

OPs only! DO NOT  use  ASEG  (absolute segment), as this produces  object code

which is not relocatable and thus not usable in a general purpose library.

Libraries of related routines must be constructed  in a particular order.

This order  is  determined  by  each module's  external  references.  If,  for

instance, module A  references module B, then module B MUST FOLLOW module A in

the library.  Otherwise,  a backward reference will occur, and MLINK may force

you to search  the  library TWICE in order  to  satisfy the  reference. If the

library is constructed as an IRL, MLINK  will perform  multiple searches auto-

matically; however,  minimum processing time  will  occur  in searching an IRL

when no  external  reference is  to be  resolved by  a module which is  stored

before the module having the extern.

You may encounter a problem if  you  try  to use MLIB  on  libraries  not

created  with MLIB. This is because the end-of-file  marker in these files may

not match the EOF in the  directory. There is a simple solution: create a text

file using SAID which  contains a single character of value  9EH;  append this

file to the problem file; load  the  /REL file into MLIB,  and immediately re-

save  it. This caveat  also applies to any  /REL files  constructed under CP/M

which have been transferred over to your system.

# Microsoft compatible 'REL' format

All Z80 assemblers work  in a similar  fashion,  in  that  they convert a

file containing SOURCE CODE,  written in Z80  assembly language mnemonics,  to

OBJECT  CODE in some binary format. In  ABSOLUTE  assemblers, this binary data

is  a faithful  representation of the actual machine language (ones and zeros)

that the Z80 will execute when  you want your program to run. This object code

can only load  and execute at a  FIXED address  in the Z80's memory space.  On

the  other hand, a RELOCATABLE assembler,  such as MRAS, will  generate object

code  which can  be  relocated to  any address  in the  Z80's 64K memory space

before  the program  is  to  be  executed. MRAS and MLINK  support a Microsoft

compatible relocation format.

Technical Information

7 - 1

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

Let's look at an example of absolute assembly. The  following program has

been assembled at an ORIGIN of  0100H.  Notice  especially the values assigned

to the memory addresses @DATE, @EXIT, @DSPLY, START, and BUFFER:

0100          00100         ORG     0100H

4470          00110 @DATE   EQU     4470H

402D          00120 @EXIT   EQU     402DH

4467          00130 @DSPLY  EQU     4467H

000D          00140 CR      EQU     0DH

0100 211401   00150 START:  LD      HL,BUFFER

0103 CD7044   00160         CALL    @DATE

0106 3E0D     00170         LD      A,CR

0108 321C01   00180         LD      (BUFFER+8),A

010B 211401   00190         LD      HL,BUFFER

010E CD6744   00200         CALL    @DSPLY

0111 C32D40   00210         JP      @EXIT

0114          00220 BUFFER: DS      9

0100          00230         END     START

@DATE           4470 @DSPLY          4467 @EXIT           402D

BUFFER          0114 CR              000D START           0100

The  program has been reassembled below at a new  origin, 0200H.  Some of

the addresses for the above labels have changed, while some remain the same:

0200          00100         ORG     0200H

4470          00110 @DATE   EQU     4470H

402D          00120 @EXIT   EQU     402DH

4467          00130 @DSPLY  EQU     4467H

000D          00140 CR      EQU     0DH

0200 211402   00150 START:  LD      HL,BUFFER

0203 CD7044   00160         CALL    @DATE

0206 3E0D     00170         LD      A,CR

0208 321C02   00180         LD      (BUFFER+8),A

020B 211402   00190         LD      HL,BUFFER

020E CD6744   00200         CALL    @DSPLY

0211 C32D40   00210         JP      @EXIT

0214          00220 BUFFER: DS      9

0200          00230         END     START

@DATE           4470  @DSPLY          4467  @EXIT           402D

BUFFER          0214  CR              000D  START           0200

To be  specific,  START and  BUFFER  have  changed, while  the  others are un-

changed.  Both  START and BUFFER have  been relocated! START, instead of being

at  0100H is  now  at 0200H, and  BUFFER has moved from 0114H to  0214H.  This

offset  of 0100H is due  to  the changed origin, 0100H versus 0200H. START and

BUFFER are therefore internally relocatable values,  while @DATE, for example,

will always be 4470H, and is thus known as an absolute value.

The same program, as assembled using relocation looks like this:

4470                          @DATE   EQU     4470H

402D                          @EXIT   EQU     402DH

## Technical Information

7 - 2

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

4467                          @DSPLY  EQU     4467H

000D                          CR      EQU     0DH

0000'   21 0014'              START:  LD      HL,BUFFER

0003'   CD 4470                       CALL    @DATE

0006'   3E 0D                         LD      A,CR

0008'   32 001C'                      LD      (BUFFER+8),A

000B'   21 0014'                      LD      HL,BUFFER

000E'   CD 4467                       CALL    @DSPLY

0011'   C3 402D                       JP      @EXIT

0014'                         BUFFER: DS      9

END     START

@DATE           4470  @DSPLY          4467  @EXIT           402D

BUFFER          0014' CR              000D  START           0000'

All of the internal  program addresses have been  assembled as if  the program

had an origin of 0000H, and  are noted with a following single-quote ('). This

is relocation at work.  The  binary output of this assembly (a /REL file) can-

not  be  executed by the Z80 until you choose an  origin for the program; this

is  done  by the  MISOSYS linker, MLINK), and  can be ANY address  in the  Z80

memory space. MLINK will determine, from the  origin  you have selected, where

START and BUFFER really will be when the program  is run.  If you choose 0100H

as the  origin,  then START will  be located at 0100H,  and  BUFFER at  0114H.

Other origins will produce similar results;  START and  BUFFER will be at dif-

ferent addresses,  but the  offset between  them  (0014H)  will  always be the

same.

This characteristic  of relocatable object files, that they can be LINKED

at  any origin, is extended  by a further capability: relocatable object files

may be linked TOGETHER  to  form a  complete program from many smaller pieces.

This allows you to  write  a  very large  program  in  lesser chunks which are

easier  to edit  and to understand. In addition, you can  develop libraries of

standard  and useful subroutines,  each thoroughly tested and debugged,  which

any main program may call  upon when necessary. The Microsoft FORTRAN  library

(FORLIB/REL), for  example,  thus contains many subroutines which  can be used

by any FORTRAN or Z80 assembler program.

The  mechanism of program and subroutine  linkage  that is  often used is

implemented by the ENTRY  and EXTERNAL attributes.  A  label which is declared

ENTRY (or GLOBAL or PUBLIC)  in  one module  can be accessed by another module

in the following way:

;Module 1

ENTRY   LABEL1        ;this is an entry point

LABEL1:

<code follows>

END                   ;end of module 1

;Module 2

EXTRN   LABEL1        ;this is an EXTERNAL declaration

;could also be EXT.

CALL    LABEL1        ;and this is a reference to the

;external

END                   ;end of module 2

## Technical Information

7 - 3

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

The  relocatable  format  also  allows you  to  do other things.  In many

applications, program code and  data areas must  be separated. This most often

occurs when code must be  placed  in ROM, such  as the BASIC interpreter in  a

TRS-80. However, the data areas cannot be in ROM;  they must  be  in writeable

memory (RAM), and  thus  must be separated  from the  code areas. This  can be

accomplished by use of the CSEG  and DSEG commands  to the MRAS  assembler.  A

CSEG pseudo-operation  signals the start  of a  code area,  while a DSEG indi-

cates the start of a data area. Code and  data SEGMENTS may be intermixed in a

program source  file, and the assembler will automatically keep  them separate

by  the use  of two distinct program or location  counters,  one for each seg-

ment. When you link the program  with MLINK,  you may tell the linker  at what

address  to place the code,  and  also where to place the  data.  Thus the two

segments  are separated.  The above  example is shown below  using  this tech-

nique:

4470                          @DATE   EQU     4470H

402D                          @EXIT   EQU     402DH

4467                          @DSPLY  EQU     4467H

000D                          CR      EQU     0DH

0000'                                 CSEG    ;code starts here

0000'   21 0000"              START:  LD      HL,BUFFER

0003'   CD 4470                       CALL    @DATE

0006'   3E 0D                         LD      A,CR

0008'   32 0008"                      LD      (BUFFER+8),A

000B'   21 0000"                      LD      HL,BUFFER

000E'   CD 4467                       CALL    @DSPLY

0011'   C3 402D                       JP      @EXIT

DSEG    ;data starts here

0000"                         BUFFER: DS      9

END     START

@DATE           4470  @DSPLY          4467  @EXIT           402D

BUFFER          0000" CR              000D  START           0000'

Notice how the label BUFFER is now located at 0000H,  but in the data segment,

as indicated by the double-quote  (") following the address. Am MLINK  session

could then be as follows with user entries in BOLDFACE:

DOS Ready

MLINK

MLINK - Ver 1.0a Copyright 1985 MISOSYS, Inc., All rights reserved

?-p=100

?-d=1000

?test

27937 Free space

P <0100-0113 0014> D <1000-1008 0009>

*test-n-e

DOS Ready

The  -p command to the  linker established  the program (or code) segment

origin, while the -d command did the same for the data  segment. After loading

TEST/REL with the  next command, the linker then  tells  us where the two seg-

ments  are located and how  long  they are.  The final  command writes  out an

Technical Information

7 - 4

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

executable command file (/CMD).  If  we were to disassemble TEST/CMD, we would

find that  START is located at 0100H and  BUFFER at 1000H. Thus the program is

separated into ROM and RAM sections.

MLINK has  other  capabilities,  such as the use of COMMON blocks,  which

are explained  in sections on  MRAS and MLINK. MRAS can also generate absolute

code, if you use the ASEG command.

Finally, we  get  to  the actual format of a Microsoft relocatable object

file. A /REL file is  composed of a bit (not byte) stream.  Each /REL file may

contain a table of ENTRY points and EXTERNAL references.  Each  ENTRY point is

identified by its  name (1 to  7  ASCII characters) and  its relative location

within  one  of  the module's code, data,  or  common segments. Each  EXTERNAL

reference is identified  by its name, and also by  a chain (or linked list) of

pointers, each of which locates  the relative  address within the module where

the external was  used. The  last  pointer in the chain is zero. The /REL file

also  contains  internal  relocation data  necessary for  resolution of  label

references  within the  module. All external  and internal relocatable  refer-

ences are  changed to absolute values at link time, when the program's segment

origins  have been  established.  The remainder of the information in the /REL

file consists of  absolute code and data bytes  which do  not need relocation,

and numerous other fields which describe  common blocks, the module  name, the

module segment lengths, and the /REL  file end (or  EOF  byte). A library file

would contain many  such  modules, each separated by  program  end indicators,

and terminated by an EOF byte.

Let's take one last look at  our example, modified slightly, to see  what

the relocatable object file assembled from this source code would look like:

NAME    ('TEST')

4470                          @DATE   EQU     4470H

402D                          @EXIT   EQU     402DH

4467                          @DSPLY  EQU     4467H

000D                          CR      EQU     0DH

CSEG    ;code starts here

ENTRY   START

EXT     MESSAGE

0000'   21 0000"              START:  LD      HL,BUFFER

0003'   CD 4470                       CALL    @DATE

0006'   3E 0D                         LD      A,CR

0008'   32 0008"                      LD      (BUFFER+8),A

000B'   21 0000"                      LD      HL,BUFFER

000E'   11 0000*                      LD      DE,MESSAGE

0011'   01 0009                       LD      BC,BUFFLEN

0014'   ED B0                         LDIR

0016'   21 0000*                      LD      HL,MESSAGE

0019'   CD 4467                       CALL    @DSPLY

001C'   C3 402D                       JP      @EXIT

DSEG    ;data starts here

ENTRY   BUFFER

0000"                         BUFFER: DS      9

0009                          BUFFLEN EQU     $-BUFFER

END     START

Technical Information

7 - 5

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

@DATE           4470  @DSPLY          4467  @EXIT           402D

BUFFER          0000" BUFFLEN         0009  CR              000D

MESSAGE         0017* START           0000'

Notice how the  external label, MESSAGE, is  defined in the symbol table;

the value  0017H represents  the relative  location  of  the LAST reference to

MESSAGE  in  the assembled  code, and the  trailing  asterisk  (*) denotes  an

external symbol both in this table and in the assembled code listing.

The  following is  a tabular picture of the  decoded  /REL file.  Each  column

represents:

(1) Absolute [0] or relocatable [1] item [1 bit]. If absolute, column (2)

shows the value in hex [8 bits].

(2) Relocation type [0 = special link item; 1, 2, or 3 = segment relative]

[2 bits]. See column (8).

(3) Special link item control field in decimal [4 bits]. See column (8).

(4) "A-field" address type, same as column (2) [2 bits].

(5) "A-field" value, displayed as high/low, but reversed in file [16 bits].

(6) "B-field" length [3 bits].

(7) "B-field" symbol in ASCII [8 bits each character].

(8) Description of the object file record as decoded.

(1) (2)  (3)  (4)    (5) (6)   (7)                   (8)

--- ---  ---  ---  ------ -- -------  -----------------------------------

1    0    2              4  TEST     program name

1    0    0              5  START    entry symbol for library search

1    0    0              6  BUFFER   entry symbol for library search

1    0   10    0   00 09             define data area size

1    0   13    1   00 1F             define program size

1    0   11    1   00 00             set loading location counter (code)

0   21                               absolute (1st byte in code segment)

1    2             00 00             data relative (ref. to BUFFER)

0   CD                               absolute

0   70                               absolute

0   44                               absolute

0   3E                               absolute

0   0D                               absolute

0   32                               absolute

1    2             00 08             data relative (ref. to BUFFER+8)

0   21                               absolute

1    2             00 00             data relative (ref. to BUFFER)

0   11                               absolute (ref. to MESSAGE follows)

0   00                               absolute (this plus next byte are

0   00                               absolute  end of external chain)

0   01                               absolute

0   09                               absolute

0   00                               absolute

0   ED                               absolute

0   B0                               absolute

0   21                               absolute (ref. to MESSAGE follows)

1    1             00 0F             program relative (link in chain)

0   CD                               absolute

0   67                               absolute

## Technical Information

7 - 6

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

0   44                               absolute

0   C3                               absolute

0   2D                               absolute

0   40                               absolute

1    0   11   2    00 00             set loading location counter (data)

1    0   11   2    00 09             set loading location counter (data)

1    0    7   2    00 00 6  BUFFER   define entry point (data)

1    0    6   1    00 17 7  MESSAGE  chain external (head of list)

1    0    7   1    00 00 5  START    define entry point (code)

1    0   14   1    00 00             end program (force to next byte)

1    0   15                          end file marker

# Relocation Format of /REL files

The REL file is an  encoded bit-stream containing relocatable object code

information. It  follows  the  format  documented  by Microsoft  for the  M-80

assembler and  L-80 linker; however,  only 16-bit externals are  supported and

the linker  currently  does  not  support chain address.  The  following  text

documents the bit stream supported by MLINK.

1) IF the  next  bit  is  a zero, THEN the following  eight  bits  are  loaded

according to the  value  of  the location  counter  currently in effect,  THEN

recycle to 1).

ELSE IF the next bit is a one, THEN the  next  two bits represent a code which

is interpreted as follows:

01 - indicates a code relative value follows. The next 16 bits are loaded

after being offset by the code segment origin, THEN recycle to 1).

10 - indicates a data relative value follows. The next 16 bits are loaded

after being offset by the data segment origin, THEN recycle to 1).

11 - indicates a common relative value follows. The next 16 bits are

loaded after being offset by the selected common segment origin,

THEN recycle to 1).

00 - Indicates a Special Link item. The SL item consists of the following

four bits which are interpreted as one of 16 different items

described below; an optional VALUE field which consists of a 2-bit

address type [00 = absolute, 01 = code relative, 10 = data relative,

11 = common relative] and a 16-bit address; and an optional NAME

field that consists of a 3-bit name length followed by the name in

8-bit bytes. SLs 0000-0100 use only a NAME field; SLs 0101-1000 use

both a VALUE field and a NAME field; SLs 1001-1110 use only a VALUE

field; SL 1111 has neither a NAME nor a VALUE field. Unless otherwise

specified, at the conclusion of processing a special link item,

processing recycles to 1). The Special Link items are as follows:

0000 - indicates an entry symbol. This is used by the linker only

when searching a library to see if the module is needed to

satisfy an undefined extern.

Technical Information

7 - 7

The MISOSYS Relocatable Macro Assembler Development System

Copyright 1985 MISOSYS, Inc., All rights reserved

0001 - Select Common Block. Used to specify the NAMEd Common Block

for subsequent common relative references.

0010 - Module name. This is the name of the module. The first one

encountered is saved by MLINK for use in generating the

optional HEADER record of the /CMD file.

0011 - Request Library Search. The library designated by the NAME

field will be searched to resolve undefined externals prior

to any object code generation. An REL will be first assumed.

If one is not found, an IRL will then be assumed.

0100 - This item is not supported by MLINK.

0101 - Define Common Size. This is used by MLINK to establish the

size of the common block designated by the NAME field.

0110 - Chain External. The VALUE field contains a pointer to the head

of a chain which ends with an absolute zero. Each 16-bit

element of the chain will be replaced with the value of the

external symbol described in the NAME field.

0111 - Define Entry Point. The VALUE field specifies the value of the

symbol described by the NAME field.

1000 - This item is not supported by MLINK.

1001 - External plus Offset. This specifies that the VALUE field must

be added to the following two bytes in the current segment

after all chain externals have been processed.

1010 - Define Data Size. The VALUE field is used by MLINK to

establish the size of the data segment of the current module.

1011 - Set Location Counter. The location counter is set to the value

identified by the VALUE field.

1100 - This item is not supported by MLINK.

1101 - Define Code Size. The VALUE field is used by MLINK to

establish the size of the code segment of the current module.

1110 - End of Module. The VALUE field defines the transfer address

for the module if other than absolute zero. This item

denotes the end of the module. The bit stream is also

advanced to a byte boundary. Recycle to 1) if loading a

module from other than a library search.

1111 - End of File. This is used to indicate the end of the file.

It is used when searching libraries or when loading modules

to detect the end of the file.

## Technical Information

7 – 8