Vswitch v0.1

A program to switch between Tandy video and VGA

Jeffrey L. Hayes

May 19, 1997

Contents

--------

Summary

Usage

Technical Stuff

Contacting the Author

Summary

-------

Vswitch is a program to switch between VGA and the onboard video in a Tandy

1000TL with 768k on the motherboard and a VGA card.  The idea is to allow

VGA to be temporarily disabled and the default video used to run programs

requiring Tandy/PCjr or Hercules video.  When I upgraded to VGA, I needed

something to do this so that I could continue to test Tandy programs for my

ftp site.  To use Vswitch, you need a VGA card, a VGA monitor, a CGA or MDA

monitor, and of course your Tandy ;-).

I cannot certify that it will work on anybody's computer but mine, though

it would probably work on the TL/2, TL/3, RL, or RL-HD, as long as there is

768k on the motherboard.  Vswitch certainly will not work with EGA.  It

does not attempt to "protect" video memory, so it won't work without 768k

unless you put something in AUTOEXEC.BAT to do that yourself (i.e.,

something similar to the Adjmem program, though I'm not sure if Adjmem

itself will work).

If Vswitch doesn't work on your system and you would like to try to get it

to, the source code is included.  It was written with the A86 shareware

assembler.

Usage

-----

To use Vswitch, install your VGA card and attach your VGA monitor to it,

then attach your old monitor to the built-in controller port.

There are two parts to the program:  VSRES.COM, a TSR to be loaded in

AUTOEXEC.BAT; and VSWITCH.COM, a command-line program for toggling between

VGA and the onboard video.

VSRES takes one optional parameter, "M" or "MONO".  That indicates that a

monochrome TTL monitor is attached to the onboard video controller.  In

that case, the built-in controller in the 1000TL's, SL's, and RL's displays

either MDA text or Hercules graphics.  If "M" is not specified, VSRES

assumes that a CGA monitor is attached to the built-in controller, in which

case the built-in controller displays CGA or Tandy/PCjr graphics.  Refer to

your owner's manual.  VSRES will tell you if it got installed or not.  For

example:

H:\>VSRES

Resident part of video switch utility installed -

onboard video monitor is CGA.

Or:

H:\>VSRES MONO

Resident part of video switch utility installed -

onboard video monitor is monochrome TTL.

Or:

H:\>VSRES

VGA not detected, resident part of video switch utility not installed.

You would also get an error message if you attempted to run VSRES on a

system that is not a Tandy 1000-series.

VSWITCH does not take any parameters.  It is a toggle:  you run VSWITCH

once to switch from VGA to Tandy video, then run it again to switch back.

VSWITCH normally will not say anything - you will just see the DOS prompt

move from one monitor to the other.  If VSRES is not loaded, VSWITCH will

display an error message:

H:\>VSWITCH

Video switch failed, VSRES not installed.

The Tandy 1000-series require that a small program be run in AUTOEXEC.BAT

to ensure that applications can detect VGA.  I have written one - Vgafix -

and there are others.  They all do the same thing.  You have to run that

program *before* loading VSRES, or it will refuse to install.

Be careful of other TSR's that deal with video.  In general, you will have

to install them *after* installing VSRES, uninstall them before running

VSWITCH, then reinstall.  That means that if you want mouse support you

will have to use MOUSE.COM, not MOUSE.SYS, and you will have to load the

mouse driver after VSRES.  For example, my AUTOEXEC.BAT file might look

like this:

C:\DOS\VGAFIXC

C:\DOS\VSRES

C:\DOS\MOUSE

Then to switch video from VGA to Tandy or vice versa, I do this:

MOUSE OFF

VSWITCH

MOUSE

That assumes that MOUSE.COM is the last thing loaded in AUTOEXEC.BAT.

Technical Stuff

---------------

When a system with VGA installed boots, the default Int 10h vector is

replaced so as to point to the BIOS on the VGA card.  To switch back to

Tandy video, we need to restore the original vector.  The default Int 10h

vector, documented for the Phoenix BIOS, is F000:F065h.  (See _System BIOS

for IBM PCs, Compatibles, and EISA Computers_, 2nd edition, Phoenix

Technologies Ltd., 1991, p. 55.)  This is confirmed for CGA and MDA by

tracing the interrupt.

The BIOS video data area must also be swapped when the video system is

changed.  This is the region between 0040:0049h and 0040:0066h, and the

region between 0040:0084h and 0040:008Ah.  (This according to the

_Programmer's Guide to PC Video Systems_, 2nd edition, by Richard Wilton,

Microsoft Press, 1994, p. 9.)  The addresses are inclusive (ibid., p. 385).

VGA uses an additional region of RAM addressed by a dword pointer at

0040:00A8h (ibid.), but CGA does not use that region (ibid., p. 388).

Hence we only need to swap the regions given above.  The addresses are

confirmed in _The New Peter Norton Programmer's Guide to the IBM PC &

PS/2_, by Peter Norton and Richard Wilton, Microsoft Press, 1988, pp. 58-

60.

According to the _Tandy 1000TL Technical Reference Manual_, software

section, pp. 52-9, the region between 0040:0049h and 0040:0066h is used,

but there is no reference to the region between 0040:0084h and 0040:008Ah.

According to the _IBM PCjr Technical Reference Manual_, the byte at

0040:008Ah is used to save a copy of the CRT/CPU page register - confirmed

for the TL by examining the ROM code with debug.  BTW, the default values

for CRT/CPU pages are 7 for both.  There are 2 bytes at 0040:00B4-5h that

are of interest.  The byte at 0040:00B4h is supposed to be 0 for "no

monochrome monitor" and 0FFh for "monochrome monitor" - but that does not

seem to be the case on the 1000TL (see below).  For the byte at 0040:00B5h:

bit 4:  0 = internal color video support enabled

1 = internal color video support disabled, external color video

enabled

bit 5:  0 = no external monochrome video installed

1 = external monochrome video installed

Bit 4 is set if VGA is installed.

The Int 1Dh vector addresses a table of video initialization values for MDA

or CGA (_Programmer's Guide to PC Video Systems_, pp. 395-6).  That vector

needs to be swapped also.  The Int 05h vector addresses the print screen

routine, which may have been modified by the VGA BIOS.  The vertical

nondisplay interrupt vector (Int 0Ah) might or might not be modified, but

it is probably a good idea *not* to swap it - IRQ 2 might be used for some

other device.  Int 1Fh addresses the character glyph table for the second

128 characters in graphics modes; in VGA, Int 43h addresses the character

glyphs for all 256 characters in graphics modes (_Programmer's Guide to PC

Video Systems_, pp. 274-5).

Tandy video is enabled by bit 2 at port 65h.  That is *not* a read/write

port, in spite of what the _1000TL Technical Reference_ says.  Other bits

enable the hard drive, the parallel port, the floppy drive, and the serial

port, and they should always be high when writing to the port.

VGA is disabled by issuing Int 10h, AH=12h, BL=32h, AL=01h.  It is enabled

with Int 10h, AH=12h, BL=32h, AL=00h (_The New Peter Norton Programmer's

Guide to the IBM PC & PS/2_, pp. 189-90).  This is confirmed by the

_Programmer's Guide to PC Video Systems_, pp. 424-5.

Since the default values for the BIOS data areas will have been overwritten

by the time the TSR portion of Vswitch gets control, it is necessary to

include them in the program.

We do not need to reprogram VGA when switching back to it, but we should

reprogram CGA, at least to the extent of setting the video mode (Int 10h,

AH=0).  That ensures that the BIOS data is in sync with the card state.

The initial video mode information in the BIOS equipment list may also need

to be modified.

The following are the values I found for the memory locations mentioned

above.  The Debug script VIDDATA.DBG was used to obtain this information:

With CGA:

655360 bytes total memory

582528 bytes free

-d 0:14 l4

0000:0010              54 FF 00 F0                               T...

-d 0:28 l4

0000:0020                          1E E6 00 F0                       ....

-d 0:74 l4

0000:0070              00 E0 00 F0                               ....

-d 0:7c l4

0000:0070                                      AD C0 00 F0               ....

-d 0:10c l4

0000:0100                                      00 00 00 00               ....

-d 40:10 11

0040:0010  2D 54                                             -T

-d 40:49 66

0040:0040                             03 50 00 00 10 00 00            .P.....

0040:0050  00 18 00 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

0040:0060  07 06 00 D4 03 29 20                              .....)

-d 40:84 8a

0040:0080              00 00 00 00-00 00 3F                      ......?

-d 40:b4 b5

0040:00B0              00 0F                                     ..

-q

With MDA:

655360 bytes total memory

582528 bytes free

-d 0:14 l4

0000:0010              54 FF 00 F0                               T...

-d 0:28 l4

0000:0020                          1E E6 00 F0                       ....

-d 0:74 l4

0000:0070              00 E0 00 F0                               ....

-d 0:7c l4

0000:0070                                      AD C0 00 F0               ....

-d 0:10c l4

0000:0100                                      00 00 00 00               ....

-d 40:10 11

0040:0010  3D 54                                             =T

-d 40:49 66

0040:0040                             07 50 00 00 40 00 00            .P..@..

0040:0050  00 18 00 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

0040:0060  0C 0B 00 B4 03 29 20                              .....)

-d 40:84 8a

0040:0080              00 00 00 00-00 00 3F                      ......?

-d 40:b4 b5

0040:00B0              00 0F                                     ..

-q

With VGA, without VGAFIXC:

655360 bytes total memory

581104 bytes free

-d 0:14 l4

0000:0010              54 FF 00 F0                               T...

-d 0:28 l4

0000:0020                          1E E6 00 F0                       ....

-d 0:74 l4

0000:0070              10 DF 00 F0                               ....

-d 0:7c l4

0000:0070                                      36 3C 00 C0               6<..

-d 0:10c l4

0000:0100                                      36 38 00 C0               68..

-d 40:10 11

0040:0010  2D 54                                             -T

-d 40:49 66

0040:0040                             03 50 00 00 10 00 00            .P.....

0040:0050  00 18 00 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

0040:0060  0E 0D 00 D4 03 29 30                              .....)0

-d 40:84 8a

0040:0080              18 10 00 60-09 11 3F                      ...`..?

-d 40:b4 b5

0040:00B0              00 1F                                     ..

-q

With VGA, with VGAFIXC:

655360 bytes total memory

581104 bytes free

-d 0:14 l4

0000:0010              54 FF 00 F0                               T...

-d 0:28 l4

0000:0020                          1E E6 00 F0                       ....

-d 0:74 l4

0000:0070              10 DF 00 F0                               ....

-d 0:7c l4

0000:0070                                      36 3C 00 C0               6<..

-d 0:10c l4

0000:0100                                      36 38 00 C0               68..

-d 40:10 11

0040:0010  2D 54                                             -T

-d 40:49 66

0040:0040                             03 50 00 00 10 00 00            .P.....

0040:0050  00 18 00 00 00 00 00 00-00 00 00 00 00 00 00 00   ................

0040:0060  0E 0D 00 D4 03 29 30                              .....)0

-d 40:84 8a

0040:0080              18 10 00 60-09 11 0B                      ...`...

-d 40:b4 b5

0040:00B0              00 1F                                     ..

-q

From the above, it appears that the only interrupt vectors changed by the

VGA BIOS in my particular system are Int 10h, Int 1Fh, and Int 43h.

Nevertheless, the vectors for Int 05h and Int 1Dh should be swapped as

well.  The vector for Int 0Ah should *not* be swapped.

Contacting the Author

---------------------

This little program is free, no warranties, etc.  If you want to contact

me, the best way is via Internet email.  My address is:

tvdog@agate.net

If you prefer, you can also use paper mail:

Jeffrey L. Hayes

130 Forest Ave., Lot 1

Orono, Me. 04473

USA

I generally monitor the Usenet newsgroup comp.sys.tandy.

The latest version of Vswitch, should there be another, would be at my

ftp/WWW site:

http://www.agate.net/~tvdog/

ftp://ftp.agate.net/users/01935/

There are a lot of programs and other stuff for the Tandy 1000-series at

the site, including some by me.  The Tandy 1000-series FAQ is also to be

found there.