���������������������������͸

�       C M O S B A K       �

�                           �

�    CMOS backup utility    �

�                           �

�        Version 1.0        �

���������������������������;

by Garry Hansen

Another Freeware Utility

from Cheapieware

February 1995

REQUIREMENTS

DOS 3.3 or later

Colour monitor

286+

WHAT DOES CMOSbak DO?

CMOSbak saves the CMOS and, optionally, the extended CMOS data to a

small binary file.  Using CMOSbak, this data can be restored later if

CMOS memory is lost or corrupted.

WHAT IS CMOS?

CMOS stands for Complementary Metal Oxide Semiconductor.  Is is a

small, low-powered portion of RAM (Random Access Memory) which

keeps its contents when your computer is turned off.  In many computers

this memory is powered by a small, barrel-shaped battery.  Newer

computers often use an "accumulator" (in essence, a battery which is

recharged whenever the computer is turned on) or have a ten-year battery

built into the chip.

CMOS RAM contains essential configuration data which is used by your

computer at start up. This information includes hard drive parameter

tables for IDE drives, video configuration, the number and kind of

floppy drives on your system, and the date and time.

WHAT IS EXTENDED CMOS?

Earlier AT-class computers contained 64 bytes of CMOS.  Later 286 and

most 386 and later PCs contain at least 128 bytes.  The 64 or more bytes

beyond the first 64 bytes of the CMOS are often referred to as "Extended

CMOS" or XCMOS. They generally contain advanced BIOS data, such as

memory wait states, clock speeds, and the number of type of SIMMs or

SIPs in your system.

If you have a 386 or a more powerful computer, you probably have

Extended CMOS.   Late-model 286s may also have XCMOS.  Ancient PCs and

XTs did not contain a CMOS at all, and the configuration data was

established by setting DIP (Dual-In-line Package) switches on the

motherboard or peripheral cards.  DO NOT use CMOSbak if you have a PC or

XT computer.

WHY SHOULD I BACKUP MY CMOS?

Your computer needs the configuration data in the CMOS to perform

correctly. Although most computers provide an option for bypassing the

Extended CMOS, restoring power-on defaults by holding down a certain key

at bootup (such as the INS key), these default values are "safe" but not

optimal.  Most importantly, the basic CMOS contains hard drive parameter

tables which are necessary to run an IDE drive.  If your CMOS fails

(often as the result of a weak or dead battery), you will lose access to

your hard drive until you restore the parameters (number of cylinders,

heads, sectors, etc.).  At the least, this is annoying; at worst,

disasterous.

HOW DO I USE CMOSbak?

First, copy CMOSBAK.EXE and its documentation to a floppy disk.  Since a

CMOS failure could prevent access to a hard drive, it is best to run

CMOSbak from a floppy disk.

Determine whether your computer contains basic or extended CMOS.

CMOSbak will give you the option to save either 64 bytes or 128 bytes of

CMOS data. Attempting to restore 128 bytes of data to a 64-byte CMOS

could be dangerous.

Since CMOSbak will attempt to create or restore CMOS.SAV from the

current directory, change to the floppy drive containing CMOSbak before

running the program.  Please ensure that your floppy disk is not write

protected.

Follow the online instructions to save your CMOS data.  Store the disk

containing CMOSbak and CMOS.SAV in a safe place.  When you need to

restore your CMOS data, simple run CMOSbak again and select the restore

command.

WILL CMOSbak MESS UP THE DATE AND TIME?

CMOS data includes date and time info.  Although CMOSbak saves this

information, it automatically skips it when restoring from a CMOS.SAV

file to avoid resetting your computer's clock and calendar.

DISTRIBUTION

CMOSbak is released as Freeware.  It may be freely used and distributed,

providing the software and documentation is complete and unaltered.

Vendors may charge a distribution fee of no more than $5.00.

DISCLAIMER

The author makes no warranties, expressed or implied, as to the quality or

performance of this program.  Your use of this program constitutes your

release of the author from liability or litigation for any indirect or

consequential damages arising from the use of this program or its associated

files.

In no event will I be liable to you for any damages, including any lost

profits, lost savings or other incidental or consequential damages arising

out of your use of inability to use the program, or for any claim by any

other party.